NSW Department of Education and Training

Manufacturing and Engineering Division

6077W

Drawing and Diagrams for Electrical Work

# End of Module Test

Time allowed: 2hrs 30 minutes

Total Marks available for this test are 100

| ## Section | ## Marks |
| --- | --- |
| A | 20 |
| B | 10 |
| C | 25 |
| D | 45 |
| Total | 100 |

Student equipment required:

- Non-programmable calculator

- Pens, pencils, erasers

- Rule

Do not mark this question paper in any way.

Section A – 20 marks

For each question, mark the correct answer in the space provide on your answer sheet.

- The electrical diagram in Fig. 1 is a/an:

- circuit diagram

- wiring diagram

- block diagram

- single line diagram

- The electrical diagram in Fig. 2 is a/an:

(A)  circuit diagram

- wiring diagram

- block diagram

- single line diagram

- A drawing is done to a scale of 100:1. The on the job length of cable run, measured at 125.5mm on the drawing, would be:

- 125.5 metres

- 12.55 metres

- 1255 metres

- 1.255 metres

- The term ‘fit out’ refers to

- running the cable on the job

- fitting cable brackets to the house frame

- tying the cable to house frame

- terminating and securing electrical equipment

- The best type of electrical diagram to use when fault finding electrical equipment is a/an:

- circuit diagram

- wiring diagram

- block diagram

- architectural diagram

- The number of intermediate switches required for 3 way switching is/are:

- 0

- 1

- 2

- 3

- A site plan shows:

- the height of one elevation

- the excavations required for the site

- the position of the dwelling on the block

- the position of the electrical supply on the block

- A floor plan shows:

- the room layout of the building

- the footing layout

- the location of piers

- details of the steel reinforcing for the slab

- When developing a wiring diagram for a lighting circuit that contains an unknown type of switch, you should first develop a/an:

- block diagram

- switching chart for the switch

- circuit diagram

- electrical schedule

- The components in a circuit diagram should be represented using:

- drawings of actual components

- numbering system

- standard symbols

- block diagrams

- Brick cavity construction is;

- timber frame with an internal and external timber cladding

- single exterior brick cladding with a space to an interior brick cladding

- single exterior brick cladding with a metal or timber frame and plaster board interior cladding

- single interior brick wall

- Looping at the light method of wiring an installation means:

- the earth and neutral wires are wired to the switch

- the earth and neutral wires are wired to the light

- the earth, active and neutral are wired to the switch

- the earth, active and neutral are wired to the light

- Looping at the switch method of wiring an installation means:

- the earth and neutral wires are wired to the switch

- the earth and neutral wires are wired to the light

- the earth, active and neutral are wired to the switch

(d) the earth, active and neutral are wired to the light

- What determines the wiring method of an installation?

- The type of cable used.

- The Inspector of the local Electrical Authority

- The type of building construction and material used

- Minimise both cable required and installation time.

- When carrying out an insulation resistance test on a lighting installation, which of the following in NOT correct:

- Isolate the power

- Disconnect the neutral at the neutral link

- Ensure 60 Watt lamps are fitted to all batten holders

- Do not test between active and neutral if electronic dimmers are used.

- When carrying out an insulation resistance test on a lighting installation, which of the following value is used:

- 240V AC

- 500V AC

- 500V DC

- 1000V DC

- What cables are fitted in the loop terminal of the batten holder using looping at the light method of wiring?

- Neutral

- Earth

- Switch Wire

- Active

- How are earth connections made at the switch in looping at the switch method of wiring?

- Two screw tunnel connector

- One screw tunnel connector

- Twisted together

- In the loop terminal

- What sequence in a building construction are the light fittings and swiches fitted?

- After the interior lining has been fitted

- After the wall tiling is completed

- After painting

- After the external cladding and before the internal cladding

- What size cable is the separate earth which is run from the switch to the batten holder in looping at the switch method of wiring?

- 0.75mm2

- 1.5mm2

- 1.0mm2

- 2.5mm2

Section B  - 10 marks

Write the answer to each question in the space provided on the answer sheet.

Do not mark the question paper.

- Explain why the ‘point of entry’ is not shown on the floor plan of a domestic              installation.

- List two parts of an electrical installation that may have their location (or route) determined from a site plan.

- Draw the architectural symbol for a two way switch

- What are the two main plans an electrician requires for house construction.

- A relay is to be replaced on a control panel, what type of diagram would an electrician use?

- What instruments/tools does an electrician use when isolating a circuit?

- When are light switches connected in parallel?

- What purpose does the wall cavity serve in a double brick wall?

- What scale is normally used on architectural drawings for floor plans?

- Name 3 items of test equipment for testing switch continuity.

## Section C – 25 marks

- A sample switch will be provided and all the following questions relate to the supplied switch.

- Draw a switching chart for the sample switch supplied. (2 marks)

- From the results entered in the switch chart draw the correct switch symbol and number all terminals (2 marks)

- From the symbol drawn in question 2 name the type of switch ( 1 mark)

- (A) Draw a circuit diagram for two way switching controlling two lights.		 (2 marks)

(B) Convert the diagram from question 2A to a wiring diagram. (3 marks)

- Draw a section of floor plan and show the method used to indicate that a single lighting point is controlled by three switches. (use architectural symbols)

- 5 marks

- Draw the achitectural symbols for the following: (5 marks)

- 15A plug socket

- exhaust fan

- heater

- luminaire fixed to wall

- intermediate switch

- Listed below are five tradespeople an electrician may work with on a building site. Give 1 reason why the electrician may need the cooperation of each of them – 5 marks

- bricklayer

- plumber

- plasterer

- concreter

- shopfitter (cabinetmaker)

Section D – 45 marks

Install and connect a lighting circuit according to the following:

- lighting point 1 is controlled by switch 1

- lighting point 2 is controlled by switch 2

- lighting point 3 is controlled by switch 3

- the circuit is supplied from a 24V AC supply

The circuit is to be wired using 1mm2 TPS cables using either:

- 1mm2 twin and earth

- 1mm2 TPE twin

The circuit cabling is to be installed so that:

- lighting points 1 and 2 and their associated switches are installed using the ‘loop at the light’ method of wiring

- lighting point 3 and its associated switch are installed using the ‘loop at the switch’ method of wiring.

- Each lighting point must be earthed in accordance with AS3000

On your answer sheet first draw a wiring diagram of the circuit – 10 marks

Install & connect the circuit – 25 marks

Test the circuit & apply correct isolation procedures –10 marks