DO NOT use this machine unless you have been trained in its safe use and operation.

| Description of Work: |  |  | Using a power drill. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Potential Hazards: Exposed moving parts and electrical hazard with the potential to cause harm through entanglement, impact and cutting, exposure to heat, dust, projectiles and sharp objects. |  |  |  |  |
| Personal Protective Equipment (PPE) Required (Check the box for required PPE): |  |  |  |  |  |  |  |  |  |  |
| Gloves |  | Face Masks |  | #### Eye #### Protection | Welding Mask |  | Appropriate Footwear |  | Hearing Protection | Protective Clothing |
| Safe Work Procedure Checklist: |  |  |  |  |  |  |  |  |  |  |
| 1. PRE-Operation: - Task (e.g. Drawings, instructions, specifications etc.) is clearly understood. - Ensure drill bit is installed tightly into chuck and chuck key has been removed (if applicable). - Work piece is secured. - Correct drill speed has been set. - Where necessary, depth stop for drill has been set or wood to be drilled is rested on scrap. - Correct drill bit has been selected for material being drilled. - Identify ON/OFF switch and emergency stop button (if applicable). 2. Operation: - Check that drill bit runs ‘true’ and does not wobble. - Keep hands clear of work piece and away from rotating tool. - Back out frequently on deep cuts to clean and cool drill bit. - Turn off drill before speed settings or bit is changed. - Exercise caution when removing hot drill bit. 3. POST-Operation: - Switch off drill before removing waste material from the drill table. - Ensure good housekeeping practices are in place to minimise dust build-up. - Return clean drill bit to drill rack and clamps to their storage area. |  |  |  |  |  |  |  |  |  |  |
| Competent Persons (The following persons are authorised to operate, supervise and test students on the equipment/process). |  |  |  |  |  |  |  |  |  |  |
| Name: | Title: |  |  |  |  |  |  | Contact Details: |  |  |