P32Q1

- Wall plates

A wall is a collection of studs (usually sized 2×4 or 2×6) equally spaced (usually 16 in. or 24 in. on center) and sandwiched between top and bottom plates. The top plate can be either single or double. Double plating is most common on load-bearing walls unless the roof rafters or trusses and floor joists stack directly over the studs in the wall, then a single top plate can be used.

AdChoices

ADVERTISING

- Headers

Large openings in the wall are made for windows and doors. When the opening is greater in width than the stud spacing — and most windows are wider than 24 in. — then a header must be inserted to carry the load of the interrupted stud(s). A header is a simple beam sized to support the load above the opening it spans.

- Jack studs and king studs

The header is supported by a jack stud at each end. Jacks, sometimes called trimmers, fit under each end of a header, and they transfer the load that the header carries down to the bottom plate and the framing beneath. Nailed to the jacks are full-height studs called king studs; they support the assembly between the plates. Sometimes jacks must be doubled on wide openings so there’s enough supporting surface for the header to bear on. Jacks can be replaced with a steel header hanger attached to the king stud.

- Saddles and cripples

A saddle (also called a sill) forms the bottom of a window opening. It’s a piece of 2x stock laid flat and nailed between the jacks. Cripples are short pieces of 2x stock that run underneath the saddle. And, depending on a header’s height, cripples can run from the header to the plate. Cripples are located at the points where a common stud would have been located had it not been interrupted by the opening.

Mike Guertin and Rick Arnold are professional builders in Rhode Island with over 20 years experience building custom homes. In addition to being contributing editors for Fine Homebulding magazine, they have written numerous articles on homebuilding, and they conduct regular seminars for builders.

P32 Q2

P32Q3

| ##### Tips Menu |  |  |
| --- | --- | --- |

| #### PLASTERBOARD 10 mm Recessed Edge 10 mm Ceiling Board 10 mm Wet Area 13 mm Recessed Edge 13 mm Wet Area Fire Rated Plasterboard 6 mm Radius Board #### DURALINIER 6 mm Duraliner 9 mm Duraliner #### Plasterboard Accessories Fixing Products Adhesives Fasteners Nails Screws Nylon Anchors Ceiling Anchors Battens Batten Clips Finishing Products Tapes Bedding Cements Finishing Cements Metal Beads External Angles Internal Angles Stopping Beads Stopping Angles Casing Beads #### Insulation Polyester Glasswool #### Tools Trowel Boxes Power Tools Plasterers Hand Tools Other Tools #### Suspended Ceilings Screw Up Plasterboard Exposed Grid Ceilings Plasterboard Tiles Mineral Fibre Tiles Concealed Grid Ceilings #### Metal Stud Systems 51mm Stud System 64mm Stud System 76mm Stud System 92mm Stud System 150mm Stud System |  |  |  |
| --- | --- | --- | --- |
|  | Product Code | Description | Product Use |
|  | Adhesives |  |  |
|  | magrip | 5.2kg stud adhesive | Stud adhesive is used in conjunction with nails or screws to fix plasterboard to timber or metal stud frames. |
|  | bca20 | 20kg Boral Plasterboard brand cornice adhesive | Cornice adhesive is used mainly to glue cornices and other ornamental plaster products in place. Other uses for cornice adhesive include, back blocking, patching, gluing plasterboard to masonry surfaces and can be used as a base coat for joining plasterboard. As Cornice adhesive will set faster the more it is mixed, use for jointing is best for small jobs. This can save you needing to buy base coat as well as cornice adhesive on small projects. |
|  | gcad20 | 20kg CSR brand cornice adhesive |  |
|  | ma20 | 20kg masonry adhesive | Masonry adhesive is used for gluing plasterboard to masonry surfaces. Note, all surfaces should be free of dust before using masonry adhesive. |
|  | bbc20 | 20kg back blocking cement | Slow setting plaster used for back blocking plasterboard ceiling joints. Note, back blocking is recommended for all ceiling joints. |
|  | Ring Shank Nails |  |  |
|  | ri30p | 30mm ring shank clouts 2.5kg packets | Ring shank nails are used for pine fames. If you have a hardwood frame, (which is very rare in new houses today), more likely in renovations, you should use smooth shank nails as ring shank nails have a tendency to bend going into hardwood. |
|  | ri30b | 30mm rinshank nails 25kg box |  |
|  | ri40p | 40mm ring shank clouts 2.5kg packets |  |
|  | ri40b | 40mm rinshank nails 25kg box |  |
|  | BACK TO TOP |  |  |
|  | Smooth Shank Nails |  |  |
|  | sm30p | 30mm smooth shank clouts 2.5kg packets | Smooth shank nails are used for hardwood frames and should not be used on softwood frames, as there will be an increased risk of "popped" nail heads. |
|  | sm30b | 30mm smooth shank clouts 25kg box |  |
|  | sm40p | 40mm smooth shank clouts 2.5kg packets |  |
|  | sm40b | 30mm smooth shank clouts 25kg box |  |
|  | sm50p | 50mm smooth shank clouts 2.5kg packets |  |
|  | sm65p | 65mm smooth shank clouts 2.5kg packets |  |
|  | Screws "S" Point |  |  |
|  | scr25mm | 25mm "s" point screws loose, 1000 per packet | "S" point screws are standard screws used for walls and ceilings into any timber frame or up to .55 gauge steel frames or battens. |
|  | scr32mm | 32mm "s" point screws loose, 1000 per packet |  |
|  | scr40mm | 40mm "s" point screws loose, 1000 per packet |  |
|  | scr50mm | 50mm "s" point screws loose, 1000 per packet |  |
|  | scr65mm | 65mm "s" point screws loose, 1000 per packet |  |
|  | scr75mm | 75mm "s" point screws loose, 1000 per packet |  |
|  | cscr25mm | 25mm "s" point screws collated, 1000 per packet | Collated screws are used with auto feed screw guns. |
|  | cscr32mm | 32mm "s" point screws collated, 1000 per packet |  |
|  | cscr40mm | 40mm "s" point screws collated, 1000 per packet |  |
|  | cscr50mm | 50mm "s" point screws collated, 1000 per packet |  |
|  | cscr60mm | 60mm "s" point screws collated, 1000 per packet |  |
|  | BACK TO TOP |  |  |
|  | Product Code | Description | Product Use |
|  | Screws Drill Point |  |  |
|  | scr15bhd | 25mm button head drill point screws loose, 1000 per packet | Drill point screws are used for fixing plasterboard to load bearing steel frames that are over .55 gauge. Button or wafer head drill point screws are used for fixing metal framing members in metal stud partitions or suspended ceilings. |
|  | scr20bhd | 20mm button head drill point screws loose, 1000 per packet |  |
|  | scr25d | 25mm drill point screws loose, 1000 per packet |  |
|  | scr30d | 30mm drill point screws loose, 1000 per packet |  |
|  | scr45d | 45mm drill point screws loose, 1000 per packet |  |
|  | scr50d | 50mm drill point screws loose, 1000 per packet |  |
|  | scr65d | 65mm drill point screws loose, 1000 per packet |  |
|  | cscr25d | 25mm drill point screws collated, 1000 per packet |  |
|  | cscr32d | 32mm drill point screws collated, 1000 per packet |  |
|  | cscr45d | 45mm drill point screws collated, 1000 per packet |  |
|  | Other Screw Types |  |  |
|  | scr25ct | 25mm coarse thread screws loose, 1000 per packet |  |
|  | scr25hl | 25mm hi/low thread screws loose, 1000 per packet |  |
|  | scr40hl | 40mm coarse thread screws loose, 1000 per packet |  |
|  | scr38lm | 38mm coarse thread laminating screws loose, 1000 per packet |  |
|  | BACK TO TOP |  |  |
|  | Product Code | Description | Product Use |
|  | Battens |  |  |
|  | 3016 | 6m 16mm metal nail up batten | Metal battens are used to level ceilings where the existing framing is not straight or level. They are also used if the ceiling framing is greater than the required minimum for the type of plasterboard being used, (maximum span 900mm). Battens are spaced at 450mm centers for 10mm re plasterboard and 600mm centers for ceiling board. |
|  | 30148 | 4.8m 16mm metal nail up batten |  |
|  | 1296 | 6m 28mm furring channel | 28mm furring channel is used for the same purpose as metal battens except they have a maximum span of 1200mm. Furring channel is also used in suspended ceilings or to batten out a masonry wall. |
|  | 12948 | 4.8m 28mm furring channel |  |
|  | 30860 | 6m 16mm furring channel | 16mm furring channel will span 1200mm the same as 28mm furring channel. It can be used as per 28mm furring channel but is mostly used to batten out masonry wall as they take up less room space. |
|  | 30848 | 4.8m 16mm furring channel |  |
|  | 333 | 6m 13mm recessed furring channel | 13mm recessed furring channel will span 1200mm the same as 28mm furring channel. It is mainly used to fix directly to masonry walls without the use of a clip. The fixing is through the recessed member that sits flat against the surface to be fixed to. |
|  | ##### BACK TO TOP |  |  |
|  | Batten Clips |  |  |
|  | 304 | 92mm direct fix clip | Used with 16mm nail up battens where the ceiling needs to be lowered or extensive leveling is required. Maximum clip fixing centers for clips is 900mm. |
|  | 305 | 150mm direct fix clip | Used with 16mm nail up battens as above but with the ability to lower the ceiling further. |
|  | 226 | 76mm direct fix clip | Used with furring channel where the ceiling needs to be lowered or extensive leveling is required. Maximum clip fixing centers for clips is 1200mm. |
|  | 394 | 150mm direct fix clip | Used with furring channel where the ceiling needs to be lowered greater than the 226 clip. Maximum clip fixing centers for clips is 1200mm. |
|  | 386 | Furring channel to purlin clip | Used with furring channel to fit over the lip of a steel purlin. They come in 13mm, 16mm, 19mm, 22mm and 25mm. |
|  | bfc | Adjustable better fix clip | Used with furring channel to fix to masonry wall. |
|  | bfcres | Resilient better fix clip | Used with furring channel to fix to masonry or timber wall or ceiling. Where an acoustic rating is required. |
|  | Product Code | Description | Product Use |
|  | Tape |  |  |
|  | ft90 | 90m fiberglass tape | Self adhesive fiberglass tape for joining plasterboard. Fiberglass tape should only be used on recessed edge wall joins. It should never be used on villaboard, internal angles or butt joints. We do not recommend use on ceiling joints even with back blocking. |
|  | pt75 | 75m paper tape | Paper tape for joining plasterboard. Paper tape is the strongest tape for plastering and should always be used for internal corners, butt joints, villaboard and ceiling joints. |
|  | fpt30 | 30m Flexi paper tape | Stiffened paper tape used for obtuse internal and external angles. Works best on shorter runs up to 2.4m as it can be a bit difficult to keep straight over longer runs. |
|  | Bedding Cement |  |  |
|  | b300 | Boral base coat | 20kg Boral brand base coat used for taping in joints, 2nd coat joints, filling external angels and beads |
|  | gb100 | CSR base coat | 20kg CSR Gyprock brand base coat used for taping in joints, 2nd coat joints, filling external angels and beads |
|  | BACK TO TOP |  |  |
|  | Finishing Cement |  |  |
|  | topcote | Boral top coat | 20kg Boral Plaster brand top coat used for the final coat, taping and 2nd coats. When using as a base coat up to 24 hours is required between coats as this is a "drying" compound, not a "setting"compound. |
|  | jmat20 | CSR top coat | 20kg CSR Gyprock brand top coat. Joint master top coat can only be used as a final coat. |
|  | External Angles |  |  |
|  | po124 | 2.4m metal external angle | Used to reinforce and finish off external corners on walls and bulkheads |
|  | po127 | 2.7m metal external angle |  |
|  | po130 | 3.0m metal external angle |  |
|  | po136 | 3.6m metal external angle |  |
|  | po1a24 | 2.4m obtuse metal external angle | Used to reinforce and finish of 135 degree obtuse external angles on walls and bulkheads. |
|  | po1a27 | 2.7m obtuse metal external angle |  |
|  | po1a30 | 3.0m obtuse metal external angle |  |
|  | po1a36 | 3.6m obtuse metal external angle |  |
|  | Product Code | Description | Product Use |
|  | Internal Angles |  |  |
|  | ps1724 | 2.4m metal internal angle | Used as an alternative to paper tape in 90 degree internals, often used for square setting between wall and ceiling junction. This angle is good to use if the wall/ceiling line is not quite straight. |
|  | ps1727 | 2.7m metal internal angle |  |
|  | ps1730 | 3.0m metal internal angle |  |
|  | ps1736 | 3.6m metal internal angle |  |
|  | ps17a24 | 2.4m obtuse metal internal angle | Used to reinforce and finish of 135 degree obtuse internal angles on walls and bulkheads. |
|  | ps17a27 | 2.7m obtuse metal internal angle |  |
|  | ps17a30 | 3.0m obtuse metal internal angle |  |
|  | ps17a36 | 3.6m obtuse metal internal angle |  |
|  | p18 | 2.4m internal angle | Flashing angle for internal angles in bathrooms. In older houses they were used to reinforce all internal angles this is not required with current plastering systems. |
|  | be24 | 2.4m beadex internal | Reinforced paper tape set at 90 degrees. Used as an alternative for taping internal angles and square setting wall/ceiling junctions. |
|  | be27 | 2.7m beadex internal |  |
|  | be30 | 3.0m beadex internal |  |
|  | BACK TO TOP |  |  |
|  | Stopping Bead |  |  |
|  | p1224 | 2.4m 10mm stopping bead | Stopping beads cap off the end of plasterboard. Used when a clean finish is required usually where plasterboard abuts a different surface. Very similar to a casing bead but leaves no edge when stopped up. Stopping angle has replaced the use of stopping bead in many applications, as stopping angle can be attached after the plasterboard is fixed in place. |
|  | p1227 | 2.7m 10mm stopping bead |  |
|  | p1230 | 3.0m 10mm stopping bead |  |
|  | p1236 | 3.6m 10mm stopping bead |  |
|  | p1324 | 2.4m 13mm stopping bead |  |
|  | p1327 | 2.7m 13mm stopping bead |  |
|  | p1330 | 3.0m 13mm stopping bead |  |
|  | p1336 | 3.6m 13mm stopping bead |  |
|  | p1130 | 3.0m 6mm stopping bead |  |
|  | p1430 | 3.0m 16mm stopping bead |  |
|  | Stopping Angle |  |  |
|  | p2530 | 3.0m 10mm stopping angle | Stopping angle caps off the end of plasterboard. Used when a clean finish is required usually where plasterboard abuts a different surface. Very similar to a casing bead but leaves no edge when stopped up. Stopping angle has replaced the use of stopping bead in many applications, as stopping angle can be attached after the plasterboard is fixed in place. |
|  | p2630 | 3.0m 13mm stopping angle |  |
|  | p2730 | 3.0m 16mm stopping angle |  |
|  | p5030 | 3.0m shadowline stopping angle | P50 creates a 10mm x 10mm shadowline and is often used around doors & windows and along the floor line. Often used in commercial applications. |
|  | BACK TO TOP |  |  |
|  | Casing Bead |  |  |
|  | p0524 | 2.4m 10mm casing bead | Casing beads cap off the end of plasterboard. Usually where plasterboard abuts a different surface. Casing bead leaves an edge and is not stopped up. |
|  | p0527 | 2.7m 10mm casing bead |  |
|  | p0530 | 3.0m 10mm casing bead |  |
|  | p0536 | 3.6m 10mm casing bead |  |
|  | p0630 | 3.0m 10mm shadowline casing bead |  |
|  | p0724 | 2.4m 13mm casing bead |  |
|  | p0727 | 2.7m 13mm casing bead |  |
|  | p0730 | 3.0m 13mm casing bead |  |
|  | p0736 | 3.6m 13mm casing bead |  |
|  | p0930 | 3.0m 13mm shadowline casing bead |  |
|  | p0430 | 3.0m 6mm casing bead |  |
|  | p0830 | 3.0m 16mm casing bead |  |

P32Q4