- Modeling Diode Circuits with MicroCap 7.0

Dr. Tony Richardson

Department of Electrical Engineering and Computer Science

University of Evansville

September 2, 2002

- Abstract

This tutorial describes how to model diodes using the MicroCap 7.0 circuit simulation program. We will focus on models appropriate for use in DC and low frequency AC circuits. High frequency models will be the topic of a future tutorial.

The basic diode model parameters are first described. We then discuss how to set model parameters to match a particular diode characteristic curve. Finally we develop appropriate models for ideal, small-signal, rectifier, LED, and Zener diodes.

- Modeling the Forward Bias Region

The Basic Model Parameters: LEVEL, IS and N

Suppose that we want to simulate a diode circuit using MicroCap. We start by placing a diode in the schematic diagram. You can do this by selecting Diode from the ComponentAnalog PrimitivesPassive Devices menu. After dropping the diode onto the schematic diagram the following window should appear:

Figure 1: MicroCap Diode Model Parameter Window

You can select one of the standard diode models from the list on the right side of the window. For most of our homework and project problems we will want to create our own diode model. This is easy to do. Just type in a model name. (I will use the name simple as an example.) Click on the Edit button in the lower right portion of the window. The window should now look like this:

Figure 2: MicroCap Diode Model Parameter Window

The window now includes a list of model parameter files that may be changed as required. There are over 30 parameters values that may are used in the MicroCap diode model, fortunately only a few of these affect the low-frequency behavior of the diode. The complete list of parameters is shown in Table 1 on the last page of this tutorial.

The three most important parameters to us are the first three shown in Figure 2: LEVEL, IS, and N. MicroCap provides two different diode models. We can select which model we want to use by setting the value of LEVEL. The default value of 1 selects the simpler of the two models, but this is a model which is sufficiently accurate for problems which we will encounter, so leave LEVEL set to 1. IS is the saturation current and N is the emission coefficient. IS and N appear in the equation describing the ideal diode current-voltage relationship:

In this equation, vD and iD are the diode voltage and current and VT is the thermal voltage. (VT = kT/q, k = Boltzmann's constant, T = absolute temperature, and q = the magnitude of the charge on an electron.) At room temperature (T = 300 ºK), VT = 26 mV.

To accurately model an arbitrary diode we just need two points (two iD - vD pairs) from the forward region of the diode characteristic curve. We can then find corresponding values of IS and N from:

For example, assume we want to model a diode that has a characteristic curve that passes through the points (0.70 V, 10 mA) and (0.72 V, 20 mA). Using the above formulas yields:

N = 1.11

IS = 2.91x10-13 A

The circuit shown in Figure 3 was simulated using MicroCap. The values of N and IS just found were used in the diode model. All other parameters were left set to their default values. Figure 4 shows a plot of the diode current versus the diode voltage as the source voltage is swept from 0 V to 50 V.

Figure 3: MicroCap Simple Diode Test Circuit

We can see from Figure 4 that the simulated current-voltage characteristic passes through the same two points as the characteristic curve we are modeling.

Figure 4: MicroCap Simple Diode Test Results

- Modeling the Reverse Bias Region

The Reverse Breakdown Parameters: BV and IBV

The default reverse breakdown (Zener) voltage is infinite. Obviously we need to change this if we are trying to accurately model a Zener diode. There are two parameters that need to be set to model the Zener breakdown region: BV is the breakdown voltage and IBV is current at the breakdown voltage. To model an 8 V Zener diode, typical values for these parameters might be:

BV = 8 V

IBV = 20 mA

Note that BV is set to a positive value although on a typical characteristic curve the breakdown voltage is negative. These values were used in the diode in Figure 3. A DC analysis was performed and the source voltage was swept from -60 V to 50 V. Figure 5 shows the result.

Figure 5: MicroCap Zener Diode Test Results

- Standard Diode Models

- The Ideal Diode

An ideal diode would have a 0 V knee or threshold voltage. We can find values of N and IS to approximate an ideal diode using the formulas given earlier. Assuming the approximate ideal characteristic passes through the two points (10 mV, 10 mA) and (20 mV, 2000 mA) the corresponding values of N and IS are:

N = 0.072

IS = 5x10-5 A

Figure 6 shows a circuit in which we compare two half-wave rectifiers. One uses the simple diode from an earlier circuit and the other uses an "ideal" diode using the values of N and IS from above. The voltage source is a 60 Hz, 5 VPEAK sine wave source. Figure 7 shows the results. The approximate 0.7 V drop across the simple diode and the 0 V drop across the "ideal" diode during the positive half-cycle of the input can clearly be seen in the figure.

- Small Signal Diodes

Using the data sheet of a 1N4148 diode for data, two points on a typical characteristic curve are (0.62 V, 1 mA) and (0.70 V, 5 mA). Corresponding values of N and IS are:

N = 1.91

IS = 3.83x10-9 A

Figure 6: MicroCap "Ideal" vs Standard Diode Test Circuit

Figure 7: MicroCap "Ideal" vs Standard Diode Test Results

- Rectifying Diodes

Using the data sheet of a 1N4001 diode as representative of a rectifying diode, we find:

N = 2.57

IS = 1.25x10-6 A

- Light Emitting Diodes (LEDs)

LEDs have greater forward voltage drops than small-signal or rectifying diodes. The forward voltage drop depends on the color and 2 V is a typically value for red LEDs. Using (2.0 V, 20 mA) and (2.3 V, 50 mA) as points on the characteristic curve and calculating N and IS gives:

N = 12.59

IS = 4.45x10-5 A

- Zener Diodes

Zener diodes have forward characteristics similar to that of a small-signal diode, so use the same values of N and IS as for the small-signal diode. The paramter BV should be set to the magnitude of the zener voltage and IBV should be set to a typical value of reverse current (20 mA is often used on data sheets).

BV = VZ

IBV = 20 mA

Diode Model Parameter Table

Name Parameter Units Default Area Level

LEVEL 1=Spice2G 2=PSpice 1

IS Saturation current Amps 1E-14 * 1,2

N Emission coefficient 1.0 1,2

ISR Recombination current parameter Amps 0 * 2

NR Emission coefficient for ISR 2 2

IKF High-injection "knee" current Amps INF * 2

BV Reverse breakdown "knee" voltage Volts INF 1,2

IBV Reverse breakdown "knee" current Amps 1E-10 * 1,2

NBV Reverse breakdown ideality factor 1 2

IBVL Low-level rev. breakdown "knee" current Amps 0 2

NBVL Low-level rev. breakdown ideality factor 1 2

RS Series resistance Ohms 0 / 1,2

TT Transit time sec 0.0 1,2

CJO Zero-bias junction capacitance Farad 0.0 * 1,2

VJ Junction potential Volts 1.0 1,2

M Junction grading coefficient 0.5 1,2

FC Forward bias depletion coefficient 0.5 1,2

EG Energy gap eV 1.11 1,2

XTI Temperature exponent for IS 3.0 1,2

TIKF IKF temperature coefficient (linear) 1/Celsius 0

TBV1 BV temperature coefficient (linear) 1/Celsius 0

TBV2 BV temperature coefficient (quadratic) 1/(Celsius^2) 0

TRS1 RS temperature coefficient (linear) 1/Celsius 0

TRS2 RS temperature coefficient (quadratic) 1/(Celsius^2) 0

KF Flicker noise coefficient 0.0 1,2

AF Flicker noise exponent 1.0 1,2

RL Junction Leakage Resistance Ohms INF 1

T_MEASURED Measured temperature Celsius undefined 1,2

T_ABS Absolute temperature Celsius undefined 1,2

T_REL_GLOBAL Relative to current temperature Celsius undefined 1,2

T_REL_LOCAL Relative to AKO model temperature Celsius undefined 1,2

A.M. Richardson Page 8 of 8 September 2, 2002