AUSTRALIAN STANDARD REQUIREMEMTS

The installation of fluorescent lighting, as with the other types, must comply with

Clause 4.5 of the Wiring Rules, which requires that luminaires and ancillary

equipment be installed so they do not cause –

All lampholders shall be located to be adequately protected against

damage that might reasonably be expected.

In order to protect against inadvertent contact with live parts, low voltage

lampholders shall be located to minimize the risk of—

(a) direct contact with live parts of a lampholder when the lamp is

removed; and

(b) mechanical damage to the lamp or lampholder.

These requirements may be satisfied by—

(i) installation not less than 1.8 m above the ground, floor or platform; or

(ii) provision of a suitable guard for the lamp and its lampholder.

A pendant lampholder shall not be exposed to the effects of significant

movement of air, e.g. outdoors or where exposed

Applications for fluorescent lighting

T8 fluorescents are the most common for commercial and industrial use. Smaller fluorescent lamps such as T5's are used in sign and display applications such as jewelry stores. Ceiling fixtures tend use a circline fluorescent light which can be found in residential applications.