Cogeneration (also combined heat and power, CHP) is the use of a heat engine or a power station to simultaneously generate both electricity and useful heat.

Conventional power plants emit the heat created as a byproduct of electricity generation into the environment through cooling towers, flue gas, or by other means. CHP or a bottoming cycle captures the byproduct heat for domestic or industrial heating purposes, either very close to the plant, or —especially in Scandinavia and eastern Europe—for distribution through pipes to heat local housing.

In the United States, Con Edison produces 30 billion pounds of steam each year through its seven cogeneration plants (which boil water to 1,000°F/538°C before pumping it to 100,000 buildings in Manhattan—the biggest commercial steam system in the world.[1][2]

Byproduct heat at moderate temperatures (212-356°F/100-180°C) can also be used in absorption chillers for cooling. A plant producing electricity, heat and cold is sometimes called trigeneration or more generally: polygeneration plant.

Cogeneration is a thermodynamically efficient use of fuel. In separate production of electricity some energy must be rejected as waste heat, but in cogeneration this thermal energy is put to good use.

- Contents [hide] - 1 Overview - 2 Types of plants - 2.1 MicroCHP - 2.2 MiniCHP - 3 History - 4 See also - 5 References - 6 External links

- [edit] Overview

Masnedø CHP power station in Denmark. This station burns straw as fuel. The adjacent greenhouses are heated by district heating from the plant.

Thermal power plants (including those that use fissile elements or burn coal, petroleum, or natural gas), and heat engines in general, do not convert all of their available energy into electricity. In most heat engines, a bit more than half is wasted as excess heat (see: Second law of thermodynamics). By capturing the excess heat, CHP uses heat that would be wasted in a conventional power plant, potentially reaching an efficiency of up to 89%, compared with 55%[3] for the best conventional plants. This means that less fuel needs to be consumed to produce the same amount of useful energy. Also, less pollution is produced for a given economic benefit.

Some tri-cycle plants have utilized a combined cycle in which several thermodynamic cycles produced electricity, and then a heating system was used as a condenser of the power plant's bottoming cycle. For example, the RU-25 MHD generator in Moscow heated a boiler for a conventional steam powerplant, whose condensate was then used for space heat. A more modern system might use a gas turbine powered by natural gas, whose exhaust powers a steam plant, whose condensate provides heat. Tri-cycle plants can have thermal efficiencies above 80%.

An exact match between the heat and electricity needs rarely exists. A CHP plant can either meet the need for heat (heat driven operation) or be run as a power plant with some use of its waste heat.

CHP is most efficient when the heat can be used on site or very close to it. Overall efficiency is reduced when the heat must be transported over longer distances. This requires heavily insulated pipes, which are expensive and inefficient; whereas electricity can be transmitted along a comparatively simple wire, and over much longer distances for the same energy loss.

A car engine becomes a CHP plant in winter, when the reject heat is useful for warming the interior of the vehicle. This example illustrates the point that deployment of CHP depends on heat uses in the vicinity of the heat engine.

Cogeneration plants are commonly found in district heating systems of big towns, hospitals, prisons, oil refineries, paper mills, wastewater treatment plants, thermal enhanced oil recovery wells and industrial plants with large heating needs.

Thermally enhanced oil recovery (TEOR) plants often produce a substantial amount of excess electricity. After generating electricity, these plants pump leftover steam into heavy oil wells so that the oil will flow more easily, increasing production. TEOR cogeneration plants in Kern County, California produce so much electricity that it cannot all be used locally and is transmitted to Los Angeles[citation needed].

- [edit] Types of plants

Topping cycle plants produce electricity first, then the exhausted steam is used for heating. Flames naturally produce heat suitable for a boiler. The hot water from condensed steam is well-suited for space and water heating.

Bottoming cycle plants produce high heats for an industrial process, then a waste heat recovery boiler feeds an electrical plant. Bottoming cycle plants are only used when the industrial process requires very high temperatures, such as furnaces for glass and metal manufacturing, so they are rarer.

Large cogeneration systems provide heating water and power for an industrial site or an entire town. Common CHP plant types are:

- Gas turbine CHP plants using the waste heat in the flue gas of gas turbines

- Combined cycle power plants adapted for CHP

- Steam turbine CHP plants that use the heating system as the steam condenser for the steam turbine.

- Molten-carbonate fuel cells have a hot exhaust, very suitable for heating.

Smaller cogeneration units may use a reciprocating engine or Stirling engine. The heat is removed from the exhaust and the radiator. These systems are popular in small sizes because small gas and diesel engines are less expensive than small gas- or oil-fired steam-electric plants.

Some cogeneration plants are fired by biomass [4], or industrial and municipal waste (see incineration).

- [edit] MicroCHP

"Micro cogeneration" is a so called distributed energy resource (DER). the installation is usually less than 5 kWe in a house or small business[1]. Instead of burning fuel to merely heat space or water, some of the energy is converted to electricity in addition to heat. This electricity can be used within the home or business, or (if permitted by the grid management) sold back into the electric power grid.

- [edit] MiniCHP

"Mini cogeneration" is a so called distributed energy resource (DER). the installation is usually more than 5 kWe and less than 500 kWe in a building or medium sized business [2].

Current (2007) Micro- andMiniCHP installations use five different technologies: microturbines, internal combustion engines, stirling engines, closed cycle steam engines and fuel cells.

- [edit] History

Perhaps the first modern use of energy recycling was done by Thomas Edison. His 1882 Pearl Street Station, the world’s first commercial power plant, was a combined heat and power plant, producing both electricity and thermal energy while using waste heat to warm neighboring buildings.[5] Recycling allowed Edison’s plant to achieve approximately 50 percent efficiency.

By the early 1900s, regulations emerged to promote rural electrification through the construction of centralized plants managed by regional utilities. These regulations not only promoted electrification throughout the countryside, but they also discouraged decentralized power generation, such as cogeneration. They even went so far as to make it illegal for non-utilities to sell power.[6]

By 1978, Congress recognized that efficiency at central power plants had stagnated and sought to encourage improved efficiency with the Public Utility Regulatory Policies Act (PURPA), which encouraged utilities to buy power from other energy producers. Cogeneration plants proliferated, soon producing about 8 percent of all energy in the U.S.[7] However, the bill left implementation and enforcement up to individual states, resulting in little or nothing being done in many parts of the country.

Outside the U.S., energy recycling is more common. Denmark is probably the most active energy recycler, obtaining about 55% of its energy from cogeneration and waste heat recovery. Other large countries, including Germany, Russia, and India, also obtain a much higher share of their energy from decentralized sources.[8][9]

- Cogeneration-Combined Heat and Power (Electricity) Generation

- Michael Roarty 
Science, Technology, Environment and Resources Group 
11 May 1999

Definition

Cogeneration or CHP (combined heat and power) is the simultaneous production of electricity and heat using a single fuel such as natural gas, although a variety of fuels can be used (refer to 'Cogeneration capacity by primary fuel'). The heat produced from the electricity generating process (for example from the exhaust systems of a gas turbine) is captured and utilised to produce high and low level steam. The steam can be used as a heat source for both industrial and domestic purposes and can be used in steam turbines to generate additional electricity (combined cycle power). Cogeneration for on-site power and heat is well established overseas, especially in Scandinavian countries. Its use is gradually increasing in Australia, although optimistic forecasts of rapid implementation and growth in the last couple of years have yet to be realised.

Advantages of cogeneration

Cogeneration technology provides greater conversion efficiencies than traditional generation methods as it harnesses heat that would otherwise be wasted. This can result in up to more than a doubling of thermal efficiency or higher heat values (HHV) (see Figure 1). Also, carbon dioxide emissions can be substantially reduced. Furthermore, the heat by-product is available for use without the need for the further burning of a primary fuel. Cogeneration systems predominantly use natural gas, a fuel source which emits less than half the greenhouse gas, per unit of energy produced than the cleanest available thermal power station.

Figure 1: Electricity generation efficiencies

Source: Australian Cogeneration Association (ACA)

Figure 1 shows the increasing thermal efficiency from the use of thermal brown coal to cogeneration and an inverse relationship to carbon dioxide emissions. At a time when there is increased emphasis on both increased thermal efficiency and the mitigation of carbon dioxide emissions, electricity generation by cogeneration appears a logical choice. An additional advantage of cogeneration is that the plant is usually located near the end user (termed distributed or embedded generation) and as such no power transmission losses are suffered. Cogeneration systems compete with electricity provided from large-scale power stations, remote from electricity consumers and as such require long distance, high voltage transmission networks (referred to as centralised systems). It is claimed by the Australian Cogeneration Association (ACA) that cogeneration systems are more environmentally friendly, flexible, efficient and can be more cost-effective than traditional systems-especially when network costs and losses are taken into account.

A major reason that cogeneration systems have not progressed as rapidly as was forecast is that the price of electricity has declined markedly since the development of the national electricity market (NEM). Present prices are believed to be well below long-run generating cost.

Global trends in cogeneration

A survey current at the end of 1996 conducted by the International Cogeneration Alliance showed that Australia lagged the world in terms of the installed cogeneration capacity. The Netherlands was the world's leader with installed generation capacity of 40 per cent of total capacity and was closely followed by other Scandinavian countries such as Denmark and Finland. Australia recorded a low 3 per cent of total installed generation capacity. The Netherlands Government has set a cogeneration target of 50 per cent of total installed cogeneration capacity by 2000 with a longer-term 70 per cent target and the European Union has set its sights on increasing capacity to 30 per cent of the total by 2010.

By the end of 1998, cogeneration installed capacity in Australia had increased to close to 5 per cent of total capacity. At this date there were 122 projects operating in Australia with an operating capacity of 2,084 megawatts electricity (MWe). The extent to which increases of cogeneration capacity occur in Australia is contingent on addressing market hurdles to the further development of embedded generation, and the recognition of its significantly lower greenhouse gas emissions.

Cogeneration capacity by primary fuel

Cogeneration capacity by primary fuel type is outlined in Figure 2. Natural gas accounts for over 50 per cent of primary fuel type.

Figure 2: Cogeneration by fuel type

Source: ACA

Important contributions to fuel type are being made from waste products such as cane residue (bagasse) in sugar cane growing regions, sludge gas from sewerage treatment works and methane from either land fill sites or coal bed methane. Where a cogeneration plant is powered by waste gases, especially coal bed methane and methane recovered from landfill sites, then fugitive gases that are naturally escaping into the atmosphere and act as particularly potent greenhouse gases, are captured and utilised to drive gas turbines which in turn generate electricity.

New plant installations

During 1998, six new cogeneration plants were commissioned in Australia. The combined capacity of these projects amounted to 285MWe, a significantly higher figure than the 221 MWe commissioned in 1997. In addition there were a further 9 plants under construction with a further 243 MWe of generating capacity.

Large cogeneration plants that have come on line in recent times are Sithe Energy's Smithfield plant (1997) in Western Sydney and CU Power and Boral's Osborne plant (1998) in South Australia.

The Smithfield Energy Facility is a 162 MWe state of the art gas fired cogeneration plant. It has three General Electric Frame 6 gas turbines (each 38 MWe) in combined cycle operation and three heat recovery steam generators producing steam for both sale and to power a 65 MWe steam turbine. Both high and low pressure steam is sold under long term contract to Visy Industries (a division of Pratt Industries) who are Australia's largest manufacturer of cardboard boxes. The energy cost savings attributable to the cogeneration plant has enabled Visy to increase cardboard box production to close on 50 per cent. The electricity is sold to Integral Energy on long term contract.

Osborne is the largest (180 MWe) most modern cogeneration plant in Australia. A high degree of flexibility has been built into the plant that allows it to produce steam for both an industrial process and for electricity production. The electrical efficiency of the plant at 49 per cent is markedly higher than the 33 per cent that can be achieved from conventional plants and when the thermal efficiency is factored in from the production of steam the total efficiency approached 70 per cent. Low-cost steam is supplied to Penrice Soda Products (the largest producer of soda ash in Australia) and electricity is sold to ETSA Power under long term contract.

Future trends in electricity generation

The traditional electricity system as we know it may well evolve beyond recognition as global and national pressures gain momentum to reduce emissions and a more holistic approach is taken to evaluating power generation, supply, and the provision of energy services to end users.

In time an increasing proportion of new power will come from a range of small embedded generators, including cogeneration.

Clean Energy Fact Sheets

All about Cogeneration

www.cleanenergycouncil.org.au © Clean Energy Council 2007 ABN 8412 710 2443

Why we need clean energy

Energy drives the economy; however Australia’s stationary energy sector is derived mostly from highemission

fossil fuels and is therefore responsible for 50% of our greenhouse gas emissions. Australia’s

economic security is at risk from climate change unless we can compete in a low carbon world. Any

successful climate change solution must target the energy sector specifically.

Australia has some of the world’s best renewable and low-emission power sources. Some of these

technologies are in development and will soon join the existing mature clean energy technologies that are

already meeting the growing energy demand while giving us a clean-powered, sustainable economy. This

Fact Sheet is part of a series that provide the basics about clean energy.

Cogeneration – how it works

Burning fuel in power plants usually ends up producing more heat than electricity. Coal-fired power plants

are typically only 30% efficient in converting the energy of the fuel into electricity, with the rest of the

energy lost as waste heat. In addition further energy is lost in transporting the electricity often vast

distances to where it is actually consumed in homes and businesses – the losses in transporting the

electricity are often as high as 10%. In addition, the electricity network – the poles and wires – that

transport power are very expensive. Over the next five years Australia will spend over $24 billion dollars on

upgrading and growing Australia’s electricity network.

However there is a more efficient way to produce electricity. Smaller, gas-fired power plants can be located

nearby to customers, where the heat as well as the electricity can be usefully employed. This achieves

significantly improved energy efficiency - above 70% is typical – and is known as cogeneration or co-gen.

Cogeneration power plants can also reduce the amount of costly electricity network infrastructure required

to transport electricity to where it is needed as the power plants are located right near customers.

Greenhouse gas savings

Cogeneration power plants have a third of the emissions associated with producing electricity from coal

power plants and the increased efficiency resulting from cogeneration means less greenhouse emissions.

In Australia

Australia has 2667 megawatts of installed cogeneration power plant capacity, with much of it used in

heavy industries such as metals, paper and chemicals.

Potential

There is potential for significantly more cogeneration in Australia. There are a number of industrial plants,

as well as commercial and residential facilities and buildings, which, for example consume gas and

electricity to produce heat and steam for general heating, water heating, and run production processes.

Cogeneration could supply the heat and steam needs while at the same time producing electricity much

more efficiently with significantly lower greenhouse emissions. Gas used in more efficient cogeneration

power plants is a major opportunity for low-cost greenhouse abatement in Australia through improved

energy efficiency. It could also provide major savings through reducing the more than $24 billion planned

to be spent on networks over the next five years.

Global view

In Europe it is very common for cogeneration power plants to be used to supply the heat needs of

buildings, yet in Australia there are only a handful of examples.

Clean Energy Fact Sheets

All about Cogeneration

www.cleanenergycouncil.org.au © Clean Energy Council 2007 ABN 8412 710 2443

Current Issues

Unfortunately the way in which electricity network services are managed and priced makes it very difficult

for cogeneration power plants to get financial recognition for the benefits they provide in avoided network

infrastructure costs for the broader electricity system. In essence the way regulators treat electricity

generators ignores their associated transportation costs. This is actually a very significant omission

because transportation costs – the costs of networks – make up more than half the cost of electricity for

most consumers.

Also because Australia has not put in place any legal cap or target on greenhouse emissions the lower

emissions of cogeneration do not receive any financial recognition.

About the Clean Energy Council

The Clean Energy Council is the peak industry association that brings together the three fields of energy

supply, business outcomes and environmental priorities. With over 410 members, providing close to a

quarter of Australia’s electricity, $5.7 billion in annual sales and employment to over 20,000 people, our

members have the knowledge and the technologies to drive the new clean-powered and efficeint economy.

Go to www.cleanenergycouncil.org.au and register for info, events and updates.

# Prime Movers

Cogeneration plant consists of four basic elements: - A prime mover (engine); - An electricity generator; - A heat recovery system; - A control system.

Depending on the site requirements, the prime mover may be a steam turbine, reciprocating engine or gas turbine. The prime mover drives the electricity generator and waste heat is recovered. The basic elements are all well established items of equipment, of proven performance and reliability.

### Prime Movers

Cogeneration units are generally classified by the type of prime mover (i.e. drive system), generator and fuel used. The following sections examine the main types.

### Steam Turbines

Steam turbines have been used as prime movers for industrial cogeneration systems for many years. High-pressure steam raised in a conventional boiler is expanded within the turbine to produce mechanical energy, which may then be used to drive an electric generator. This system generates less electrical energy per unit of fuel than a gas turbine or reciprocating engine-driven cogeneration system, although its overall efficiency may be higher, achieving up to 84% (based on fuel gross calorific value).

For viable power generation, steam input must be at a high pressure and temperature. The plant is capital intensive because a highpressure boiler is required to produce the motive steam. At existing sites, where steam systems are supplied by low-pressure boilers, it will be necessary to replace these boilers with highpressure plant, possibly retaining the original equipment as stand-by.

Steam cycles typically produce a large amount of heat compared with the electrical output, resulting in a high cost installation in terms of Euro/kWe.

### Gas Turbines

The gas turbine has become the most widely used prime mover for large-scale cogeneration in recent years, typically generating 1-100 MWe. A gas turbine based system is much easier to install on an existing site than high-pressure boiler plant and a steam turbine. On many sites plot space is at a premium, a factor weighing heavily in favour of gas turbines. This, together with reduced capital cost and the improved reliability of modern machines, often makes gas turbines the optimum choice.

The fuel is burnt in a pressurised combustion chamber using combustion air supplied by a compressor that is integral with the gas turbine. The very hot (900ºC- 1200oC) pressurised gases are used to turn a series of fan blades, and the shaft on which they are mounted, to produce mechanical energy. Residual energy in the form of a high flow of hot exhaust gases can be used to meet, wholly or partly, the thermal demand of the site.

The available mechanical energy can be applied in the following ways:

- to produce electricity with a generator (most applications);

- to drive pumps, compressors, blowers, etc.

A gas turbine operates under exacting conditions of high speed and high temperature. High-premium fuels are therefore most often used, particularly natural gas. Distillate oils such as gas oil are also suitable, and sets capable of using both are often installed to take advantage of cheaper interruptible gas tariffs.

Waste gases are exhausted from the turbine at 450oC to 550oC, making the gas turbine particularly suitable for high-grade heat supply. The usable heat to power ratio ranges from 1.5:1 to 3:1 depending on the characteristics of the particular gas turbine. Supplementary firing may be used to increase exhaust gas temperatures to 1,000oC or more, raising the overall heat to power ratio to as much as 10:1.

Supplementary firing is highly efficient as no additional combustion air is required to burn extra fuel. Efficiencies of 95% or more are typical for the fuel burned in supplementary firing systems.

Exhaust gases can be used in either of the following ways:

- For direct firing and drying processes. The single flow of heat at high temperature is suitable for processes in which direct contact with combustion gases is permissible.

- To raise steam at medium or low pressure for process or space heating in an open-cycle gas turbine cogeneration plant which comprises a gas turbine-alternator unit and a heat recovery boiler.

- To generate hot water, best for high temperature hot water applications where temperatures in excess of 140°C are required.

- To raise steam in a HRSG at high pressure for use in a steam turbine

Gas turbines are available in a wide power output range from 250 kWe to over 200 MWe, although sets smaller than 1 MWe have so far been generally uneconomic due to their comparatively low electrical efficiency and consequent high cost per kWe output. This is starting to change. The turbine is typically mounted on the same sub-base as its generator, with a stepdown gearbox between the two to reduce the high shaft speed of the turbine to a speed suitable for the generator. A gas turbo-generator is extremely noisy and generally housed in an acoustic enclosure for noise attenuation.

### Reciprocating Engines

The reciprocating engines used in cogeneration are internal combustion engines operating on the same familiar principles as their petrol and diesel engine automotive counterparts. Although conceptually the system differs very little from that of gas turbines, there are important differences. Reciprocating engines give a higher electrical efficiency, but it is more difficult to use the thermal energy they produce, since it is generally at lower temperatures and is dispersed between exhaust gases and cooling systems.

The usable heat:power ratio range is normally in the range 0.5:1 to 2:1. However, as the exhaust contains large amounts of excess air, supplementary firing is feasible, raising the ratio to a maximum of 5:1.

#### Compression-ignition ('diesel') engines

Compression-ignition ('diesel') engines for large-scale cogeneration are predominantly four-stroke direct-injection machines fitted with turbochargers and intercoolers. Diesel engines will accept gas oil, HFO and natural gas. The latter is in reality a dual-fuel mode, as a small quantity of gas oil (about 5% of the total heat input) has to be injected with the gas to ensure ignition; as the engine can also run on gas oil only it is suited to interruptible gas supplies. Cooling systems are more complex than on spark-ignition engines and temperatures are often lower, typically 85oC maximum, thereby limiting the scope for heat recovery. Exhaust excess air levels are high and supplementary firing is practicable. Compression-ignition engines run at speeds of between 500 and 1500 rev/min. In general, engines up to about 500 kWe (and sometimes up to 2 MWe) are derivatives of the original automotive diesels, operating on gas oil and running at the upper end of their speed range. Engines from 500 kWe to 20 MWe evolved from marine diesels and are dual-fuel or residual fuel oil machines running at medium to low speed.

#### Spark-ignition engines

Spark-ignition engines are derivatives of their diesel engine equivalents and have their same parameter equivalents as 90°C cooling water.

Traditionally, shaft efficiency has been lower than for compression ignition engines. The output of a spark-ignition engine is a little smaller, typically 83% of the diesel engines.

They are suited to smaller, simpler cogeneration installations, often with cooling and exhaust heat recovery cascaded together with a waste heat boiler providing medium or low temperature hot water to site.

Spark-ignition engines operate on clean gaseous fuels, natural gas being the most popular. Biogas and similar recovered gases are also used but, because of their lower calorific value, output is reduced for a given engine size. Spark-ignition engines give up less heat to the exhaust gases than diesel engines. The large lean-burn engines have typically 12% oxygen in exhaust gases, and this can be used with supplementary firing. The following are among the most common applications for the thermal energy produced by reciprocating engines:

- production of up to 15 bar steam utilising the heat of exhaust gases; and separate production of hot water at 85-90oC from the cooling system of the engine;

- production of hot water at 90oC, supplementing the temperature of cooling system water with heat from the gases;

- Exhaust fumes can be used directly in certain processes, such as drying, CO2 production, etc;

- generation of hot air. All the residual energies from the engine can be used, through the installation of suitable exchange devices, for the generation of hot air.

Reciprocating machines by their nature have more moving parts, some of which wear more rapidly than those in purely rotating machines, and have running as well as shutdown maintenance requirements. Nevertheless, typical availability is about 90-96%.

Gas engines are operated under two distinct air/fuel ration regimes that have a market effect upon environmental performance:

- Stoichiometric engines;

- Lean-burn engines.

NOx emissions can be reduced markedly by operating with large excess of combustion air (lean-burn). However, this has an adverse effect upon the engine’s power output.

Stoichiometric engines tend to be smaller (typically <300 kWe) than their lean-burn counterparts and are based upon standard vehicle engine blocks with adapted cylinder heads and spark ignition systems.

Cogeneration diesel plants HFO systems have been built in those places where gas is not available. This includes many islands and developing countries. In places where gas availability will arrive later, the plants can use HFO at the beginning and later switch to gas, or use HFO in winter and gas in summer.

| #### Type | #### Output Range | #### Typical Fuels | #### Typical Heat to Power Ratio | #### Grade of Heat Output |
| --- | --- | --- | --- | --- |
| #### Gas Turbine | 0.5MW upwards | Natural gas, Gas Oil, Landfill gas, Biogas, Mine gas | 1.6:1 up to 5:1 with after firing | High |
| #### Compression ignition gas engine | 2MW upwards | Natural gas with gas oil, Heavy fuel oil | 1:1 to 1.5:1 up to 2.5:1 with after firing | Low and High |
| #### Spark Ignition | Up to 4MW | Natural gas, Landfill gas, Biogas, Mine gas | 1:1 to 1.7:1 | Low and High |
| #### Steam Turbine | 0.5MW upwards | Any, but converted to steam | 3:1 to 10:1 | Meduim |
| #### CCGT | 10MW upwards | As gas turbine | Down to 0.7:1 | Meduim |

### Combined Cycles

Some large systems utilise a combination of gas turbine and steam turbine, with the hot exhaust gases from the gas turbine being used to produce the steam for the steam turbine. This is called a combined cycle.

Gas turbine combined cycle (CCGT) systems have been adopted by public utility companies where supplies of natural gas are plentiful: power stations of up to 1,800 MWe have been constructed. In cogeneration applications of the CCGT, exhaust or pass-out steam from the steam turbine is used for process or other heating duties. The main advantage of CCGT cogeneration is its greater overall efficiency in the production of electricity.

### Waste heat recovery units

The heat recovery boiler is an essential component of the cogeneration installation. It recovers the heat from the exhaust gases of gas turbines or reciprocating engines. The simplest one is a heat exchanger through which the exhaust gases pass and the heat is transferred to the boiler feedwater to raise steam. The cooled gases then pass on the exhaust pipe or chimney and are discharged into the atmosphere. In this case, the composition or constituents of the exhaust gases from the prime mover are not changed. The exhaust gases discharged, contain significant quantities of heat, but not all can be recovered in a boiler. One typical feature of the exhaust heat boiler (or waste heat recovery unit) is that the typical size is bigger than a conventional fuel-burning unit. This is for two main reasons:

- The lower exhaust gas temperatures require a greater heat transfer area in the boiler;

- There are practical limitations on the flow restriction.

Excessive flow resistance in the exhaust gas stream must be avoided as this can adversely affect operation of the turbine or engine.

Exhaust heat boilers are not, therefore,‘offthe- shelf’ items: they need to be designed for the particular exhaust conditions of the specified turbine or engine. The usual procedure is to provide the boiler supplier with details of the exhaust gas flow from which the heat is to be recovered, and with the temperature and pressure conditions of the required heat output. The boiler supplier will then be able to advise on the quantity of heat that can be recovered, and the temperature at which the exhaust gas will be discharged from the boiler.

The choice of prime mover is based on a number of factors and even with similar energy requirements, no two sites are the same.

The critical factor is the Heat to Power ratio of site demand. Where the electrical power requirement is relatively high as a proportion of total energy this tends to favour engines. Conversely where heat demand is typically more than 3 or 4 times electrical demand the turbine begins to have an advantage. Another key factor is the quality of heat required by the customer site. Some industrial processes have little use for low grade heat – the hot water produced in enginesbased schemes. Where high temperature steam is the primary heat requirement then the turbine is clearly superior.

Cogeneration: a regulatory framework toward growth

E. Cardona, and A. Piacentino

DREAM—Department of Energetic and Environmental Researches, Engineering Faculty—Università di Palermo, Viale delle Scienze, Palermo 90128, Italy

Available online 8 June 2004.

References and further reading may be available for this article. To view references and further reading you must purchase this article.

### Abstract

After having defined the general rules for the European liberalised energy market, new actions are needed to improve the energy system.

In the rising common framework, the environmental policies and particularly the cost-internalising actions are being empowered, in order to favour improvements in conversion efficiencies and to promote low environmental impact technologies.

The emerging EU policy for the promotion of cogeneration and trigeneration plants that fixes the eligibility criteria for high conversion efficiency plants is here discussed.

On the basis of thermodynamic remarks and of a comparative analysis of the evaluation criteria applied to most industrialised countries, the new EU directive is critically examined.

The approaches that are not thermodynamically appropriate and could affect the EU provisions concerning the internal competition without ensuring benefits in terms of energy saving are also underlined in this report.

Where available, information about the expected or stated impacts of the different evaluation criteria are also discussed.

Author Keywords: Cogeneration; Legislation; Efficiency

### Nomenclature

Nomenclature

DG

distributed generation

DER

distributed energy resources

ECHP

electricity assessed from high efficiency CHP

HCHP

heat produced from CHP process

Kprime mover

power/heat ratio of a prime mover

CHP Hη

thermal efficiency of cogeneration, i.e ratio of annual heat from cogeneration and fuel cons.

CHP Eη

Electric efficiency of cogeneration, i.e. ratio of annual electricity from cogeneration and fuel

Ref Hη

reference efficiency for heat production in a standard boiler

Ref Eη

reference efficiency for electricity production in a power plant

E

electricity yearly produced in a CHP plant (in CHP and non-CHP activity)

H

heat yearly produced in a CHP plant

Ef

energy yearly introduced by fuel in a CHP plant

Hη

thermal efficiency of the process, i.e. H/Ef ratio

Eη

electric efficiency of the process, i.e. E/Ef ratio

Ien

Indice Energetico (Energy Index)

PES

primary Energy Saving

Estandard

yearly energy consumption for producing E and H by separate processes

Hciv

heat for to civil uses (like in the tertiary sector)

Hind

heat for industrial processes

Ref Hciv–η

reference value for separate production of heat for civil uses

Ref Hind–η

reference value for separate heat production of heat for industrial processes

IRE

Indice di Risparmio Energetico (energy saving index)

LT

Limite Termico (thermal limit)

P

coefficient accounting for the avoided losses on the grid

QI

quality index

X, Y

multiplying factors respectively for electric and thermal CHP efficiencies

EEL

electric efficiency

CHPGood Quality Fuel

Fuel consumption assessed as Good Quality CHP

Hη′

fictitious thermal efficiency

Eel, Good Quality

electricity assessed as Good Quality CHP

EK–Kpm

electricity not produced from CHP and improperly eligible for support

K

multiplying factor

Kuses

ratio between electric and heat yearly consumption

ηel.eq.

equivalent electric efficiency

ηtot

overall efficiency, ratio between total output and fuel input

PC,PV,PA

respectively constant, variable and avoided emissions-related fractions of revenue

t

temperature of heat requirements (°C)

ASE

average specific emissions

Emtot

total emissions

SEE

specific emission for electricity production

EmH

emissions for producing H in a conventional boiler

### Article Outline

Nomenclature

1. Introduction

2. The new EU Directive

3. A thermodynamic comparison of evaluation criteria

3.1. The Italian case: a long definition process

3.2. The English case: an easy eligibility for a troubled CHP market

3.3. The Spanish case: a difficult eligibility for a developing market

3.4. Other European countries

4. CHP evaluation in USA

5. Results of comparative analysis

6. Conclusions

References

Fig. 1. Graphical comparison between the evaluation criteria adopted in Italy since 1992.

View Within Article

Cogeneration (Combined Heat and Power or CHP) is the simultaneous production of electricity and heat, both of which are used. The central and most fundamental principle of cogeneration is that, in order to maximise the many benefits that arise from it, systems should be based according to the heat demand of the application. This can be an individual building, an industrial factory or a town/city served by district heat/cooling. Through the utilisation of the heat, the efficiency of cogeneration plant can reach 90% or more. Cogeneration therefore offers energy savings ranging between 15-40% when compared against the supply of electricity and heat from conventional power stations and boilers:

Typical Cogeneration Scheme:

Tyical conventional power generation scheme:

Cogeneration optimises the energy supply to all types of consumers with the following benefits to both users and society at large:

- Increased efficiency of energy conversion and use. Cogeneration is the most effective and efficient form of power generation

- Lower emissions to the environment, in particular of CO2, the main greenhouse gas. Cogeneration is the single biggest solution to the Kyoto targets

- Large cost savings, providing additional competitiveness for industrial and commercial users, and offering affordable heat for domestic users

- An opportunity to move towards more decentralised forms of electricity generation, where plant is designed to meet the needs of local consumers, providing high efficiency, avoiding transmission losses and increasing flexibility in system use. This will particularly be the case if natural gas is the energy carrier

- Improved local and general security of supply - local generation, through cogeneration, can reduce the risk that consumers are left without supplies of electricity and/or heating. In addition, the reduced fuel need which cogeneration provides reduces the import dependency - a key challenge for Europe's energy future

- An opportunity to increase the diversity of generation plant, and provide competition in generation. Cogeneration provides one of the most important vehicles for promoting liberalisation in energy markets

- Increased employment - a number of studies have now concluded that the development of CHP systems is a generator of jobs.

More in-depth information on cogeneration is provided in the following publications:

The method for the production of electrical power by a diesel engine (9) connected to an alternator (10) which has biodiesel as fuel is described. The waste heat of the diesel engine is applied for the production of biodiesel.This cogeneration system contributes to the demand for so called "green" electrical and automotive energy by using fuels produced out of (waste) oils and fats of vegetable and animal origin. The carbondioxide emission in burning these fuels, is neutral to the environment

# What is the fuel type for my cogeneration system?

Fuel types for cogeneration systems available in BASIX:

- biodiesel (an alternative fuel or fuel additive for diesel engines, which is renewable, biodegradable and has low greenhouse gas emissions)

- gas

- diesel

These fuel types are ranked in order of greenhouse efficiency, with biodiesel the most greenhouse efficient fuel type, followed by gas, then diesel.

If you are selecting the biodiesel option, the calculations assume that B60 biodiesel will be used (a blend of 60% biodiesel, 40% diesel). If the system is designed to take B100, please apply through the BASIX Alternative Assessment process.

What is a Combined Cycle Power Plant?

As opposed to a Simple Cycle Power Plant that is not very efficient as the heat from the turbine is "wasted" to the atmosphere, a Combined Cycle Power Plant ("CCPP") recovers this heat.  CCPP's, also referred to as "Combined Cycle Cogeneration, is a natural gas (or diesel) turbine coupled with an electrical  generator, together, referred to as a "genset." The exhaust heat from the gas turbine is directed to a waste heat recovery boiler ("WHRB") or heat-recovery steam generator ("HRSG"). The steam from the WHRB or HRSG where the steam is directed to a steam turbine generator where the steam is used to power the steam turbine genset. By capturing the waste heat of the gas turbine in a Combined Cycle Power Plant, and putting it to work, the overall thermal efficiency of the plant is increased. In a typical  cogeneration plant, electric power is generated but some of the steam from the WHRB or HRSG is used for process heat.  By diagram below, the combined-cycle power plant combines the Rankine (steam turbine) and Brayton (gas turbine) thermodynamic cycles by using heat recovery boilers to capture the energy in the gas turbine exhaust gases for steam production to supply a steam turbine as shown in the figure "Combined-Cycle Cogeneration". Process steam can be also provided for industrial purposes.

Fossil fuel-fired (central) power plants use either steam or combustion turbines to provide the mechanical power to electrical generators. Pressurized high temperature steam or gas expands through various stages of a turbine, transferring energy to the rotating turbine blades. The turbine is mechanically coupled to a generator, which produces electricity.

Steam Turbine Power Plants:

Fossil fuel-fired (central) power plants use either steam or combustion turbines to provide the mechanical power to electrical generators. Pressurized high temperature steam or gas expands through various stages of a turbine, transferring energy to the rotating turbine blades. The turbine is mechanically coupled to a generator, which produces electricity.

Steam Turbine Power Plants:

Steam turbine power plants operate on a Rankine cycle. The steam is created by a boiler, where pure water passes through a series of tubes to capture heat from the firebox and then boils under high pressure to become superheated steam. The heat in the firebox is normally provided by burning fossil fuel (e.g. coal, fuel oil or natural gas). However, the heat can also be provided by biomass, solar energy or nuclear fuel.  The superheated steam leaving the boiler then enters the steam turbine throttle, where it powers the turbine and connected generator to make electricity.  After the steam expands through the turbine, it exits the back end of the turbine, where it is cooled and condensed back to water in the surface condenser.  This condensate is then returned to the boiler through high-pressure feedpumps for reuse.  Heat from the condensing steam is normally rejected from the condenser to a body of water, such as a river or cooling tower.

Steam turbine plants generally have a history of achieving up to 95% availability and can operate for more than a year between shutdowns for maintenance and inspections. Their unplanned or forced outage rates are typically less than 2% or less than one week per year.

Modern large steam turbine plants (over 500 MW) have efficiencies approaching 40-45%.  These plants have  installed costs between $800 and$2000/kW, depending on environmental permitting requirements.

Combustion (Gas) Turbines:

Combustion turbine plants operate on the Brayton cycle.  They use a compressor to compress the inlet air upstream of a combustion chamber. Then the fuel is introduced and ignited to produce a high temperature, high-pressure gas that enters and expands through the turbine section.  The turbine section powers both the generator and compressor. Combustion turbines are also able to burn a wide range of liquid and gaseous fuels from crude oil to natural gas.

The combustion turbine’s energy conversion typically ranges between 25% to 35% efficiency as a simple cycle.  The simple cycle efficiency can be increased by installing a recuperator or waste heat boiler onto the turbine’s exhaust.  A recuperator captures waste heat in the turbine exhaust stream to preheat the compressor discharge air before it enters the combustion chamber.  A waste heat boiler generates steam by capturing heat form the turbine exhaust.  These boilers are known as heat recovery steam generators (HRSG). They can provide steam for heating or industrial processes, which is called cogeneration.  High-pressure steam from these boilers can also generate power with steam turbines, which is called a combined cycle (steam and combustion turbine operation).   Recuperators and HRSGs can increase the combustion turbine’s overall energy cycle efficiency up to 80%.

Combustion (natural gas) turbine development increased in the 1930’s as a means of jet aircraft propulsion. In the early 1980’s, the efficiency and reliability of gas turbines had progressed sufficiently to be widely adopted for stationary power applications. Gas turbines range in size from 30 kW (micro-turbines) to 250 MW (industrial frames).  Industrial gas turbines have efficiencies approaching 40% and 60% for simple and combined cycles respectively.

The gas turbine share of the world power generation market has climbed from 20 % to 40 % of capacity additions over the past 20 years with this technology seeing increased use for base load power generation. Much of this growth can be accredited to large (>500 MW) combined cycle power plants that exhibit low capital cost (less than $550/kW) and high thermal efficiency.

The capital cost of a gas turbine power plant can vary between $35000-$950/kW with the lower end applying to large industrial frame turbines in combined cycle configurations. Availability of natural gas-fired plants can exceed 95%.  In Canada, there are 28 natural gas-fired combined cycle and cogeneration plants with an average efficiency of 48 %. The average power output for each plant was 236 MW with an installed cost of around $ 500/kW.

## Simple Cycle Power Plants (Open Cycle)

The modern power gas turbine is a high-technology package that is comprised of a compressor, combustor, power turbine, and generator, as shown in the figure "Simple-Cycle Gas Turbine".

In a gas turbine, large volumes of air are compressed to high pressure in a multistage compressor for distribution to one or more combustion gases from the combustion chambers power an axial turbine that drives the compressor and the generator before exhausting to atmosphere. In this way, the combustion gases in a gas turbine power the turbine directly, rather than requiring heat transfer to a water/steam cycle to power a steam turbine, as in the steam plant. The latest gas turbine designs use turbine inlet temperatures of 1,500C (2,730F) and compression ratios as high as 30:1 (for aeroderivatives) giving thermal efficiencies of 35 percent or more for a simple-cycle gas turbine.

## Process Industries Power the Pacific Rim: Sixth Conference of the Asia Pacific Confederation of Chemical Engineering; Twenty-first Australasian Chemical Engineering Conference; Official Proceedings of Combined Conference 1993

### Gas Turbine Cogeneration in the Australian Economic Environment

Poole, ML1; Prince, RGH2

Abstract: The viability of a cogeneration project is affected by a number of factors. A model has been developed to analyse the economics of retrofitting cogeneration to sites with suitable heat and power demands. The model compares the cost of operating the gas turbine cogeneration plant with the cost of generating the heat in gas fired package boilers and purchasing the electricity from the local Council, and calculates a simple payback period for the cogeneration unit. 

The model allows optimisation of the electrical size, the heat to power ratio and the operating hours of a gas turbine powered cogeneration installation so as to minimise the payback period. The optimum arrangement depends on the buyback arrangements available for surplus electricity, and for different buyback prices there are widely different solutions.

What is VHTR Cogeneration System ?

1. Why Nuclear Heat Application ?

Most of the energy supply in our country comes from combustion of fossil fuels. Nuclear power plants emit almost no CO2 and its amount is less than that of hydraulic power generations and solar energy generations.

Though nuclear power plants in Japan is currently limited to electrical applications, it can also contribute to the non-electrical application and to prevent global warming. Extension of the nuclear application fields can reduce the excessive dependence on fossil fuels imported from foreign countries. From the point of energy security, nuclear heat application is also important. VHTR is the most qualified candidate for the nuclear heat application.

2．Advantages of the cogeneration system

(1) High Power Generation Efficiency (For Power Generation)

Since VHTR can produce high temperature gas around 1000deg C, its heat can apply to most of the industry fields. Its gas turbine system can generate electricity at an efficiency of 50 % （Existing nuclear power plant : 33%）.

(2) Heat Application for Various Industry Fields (For Industrial Use）

VHTR can produce heat or electricity to chemical complexes because of its various temperature range usage. VHTR also able to be constructed close to the point of demand due to its inherent safety. Replacement of the 1000MW grade oil-fired power plant by nuclear power plant can reduce 10% of reduction targets on greenhouse gas emissions (About 65Mt) based on energy saving law.

(3) Contribution to Ecological Hydrogen Society (For Transportation)

For the protection of the global environment, it is estimated that fuel vehicles will be disseminated widely. Since VHTR can supply hydrogen to 180,000 fuel cell vehicles per one unit, it can contribute to the hydrogen society in the near future.

(4) Contribution to Locality (For Civilian use）

In low temperature ranges the heat can be used for agricultural and fishery field such as aqua firming, greenhouse cultivation, etc. civilian use such as district heating, and desalination. Energy demand for civilian use makes up 28.7% (FY2001) of the whole energy demand and 50% of them is used for residual heating and cooling. Since VHTR can be constructed close to the point of demand due to its inherent safety, its heat can be used for various heat applications such as hydrogen production, district heating, etc., and able to contribute to the regional revitalization and establishment of new industries.

1. A cogeneration system suitable for residential applications, comprising:

A. an engine for providing both heat and motive power,

B. a generator coupled to said engine for providing electric power,

C. electric heating means for providing additional heat, comprising a first
electrical heat source and a second electrical heat source, and

D. controller means for controlling the energization of said first and
second electrical heat sources during a given period in accordance with
measured operating characteristics of said system during a defined prior
time interval which is representative of overall system energy demand for
a next operating time period, for applying the measured operating
characteristic to control energization of the electric heating means for
the next operating time period by computing a duty cycle band value
indicative of the measured operating characteristic and computing a first
delay value and a second delay value based upon said duty cycle band,
wherein each of said delay values is indicative of a time delay before
said first and second electrical heat sources, respectively, will be
energized.

2. A cogeneration system according to claim 1 in which said electric
heating means is connected for energization by said generator to the
extent that said generator provides electrical output in excess of the
electrical load imposed by said electric heating means.

3. A cogeneration system according to claim 2 in which said electric
heating means is connected for energization by an electric utility to the
extent that said generator provides electrical output less than the
electrical load imposed by said electric heating means.

4. A cogeneration system according to claim 1 in which said first and
second heating sources are energized by said controller in accordance with
separate control cycles.

5. A cogeneration system as in claim 1 wherein said measured operating
characteristics consist of a measurement of the accumulated system
operating time during the selected prior time interval which occurred
during the previous day.

6. The cogeneration system of claim 1 wherein the relationship between said
duty cycle band and said first delay value is characterized by a generally
linear paradigm, while the relationship between said duty cycle band and
said second delay value is characterized by a generally geometric
paradigm.

7. A cogeneration system suitable for residential applications, comprising:

A. an engine for providing both heat and motive power,

B. a generator coupled to said engine for providing electric power,

C. e

ectric heating means for providing additional heat, said electric
heating means comprising first and second heating means independently
actuable, and

D. a controller controlling the energization of said electric heating means
during a given period in accordance with measured operating
characteristics of said system during a selected prior time interval, in
which said controller energizes said electric heating means as a function
of the duty cycle of said system during said time interval, in which said
first heating means is energized by said controller in accordance with a
generally linear paradigm when said system is in a duty cycle state in
excess of a first state, and said second heating means is energized in
accordance with a generally geometric paradigm when said system is in a
duty cycle state corresponding to a second state.

8. In a cogeneration system having an engine for generating heat and an
induction generator driven from said engine, the improvement comprising
means for operating said generator as a motor, comprising

A. an auxiliary winding in said generator,

B. means for switching alternating current through said auxiliary winding
in a first phase to operate said generator as a motor, and means for
switching current through said auxiliary winding in a second phase to
operate said generator as a generator.

9. A supplemental heating system in which a controller energizes an
electric heating means as a function of the duty cycle of a primary
heating system during a prior time interval, and said electric heating
means comprises first and second heating means independently actuable, and
said first and second heating means are energized by said controller in
accordance with separate control cycles,

in which said first heating means is energized by said controller in
accordance with a generally linear paradigm when said system is in a duty
cycle state in excess of a first state, and said second heating means is
energized in accordance with a generally geometric paradigm when said
system is in a duty cycle state corresponding to a second state.

10. A cogeneration system suitable for residential applications,
comprising:

an engine for providing heat and motive power;

a generator coupled to said engine for providing electric power;

an acoustical noise attenuating housing within which said engine and
generator reside, wherein said housing attenuates acoustic noise
originating within said housing to reduce the level of noise external to
said housing; and

a thermal barrier located within said housing and disposed between said
engine and said generator to establish a thermal barrier between said
engine and said generator.

### Keywords

Co-generation, Safety, Allocation, Factors

### Abstract

The present work analyses the most important safety allocation techniques available in literature. Subsequently, we have proposed and validated a new method, called: Integrated Dangers Method (IDM). The study starts from the application of the above techniques to a co-generation system. In particular, the new method considers a big number of factors able to assure its applicability to a wide variety of systems. The chosen input data is the “Not Safety”, corresponding to the number of undesired events. The proposed method is characterized by a very simple analytical formulation. By comparing the allocation results of the new technique with other techniques and with historical data of the system, we have appreciated its validity. The new method is able to adapt the available methodologies to the different design phases, according to a correct analysis of RAMS parameters.

Cogeneration is the simultaneous generation of electricity and heat power, in a single process from the same fuel.  Cogeneration is not new.  Thomas Edison's first electric generating plant used combined heat and power generation.  The principal of co-generation is simple.  Conventional power generation on average is only 35% - 40% efficient.  Up to 65% of energy potential is released in waste heat.

Out of 100% of the BTUs in diesel fuel only a bit more than 35% is converted to electricity.  35% is expelled out of the radiator as heat and the remaining 30%+ is heat lost through the exhaust and radiant heat.   A 12kW genset can produce up to 22,500 BTUs of recoverable heat per hour from the radiator when running under load.

A cogeneration system, as we build them, consists of a engine which powers your generator to produce electricity and a waste heat recovery system to capture heat from the water cooling system and exhaust. Energy efficiency can be more than doubled with co-generation.

Until recently cogeneration implementation has been limited to large industrial applications and commercial facilities. Our implementation of cogeneration would be termed as micro cogeneration or micro CHP (Combined Heat and Power) production.

Harvesting heat before it is expelled from the radiator is the simplest and easiest method of micro cogeneration.  This is because it doesn't affect the flow of water and air through the radiator and it does not affect engine performance.  Because water cooled engines manage their heat at the thermostat you can't take too much heat from the engine by tapping heat beyond the thermostat.  Harvesting heat directly from the engine block is also safe needing only a heat sensor to avoid taping too much heat and it is inexpensive.

Depending on your application, the integration of power and heat production into one on-site cogeneration system can quickly generate savings of up to 35%. The more heat energy you are able to utilize the more you can save.  The best candidates for co-generation are those that can utilize the most heat energy.  From owners of factories to remote camps, clients are taking advantage of co-generation.  Thermal energy is most frequently used for space and water heating, however commercial applications for the use of heat energy abound.

Our longest running and most durable generators units are powered by Perkins, Lombardini, Cummins and Kubota diesels.  You can expect at least 30,000 hours before requiring a rebuild.  That's three and a half years of continuous running 7/24.  We manufacture generators using these engines and many others.

### Our co-generation solution:

### We’ve developed a solution which:

- Is easy, simple and requires little or no maintenance.

- Does not rely on additional infrastructure for safety or controls.

- Will not damage the generator if other downstream systems malfunction.

- Is not dependant on downstream systems (will not cause malfunction or shutdown if there is a downstream leak or a circulating pump fails).

- Is extensible:  The heat exchangers, spliced into the coolant line, remain in place and can operate without damage (brazing problems, metal stressing...) even if there is NO water being pumped into them on the secondary side.  Of course if you allow the secondary system to freeze damage will be done to the Generator's heat exchange system.

- Is able to use normal and proven, simple equipment (regular household hot water heaters can store the heat).

- Is able to produce the greatest return on your investment.

Our system with a high efficiency heat exchanger within the coolant circulation system is all of the above.  If sufficient heat is not removed from the water within the heat exchanger to your heat reservoir, the excess heat is safely pumped out of the radiator into the air.  Circulation and heat management are already in place as is shutdown protection should heat be too great within the engine (standard on all of our generators).

You must design your own secondary system with safety systems within the secondary system  because input heat is at 160-180 deg F and must be managed carefully.  If your system doesn't have capacity or isn't sufficiently protected for pressurization something could pop, even if the temperature is not at boiling point.

To maximize heat capture for cogeneration and get more energy out of the generator the regular engine mounted fan can optionally be replaced with an electric fan and separate heat sensor on the radiator.  This may provide as much as 1-1.5kW additional power from the engine/generator because there’s no fan for the engine to push.  This additional modification will also “save” more heat for co-generation, not expelling it into the atmosphere until safety and engine heat management require additional heat dissipation.  This modification includes an electric fan replacement and sensor/relay installation within the radiator.

The manifold and engine casing depend on airflow for some cooling.  The alternator cooling fan pushes a significant amount of air.  Unless the generator is located in a small enclosed space, sufficient air movement is created, avoiding problems.

### Heat capture from the exhaust system

### The exhaust system dissipates of almost 30% of the BTUs  burned within a generator.  This heat would otherwise also be lost.  The BTU from exhaust gas for a 12kW diesel genset running at load is approximately 14,000 BTU/hr.

### Design Considerations:

- Requires a separate circulation system for both heat capture and secondary system.

- Requires management of excess heat not absorbed by the heat sink.

- Requires additional heat sensing, heat overload safety shutdown (otherwise a steam explosion could result).

- If too much heat is extracted before the muffler a (smelly) tar like residue may be condensed within the muffler as exhaust gas condenses and mixes with hydrocarbon residue.

This system, while not rocket science, is not fool proof primarily because safety systems for heat dissipation must be incorporated.  It is not as cheap and easy as capturing heat from the engine coolant system but it provides additional “free BTUs” of heat for a relatively small investment.

This second heat capture system must be kept separate from the heat capture system from the coolant system because the radiator and fan capacity supplied is insufficient to remove this additional heat through the radiator should it be required to do so.

We have water cooled exhaust manifolds for several engines and can have a water cooled manifold designed for any engine available.  Double wall, water cooled exhaust stacks and pipes are also available.

### Heat Utilization and Storage

We like to keep things as simple and inexpensive as possible.  Exotic systems and technology require longer pay-back times.  For example, heat storage can be as simple as a one or a series of household water heating tanks if you want to start of as simply and cheaply as possible.

Cogeneration can reduce diesel generation costs down to the current cost of grid electricity. We invite you to call us to assist you in making your electrical generation solution more economical.

### Fuelcell Based Cogeneration (Combined Heat and Power) Systems.

### A bridge to a hydrogen-powered world.

If you study your gas and electricity bills you will see there is a big difference between the energy cost of gas and electricity. For example in March 2000 the price quoted by Powergen for gas was 1.235 pence / kWh. and for electricity 6.08 pence / kWh. the difference between the two being 4.845 pence / kWh.

If it were possible to generate electricity easily and quietly from natural gas at home then this difference in price would be available to pay the capital cost of the device to do so. Such a device could be a fuelcell running on natural gas.

The annual average electrical power demand for a small house is 1 to 2 kW per hour.

If say 0.5kW per hour of this electrical energy requirement was a base load to be provided by a fuelcell running on natural gas then the energy provided per year would be 0.5 x 365 x 24 = 4,380 kWh and the difference in cost would be 4,380 x 4.845 / 100 = £212 per year. If the fuelcell had a life of at least 5 years this would generate available capital of £1060 for a fuelcell based system with a power of 0.5 kW which is equal to £2120 / kW of fuelcell capacity. Over the next year or two in the USA domestic fuelcell systems are coming to market in this price range. The proposed fuelcells will last longer than 5 years and so should provide a cheaper source of electricity for the homeowner over the lifetime of the fuelcell.

At first impression it may not seem worthwhile to go to all the trouble of installing fuelcells in homes if it isn't going to save much money for the homeowner, but saving money is not the main attraction. It is the possibility of reducing CO2 output and conserving gas supplies that makes domestic fuelcells worth considering. The proposed fuelcells will be very efficient converters of gas into energy because they will be cogeneration units in which waste heat from the electricity generation part of the process is used to heat domestic hot water and for central heating of the home. Also the electricity is generated in the home and so there are none of the transmission losses associated with central generation.

It should be possible to achieve total energy conversion efficiencies that are 50% higher than the average efficiency achieved by gas fired power stations generating electricity centrally where the waste heat is lost and energy is lost in transmission. So from a given amount of gas 50% more energy is produced which means that the amount of CO2 produced per unit of energy is reduced by 33%. This assumes that the fuelcell runs all the time which might require reversible electricity metering so that surplus electricity can be fed to the local electricity grid

The 50% improvement in efficiency of energy transformation that could be achieved by domestic fuelcells is derived as follows:

The best combined cycle gas turbines (CCGTs) now used at gas-powered central electricity generating stations achieve an average energy conversion efficiency of 65% - assuming continuous operation. However, not all gas-fired generating plants are CCGTs and continuous running is not always available so assume an efficiency of 60%.

Domestic fuelcells in a cogeneration unit producing base load electricity (i.e. 24 hrs per day running to supply the basic needs of the house, and export surplus to the local grid) and domestic hot water and central heating will achieve an energy recovery of 85%.

Because there are no long distance electrical transmission losses, there is a further gain in conversion efficiency of, say, 6%.

The overall improvement in energy conversion efficiency is therefore:

If we now look at the bigger picture an interesting possibility emerges. Excluding Sizewell B the total generating capacity of the UK nuclear power stations is approximately 10 Gigawatts and all of this plant is going to close in the next ten to twenty years. This generating plant will have to be replaced and to continue to meet our need to reduce CO2 emissions it will have to be replaced with generating plant that emits no CO2.

The total generating capacity of all the UK gas fired power stations already built or in the process of being built is over 20 Gigawatts. If the natural gas these power stations burn was used in domestic fuelcells as described above then 50% more energy could be extracted from the gas without producing any more CO2. Now 50% of 20 Gigawatts is 10 Gigawatts, so we could replace all the electrical energy we currently get from nuclear power and gas-fired power stations by switching the gas from gas fired power stations to domestic fuelcells as the nuclear power stations close.

To achieve this relocation of generation capacity from centralised power stations to distributed domestic generation will require 20 Gigawatts of fuelcell capacity being installed.

The example given at the beginning of this section assumes a domestic baseload of 0.50 kW to be supplied by a fuelcell in each home.

Assuming 20 million homes, this gives a total installed fuelcell capacity of 10 Gigawatts. But it would be possible to accommodate more fuelcell capacity in homes if reversible electricity metering were introduced (i.e. the home owner received a credit at the retail price for all electricity returned to the grid) and if heat storage systems were incorporated in the home to absorb surplus heat for later use.

The domestic fuelcell capacity is only limited by the demand for heat in the home. The improved efficiency and energy saving given by the fuelcell is achieved by using the heat that would be wasted if the electricity generated by the fuelcell were generated at a remote power station.

It may be the case that, due to global warming, the demand for air conditioning in the UK will increase. Using absorption chillers, the heat from the domestic fuelcell could be used to drive a domestic air conditioning system which would absorb surplus fuelcell heat in summer. This approach is already used to achieve 85% energy recovery in centralised combined heat and power systems based on diesel engines or gas turbines.

The point we are making is that, with lateral thinking and imagination, we can move towards much more efficient energy conversion systems that will also establish the infrastructure we will need when natural gas runs out and we have to move to a hydrogen-powered world.

Just as the nuclear power stations will all be closed in the next 10 to 20 years, all the gas-fired power stations will be paid for and worn out during the same 20 year period. So, as the gas stations are closed, the gas becomes available for the domestic fuelcells and the extra 50% of energy utilisation makes up the energy supply losses from the closing nuclear stations. It could all be synchronised to effect an orderly change over a twenty year period. It does not matter that some of the 50% extra energy from the gas is made available as heat whereas the nuclear energy it replaces is in the form of electricity. The short fall in electricity could be made up using renewable electricity generation such as offshore wind powered generators and the extra heat available displaces other gas burning for domestic central heating.

In the longer term there is going to be a shortage of natural gas but the above change will not aggravate the shortage because no additional natural gas is required. The proposals will however help prepare us for when we do run out of natural gas because the gas supplied to the domestic fuelcells can be enriched with hydrogen made from renewable electricity. In due course areas of the country could be changed over to 100% hydrogen.

In this way the whole country would go over to hydrogen as the natural gas supplies were exhausted.

The changeover described here from today's nuclear and gas powered centralised electricity generating stations to decentralised domestic hydrogen-powered-fuelcell cogeneration systems could all be achieved over a 30 year period starting now in step with existing nuclear plant reaching the end of it's useful life and so avoid the waste of valuable capital assets. Also the proposed change over is in step with the likely run down in economic natural gas supplies for the UK and provides security of energy supplies without relying on nuclear power which could be phased out in an orderly manner.

To achieve the above scenario will require urgent development of fuelcells for domestic cogeneration applications. We can do this but it will require a greater UK commitment of funds for fuelcell research and development than is currently the case.

The UK market for 20 Gigawatts of domestic fuelcells would be worth £40 billion, the European market for domestic fuelcells will be worth £200 billion.

# 3. a Prime Movers

Cogeneration plant consists of four basic elements: - A prime mover (engine); - An electricity generator; - A heat recovery system; - A control system.

Depending on the site requirements, the prime mover may be a steam turbine, reciprocating engine or gas turbine. The prime mover drives the electricity generator and waste heat is recovered. The basic elements are all well established items of equipment, of proven performance and reliability.

### Prime Movers

Cogeneration units are generally classified by the type of prime mover (i.e. drive system), generator and fuel used. The following sections examine the main types.

### Steam Turbines

Steam turbines have been used as prime movers for industrial cogeneration systems for many years. High-pressure steam raised in a conventional boiler is expanded within the turbine to produce mechanical energy, which may then be used to drive an electric generator. This system generates less electrical energy per unit of fuel than a gas turbine or reciprocating engine-driven cogeneration system, although its overall efficiency may be higher, achieving up to 84% (based on fuel gross calorific value).

For viable power generation, steam input must be at a high pressure and temperature. The plant is capital intensive because a highpressure boiler is required to produce the motive steam. At existing sites, where steam systems are supplied by low-pressure boilers, it will be necessary to replace these boilers with highpressure plant, possibly retaining the original equipment as stand-by.

Steam cycles typically produce a large amount of heat compared with the electrical output, resulting in a high cost installation in terms of Euro/kWe.

### Gas Turbines

The gas turbine has become the most widely used prime mover for large-scale cogeneration in recent years, typically generating 1-100 MWe. A gas turbine based system is much easier to install on an existing site than high-pressure boiler plant and a steam turbine. On many sites plot space is at a premium, a factor weighing heavily in favour of gas turbines. This, together with reduced capital cost and the improved reliability of modern machines, often makes gas turbines the optimum choice.

The fuel is burnt in a pressurised combustion chamber using combustion air supplied by a compressor that is integral with the gas turbine. The very hot (900ºC- 1200oC) pressurised gases are used to turn a series of fan blades, and the shaft on which they are mounted, to produce mechanical energy. Residual energy in the form of a high flow of hot exhaust gases can be used to meet, wholly or partly, the thermal demand of the site.

The available mechanical energy can be applied in the following ways:

- to produce electricity with a generator (most applications);

- to drive pumps, compressors, blowers, etc.

A gas turbine operates under exacting conditions of high speed and high temperature. High-premium fuels are therefore most often used, particularly natural gas. Distillate oils such as gas oil are also suitable, and sets capable of using both are often installed to take advantage of cheaper interruptible gas tariffs.

Waste gases are exhausted from the turbine at 450oC to 550oC, making the gas turbine particularly suitable for high-grade heat supply. The usable heat to power ratio ranges from 1.5:1 to 3:1 depending on the characteristics of the particular gas turbine. Supplementary firing may be used to increase exhaust gas temperatures to 1,000oC or more, raising the overall heat to power ratio to as much as 10:1.

Supplementary firing is highly efficient as no additional combustion air is required to burn extra fuel. Efficiencies of 95% or more are typical for the fuel burned in supplementary firing systems.

Exhaust gases can be used in either of the following ways:

- For direct firing and drying processes. The single flow of heat at high temperature is suitable for processes in which direct contact with combustion gases is permissible.

- To raise steam at medium or low pressure for process or space heating in an open-cycle gas turbine cogeneration plant which comprises a gas turbine-alternator unit and a heat recovery boiler.

- To generate hot water, best for high temperature hot water applications where temperatures in excess of 140°C are required.

- To raise steam in a HRSG at high pressure for use in a steam turbine

Gas turbines are available in a wide power output range from 250 kWe to over 200 MWe, although sets smaller than 1 MWe have so far been generally uneconomic due to their comparatively low electrical efficiency and consequent high cost per kWe output. This is starting to change. The turbine is typically mounted on the same sub-base as its generator, with a stepdown gearbox between the two to reduce the high shaft speed of the turbine to a speed suitable for the generator. A gas turbo-generator is extremely noisy and generally housed in an acoustic enclosure for noise attenuation.

### Reciprocating Engines

The reciprocating engines used in cogeneration are internal combustion engines operating on the same familiar principles as their petrol and diesel engine automotive counterparts. Although conceptually the system differs very little from that of gas turbines, there are important differences. Reciprocating engines give a higher electrical efficiency, but it is more difficult to use the thermal energy they produce, since it is generally at lower temperatures and is dispersed between exhaust gases and cooling systems.

The usable heat:power ratio range is normally in the range 0.5:1 to 2:1. However, as the exhaust contains large amounts of excess air, supplementary firing is feasible, raising the ratio to a maximum of 5:1.

#### Compression-ignition ('diesel') engines

Compression-ignition ('diesel') engines for large-scale cogeneration are predominantly four-stroke direct-injection machines fitted with turbochargers and intercoolers. Diesel engines will accept gas oil, HFO and natural gas. The latter is in reality a dual-fuel mode, as a small quantity of gas oil (about 5% of the total heat input) has to be injected with the gas to ensure ignition; as the engine can also run on gas oil only it is suited to interruptible gas supplies. Cooling systems are more complex than on spark-ignition engines and temperatures are often lower, typically 85oC maximum, thereby limiting the scope for heat recovery. Exhaust excess air levels are high and supplementary firing is practicable. Compression-ignition engines run at speeds of between 500 and 1500 rev/min. In general, engines up to about 500 kWe (and sometimes up to 2 MWe) are derivatives of the original automotive diesels, operating on gas oil and running at the upper end of their speed range. Engines from 500 kWe to 20 MWe evolved from marine diesels and are dual-fuel or residual fuel oil machines running at medium to low speed.

#### Spark-ignition engines

Spark-ignition engines are derivatives of their diesel engine equivalents and have their same parameter equivalents as 90°C cooling water.

Traditionally, shaft efficiency has been lower than for compression ignition engines. The output of a spark-ignition engine is a little smaller, typically 83% of the diesel engines.

They are suited to smaller, simpler cogeneration installations, often with cooling and exhaust heat recovery cascaded together with a waste heat boiler providing medium or low temperature hot water to site.

Spark-ignition engines operate on clean gaseous fuels, natural gas being the most popular. Biogas and similar recovered gases are also used but, because of their lower calorific value, output is reduced for a given engine size. Spark-ignition engines give up less heat to the exhaust gases than diesel engines. The large lean-burn engines have typically 12% oxygen in exhaust gases, and this can be used with supplementary firing. The following are among the most common applications for the thermal energy produced by reciprocating engines:

- production of up to 15 bar steam utilising the heat of exhaust gases; and separate production of hot water at 85-90oC from the cooling system of the engine;

- production of hot water at 90oC, supplementing the temperature of cooling system water with heat from the gases;

- Exhaust fumes can be used directly in certain processes, such as drying, CO2 production, etc;

- generation of hot air. All the residual energies from the engine can be used, through the installation of suitable exchange devices, for the generation of hot air.

Reciprocating machines by their nature have more moving parts, some of which wear more rapidly than those in purely rotating machines, and have running as well as shutdown maintenance requirements. Nevertheless, typical availability is about 90-96%.

Gas engines are operated under two distinct air/fuel ration regimes that have a market effect upon environmental performance:

- Stoichiometric engines;

- Lean-burn engines.

NOx emissions can be reduced markedly by operating with large excess of combustion air (lean-burn). However, this has an adverse effect upon the engine’s power output.

Stoichiometric engines tend to be smaller (typically <300 kWe) than their lean-burn counterparts and are based upon standard vehicle engine blocks with adapted cylinder heads and spark ignition systems.

Cogeneration diesel plants HFO systems have been built in those places where gas is not available. This includes many islands and developing countries. In places where gas availability will arrive later, the plants can use HFO at the beginning and later switch to gas, or use HFO in winter and gas in summer.

| #### Type | #### Output Range | #### Typical Fuels | #### Typical Heat to Power Ratio | #### Grade of Heat Output |
| --- | --- | --- | --- | --- |
| #### Gas Turbine | 0.5MW upwards | Natural gas, Gas Oil, Landfill gas, Biogas, Mine gas | 1.6:1 up to 5:1 with after firing | High |
| #### Compression ignition gas engine | 2MW upwards | Natural gas with gas oil, Heavy fuel oil | 1:1 to 1.5:1 up to 2.5:1 with after firing | Low and High |
| #### Spark Ignition | Up to 4MW | Natural gas, Landfill gas, Biogas, Mine gas | 1:1 to 1.7:1 | Low and High |
| #### Steam Turbine | 0.5MW upwards | Any, but converted to steam | 3:1 to 10:1 | Meduim |
| #### CCGT | 10MW upwards | As gas turbine | Down to 0.7:1 | Meduim |

### Combined Cycles

Some large systems utilise a combination of gas turbine and steam turbine, with the hot exhaust gases from the gas turbine being used to produce the steam for the steam turbine. This is called a combined cycle.

Gas turbine combined cycle (CCGT) systems have been adopted by public utility companies where supplies of natural gas are plentiful: power stations of up to 1,800 MWe have been constructed. In cogeneration applications of the CCGT, exhaust or pass-out steam from the steam turbine is used for process or other heating duties. The main advantage of CCGT cogeneration is its greater overall efficiency in the production of electricity.

### Waste heat recovery units

The heat recovery boiler is an essential component of the cogeneration installation. It recovers the heat from the exhaust gases of gas turbines or reciprocating engines. The simplest one is a heat exchanger through which the exhaust gases pass and the heat is transferred to the boiler feedwater to raise steam. The cooled gases then pass on the exhaust pipe or chimney and are discharged into the atmosphere. In this case, the composition or constituents of the exhaust gases from the prime mover are not changed. The exhaust gases discharged, contain significant quantities of heat, but not all can be recovered in a boiler. One typical feature of the exhaust heat boiler (or waste heat recovery unit) is that the typical size is bigger than a conventional fuel-burning unit. This is for two main reasons:

- The lower exhaust gas temperatures require a greater heat transfer area in the boiler;

- There are practical limitations on the flow restriction.

Excessive flow resistance in the exhaust gas stream must be avoided as this can adversely affect operation of the turbine or engine.

Exhaust heat boilers are not, therefore,‘offthe- shelf’ items: they need to be designed for the particular exhaust conditions of the specified turbine or engine. The usual procedure is to provide the boiler supplier with details of the exhaust gas flow from which the heat is to be recovered, and with the temperature and pressure conditions of the required heat output. The boiler supplier will then be able to advise on the quantity of heat that can be recovered, and the temperature at which the exhaust gas will be discharged from the boiler.

The choice of prime mover is based on a number of factors and even with similar energy requirements, no two sites are the same.

The critical factor is the Heat to Power ratio of site demand. Where the electrical power requirement is relatively high as a proportion of total energy this tends to favour engines. Conversely where heat demand is typically more than 3 or 4 times electrical demand the turbine begins to have an advantage. Another key factor is the quality of heat required by the customer site. Some industrial processes have little use for low grade heat – the hot water produced in enginesbased schemes. Where high temperature steam is the primary heat requirement then the turbine is clearly superior.

A method of driving a large cooling system having a compressor operating as a chiller driven by a prime mover, such as a gas turbine, reciprocating engine or steam turbine, in conjunction with an electric motor/generator coaxially arranged on a common shaft with the other units, so that as the ambient temperature rises and the power needed to produce the cooling output increases, and as there is a corresponding drop in power output if the prime mover is a gas turbine or condensing steam turbine, the required additional driving power is supplied by the motor. Conversely, as the ambient temperature drops and required cooling power lessens, the electric motor operates as a generator to convert the excess prime mover power to electrical power.

Co-generation system control

BACKGROUND OF THE INVENTION

A. FIELD OF INVENTION

The invention relates to cogeneration systems, and particularly to
residential cogeneration systems although it is not limited thereto.

B. PRIOR ART

Cogeneration systems are used to provide both heat and electrical power.
Such systems offer potential savings to large consumers of electricity,
and are used most commonly in a commercial setting. It has occasionally
been proposed to use cogeneration systems in a residential environment,
but numerous problems such as safety concerns, noise, maintenance issues,
lower efficiency of smaller units and the like concerns have greatly
limited the actual implementation of such systems.

OBJECTS OF THE INVENTION

A. Objects

Accordingly, it is an object of the invention to provide an improved
cogeneration system.

Further, it is an object of the invention to provide an improved
cogeneration system that is compact and efficient and is particularly
suited for residential applications.

Yet a further object of the invention is to provide an improved controller
for cogeneration systems.

B. BRIEF DESCRIPTION OF THE INVENTION

In accordance with the preferred embodiment of the present invention
described herein, a cogeneration system comprises a prime mover in the
form of a diesel engine coupled to an induction generator. A heat-transfer
agent such as water is passed through a heat exchanger through which the
engine exhaust gasses are removed, and is thereafter passed through the
engine block to provide cooling therefor while at the same time receiving
further heat therefrom. A supplemental electric heater is positioned to
supply further heat to the water when necessary. The heated water is
thereafter conducted to the space to be heated.

The induction generator generates electricity whenever the engine is
operating. The electricity thereby generated is made available for
operating the electrical systems of the facility in which it is located.
During periods in which the engine is operating for a significant portion
of the day (e.g., during colder weather), an excess of electricity is
generated. This excess is fed back into the electric utility lines, and
the utility then credits the facility with the appropriate amount of
generation.

In accordance with the present invention, the engine and the generator are
located in separate, thermally isolated compartments. This allows the
engine compartment to run at a relatively high temperature (on the order
of 160 degrees Fahrenheit) and greatly improves the heat transfer to the
water and thus

the efficiency of the system.

A controller for the system further enhances the efficiency of the system
by allowing the use of a system of smaller generated thermal capacity than
commonly provided, which is then run at higher duty cycle. Further, unlike
many systems which use outdoor air temperature as the reference for
setting operating points, the present system is referenced to the
particular usage characteristics of the facility in which it is installed.
Thus, it can provide the necessary heating at relatively high efficiency.

In particular, in the present invention, the usage of the supplemental
heater is tightly controlled to ensure maximum economy while providing
sufficient heat to maintain the desired level of comfort. This is
accomplished by measuring the accumulated system operating time during a
defined period and setting the operating characteristics of the system for
the next day in accordance with the measured result. In the preferred
embodiment, the defined period is from 7:00 PM to 11:00 PM. The percentage
of time during which the engine operates during this period is determined,
and the controller then sets the system to one of a specified number of
operating "bands" for the next day's operation. For example, we utilize a
16-band operation for residential systems. If it is found that the engine
operates for a total of two and one-half hours out of four during the 7-11
PM measuring period, the controller sets the system to operate in band 10
( (2.5/4)*16=10) for the following day. Each band defines a distinct delay
time in each cycle of engine operation before the supplemental heater is
turned on. In general, the lower the band number, the longer the delay
before turning on the supplemental heater.

Further in accordance with the present invention, the supplemental heater
itself is divided into two sections, operable independently. The
energization of the two sections is controlled in accordance with two
different rules so as to further optimize the efficiency and economy of
the present system. In the preferred embodiment described herein, the
energization of the first ("locally generated power") section is performed
in accordance with a linear characteristic, while the energization of the
second ("purchased power") section is performed in accordance with a
geometric characteristic. The supplemental heater is connected to receive
power from the local generator as well as from the utility. The system is
structured such that, for a large portion of the time, the thermal demands
on the system will be satisfied by the engine alone, and neither heater
section will be energized. As greater thermal demands are imposed on the
system, the first heater will be energized and, if the demands continue,
the second heater will also be energized. Initially, all the power for the
heaters will come from the local generator. When the capacity of this unit
is exhausted, the remaining power necessary to satisfy the thermal demands
will be drawn from the utility. Provisions are made to manually override
the settings when desired so that, for example, additional heat can be
made available for above average heating or hot water needs.

Yet a further aspect of the present invention is a control circuit for the
generator which allows its efficient operation as either a motor or as a
generator. Thus, the engine of the present system can be started with the
generator, without storage batteries, chargers, or separate starting
motors. In particular, the generator comprises an induction generator
having the usual main winding and a switchable auxiliary winding that
enables its function as a motor as well. The auxiliary winding is switched
into circuit in a first polarity to run the generator as a starting motor
for the engine, and is thereafter switched into circuit with a second,
reverse polarity to run the generator in its normal generator mode.

DETAILED DESCRIPTION OF THE INVENTION

The foregoing and other and further objects and features of the invention
will be more readily understood from the following detailed description of
the invention, when taken in conjunction with the accompanying drawings,
in which:

FIG. 1 is block and line sketch of a cogeneration system constructed in
accordance with the present invention;

FIG. 2 is a circuit diagram of the generator/motor circuit of the present
invention; and

FIG. 3 is a graph showing the control paradigm for the supplemental heaters
in accordance with the present invention.

Turning now to FIG. 1, a cogeneration system 10 in accordance with the
present invention comprises a housing 12 divided into thermally-separated
compartments 14, 16, respectively, by a thermal barrier 18. An engine 20
is located in compartment 16, and a generator 22 is located in compartment
14. A conduit 24 supplies outside cooling air to generator 22. The air
flows downwardly to cooling vents 22a in generator 22. After passing
through the generator, it exits the system 10 by way of a plenum 26.

Air from inlet 24 is also fed to engine 20 via a conduit 30. Exhaust air
from the engine 20 is discharged into a pipe 32, an expansion chamber 34,
a pipe 36, and thence into a heat exchanger 38. Heat exchanger 38 is
preferably a conventional multi-tube element in which the heating fluid is
carried within a first set of tubes within the exchanger shell and the
fluid to be heated is carried in countercurrent direction outside the
tubes but within the shell. Such a construction is well known and thus
will not be illustrated in detail. The exhaust gas is discharged from the
heat exchanger into a muffler 40, and is then carried to the exterior of
the installation for discharge into the atmosphere by means of pipe 42. An
external oil reservoir 44 is connected to the engine via pipes 46 and 48.

Water to be heated is supplied to the system via a pipe 50 which connects
to the heat exchanger 38 where heat is transferred to the water as it
transits the exchanger. The heated water exits the heat exchanger via pipe
52 which carries it into the engine for further collection of heat. The
heated water is discharged from the engine into a pipe 54 which carries it
into the residential area to be heated. A supplemental heater in the form
of a dual-section electric resistance heater 56 having first and second
windings 57a and 57b, respectively, is connected via independently
operable switches 59a and 59b, respectively, to the generator 22 as well
as to the utility lines for energization by either or both. When
energized, the heater 56 provides additional heat input to the water in
pipe 50.

In accordance with the present invention, the engine 20 is a diesel engine,
having a power rating on the order of 11 horsepower (and a thermal output
of approximately 50,000 BTU) for a typical residential home in the
northeastern sector of the United States, although the exact capacity will
be selected in accordance with the size of the area to be heated, the
specific needs of the inhabitants (e.g., a larger family may require more
water for washing, etc.), and the area of the country. Such a capacity is
considerably less than conventional residential heating wisdom teaches is
necessary for ensuring adequate heat during periods of high thermal
demand, such as continued cold weather or high demand for water for
washing, etc. However, by using an engine of this size, we are able to
operate it with a higher duty cycle than is common in residential heating
systems, and thereby considerably improve heating efficiency.
Correspondingly, the generator 22 preferably has a rating on the order of
5 kilowatts. This is generally sufficient to supply the entire electric
energy requirements of a typical single-family residence during the
heating season and, additionally, provide further capacity for
supplemental heating when required.

External oil reservoir 44 has a substantially greater oil capacity than
engine 20 and, because it is located external to the engine, has a
significantly lower ambient temperature than that within the engine block.
For example, for an 11 horsepower system, the engine 20 typically has an
oil capacity of less than a gallon, while the external reservoir has a
capacity in excess of ten gallons. By recirculating the oil through the
external reservoir while the engine is on, the average oil temperature is
considerably decreased and its life, and thus the life of the engine, is
considerably prolonged, thereby facilitating the utilization of the
cogeneration system in an environment (residential heating) where it is
not normally used.

A further provision facilitating utilization of the present system in a
residential environment is the use of a resilient muffler of the type
commonly used in marine exhaust systems (e.g., in pleasure boats) for the
muffler 40. Such a muffler is commonly made of a tough but flexible rubber
or plastic material, and we have found that it provides excellent
vibration isolation between the heating system and the external vent pipe
42. We do not believe that mufflers of this type have heretofore been
applied to cogeneration systems, particularly residential cogeneration
systems. Similarly, pipe 42 is preferably of plastic. This again is
uncommon for exhaust discharge systems, particularly cogeneration systems,
but has proven to be of significant benefit in prolonging system life by
resisting the corrosive discharges characteristic of diesel systems.

Thermal barrier 18 provides heat insulation between the compartments 14 and
16. This allows compartment 16 to remain at a relatively high temperature
(e.g., approximately 160 degrees Fahrenheit) during operation of the
engine 20, while allowing compartment 14 to operate at a much lower
temperature (e.g., approximately 100 degrees Fahrenheit). The higher
temperature in compartment 16 enhances the efficiency of heat capture and
transfer to the residential heating water. The lower temperature in
compartment 14 enhances the life of generator 22 while simultaneously
decreasing the amount of cooling air needed from the outside to maintain
the lower temperature and thus further increasing the efficiency of the
system. Indeed, as a result of the system design as described herein, we
have determined that an operating efficiency of approximately 93% has been
achieved.

The system of the present invention operates in response to a heat demand
element such as a thermostat. When the thermostat calls for additional
heat, engine 20 is started and the heat from its operation is transferred
to the circulating water as described above. At the same time, generator
22, which is connected to engine 20, is rotated by the engine to thereby
generate electricity. The electricity so generated is connected to the
household main supply line, which in turn is connected to the utility line
through an electric metering system which measures household electric use
for billing purposes. To the extent that the household demand for
electricity (including the demand for electricity in the supplemental
heater 56) is equal to or less than the output of generator 22, the demand
is met by the generator output and the excess is fed back to the utility
to obtain an electric power generation credit. Conversely, to the extent
that the demand is greater than the output of generator 22, the demand is
met to the extent of this output and the deficiency is made up by drawing
power from the utility.

In accordance with the present invention, generator 22 is an induction
generator. Such a generator has the desirable characteristic that it
inherently adjusts to the utility line voltage and frequency when it is
connected to it. This considerably simplifies the interconnection to the
utility line. Further, having much simpler construction, it has an
inherently greater life than generators of the synchronous type.

In the present invention, generator 22, in addition to generating electric
power, serves as a starting motor for engine 20. FIG. 2 shows how this is
accomplished. Generator 22 has a main winding 100 and an auxiliary winding
102 as is known in the art. The main winding is connected in parallel
across terminals 23a and 23b via a main switch 104 in the usual manner.
However, in accordance with the present invention, the auxiliary winding
102 is connected across main winding 100 by means of single pole, two
position switch 106. The opposite ends of the auxiliary winding 102 are
connected to terminal 23b by a first capacitor 108 and switch 110, and by
a second capacitor 112 and switch 114, respectively.

When heat is demanded from the system, the main controller (described in
more detail below) responds by closing switches 104 and 114 of generator
22. This connects the generator to the main terminals 23a and 23b, and
current is drawn from the utility system. With switch 106 positioned as
shown in FIG. 2, this current flows through winding 102 from left to right
as shown by arrows 116, and thence downwardly through capacitor 112 and
switch 114. Generator 22 then operates as a motor, and turns the shaft of
engine 20 to start it. When the shaft reaches a defined speed, switch 114
is opened, then the pole of switch 106 is switched to terminal 106b, and
switch 110 is also closed. The current through the auxiliary winding then
flows in the reverse direction, from right to left in FIG. 2, and the
generator 22 then begins operation in generator mode.

It will be seen from FIG. 2 that connection of the generator 22 to the
electric utility is accomplished through a single switch. This greatly
simplifies the interconnect, lowers its cost, and helps to make the
present system economically feasible for residential use. Further, the
generator control circuit of the present invention significantly
contributes to the economic construction of the system, as well as its
reliability and maintainability, by eliminating a separate starting motor
and related equipment.

Turning now to FIG. 3, a graph showing the relationship between the
operation of the supplemental heating element and the duty cycle band of
the system is set forth. The vertical axis shows the delay (in minutes)
between the time that a call is generated by the system's heat demand
element (e.g., the thermostat) and the duty cycle band that characterizes
the state of the system at a given time; the horizontal axis indicates the
various bands. For purposes of illustration, the system is shown as
divided into 16 bands, numbered =b 0-15 on FIG. 3, although it will be
understood that a greater of lesser number of bands may be used. Each band
corresponds to the fractional amount of time that the system has operated
during a selected time interval, e.g., during the time period from 7 PM to
11 PM of the preceeding day. A first curve 130 shows the relationship for
the first heater element, coil 57a, while curve 132 shows the relationship
for the second supplemental heater element, coil 57b.

As long as the duty cycle is less than a certain fractional amount, neither
heater is energized. For example, as shown in FIG. 3, as long as the
system duty cycle for the time in question is less than 8/16, the first
heater element 130 is not energized. Similarly, as long as the system is
in a state in which the duty cycle is less than 9/16, the second heater is
not energized. If, now, the system is in a state corresponding to a duty
cycle higher than that corresponding to the initial delay interval, e.g.,
state 11 (corresponding to a duty cycle of 11/16, i.e., approximately
70%), the first and second heaters are energized only after a delay time
of D'1 and D'2, respectively.

In accordance with a further aspect of the invention, the rate of decrease
of the energization delay time of the heaters is established in accordance
with two different paradigms, the first being essentially a linear
decrease, and the second being essentially a geometric decrease, as shown
in FIG. 3. We have found that this establishes a desirable balance between
the need for heat and the desire to supply this need as much as possible
from the thermally generated heat of the engine, as opposed to heat
generated via the supplemental heater.

As a specific example of the foregoing, the following has been found to be
a useful control paradigm for the heaters:

______________________________________
 System State No.
 Heater #1 Heater #2
 ______________________________________
 8 60 255
 9 50 255
 10 40 240
 11 30 180
 12 20 120
 13 15 60
 14 10 30
 15 5 15
 16 5 10
 ______________________________________

Here, the numbers in the table for each of the heater elements indicate the
delay in minutes before the corresponding heater element is turned on.

Briefly summarizing the operation of the heater 56, when the heat demand
element (e.g., thermostat) call for heat, the system attempts to supply
the heat first from the thermal output of the engine 20. As long as the
duty cycle state of the system is sufficiently low, the heater 56 will not
be utilized, since the delay in energization of the heater elements will
exceed the time required for the thermal output of the engine to satisfy
the heat demand. As the duty cycle state increases, the time required for
the engine to generate the requisite thermal output increases
correspondingly, and at some point will exceed the energization delay time
of one or both of the heater elements. One or both of these elements will
then be energized in order to generate supplemental heat for the heating
medium (.e.g., water, in the case of a forced hot water system) for the
residence. The electrical energy for these heaters is supplied from the
generator 22 to the extent that the demands on the generator from other
household systems (e.g., washers, dryers, electric stoves, lights, etc.)
leaves an excess. To the extent that there is a deficiency in the required
power, however, it is supplied from the electric utility. From data
obtained in one test installation, we have determined that purchase of
electric power from the electric utility should be necessary for only a
relatively few hours per year (generally during the coldest months), and
have found that power in fact will be supplied to the electric utility by
the system for a substantial number of hours each year, thus earning power
usage credits from the utility.

CONCLUSION

From the foregoing it will be seen that we have provided an improved
cogeneration system. The system is particularly suited to residential
applications, and is highly efficient. A unique controller enables use of
a prime heat generator of comparatively smaller capacity than commonly
believed feasible, while providing sufficient capability to meet even
extreme needs. Supplemental heating means driven first from locally
generated electricity, and then from purchased electricity, ensures that
the system provides adequate heat at all times but in an economical
manner.

It will be understood that the foregoing has described an illustrative
embodiment only, and that various changes may be made therein without
departing from either the spirit or the scope of the invention, the
invention being defined with particularity in the claims appended hereto.

THE FUTURE OF THE HYDROGEN ECONOMY: BRIGHT OR BLEAK?

Ulf Bossel, Ph.D. A1, Baldur Eliasson, Ph.D. A2, Gordon Taylor A3

A1 Fuel Cell Consultant
A2 ABB Switzerland Ltd.
A3 G T Systems

Abstract:

In the past, many have considered the production and use of hydrogen, assuming that it is just another gaseous fuel and can be handled much like natural gas in today's energy economy. With this study we present an analysis of the energy required to operate an elemental hydrogen economy, with particular reference to road transport. High-grade electricity from renewable or nuclear sources is needed not only to generate hydrogen, but also for all the other essential stages. However, because of the molecular structure of hydrogen, the infrastructure is much more energy-intensive than in an oil and natural gas economy.

In a "Hydrogen Economy" the hydrogen, like any other commercial product, is subject to several stages between production and use. Hydrogen has to be packaged by compression or liquefaction, transported by surface vehicles or pipelines, stored, and transferred to the end user. Whether generated by electrolysis or by chemistry, and even if produced locally at filling stations, the gaseous or liquid hydrogen has to undergo these market processes before it can be used by the customer. Hydrogen can also be derived chemically at relatively low cost from natural gas or other hydrocarbons. However, because there are no energetic or environmental advantages, we do not consider this option.

In this study, the energy consumed by each stage is related to the true energy content--the higher heating value (HHV)--of the delivered hydrogen. The analysis reveals that much more energy is needed to operate a hydrogen economy than is required for fossil energy supply and distribution today. In fact, the input of electrical energy to make, package, transport, store and transfer hydrogen may easily exceed the hydrogen energy delivered to the end user--implying a well-to-tank efficiency of less than 50%. However, precious energy can be saved by packaging hydrogen chemically in a synthetic liquid hydrocarbon like methanol or ethanol. To decouple energy use from global warming, the use of "geo-carbons" from fossil sources should be avoided. However, carbon atoms from biomass, organic waste materials or recycled carbon dioxide could become the carriers for hydrogen atoms. Furthermore, energy intensive electrolysis may be partially replaced by the less energy intensive chemical transformation of water and carbon to natural and synthetic hydrocarbons, including bio-methanol and bio-ethanol. Hence, the closed natural hydrogen (water) cycle and the closed natural carbon (CO2) cycle may be used to produce synthetic hydrocarbons for a post-fossil fuel energy economy. As long as the carbon comes from the biosphere ("bio-carbon"), the synthetic hydrocarbon economy would be far better than the elemental hydrogen economy--both energetically and thus environmentally.

Mechanical arrangements with primary application to single-acting, multiple-piston, Stirling-cycle machines providing a dramatic reduction in mechanical complexity and production cost. Two specific new machines are disclosed in detail, a single-acting, two-piston "ducted axle" machine and a quasi double-acting, four piston "drum cam" machine. A power level control subsystem inherent in the ducted axle machine permits engine output to vary from maximum positive through zero to maximum negative under full load conditions by means of the simple rotation of a single moving part.

Solar Thermal Cogeneration
[Overview] [Program] [System] [Collector] [Turbine] [Generator] [Controller] [Battery]

Overview

| - Introduction - Description - Roof Mount for Economy and Aesthetics - Comparison With Alternative Approaches - Residential Energy Needs | - Small-Scale Energy Systems - Local Craftsman Industry - Photovoltaic Material Shortage - Global Warming |
| --- | --- |

Introduction

By drawing our energy directly from the sun we minimize geological, ecological and social disturbance.  The Solar-Thermal Cogeneration (STC) System is a complete residential energy system, providing both electrical and thermal energy.  Solar thermal describes converting the sun's radiation into usable heat energy and cogeneration describes generating electricity while using excess heat for thermal needs such as space heating/cooling.  Synonyms for cogeneration include combined-heat/power and combined cycle.  High efficiencies can be achieved with cogeneration, making the STC competitive when full costs and benefits are considered.  A drawback of cogeneration is less flexibility in independently specifying the electric and thermal capacities.


Description

The STC system provides residential energy in a flat-roof-mount configuration.  The system can supply all electric and thermal needs for a conservation-oriented residence.  On a clear winter day in a sunny climate in mid-latitude an STC system occupying 1000 sq. ft. of roof area can provide 25 kilowatt-hours of electric energy and another 56 kilowatt-hours of thermal energy for space heating, water heating and refrigeration.  This may provide 16 to 19 kilowatt-hours for residential use while providing 15 to 30 miles of range per day for an electric vehicle.  In summer these outputs are multiplied by 2.5, providing an extra 125 miles of range per day for electric vehicles, and plenty of thermal energy for space cooling.

The STC system employs a Rankine cycle heat engine with a closed-loop water-steam boiler.  Components include a solar collector, turbine/generator, and electrical and thermal output subsystems. The collector consists of a concentrator (optical reflector) and a receiver (heat pipe).  The collector geometry is parabolic trough, a practical geometry for low-profile roof mounting.  A tracking mechanism maintains concentrator focus as the sun traverses the sky.  The turbine is of the boundary-layer type and drives a permanent magnet generator.  The electrical output subsystem includes lead-acid batteries, high-efficiency charge controller and AC /DC distribution controllers.  The thermal output subsystem includes ammonia/water absorption space cooling/refrigeration, and space heating/water heating.  The steam circuit is operated with water at night to radiate the system's waste heat to the sky and to supplement space cooling.

The STC program provides extensive design information, fabrication procedures, and supplemental resources, enabling widespread production of STC systems in small workshops with simple tools, limited skills, and available materials.


Roof Mount for Economy and Aesthetics

The STC is intended for mounting on a flat roof.   A roof parapet may completely or partially hide the concentrator.  The concentrator might partly show above the parapet during the winter to reach more sun in the low sky but hide below the parapet during the summer.  The equator-facing wall might have a lower parapet for better exposure.  Besides aesthetics, roof mounting preserves wildlife habitats, economizes space, economizes energy transfer, and keeps sunlight off much of the roof.  

A design that provides for a hybrid pitched-flat roof would orient the front of the structure away from the equator and make the front half pitched and the rear half flat with parapets.  Far from the equator, the equator-facing exposure should be maximized for in the winter season when less energy is available.  The east/west sides may have limited pitch that reduces exposure in the summer season where less energy is needed, e.g. in mild marine climates.  Near the equator, reduced east/west exposure would reduce energy availability year-round, so in this region, limited pitch on the north/south sides is more acceptable.


Comparison With Alternative Approaches

Photovoltaic and wind systems are two alternative approaches to generating electric power.  Photovoltaic has no moving parts and requires less maintenance than solar-thermal systems.  Photovoltaic panels are becoming less expensive but their manufacture is enormously resource-intensive (electricity, water) and capital-intensive, with associated environmental and social costs.  Schott has calculated that photovoltaics take five years to produce the energy embodied in their manufacture while solar-thermal and wind systems take five months.  

In 2005, high labor costs kept photovoltaic costs high and material shortages were expected to persist for years. Photovoltaic panels lose as much as 90% of energy absorbed to heat.  This heat may be collected and utilized but it is not as versatile (i.e. hot) as that produced by concentrated solar-thermal.  Photovoltaic panels are also expensive to recycle.  

Wind systems may be more economical than solar-thermal systems, having fewer mechanical/thermal components, but they can be lethal to wildlife and an aural and visual blight for some people.  In a six-week period, FPL's 44-turbine wind facility in West Virginia killed an estimated 1,364 to 1,980 bats [1].  Complaints are fairly frequent and zoning restrictions and permit fees are likely in suburban/urban communities [2].  Wind energy is more erratic and off-peak than solar, often requiring a larger battery reserve.  

Photovoltaic and wind systems may generate electricity in tandem with a low-temperature solar thermal system for hot water and space heat.  Refrigeration and space cooling require doubling to tripling the electrical capacity, including batteries, or fuel heat.  Solar-thermal systems economize by utilizing the waste heat from electric generation to supply all thermal loads.  Improvements will continue to be made to both photovoltaic and wind technologies but solar-thermal is a complete energy system with low full costs and is especially good for a flat-roof structure in a sunny climate. 

Availability of either wind or solar energy at a given location should be researched early.  Wind and/or solar measurements should be collected over a period of one year, or preferably more, in the immediate vicinity of the installation.  Buildings and trees may interfere with both wind and solar systems, probably more with wind.

[1] Yellow light for a 'green' energy source | csmonitor.com
[2] A War Over Wind Turbines - Los Angeles Times - December 7, 2005


Residential Energy Needs

Residential energy has been conventionally supplied by grid electricity and piped natural gas and fuel oil.  The STC may supplement these sources for flexibility, or entirely replace them, which may be simpler and in some cases more feasible.  When replacing them, it is important to size the STC carefully.

Estimating residential energy needs is an art, not a science.  Energy needs can change, for example, with the number of residents.  A strategy to address this dynamic, and reduce investment risk, is to start with a minimum system and expand as needed.  The STC's solar collector, thermal banks and batteries are expandable.  The rest of the system must be sized initially to accommodate the later expansion.

While the the energy from the sun is free, energy systems still have embedded costs.  To minimize the needed system capacity for active heating/cooling, ideally, new construction would employ passive solar/passive design techniques.  Passive design can completely satisfy space heating/cooling requirements in many climates.   If an electric vehicle is planned that would likely become the primary energy consumer resulting in a surplus of thermal energy.  

The table below shows the US DOE's data of average residential energy consumption in four large US states.  Space heating and cooling represents a minority percent of total energy usage in warmer climates.  The remaining applications include water heaters, refrigerators, stoves/ovens, lights, clothes and dish washers, televisions and computers.

| 1997 residential energy consumption (million BTUs/year) | space heating | space cooling | other | --total-- |
| --- | --- | --- | --- | --- |
| Florida | 5 | 13 | 37 | 55 |
| Texas | 31 | 12 | 53 | 96 |
| New York | 77 | 2 | 44 | 123 |
| California | 20 | 4 | 40 | 64 |

In a conservation-oriented household powered by the STC, water heating and refrigeration are handled as thermal loads.  Clothes and dish washing and drying can be handled the same (the STC program will later expand to include appliance designs).  Cooking duty might be shared between a biofuel stove and a solar oven.  Low-power appliances utilize efficient motors and avoid large transformers and illuminated displays.  Low-power televisions and computers are available with LED-backlit LCD displays.  Generally, laptop computers are lower power than desktop models.  


Small-Scale Distributed Energy Systems

Small-scale distributed systems enable the efficiency of cogeneration but have additional advantages over large-scale systems.   Small-scale distributed systems have reduced development, security, environmental, and transmission costs.  Small-scale systems can preserve the landscape if held to certain constrains, e.g. solar-thermal systems mounted in low-profile configurations on the roofs of building structures.  The materials cost of small-scale energy systems beyond that of large-scale production/transmission is very small.  Small-scale systems protect consumers from market turmoil that may arise from speculation, geopolitics, and natural disasters.  

In turbine/generators, the material's molecular bonding strength is the same in small and large scale equipment, so the large scale equipment has considerably more mechanical stress/fatigue problems.  Large-scale turbine/generators must be taken offline and rebuilt as often as every six months.  A small-scale turbine/generator may last a decade before it needs rebuilding.  The engineering cost of small scale systems is much lower.

One of the environmental costs of centralized energy production is the condensing stage of the Rankine cycle, which is used in the vast majority of electric generating plants.  This process alters ecosystems, by evaporating and/or warming the water that is the habitat of many species.   In industrialized nations the volume of water affected is nearly equal to the total amount otherwise consumed by the human population.  In 2006 it was reported that Global warming is inducing water shortage, forcing centralized electric plants to scale back production (see Global Warming).  In 2007, the US EPA was seeking to classify water vapor expelled into the atmosphere by human activities as a pollutant *.

Environmental effects of the waste heat from the distributed heating/cooling of residential and commerical structures are not well known at this point but one of the STC design goals is to emit as much of the waste heat as possible to the sky.

Merging new power with old system proves costly
Microgeneration Technology: Shaping Energy Markets


Local Craftsman Industry

The STC is designed to be built in local workshops from as raw materials as possible.  Things that can be built and maintained locally bring economic opportunity to the local level.  The knowledge, skills and tools acquired to build STCs can be used to build other things. 

The STC design is fully documented online. High quality documentation helps enable better construction and maintenance of the systems, extending system life and reducing costs.  Undocumented designs can result in poor maintenance and shortened system life.  The STC documentation helps reasonably skilled craftsmen successfully construct and maintain the systems and make successful material substitutions and design alterations. 

Commercial components may be purchased and connected together to make a solar-thermal cogeneration system.  These components should be high-quality and well-documented to meet the target service life.  The STC design might include commercial components which meet specifications and are allowed to be fully documented on the STC program website under the free license.  


Photovoltaic Material Shortage

Silicon Still in Short Supply - June 21, 2005 - The Energy Blog
The shortage on Solar Grade Silicon was evident at the 20th European Photovoltaic Solar Energy Conference, Barcelona, Spain and Exhibition last week. With the resulting shortages in wafers, solar cells and modules, it was a major topic and it seems it will remain high on the agenda for at least the coming 2 years. 

Shortages Stifle a Boom Time for the Solar Industry - August 6, 2005 - New York Times
American suppliers for the solar energy industry say that burgeoning demand both domestically and overseas, a weak dollar and shortages of raw material have created back orders of several months on electricity-generating photovoltaic, or PV, panels.

11-02-05- divpower.com
Due to a world-wide shortage of Photovoltaic Panels we are unable to offer specific models for purchase here at this time, however, with our connections to suppliers and manufacturers, we are able to find panels to meet your needs.

Global Warming

Global warming is a trend of increased atmospheric and terrstrial temperature due mainly to the greenhouse effect from carbon dioxide from combustion of fossil fuels and livestock, and increasing feedback effects such as the albedo change in the arctic with ice melt, methane release from thawing Siberian peat bogs, etc.  The increased temperature leads to erratic weather patterns, rising sea levels causing coastal flooding, and adverse pressures on societies, ecosystems and biodiversity, i.e. species extinctions.  Additional pollutants from combustion of fossil fuels include sulfur oxides, nitrous oxides, carbon monoxide, ozone, particulates and mercury.

(sierra club)

| Greenhouse Gas Emissions | electricity | industrial | transport | commercial | residential | agriculture |
| --- | --- | --- | --- | --- | --- | --- |
| Calif. 1999 (%) | 16 | 6 | 56 | 13 | 9 |  |
| USA 1998 (%) | 32 | 23 | 26 | 4 | 8 | 7 |
| USA 1997 (*) |  | 620 | 460 | 270 | 330 | 130 |
| USA gas (*) | 35 | 150 | 10 | 45 | 70 |  |
| USA oil (*) | 20 | 110 | 450 | 10 | 20 |  |
| USA coal (*) | 470 | 60 | 0 | 10 | 10 |  |

*millions of metric tons of carbon equivalent


Nigeria kidnappers demand $1.5bn and control of oil, Times Online, 1-17-2006

Hundreds burned alive in Lagos pipeline blaze - The Guardian UK, 12-27-2006

More than 260 people were killed yesterday - burned alive when a ruptured oil pipeline burst into flames in Lagos. Crowds of local residents had gone to scoop up petrol using plastic containers after an armed gang punctured an underground pipeline to illegally siphon off fuel.  The Christmas shortage has caused all the more hardship as petrol is not only essential for transport, but for the generators on which most homes and businesses rely in the absence of reliable mains electricity.

Global Warming, Not Just Heat Wave, Inter Press Service, 7-21-06

Gerstengarbe said that over the last century temperatures in Germany rose 0.8 degrees. "Over the next 75 years, we expect a warming of between 1.8 to 3..6 degrees for our region."  The heat is also taking its toll on agriculture, and affecting the generation of electricity, especially in nuclear power plants.  The lack of fresh water for the nuclear plants' cooling systems has led German private electricity suppliers to slow down their generators.  In France, the state-owned Electricité de France (EdF) was allowed to continue to drain hot water from the cooling system into rivers, although the water temperatures exceeded the limits imposed by environmental authorities. But output has had to be lowered.  EdF has been importing electricity to compensate the nuclear power plants' lower performance. In Italy, hydroelectric plants have had to slow down due to a shortage of water in rivers.

Shaft turning parking bus for multiple unit installations utilizing a single motorized generator control system

In an electrical power generating installation which includes a plurality of turbine-generator sets, a parking bus is disclosed, which is provided with a potential regulated at a relatively low and constant frequency of the order of 1-2 Hz. Switching and phase and frequency matching means are provided, which enable the same controlled variable frequency starting system used for start-up of the turbine-generator sets, to be used for gradually adjusting the rotational speed of a selected turbine-generator set to a point where phase and frequency may be matched with that of the parking bus, to then permit the generator member of such set to be electrically transferred to the bus. By means of such arrangement, any number of turbine-generator sets may be connected in parallel to the parking bus which thereupon effects a simultaneous, continued low speed rotation of the sets for cooling or other purposes. The sophisticated, complex, and costly start-up system, is thus freed of any requirement for continuing rotation of the sets during the long cool-down period, whereby in typical installations but a single such start-up system is required.

### RELIABILITY FEATURES

Simplified design crammed with desirable features; benefits from several preceding designs in an evolutionary process which preserved the best features of the predecessors while using the latest design innovations developed by STADCO for enclosed power units.

Prime mover is Deutz FM type liquid cooled direct injection diesel engine. This news series of emissions engines are brand new from the ground up, designed with leading edge technology throughout to meet proposed worldwide emissions legislation. Exceptionally low exhaust emissions and low noise levels are the hallmarks of this new family of Deutz engines. The future oriented high pressure fuel injection system and combustion concepts include massive individual injection pumps, precise fuel tiring and low swirl combustion process. All engines are able to meet present and future U.S. industrial and European emission regulations, with extremely low fuel consumption and with lower than average engine noise levels. Engine levels are 4 decibels below the quietest rival. M/D units will operate satisfactorily on both #2D Red Diesel fuel, without injector fouling.

The generator is a new type which delivers improved efficiency in a smaller package. Manufactured by the worlds best recognized builder; now produced here in the USA.

Wherever possible engine accessories and components are mounted off the engine to eliminate the destructive effects of engine vibrations on operating systems and accessories.

Dual fuel filter system provides an additional measure of protection to trap water and contaminants which could otherwise damage the fuel injection system.

The cold weather starting systems are the latest state of the art, with sealed heat bulbs located directly in the combustion chamber where auxiliary heating is safest and most effective. The systems operate with 12V. current and are self contained.

Cooling air intake area located entirely on Service side, toward platform.- draws coolest available air in the gen set environment. Simplifies installation on skirting-equipped railcars. Strategic location of air intake panel also reduces to minimum the ingestion of road dust and dirt. All models are equipped with cleanable media filters for cooling air stream serviceable from the outside.

Main frame houses all engine/generator operating systems inboard for maximum reliability and protection from vibration and damage.

### CURRENT MODELS LIST

Please see the current models list of Stadco Generator Sets designed for service in the railroad industry.

### DESIGN FEATURES

MAIN FRAME & ENCLOSURE PANELS: Welded main frame assembly constructed of formed rectangular steel tubing and prestressed steel plates. Narrow configuration main frame with generous triangulation for maximum strength-to-flexibility ratio to absorb shock loads. Tuned rubber mounts operated in compression and shear to isolate main frame from engine vibration. Welded steel crash stops prevent excessive engine deflection in the event of coupling impact. All side/top panels and belly pan externally removable for service access. Two hinged access doors secured by padlockable double bolt latch provide convenient routine service access.

ENGINES: Deutz liquid cooled diesel engines, specific engine per unit as shown.

Deutz diesels are premium quality engines which include these long-life feature: Cam shaft forged steel alloy Connecting rods-forged steel alloy. Camshaft-forged steel alloy. Cylinder head-replaceable valve seats and valve guides. Fuel system-ultra reliable Bosch type in-line multi-plunger injection pump or individual injection pumps. Lube system-gear oil pump and individual jets for oil cooling the pistons. Lube oil cooler Pistons-aluminum alloy.

FUEL SYSTEM: Primary water separator fuel filter and secondary fine fuel filter. Bulkhead fuel supply connections located on outside rear of unit. 12V. operated fuel solenoid provides instant engine shutdown capability. Wire reinforced fuel supply hoses.

COLD STARTING SYSTEM: 12V. sealed pintle type heaters direct in combustion chambers.

AIR CLEANER: Dry type dual element air cleaner with self-cleaning precleaner, low restrictions for improved fuel economy and extended service interval. Filter service indicator included. Rotary type centrifugal precleaners extend main air filters cartridge life.

EXHAUST SYSTEM: Turbo exhaust manifold and tailpipe wrapped with removable insulation blankets to reduce radiator heat saturation.

RADIATOR: Specially designed radiator with 5-row core and brazed support frame for extra vibration resistance. Rated for safe operation at rated temperatures and climatic conditions. Elastomeric mounts provide double isolation from engine vibration and single isolation from road vibration and main frame stresses.

BATTERY: None included. Steel battery enclosure mounted to unit end panel. Removable. Battery cables included.

COOLING SYSTEM: Positive forced ventilation of engine/generator compartment with positive ambient temperature air supply to both engine and generator cooling systems. Cooling air flows straight through enclosure. Cooling air inlet separated as far as possible from hot air outlet to reduce potential for heat recirculation. Cleanable foam media filter reduces accumulation of air-bourne dirt and debris inside the enclosure.

LUBE OIL LEVEL SENSOR: Mounted directly in oil pan at minimum safe oil level (12V. contactor).

LUBE OIL FILL & DIPSTICK: Located directly inside service access doors.

CAUTIONARY DECALS: Danger/Caution decals applied in all locations deemed as potentially dangerous.

GENERATOR: Totally brushless revolving field alternator built in the USA. High temp Class H insulation throughout for continuous duty at 105 Deg. C temp rise. Solid state type voltage regulation system with adjustable voltage output and voltage ramp-down. Voltage regulator is thermally load protected. 12-Lead reconnectable wiring configuration. Voltage regulation = +/- 1.5% at constant speed. Voltage adjustment = +/- 10%. Adjustable Volts-per-Hertz voltage ramp-down feature for motor protection. Frequency regulation = 3% from no load to full load. High efficiency performance for maximum fuel economy

AC LOAD WIRING: Unit wiring is Polyrad 60OV, Amtrak Spec #323 approved, irradiated Polyolefin insulation. Wired to terminal posts located in main power distribution box located on rear side of unit.

AC EQUIPMENT PROTECTION: Special inboard enclosure box, accessible from service side, protects voltage regulator and AC gear from dirt and moisture.

WIRE SAFETY: Poly-Rad wiring and noncombustible fiberglass wire loom.

Primary fuel filter (with drain) located outside enclosure.

Fuel supply hoses are wire reinforced to provide fire resistance and prevent failures due to chafing, cutting or pinching. Fire detection devices with engine shut down protection, on Model 5-65DE & up.

### SILENCING PACKAGE

Silencing package totally encloses unit with steel panels designed to produce one of the quietest railcar gen-sets available to date at a popular price.

Enclosure panels are constructed of solid sheet steel, insulated with heavy multilayer high performance composite sound insulation material to absorb mechanical sounds and engine air intake sounds. All panels facing toward railcar are totally solid (without openings) for improved noise control. No straight-through paths for sound emissions through top or sides.

Quiet engine intake and exhaust characteristics: Care is exercised to engineer out any annoying engine air-intake noise. Standard Critical grade muffler typically reduces tailpipe sound emissions to below 80 dBA (at 1 meter) at full load.

Tuned rubber mounts between the power unit and main frame substantially reduce the transmission of noise and vibration. Transmitting of Secondary vibration to railcar frame is virtually eliminated.

SOUND LEVELS: Results of testing available on request.

### SERVICABILITY

Easy routine servicing. Logical arrangement of routine service items and engine service points provide the easiest possible access for routine servicing. All routine service points and engine control panel are accessible from the (2) main service doors. Air cleaner also accessible from service side.

Easy to keep clean by design. Exceptionally clean design with an absolute minimum of joints and connections to minimize oil leaks. Lube oil drains directly down through an access plate in the belly pan via self-locking oil drain valve.

Coolant recovery system with see-through reservoir for easy visual monitoring of coolant level and convenient replenishment of coolant.

Optional available Roll Out service track system provides unrestricted access for major service.

Comprehensively written 125 page Operators Instruction Manual and trouble shooting charts make self servicing as easy as possible. Includes unit installation instructions and diagrams.

### EXTENDED OPERATING TIMES

Lube oil change interval is 500 hours on most models; refer to Brief Spec Sheets #S32. Oil capacity is entirely self contained without external reservoir tanks, connections or hoses of any kind to leak or trap oil.

100 hours (average min.) running time for FULL or ADD level on lube oil dipstick (after engine break-in period).

### SIMPLIFED INSTALLATION

Full sized prestressed steel end plates on mainframe provide numerous options for mounting generator set to rail car frame. The installer has numerous installation options over a wide area of the end plates.

Cooling air intake area on Service side (outside) to simplify installation on Skirting equipped cars. Reduces heat recirculation potential on all types of cars. See Sales drawings for further installation information.

Optionally available Roll Out service track system provides convenient access to entire unit for major service procedures. In transport position unit rests on (4) bolted rigid mounts. In service position unit rolls out on 48 inch long track extensions. Storage required for track extensions and ground support jacks. ROLL-OUT service track attaches to predrilled holes in generator unit frame using hardware included with track assembly. Installation drawing and instructions included.

A tucked-in configuration belly pan provides additional installation clearance in a critical area.

All electrical connections are wired to terminal strips in dedicated terminal boxes for both AC and DC circuits. Likewise, Remote control panels are equipped with terminal strips to receive conductors from main unit.

Fuel connections (2) are located on outside rear side of the unit.

STADCO supplied exhaust system quickly and easily disconnected by a stainless steel connector.

Current distribution box, located on back side of unit provides for a choice of outlet cable and control cable connections. Cable entry is from the sides of the box. (Distribution box can be relocated to end panel if required for clearance).

All-steel battery compartment designed to mount directly to unit frame. All necessary mounting hardware and battery cables included. Design also permits remote location of battery.

### MISCELLANEOUS

Standard finish: Silver gray enamel interior and exterior finish. Load test and certification of performance-. 4 hours controlled load test run on resistive load bank.

Predelivery Quality Control: Check engine fuel delivery timing and adjust valve clearance, check entire unit against Bill of Materials and against Quality Control check list.

Publications: Electrical diagrams for AC and DC circuit, 125 page operators manual with trouble shooting charts. Complete parts manual containing engine parts numbers in addition to those for all other components.

Heavy timber shipping skid for motor freight shipment to prevent shipment damage.

Specifications and features subject to change without notice. For construction we reserve the right to substitute components or equipment which do not substantially alter Gen-set specifications and overall performance, without notice to customers.

Engine driven power inverter system with cogeneration

A combined heat and AC power generating system is disclosed having black start capability for the full time, simultaneous production of both electricity and heat. Heat generated within the system is captured and used for heating applications such as heating building air and tap water. The power generating system comprises an engine, generator, rectifier, variable frequency drive inverter, and inverter control electronics. The system provides improved efficiency and prolonged engine life by always operating the engine with its throttle fully open to obtain maximum efficiency and the engine is normally operated near its stall point. In this operating state the inverter control electronics adjust the power output from the inverter to control the speed of the engine. This is done by increasing the power output from the inverter when the power drawn by the load decreases. This causes the engine to operate closer to its stall point and it slows down. The power from the inverter is decreased when the power drawn by the load increases. This causes the engine to operate further from its stall point and it speeds up. The slight power changes to facilitate this operation will always be drawn from or distributed to the electric utility grid so there is no wasted power. The inverter control electronics are responsive to signals from loads such as refrigeration loads to cause the frequency of the inverter output to change to permit the refrigeration load to operate more efficiently.

Be the first to comment on this page

### US Patent 4752697 - Cogeneration system and method

. A cogeneration system for producing thermal and electrical energy at a site having a thermal load and an electrical load coupled through electrical distribution to a
utility central generating station comprising:

cogeneration means including heat engine means, electrical generating means mechanically coupled to said engine means, electrical coupling means for coupling the electrical output of said generating means to said distribution lines, and heat
transfer means for transferring heat from said engine means to said thermal load; and supervisory means for monitoring cogeneration system operating conditions and controlling cogeneration system operation, said supervisory means including

electrical monitoring means for monitoring the electrical energy or power of said electrical load, said generating means, and said utility;

real time clock means for providing real time data; and

cogeneration control means for controlling the operation of said cogeneration system in response to said monitoring means and said clock means.

2. System according to claim 1, wherein said supervisory means comprises data storage means for retaining stored data, and said supervisory means stores data representing monitored operating conditions.

3. System according to claim 2, wherein said supervisory means comprises remote input/output means for transferring data between said supervisory means and a communication channel.

4. System according to claim 3, wherein said remote input/output means comprises a modem.

5. System according to claim 3, wherein said remote input/output means comprises radio transmitter/receiver means.

6. System according to claim 1, wherein said supervisory means comprises thermal monitoring means for monitoring the heat transferred between said engine means and said thermal load.

7. System according to claim 6, wherein said heat transfer means comprises means for circulating a fluid through a fluid circuit thermally coupled to said engine means and to said thermal load, and said thermal monitoring means comprises means
for measuring the difference in temperature of said fluid circulated to and from said thermal load and means for measuring the flow of fluid through said fluid circuit.

8. System according to claim 2, wherein said stored data includes data representing the rate structure of said utility.

9. System according to claim 8, wherein said supervisory means controls operation of said cogeneration means in response to said utility rate structure data.

10. System according to claim 9, wherein said utility rate structure data includes data representing utility peak demand measuring periods, and said supervisory means controls operation of said cogeneration system in response to said rate
structure data and said real time data.

11. System accordin

to claim 10, wherein said utility rate structure data includes data representing the times at which different utility energy rates are in effect, and said supervisory means controls operation of said cogeneration system in
response to said rate structure data and said real time data.

12. System according to claim 2, wherein said supervisory means controls the output of said generator means in response to monitored utility power data.

13. System according to claim 12, wherein said supervisory means controls the output of said cogeneration means so that the monitored utility power is substantially zero.

14. System according to claim 2, wherein said stored data includes data representing alarm limits for said monitored operating conditions, and said supervisory means compares said monitored operating condition data with corresponding alarm limit
data and generates alarm outputs when said operation conditions exceeds corresponding alarm limits.

15. System according to claim 14, wherein said supervisory means generates remote alarm outputs via remote input/output means.

16. System according to claim 2, wherein said supervisory means comprises input/output means for entry and display of stored data.