Where does Wind Energy come From?

All renewable energy (except tidal and geothermal power), and even the energy in fossil fuels, ultimately comes from the sun. The sun radiates 174,423,000,000,000 kilowatt hours of energy to the earth per hour. In other words, the earth receives 1.74 x 10 17 watts of power . 1)

About 1 to 2 per cent of the energy coming from the sun is converted into wind energy. That is about 50 to 100 times more than the energy converted into biomass by all plants on earth. 2)

Temperature Differences Drive Air Circulation

The regions around equator, at 0° latitude are heated more by the sun than the rest of the globe. These hot areas are indicated in the warm colours, red, orange and yellow in this infrared picture of sea surface temperatures (taken from a NASA satellite, NOAA-7 in July 1984).

Hot air is lighter than cold air and will rise into the sky until it reaches approximately 10 km (6 miles) altitude and will spread to the North and the South. If the globe did not rotate, the air would simply arrive at the North Pole and the South Pole, sink down, and return to the equator.

1) The power emission from the sun is 1.37 kW/m 2 on the surface of the sphere, which has the sun as its centre and the average radius of the earth trajectory. The power hits a circular disc with an area of of 1.27 x 10 14 m 2 . The power emitted to the earth is thus 1.74 x 10 17 W.

2) On average, plant net primary production is about 4.95 x 10 6 calories per square metre per year. This is global NPP, Global net primary production , i.e. the amount of energy available to all subsequent links in the food/energy chain. The earth's surface area is 5.09 x 10 14 m 2 . The net power output stored by plants is thus 1.91 x 10 13 W, or 0.011% of the power emitted to earth. You may find the conversion factor between the energy units calories and Joule in the reference manual .

| The Coriolis Force Since the globe is rotating, any movement on the Northern hemisphere is diverted to the right, if we look at it from our own position on the ground. (In the southern hemisphere it is bent to the left). This apparent bending force is known as the Coriolis force. (Named after the French mathematician Gustave Gaspard Coriolis 1792-1843). It may not be obvious to you that a particle moving on the northern hemisphere will be bending towards the right. |  |
| --- | --- |
|  | Consider this red cone moving southward in the direction of the tip of the cone. The earth is spinning, while we watch the spectacle from a camera fixed in outer space. The cone is moving straight towards the south. |
|  | Note that the red cone is veering in a curve towards the right as it moves. The reason why it is not following the direction in which the cone is pointing is, of course, that we as observers are rotating along with the globe. |
|  | Here we show the same image with the camera locked on to the globe. |
|  | Here we show the same image,with the camera fixed in outer space, while the earth rotates. |
|  | The Coriolis force is a visible phenomenon. Railroad tracks wear out faster on one side than the other. River beds are dug deeper on one side than the other. (Which side depends on which hemisphere we are in: In the Northern hemisphere moving particles are bent towards the right). In the Northern hemisphere the wind tends to rotate counterclockwise (as seen from above) as it approaches a low pressure area. In the Southern hemisphere the wind rotates clockwise around low pressure areas. |

Wind Energy Resources: Global Winds

How the Coriolis Force Affects Global Winds

The wind rises from the equator and moves north and south in the higher layers of the atmosphere.

Around 30°; latitude in both hemispheres the Coriolis force prevents the air from moving much farther. At this latitude there is a high pressure area, as the air begins sinking down again.

As the wind rises from the equator there will be a low pressure area close to ground level attracting winds from the North and South.

At the Poles, there will be high pressure due to the cooling of the air.

Keeping in mind the bending force of the Coriolis force, we thus have the following general results for the prevailing wind direction:

Prevailing Wind Directions

| Latitude | 90-60°N | 60-30°N | 30-0°N | 0-30°S | 30-60°S | 60-90°S |
| --- | --- | --- | --- | --- | --- | --- |
| Direction | NE | SW | NE | SE | NW | SE |

The size of the atmosphere is grossly exaggerated in the picture above (which was made on a photograph from the NASA GOES-8 satellite). In reality the atmosphere is only 10 km thick, i.e. 1/1200 of the diameter of the globe. That part of the atmosphere is more accurately known as the troposphere. This is where all of our weather (and the greenhouse effect) occurs.The prevailing wind directions are important when siting wind turbines, since we obviously want to place them in the areas with least obstacles from the prevailing wind directions. Local geography, however, may influence the general results in the table above, cf. the following pages.

| The Geostrophic Wind The Atmosphere (Troposphere) The atmosphere around the globe is a very thin layer. The globe has a diameter of 12,000 km. The troposphere, which extends to about 11 km (36,000 ft.) altitude, is where all of our weather, and the greenhouse effect occurs. |  |
| --- | --- |
|  | On the picture you can see a stretch of islands 300 km (200 miles) across, and the approximate height of the troposphere. To look at it at a different scale: If the globe were a ball with a diameter of 1.2 metres (4 ft.), the atmosphere would only be 1 mm (1/25") thick. |
|  | The Geostrophic Wind The winds we have been considering on the previous pages on global winds are actually the geostrophic winds. The geostrophic winds are largely driven by temperature differences, and thus pressure differences, and are not very much influenced by the surface of the earth. The geostrophic wind is found at altitudes above 1000 metres (3300 ft.) above ground level. The geostrophic wind speed may be measured using weather balloons. Surface Winds Winds are very much influenced by the ground surface at altitudes up to 100 metres. The wind will be slowed down by the earth's surface roughness and obstacles , as we will learn in a moment. Wind directions near the surface will be slightly different from the direction of the geostrophic wind because of the earth's rotation (cf. the Coriolis force ). When dealing with wind energy, we are concerned with surface winds, and how to calculate the usable energy content of the wind. |

Local Winds: Sea Breezes

Although global winds are important in determining the prevailing winds in a given area, local climatic conditions may wield an influence on the most common wind directions.

Local winds are always superimposed upon the larger scale wind systems, i.e. the wind direction is influenced by the sum of global and local effects. When larger scale winds are light, local winds may dominate the wind patterns.

Sea Breezes

Land masses are heated by the sun more quickly than the sea in the daytime. The air rises, flows out to the sea, and creates a low pressure at ground level which attracts the cool air from the sea. This is called a sea breeze. At nightfall there is often a period of calm when land and sea temperatures are equal.

At night the wind blows in the opposite direction. The land breeze at night generally has lower wind speeds, because the temperature difference between land and sea is smaller at night.

The monsoon known from South-East Asia is in reality a large-scale form of the sea breeze and land breeze, varying in its direction between seasons, because land masses are heated or cooled more quickly than the sea.

| Local Winds: Mountain Winds |  |
| --- | --- |
|  | Mountain regions display many interesting weather patterns. One example is the valley wind which originates on south-facing slopes (north-facing in the southern hemisphere). When the slopes and the neighbouring air are heated the density of the air decreases, and the air ascends towards the top following the surface of the slope. At night the wind direction is reversed, and turns into a downslope wind. If the valley floor is sloped, the air may move down or up the valley, as a canyon wind. Winds flowing down the leeward sides of mountains can be quite powerful: Examples are the Foehn in the Alps in Europe, the Chinook in the Rocky Mountains, and the Zonda in the Andes. Examples of other local wind systems are the Mistral flowing down the Rhone valley into the Mediterranean Sea, the Scirocco, a southerly wind from Sahara blowing into the Mediterranean sea. |

The Energy in the Wind: Air Density and Rotor Area

A wind turbine obtains its power input by converting the force of the wind into a torque (turning force) acting on the rotor blades. The amount of energy which the wind transfers to the rotor depends on the density of the air, the rotor area, and the wind speed.

The cartoon shows how a cylindrical slice of air 1 metre thick moves through the 2,300 m 2 rotor of a typical 1,000 kilowatt wind turbine.

With a 54 metre rotor diameter each cylinder actually weighs 2.8 tonnes, i.e. 2,300 times 1.225 kilogrammes.

Density of Air

The kinetic energy of a moving body is proportional to its mass (or weight). The kinetic energy in the wind thus depends on the density of the air, i.e. its mass per unit of volume.

In other words, the "heavier" the air, the more energy is received by the turbine.

At normal atmospheric pressure and at 15° Celsius air weighs some 1.225 kilogrammes per cubic metre, but the density decreases slightly with increasing humidity.

Also, the air is denser when it is cold than when it is warm. At high altitudes, (in mountains) the air pressure is lower, and the air is less dense.

Rotor Area

A typical 1,000 kW wind turbine has a rotor diameter of 54 metres, i.e. a rotor area of some 2,300 square metres. The rotor area determines how much energy a wind turbine is able to harvest from the wind.

Since the rotor area increases with the square of the rotor diameter, a turbine which is twice as large will receive 2 2 = 2 x 2 = four times as much energy. The page on the size of wind turbines gives you more details.

Wind Turbines Deflect the Wind

The image on the previous page on the energy in the wind is a bit simplified. In reality, a wind turbine will deflect the wind, even before the wind reaches the rotor plane. This means that we will never be able to capture all of the energy in the wind using a wind turbine. We will discuss this later, when we get to Betz' Law.

In the image above we have the wind coming from the right, and we use a device to capture part of the kinetic energy in the wind. (In this case we use a three bladed rotor, but it could be some other mechanical device).

The Stream Tube

The wind turbine rotor must obviously slow down the wind as it captures its kinetic energy and converts it into rotational energy. This means that the wind will be moving more slowly to the left of the rotor than to the right of the rotor.

Since the amount of air entering through the swept rotor area from the right (every second) must be the same as the amount of air leaving the rotor area to the left, the air will have to occupy a larger cross section (diameter) behind the rotor plane.

In the image above we have illustrated this by showing an imaginary tube, a so called stream tube around the wind turbine rotor. The stream tube shows how the slow moving wind to the left in the picture will occupy a large volume behind the rotor.

The wind will not be slowed down to its final speed immediately behind the rotor plane. The slowdown will happen gradually behind the rotor, until the speed becomes almost constant.

The Air Pressure Distribution in Front of and Behind the Rotor

The graph to the left shows the air pressure plotted vertically, while the horizontal axis indicates the distance from the rotor plane. The wind is coming from the right, and the rotor is in the middle of the graph.

As the wind approaches the rotor from the right, the air pressure increases gradually, since the rotor acts as a barrier to the wind. Note, that the air pressure will drop immediately behind the rotor plane (to the left). It then gradually increases to the normal air pressure level in the area.

What Happens Farther Downstream?

If we move farther downstream the turbulence in the wind will cause the slow wind behind the rotor to mix with the faster moving wind from the surrounding area. The wind shade behind the rotor will therefore gradually diminish as we move away from the turbine. We will discus this further on the page about the park effect.

Why not a Cylindrical Stream Tube?

Now, you may object that a turbine would be rotating, even if we placed it within a normal, cylindrical tube, like the one below. Why do we insist that the stream tube is bottle-shaped? Of course you would be right that the turbine rotor could turn if it were placed in a large glass tube like the one above, but let us consider what happens:

The wind to the left of the rotor moves with a lower speed than the wind to the right of the rotor. But at the same time we know that the volume of air entering the tube from the right each second must be the same as the volume of air leaving the tube to the left. We can therefore deduce that if we have some obstacle to the wind (in this case our rotor) within the tube, then some of the air coming from the right must be deflected from entering the tube (due to the high air pressure in the right ende of the tube).

So, the cylindrical tube is not an accurate picture of what happens to the wind when it meets a wind turbine. This picture at the top of the page is the correct picture.

| The Power of the Wind: Cube of Wind Speed The wind speed is extremely important for the amount of energy a wind turbine can convert to electricity: The energy content of the wind varies with the cube (the third power) of the average wind speed, e.g. if the wind speed is twice as high it contains 2 3 = 2 x 2 x 2 = eight times as much energy. Now, why does the energy in the wind vary with the third power of wind speed? Well, from everyday knowledge you may be aware that if you double the speed of a car, it takes four times as much energy to brake it down to a standstill. (Essentially this is Newton's second law of motion). In the case of the wind turbine we use the energy from braking the wind, and if we double the wind speed, we get twice as many slices of wind moving through the rotor every second, and each of those slices contains four times as much energy, as we learned from the example of braking a car. The graph shows that at a wind speed of 8 metres per second we get a power (amount of energy per second) of 314 Watts per square metre exposed to the wind (the wind is coming from a direction perpendicular to the swept rotor area). At 16 m/s we get eight times as much power, i.e. 2509 W/m 2 . The table in the Reference Manual section gives you the power per square metre exposed to the wind for different wind speeds. |  |
| --- | --- |
|  | Power of the Wind Formula The power of the wind passing perpendicularly through a circular area is: P = 1/2 v3 r2 Where P = the power of the wind measured in W (Watt). = (rho) = the density of dry air = 1.225 measured in kg/m 3 (kilogrammes per cubic metre, at average atmospheric pressure at sea level at 15° C). v = the velocity of the wind measured in m/s (metres per second). = (pi) = 3.1415926535... r = the radius (i.e. half the diameter) of the rotor measured in m (metres). |

Wind Speed Measurement: Anemometers

The measurement of wind speeds is usually done using a cup anemometer, such as the one in the picture to the left. The cup anemometer has a vertical axis and three cups which capture the wind. The number of revolutions per minute is registered electronically.

Normally, the anemometer is fitted with a wind vane to detect the wind direction.

Instead of cups, anemometers may be fitted with propellers, although this is not common.

Other anemometer types include ultrasonic or laser anemometers which detect the phase shifting of sound or coherent light reflected from the air molecules. Hot wire anemometers detect the wind speed through minute temperature differences between wires placed in the wind and in the wind shade (the lee side).

The advantage of non-mechanical anemometers may be that they are less sensitive to icing. In practice, however, cup anemometers tend to be used everywhere, and special models with electrically heated shafts and cups may be used in arctic areas.

Quality Anemometers are a Necessity for Wind Energy Measurement

You often get what you pay for, when you buy something. That also applies to anemometers. You can buy surprisingly cheap anemometers from some of the major vendors in the business. They may be OK for meteorology, and they are OK to mount on a wind turbine, where a large accuracy is not really important. *) But cheap anemometers are not usable for wind speed measurement in the wind energy industry, since they may be very inaccurate and calibrated poorly, with measurement errors of maybe 5 per cent or even 10 per cent.

If you are planning to build a wind farm it may be an economic disaster if you have an anemometer which measures wind speeds with a 10% error. In that case, you may risk counting on an energy content of the wind which is 1.1 3 - 1 = 33% higher than than it is in reality. If you have to recalculate your measurements to a different wind turbine hub height (say, from 10 to 50 m height), you may even multiply that error with a factor of 1.3, thus you end up with a 75% error on your energy calculation.

It is possible to buy a professional, well calibrated anemometer with a measurement error around 1% for about 700-900 USD. That is quite plainly peanuts compared to the risk of making a potentially disastrous economic error. Naturally, price may not always be a reliable indicator of quality, so ask someone from a well reputed wind energy research institution for advice on purchasing anemometers.

*) The anemometer on a wind turbine is really only used to determine whether there is enough wind to make it worthwhile to yaw the turbine rotor against the wind and start it.

| Wind Speed Measurement in Practice The best way of measuring wind speeds at a prospective wind turbine site is to fit an anemometer to the top of a mast which has the same height as the expected hub height of the wind turbine to be used. This way one avoids the uncertainty involved in recalculating the wind speeds to a different height. By fitting the anemometer to the top of the mast one minimises the disturbances of airflows from the mast itself. If anemometers are placed on the side of the mast it is essential to place them in the prevailing wind direction in order to minimise the wind shade from the tower. Which Tower? Guyed, thin cylindrical poles are normally preferred over lattice towers for fitting wind measurement devices in order to limit the wind shade from the tower. The poles come as kits which are easily assembled, and you can install such a mast for wind measurements at (future) turbine hub height without a crane. Anemometer, pole and data logger (mentioned below) will usually cost somewhere around 5,000 USD. |  |  |
| --- | --- | --- |
| NRG data logger Photograph by Soren Krohn © 1998 DWIA |  | Data Logging The data on both wind speeds and wind directions from the anemometer(s) are collected on electronic chips on a small computer, a data logger, which may be battery operated for a long period. An example of such a data logger is shown to the left. Once a month or so you may need to go to the logger to collect the chips and replace them with blank chips for the next month's data. (Be warned: The most common mistake by people doing wind measurements is to mix up the chips and bring the blank ones back!) Arctic Conditions If there is much freezing rain in the area, or frost from clouds in mountains, you may need a heated anemometer, which requires an electrical grid connection to run the heater. 10 Minute Averages Wind speeds are usually measured as 10 minute averages, in order to be compatible with most standard software (and literature on the subject). The result for wind speeds are different, if you use different periods for averaging, as we'll see later. |

| The Wind Rose |  |
| --- | --- |
| Wind rose from Brest, France, taken from the European Wind Atlas, Riso National Laboratory, Denmark. | You will notice that strong winds usually come from a particular direction, as discussed in the Wind Energy Resource section. To show the information about the distributions of wind speeds, and the frequency of the varying wind directions, one may draw a so-called wind rose on the basis of meteorological observations of wind speeds and wind directions. The picture shows the wind rose for Brest, on the Atlantic coast of France. We have divided the compass into 12 sectors, one for each 30 degrees of the horizon. (A wind rose may also be drawn for 8 or 16 sectors, but 12 sectors tend to be the standard set by the European Wind Atlas, from which this image was taken). The radius of the 12 outermost, wide wedges gives the relative frequency of each of the 12 wind directions, i.e. how many per cent of the time is the wind blowing from that direction. The second wedge gives the same information, but multiplied by the average wind speed in each particular direction. The result is then normalised to add up to 100 per cent. This tells you how much each sector contributes to the average wind speed at our particular location. The innermost (red) wedge gives the same information as the first, but multiplied by the cube of the wind speed in each particular location. The result is then normalised to add up to 100 per cent. This tells you how much each sector contributes to the energy content of the wind at our particular location. Remember, that the energy content of the wind varies with the cube of the wind speed, as we discussed in the page on The Energy in the Wind. So the red wedges are really the most interesting ones. They tell us where to find the most power to drive our wind turbines. In this case we can see that the prevailing wind direction is Southwest, just as we would have predicted from the page on Global Winds. A wind rose gives you information on the relative wind speeds in different directions, i.e.each of the three sets of data (frequency, mean wind speed, and mean cube of wind speed) has been multiplied by a number which ensures that the largest wedge in the set exactly matches the radius of the outermost circle in the diagram. Wind Roses Vary Wind roses vary from one location to the next. They actually are a form of meteorological fingerprint. As an example, take a look at this wind rose from Caen, France, only about 150 km (100 miles) North of Brest. Although the primary wind direction is the same, Southwest, you will notice that practically all of the wind energy comes from West and Southwest, so on this site we need not concern ourselves very much about other wind directions. Wind roses from neighbouring areas are often fairly similar, so in practice it may sometimes be safe to interpolate (take an average) of the wind roses from surrounding observations. If you have complex terrain, i.e. mountains and valleys running in different directions, or coastlines facing in different directions, it is generally not safe to make simple assumptions like these. The wind rose, once again, only tells you the relative distribution of wind directions, not the actual level of the mean wind speed. How to Use the Wind Rose A look at the wind rose is extremely useful for siting wind turbines. If a large share of the energy in the wind comes from a particular direction, then you will want to have as few obstacles as possible, and as smooth a terrain as possible in that direction, when you place wind turbines in the landscape. In these examples most of the energy comes from the Southwest. We therefore need not be very concerned about obstacles to the East or Southeast of wind turbines, since practically no wind energy would come from those directions. You should note, however, that wind patterns may vary from year to year, and the energy content may vary (typically by some ten per cent) from year to year, so it is best to have observations from several years to make a credible average. Planners of large wind parks will usually rely on one year of local measurements, and then use long-term meteorological observations from nearby weather stations to adjust their measurements to obtain a reliable long term average. Since this wind rose comes from the European Wind Atlas we are reasonably confident that we can rely on it. The European Wind Atlas contains a description of each of the measurement stations, so we may be warned about possible local disturbances to the airflow. On the page on selecting a wind turbine site, we return to the pitfalls in using meteorology data. |

|  | Wind Rose Plotter Programme CALCULATOR Plot your own wind rose |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Do not operate the form until this page and its programme have loaded completely. The explanation of the wind rose may be found on the previous page. The Wind Frequency is the percentage of the time the wind is coming from a particular direction. The first row in the table to the left corresponds to North (the top wedge). The subsequent rows correspond to the sectors of the wind rose in a clockwise direction. Use Sectors. data. to Copenhagen data. Show wind frequency. Show wind speed. Show wind energy. |  |  |  |
|  |  |  | For each of the sectors the outermost (blue) wedges show the wind frequency distribution. |  | The middle (black) wedges show the distribution of the product of the two columns, i.e. the wind speeds times their frequency. | The innermost (red) wedges show the distribution of the wind speeds cubed (i.e. the energies) multiplied by their frequencies. |

| Roughness and Wind Shear |  |
| --- | --- |
|  | High above ground level, at a height of about 1 kilometre, the wind is hardly influenced by the surface of the earth at all. In the lower layers of the atmosphere, however, wind speeds are affected by the friction against the surface of the earth. In the wind industry one distinguishes between the roughness of the terrain, the influence from obstacles , and the influence from the terrain contours, which is also called the orography of the area. We shall be dealing with orography, when we investigate so called speed up effects, i.e. tunnel effects and hill effects , later. Roughness In general, the more pronounced the roughness of the earth's surface, the more the wind will be slowed down. Forests and large cities obviously slow the wind down considerably, while concrete runways in airports will only slow the wind down a little. Water surfaces are even smoother than concrete runways, and will have even less influence on the wind, while long grass and shrubs and bushes will slow the wind down considerably. Roughness Classes and Roughness Lengths |
| Sheep are a wind turbine's best friend. In this picture from Akaroa Spit, New Zealand, the sheep keep the roughness of the landscape down through their grazing. Photograph Soren Krohn © 1998 DWIA | In the wind industry, people usually refer to roughness classes or roughness lengths, when they evaluate wind conditions in a landscape. A high roughness class of 3 to 4 refers to landscapes with many trees and buildings, while a sea surface is in roughness class 0. Concrete runways in airports are in roughness class 0.5. The same applies to the flat, open landscape to the left which has been grazed by sheep. The proper definition of roughness classes and roughness lengths may be found in the Reference Manual. The term roughness length is really the distance above ground level where the wind speed theoretically should be zero. |
|  | Wind Shear This graph was plotted with the wind speed calculator on the next page. It shows you how wind speeds vary in roughness class 2 (agricultural land with some houses and sheltering hedgerows with some 500 m intervals), if we assume that the wind is blowing at 10 m/s at a height of 100 metres. The fact that the wind profile is twisted towards a lower speed as we move closer to ground level, is usually called wind shear. Wind shear may also be important when designing wind turbines. If you consider a wind turbine with a hub height of 40 metres and a rotor diameter of 40 metres, you will notice that the wind is blowing at 9.3 m/s when the tip of the blade is in its uppermost position, and only 7.7 m/s when the tip is in the bottom position. This means that the forces acting on the rotor blade when it is in its top position are far larger than when it is in its bottom position. Wind Shear Formula *) The wind speed at a certain height above ground level is: v = v ref ln(z/z 0 )/ln(z ref /z 0 ) v = wind speed at height z above ground level. v ref = reference speed, i.e. a wind speed we already know at height z ref . ln(...) is the natural logarithm function. z = height above ground level for the desired velocity, v. z 0 = roughness length in the current wind direction. Roughness lengths may be found in the Reference Manual. z ref = reference height, i.e. the height where we know the exact wind speed v ref . In the above example, assume we know that the wind is blowing at 7.7 m/s at 20 m height. We wish to know the wind speed at 60 m height. If the roughness length is 0.1 m, then v ref = 7.7 z = 60 z 0 = 0.1 z ref = 20 hence, v = 7.7 ln(60/0.1) / ln(20/0.1) = 9.2966 m/s *) = The formula assumes so-called neutral atmospheric stability conditions, i.e. that the ground surface is neither heated nor cooled compared to the air temperature. Further details may be found in the engineering handbook Guidelines for Design of Wind Turbines from Risoe National Laboratory and DNV. |
| Wind Speed Calculator Do not operate the form until this page and its programme have loaded completely. Enter your wind speed measurement in any column at the appropriate height, e.g. 10 metres. Then click outside the field, click Submit, or use the tab key. The programme will then calculate wind speeds for other heights. You may plot your results in a separate window by clicking on Plot in the appropriate column. (If the plot window disappears, it is probably hidden behind this window). |  |

CALCULATOR

Top of Form

| roughness - class - length m | 0.0 0.0002 | 0.5 0.0024 | 1.0 0.03 | 1.5 0.055 | 2.0 0.1 | 3.0 0.4 | 4.0 1.6 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 150 m |  |  |  |  |  |  |  |  |  |
| 140 m |  |  |  |  |  |  |  |  |  |
| 130 m |  |  |  |  |  |  |  |  |  |
| 120 m |  |  |  |  |  |  |  |  |  |
| 110 m |  |  |  |  |  |  |  |  |  |
| 100 m |  |  |  |  |  |  |  |  |  |
| 90 m |  |  |  |  |  |  |  |  |  |
| 80 m |  |  |  |  |  |  |  |  |  |
| 70 m |  |  |  |  |  |  |  |  |  |
| 60 m |  |  |  |  |  |  |  |  |  |
| 50 m |  |  |  |  |  |  |  |  |  |
| 40 m |  |  |  |  |  |  |  |  |  |
| 30 m |  |  |  |  |  |  |  |  |  |
| 20 m |  |  |  |  |  |  |  |  |  |
| 10 m |  |  |  |  |  |  |  |  |  |
|  | Plot | Plot | Plot | Plot | Plot | Plot | Plot |  |  |

Bottom of Form

|  | Average wind speeds are often available from meteorological observations measured at a height of 10 metres. Hub heights of modern 600 to 1,500 kW wind turbines are usually 40 to 80 metres, however. The spreadsheet will calculate average wind speeds at different heights and roughness classes. Just enter a wind speed measured at a certain height for a given roughness class and click the Submit button. Please note, that the results are not strictly valid if there are obstacles close to the wind turbine (or the point of meteorological measurement) at or above the specified hub height. ["close" means anything up to one kilometre]. You should also note, that there may be inverse wind shear on hilltops because of the hill effect , i.e. the wind speed may actually decline with increasing height during a certain height interval above the hilltop. You should consult the European Wind Atlas mentioned in the bibliography in the Reference Manual for further information on this phenomenon. Take a look at the example below the table to make sure you understand how it works, before you start entering your data. More accurate and extensive roughness definitions may be found in the units section. An Example As an example, have a look at the spreadsheet above. We have already entered 10 m/s at 100 m height in roughness class 2. You will notice that the wind speed declines as you approach ground level. You will also notice that it declines more rapidly in rough terrain. Remember that the energy content of the wind varies with the third power of the wind speed. If you look at the column with roughness class 2, you will see that wind speeds declines 10 per cent going from 100 metres to 50 metres. But the power of the wind declines to 0.9 3 = 0.73, i.e. by 27 per cent. (From 613 to 447 W/m 2 ). If you compare the wind speeds below 100 m in roughness class 2 with roughness class 1, you will notice that for a given height the wind speeds are lower everywhere in roughness class 2. If you have a wind turbine in roughness class 2, you may consider whether it is worthwhile to invest 15,000 USD extra to get a 60 metre tower instead of a 50 metre tower. In the table you can see that it will give you 2.9 per cent more wind, and you can calculate, that it will give you 9 per cent more wind energy. You can solve this problem once you have learned how the turbine electricity production varies with the available wind energy. We will return to that question when you have learned to use the power density calculator and the wind energy economics calculator. |  |  |
| --- | --- | --- | --- |
| Wind Shear and Escarpments |  |  |  |
| Aerial photograph Soren Krohn © 1999 DWIA | Do not Include the Altitude of Your Terrain in Wind Shear Calculations The aerial photograph above shows a good site for wind turbines along a shoreline with the turbines standing on a cliff which is about 10 m (30 ft.) tall. It is a common mistake to believe that in this case one may add the height of the cliff to the height of the wind turbine tower to obtain the effective height of the wind turbine, when one is doing wind speed calculations, at least when the wind is coming from the sea. This is patently wrong. The cliff in the front of the picture will create turbulence , and brake the wind even before it reaches the cliff. It is therefore not a good idea to move the turbines closer to the cliff. That would most likely lower energy output, and cause a lower lifetime for the turbines, due to more tear and wear from the turbulence. If we had the choice, we would much rather have a nicely rounded hill in the direction facing the sea, rather than the escarpment you see in the picture. In case of a rounded hill, we might even experience a speed up effect, as we explain later when we get to the page on the hill effect. |  |  |
| The Roughness Rose |  |  |  |
|  | If we have measured the wind speed exactly at hub height over a long period at the exact spot where a wind turbine will be standing we can make very exact predictions of energy production. Usually, however, we have to recalculate wind measurements made somewhere else in the area. In practice, that can be done with great accuracy, except in cases with very complex terrain (i.e. very hilly, uneven terrain). Just like we use a wind rose to map the amount of wind energy coming from different directions, we use a roughness rose to describe the roughness of the terrain in different directions from a prospective wind turbine site. Normally, the compass is divided into 12 sectors of 30 degrees each, like in the picture to the left, but other divisions are possible. In any case, they should match our wind rose, of course. For each sector we make an estimate of the roughness of the terrain, using the definitions from the Reference Manual section. In principle, we could then use the wind speed calculator on the previous page to estimate for each sector how the average wind speed is changed by the different roughness of the terrain. Averaging Roughness in Each Sector In most cases, however, the roughness will not fall neatly into any of the roughness classes, so we'll have to do a bit of averaging. We have to be very concerned with the roughness in the prevailing wind directions. In those directions we look at a map to measure how far away we have unchanged roughness. Photograph Soren Krohn, © 1999 DWIA Accounting for Roughness Changes Within Each Sector Let us imagine that we have a sea or lake surface in the western sector (i.e. roughness class 0) some 400 m from the turbine site, and 2 kilometres away we have a forested island. If west is an important wind direction, we will definitely have to account for the change in roughness class from 1 to 0 to 3. This requires more advanced models and software than what we have shown on this web site. It is also useful to be able to use the software to manage all our wind and turbine data, so at a future update of this site we'll explain how professional wind calculation software works. Meanwhile, you may look at the Links page to find the link to Risoe's WAsP model and Energy & Environmental Data's WindPro Windows-based software. Accounting for Wind Obstacles It is extremely important to account for local wind obstacles in the prevailing wind direction near the turbine (closer than 700 m or so), if one wants to make accurate predictions about energy output. We return to that subject after a couple of pages. |  |  |
| Wind Speed Variability Short Term Variability of the Wind The wind speed is always fluctuating, and thus the energy content of the wind is always changing. Exactly how large the variation is depends both on the weather and on local surface conditions and obstacles. Energy output from a wind turbine will vary as the wind varies, although the most rapid variations will to some extent be compensated for by the inertia of the wind turbine rotor. |  |  |  |
|  |  |  | Diurnal (Night and Day) Variations of the Wind In most locations around the globe it is more windy during the daytime than at night. The graph to the left shows how the wind speed at Beldringe, Denmark varies by 3 hour intervals round the clock. (Information from the European Wind Atlas). This variation is largely due to the fact that temperature differences e.g. between the sea surface and the land surface tend to be larger during the day than at night. The wind is also more turbulent and tends to change direction more frequently during the day than at night. From the point of view of wind turbine owners, it is an advantage that most of the wind energy is produced during the daytime, since electricity consumption is higher than at night. Many power companies pay more for the electricity produced during the peak load hours of the day (when there is a shortage of cheap generating capacity). We will return to this subject in the section on Wind Turbines in the Electrical grid. Seasonal Variations of the Wind We treat this subject in the section on Wind Turbines in the Electrical grid. |

Turbulence

You have probably experienced how hailstorms or thunderstorms in particular, are associated with frequent gusts of wind which both change speed and direction.

In areas with a very uneven terrain surface, and behind obstacles such as buildings there is similarly created a lot of turbulence, with very irregular wind flows, often in whirls or vortexes in the neighbourhood.

You can see an example of how turbulence increases the fluctuations in the wind speed in the image, which you may compare with the image on the previous page.

Turbulence decreases the possibility of using the energy in the wind effectively for a wind turbine. It also imposes more tear and wear on the wind turbine, as explained in the section on fatigue loads. Towers for wind turbines are usually made tall enough to avoid turbulence from the wind close to ground level.

| Wind Obstacles This movie was shot at a coastal wind site with the wind coming from the right side of the picture. It shows an interesting phenomenon: We would really expect the wind turbine to the right (which is facing the wind directly) to be the one to start first when the wind starts blowing. But you can see, that the wind turbine to the right will not start at the low wind speeds which are sufficient to drive the other two wind turbines. The reason is the small wood in front of the wind turbines which shelters the rightmost turbine in particular. In this case, the annual production of these wind turbines is probably reduced by some 15 per cent on average, and even more in case of the rightmost turbine. (The turbines are located some five rotor diameters apart, and the wood is located at a similar distance from the first wind turbine. The reason why the turbines look like they are standing very close together, is that the movie was shot from about a mile away with the equivalent of a 1200 mm lens for a 35 mm camera). |  |  |
| --- | --- | --- |
| Side view of wind flow around an obstacle. Note the pronounced turbulent airflow downstream |  | Obstacles to the wind such as buildings, trees, rock formations etc. can decrease wind speeds significantly, and they often create turbulence in their neighbourhood. As you can see from this drawing of typical wind flows around an obstacle, the turbulent zone may extend to some three time the height of the obstacle. The turbulence is more pronounced behind the obstacle than in front of it. Therefore, it is best to avoid major obstacles close to wind turbines, particularly if they are upwind in the prevailing wind direction, i.e. "in front of" the turbine. |
| Top view of wind flow around an obstacle. |  | Shelter Behind Obstacles Obstacles will decrease the wind speed downstream from the obstacle. The decrease in wind speed depends on the porosity of the obstacle, i.e. how "open" the obstacle is. (Porosity is defined as the open area divided by the total area of the object facing the wind). A building is obviously solid, and has no porosity, whereas a fairly open tree in winter (with no leaves) may let more than half of the wind through. In summer, however, the foliage may be very dense, so as to make the porosity less than, say one third. The slowdown effect on the wind from an obstacle increases with the height and length of the obstacle. The effect is obviously more pronounced close to the obstacle, and close to the ground. When manufacturers or developers calculate the energy production for wind turbines, they always take obstacles into account if they are close to the turbine - say, less than 1 kilometre away in one of the more important wind directions. |

Wind Shade

This graph gives you an estimate of how wind speeds decrease behind a blunt obstacle, i.e. an obstacle which is not nicely streamlined. In this case we use a seven story office building, 20 metres tall and 60 metres wide placed at a distance of 300 m from a wind turbine with a 50 m hub height. You can quite literally see the wind shade as different shades of grey. The blue numbers indicate the wind speed in per cent of the wind speed without the obstacle.

At the top of the yellow wind turbine tower the wind speed has decreased by some 3 per cent to 97 per cent of the speed without the obstacle. You should note that this means a loss of wind energy of some 10 per cent, i.e. 1.03 3 - 1, as you may see in the graph at the bottom of this page.

If you have a reasonably fast computer (or a bit of patience with a slower one) you can plot tables and graphs like this one using the wind shade calculator in a couple of pages.

Guide to the Wind Shade Calculator

The calculator will quickly give you the result at hub height at the distance from the obstacle you specify. If you use the plot facility, your computer will also calculate 620 different measurement points at different heights and distances from your obstacle.

Turbine Hub Height

The higher you are above the top of the obstacle, the less wind shade. The wind shade, however, may extend to up to five times the height of the obstacle at a certain distance.

If the obstacle is taller than half the hub height, the results are more uncertain, because the detailed geometry of the obstacle, (e.g. differing slopes of the roof on buildings) will affect the result. In that case the programme will put a warning in the text box below the results.

Distance Between Obstacle and Turbine

The distance between the obstacle and the turbine is very important for the shelter effect. In general, the shelter effect will decrease as you move away from the obstacle, just like a smoke plume becomes diluted as you move away from a smokestack. In terrain with very low roughness (e.g. water surfaces) the effect of obstacles (e.g. an island) may be measurable up to 20 km away from the obstacle.

If the turbine is closer to the obstacle than five times the obstacle height, the results will be more uncertain, because they will depend on the exact geometry of the obstacle. In that case the programme will put a warning in the text box below the results.

Roughness Length or Roughness Class

The roughness of the terrain between the obstacle and the wind turbine has an important influence on how much the shelter effect is felt. Terrain with low roughness will allow the wind passing outside the obstacle to mix more easily in the wake behind the obstacle, so that it makes the wind shade relatively less important.

It may be a bit confusing at first, that we both deal with the roughness of the terrain, and with individual obstacles. A good rule of thumb is that we deal with individual obstacles which are closer than about 1000 metres from the wind turbine in the prevailing wind directions. The rest we deal with as changes in roughness classes.

Obstacle Height

The taller the obstacle, the larger the wind shade.

As we have mentioned above, if the turbine is closer to the obstacle than five times the obstacle height, or if the obstacle is taller than half the hub height, the results will be more uncertain, because they will depend on the exact geometry of the obstacle. In that case the programme will put a warning in the text box below the results.

Obstacle Width

The obstacle calculation model works on the basis of the assumption that obstacles are infinitely long, and that they are placed at a right angle (perpendicular) to the wind direction.

A very narrow object will of course cast a far smaller wind shade than a large one. For practical reasons we assume that we investigate the horizon around the wind turbine in twelve 30 degree sections.

At the bottom of the drawing on the right side of the wind shade calculator we illustrate (in 10 per cent steps) how much space the obstacle take up in such a 30 degree section. You may adjust the width of the obstacle in 10 per cent steps by clicking on the squares at the bottom of the graph.

You may also type the exact length of the obstacle (as seen from the wind turbine) directly, or you may enter the percentage of the sector width that the object fills up.

Porosity

= 0% 
= 30% 
= 50% 
= 70%

A tree without leaves will brake the wind far less than a building. Trees with dense foliage will have a braking effect somewhere in between. In general, the wind shade will be proportional to (one minus the porosity of the obstacle).

The porosity of an obstacle is a percentage indication of how open an obstacle is, i.e. how easily the wind can pass through it. A building obviously has a zero porosity. A group of buildings with some space between them with have a porosity equal to (the area of the open space) divided by (the total area of both buildings and the open space in between, as seen from the wind turbine).

You may either specify the porosity directly in the calculator, click on one of the buttons with the symbols shown above, or use the pop up menu for suggested settings for different objects.

Control Buttons

Submit calculates your latest input. You may use the tab key or just click outside the field you change instead.

Plot Wind Speed gives you a graph and a table of the percentage of the remaining wind speed at a number of heights and distances up to 1.5 times the height and distance of your wind turbine hub. The turbine tower is shown in yellow. The calculations are quite complex, so be patient if your computer is slow.

Plot Wind Energy gives you a graph and a table of the percentage of the remaining wind energy at a number of heights and distances up to 1.5 times the height and distance of your wind turbine hub. The turbine tower is shown in yellow. The calculations are quite complex, so be patient if your computer is slow.

Plot Speed Profile gives you a plot of the wind speed profile at different heights up to 100 m at the distance where you have placed your turbine. You can see directly on the red curve how the obstacle makes the wind speed drop. You can enter any wind speed you like for the hub height. (The shape of the curve remains the same, which is should, since obstacles cause a relative change in wind speed). The curve corresponds to the curves drawn by the wind speed calculator.

Results

The result line in the calculator tells you how many per cent the wind speed will decline due to the presence of the obstacle. You may plot the change in wind speeds for a number of distances and heights up to 1.5 times your present distance and height by clicking the Plot Wind Speed button.

(If you are working with a specific Weibull distribution describing the wind in this particular sector, the change in wind speed corresponds to a change in the scale factor A. If you use the results of these calculations to find a Weibull distribution, you can just adjust the scale factor, A, with this change. The shape factor, k, remains unchanged. You will get to the Weibull distribution later in this Guided Tour, when we explore how to compute the energy output from a wind turbine).

The result line also tells you the loss of wind energy due to the presence of the obstacle. You may plot the change in wind energy for a number of distances and heights up to 1.5 times your present distance and height by clicking the Plot Wind Speed button.

More Complex Obstacle Calculations

Obstacles may not be perpendicular to the centreline in the sector, and there may be several rows of obstacles. Although you can still use the basic methods in the calculator, you would probably want to use a professional wind assessment programme such as WindPro or WAsP to manage your data in such cases.

The methods used in the wind calculator are based on the European Wind Atlas. If you read chapter 8, however, you should note that there is a misprint in formula 8.25.

| Wind Shade Calculator Do not operate the form until this page and its programme have loaded completely. If you are too fast, the programme will complain about missing data, and you will have to click reload. You should have read about obstacles , roughness and porosity before using the calculator. CALCULATOR |  |  |  | Top of Form |
| --- | --- | --- | --- | --- |
| Turbine hub height m |  | Click in grey squares to insert or remove obstacles Energy in per cent of undisturbed airflow. 70; 75; 80; 85; 90; 95; 100. Select obstacle porosity: 0%= 30%= 50%= 70%= |  |  |
| Distance between obstacle and turbine m |  |  |  |  |
| Roughness length m = roughness class |  |  |  |  |
| Obstacle height m |  |  |  |  |
| Obstacle width m = % of sector width |  |  |  |  |
| Porosity % |  |  |  |  |
| for m/s hub height wind speed |  |  |  |  |
| Result: % wind speed decrease* = % energy loss in this sector* |  |  |  |  |
| * To print the results of the plotter programme you should make a screen dump |  |  |  |  |
| This calculator shows the shelter effect (wind shade) of blunt obstacles (buildings, trees) in any 30 degree sector near a wind turbine. You can change any number, except the results which are labelled with *. If the obstacle is too tall (more than half the hub height of your turbine) - or too close (less than five times the height of the obstacle) the programme will warn you that the results are uncertain, because the detailed geometry of the obstacle and the angle of the wind will have an important influence on the resulting effect. Please note that you only have to consider the percentage of wind energy coming from this direction cf. the wind rose, because the obstacle obviously only affects your turbine's energy output when the wind is coming from this particular direction. If you have a fast computer or some patience you may plot the wind speed or wind energy profile behind the obstacle. (If the plot window disappears, it is probably hidden behind another window). Bottom of Form |  |  |  |  |

| Wake Effect Since a wind turbine generates electricity from the energy in the wind, the wind leaving the turbine must have a lower energy content than the wind arriving in front of the turbine. |  |
| --- | --- |
| Wake effect from wind turbine Picture © Riso National Laboratory, Denmark | This follows directly from the fact that energy can neither be created nor consumed. If this sounds confusing, take a look at the definition of energy in the Reference Manual. A wind turbine will always cast a wind shade in the downwind direction. In fact, there will be a wake behind the turbine, i.e. a long trail of wind which is quite turbulent and slowed down, when compared to the wind arriving in front of the turbine. (The expression wake is obviously derived from the wake behind a ship). You can actually see the wake trailing behind a wind turbine, if you add smoke to the air passing through the turbine, as was done in the picture. (This particular turbine was designed to rotate in a counterclockwise direction which is somewhat unusual for modern wind turbines). Wind turbines in parks are usually spaced at least three rotor diameters from one another in order to avoid too much turbulence around the turbines downstream. In the prevailing wind direction turbines are usually spaced even farther apart, as explained on the next page. |

Park Effect

As we saw in the previous section on the wake effect , each wind turbine will slow down the wind behind it as it pulls energy out of the wind and converts it to electricity.

Ideally, we would therefore like to space turbines as far apart as possible in the prevailing wind direction. On the other hand, land use and the cost of connecting wind turbines to the electrical grid would tell us to space them closer together.

Park Layout

As a rule of thumb, turbines in wind parks are usually spaced somewhere between 5 and 9 rotor diameters apart in the prevailing wind direction, and between 3 and 5 diameters apart in the direction perpendicular to the prevailing winds.

In this picture we have placed three rows of five turbines each in a fairly typical pattern.

The turbines (the white dots) are placed 7 diameters apart in the prevailing wind direction, and 4 diameters apart in the direction perpendicular to the prevailing winds.

Energy Loss from the Park Effect

With knowledge of the wind turbine rotor, the wind rose , the Weibull distribution and the roughness in the different directions manufacturers or developers can calculate the energy loss due to wind turbines shading one another.

Typically, the energy loss will be somewhere around 5 per cent.

Speed Up Effects: Tunnel Effect

If you push an ordinary bicycle air pump, (just point to the image, do not click) you will notice that the air leaving the nozzle moves much faster than the speed with which you are pushing. The reason, of course, is that the nozzle is much narrower than the cylinder in the pump.

Tunnel Effect

If you take a walk between tall buildings, or in a narrow mountain pass, you will notice that the same effect is working:

The air becomes compressed on the windy side of the buildings or mountains, and its speed increases considerably between the obstacles to the wind. This is known as a "tunnel effect".

So, even if the general wind speed in open terrain may be, say, 6 metres per second, it can easily reach 9 metres per second in a natural "tunnel".

Placing a wind turbine in such a tunnel is one clever way of obtaining higher wind speeds than in the surrounding areas.

To obtain a good tunnel effect the tunnel should be "softly" embedded in the landscape. In case the hills are very rough and uneven, there may be lots of turbulence in the area, i.e. the wind will be whirling in a lot of different (and rapidly changing) directions.

If there is much turbulence it may negate the wind speed advantage completely, and the changing winds may inflict a lot of useless tear and wear on the wind turbine.

| Speed Up Effects: Hill Effect |  |  |  |
| --- | --- | --- | --- |
| The wind in passing the summits of mountains becomes swift and dense and as it blows beyond the mountains it becomes thin and slow, like water that issues from a narrow channel into the wide sea. Notebooks of Leonardo da Vinci (1452-1519) |  | A common way of siting wind turbines is to place them on hills or ridges overlooking the surrounding landscape. In particular, it is always an advantage to have as wide a view as possible in the prevailing wind direction in the area. On hills, one may also experience that wind speeds are higher than in the surrounding area. Once again, this is due to the fact that the wind becomes compressed on the windy side of the hill, and once the air reaches the ridge it can expand again as its soars down into the low pressure area on the lee side of the hill. |  |
|  |  |  | You may notice that the wind in the picture starts bending some time before it reaches the hill, because the high pressure area actually extends quite some distance out in front of the hill. Also, you may notice that the wind becomes very irregular, once it passes through the wind turbine rotor. As before, if the hill is steep or has an uneven surface, one may get significant amounts of turbulence , which may negate the advantage of higher wind speeds. |

| Selecting a Wind Turbine Site |  |
| --- | --- |
| Photograph Soren Krohn © 1997 DWIA | Wind Conditions Looking at nature itself is usually an excellent guide to finding a suitable wind turbine site. If there are trees and shrubs in the area, you may get a good clue about the prevailing wind direction , as you do in the picture to the left. If you move along a rugged coastline, you may also notice that centuries of erosion have worked in one particular direction. Meteorology data, ideally in terms of a wind rose calculated over 30 years is probably your best guide, but these data are rarely collected directly at your site, and here are many reasons to be careful about the use of meteorology data, as we explain in the next section. If there are already wind turbines in the area, their production results are an excellent guide to local wind conditions. In countries like Denmark and Germany where you often find a large number of turbines scattered around the countryside, manufacturers can offer guaranteed production results on the basis of wind calculations made on the site. Look for a view As you have learned from the previous pages, we would like to have as wide and open a view as possible in the prevailing wind direction, and we would like to have as few obstacles and as low a roughness as possible in that same direction. If you can find a rounded hill to place the turbines, you may even get a speed up effect in the bargain. Grid Connection Obviously, large wind turbines have to be connected to the electrical grid. For smaller projects, it is therefore essential to be reasonably close to a 10-30 kilovolt power line if the costs of extending the electrical grid are not to be prohibitively high. (It matters a lot who has to pay for the power line extension, of course). The generators in large, modern wind turbines generally produce electricity at 690 volts. A transformer located next to the turbine, or inside the turbine tower, converts the electricity to high voltage (usually 10-30 kilovolts). Grid Reinforcement The electrical grid near the wind turbine(s) should be able to receive the electricity coming from the turbine. If there are already many turbines connected to the grid, the grid may need reinforcement, i.e. a larger cable, perhaps connected closer to a higher voltage transformer station. Read the section on Electrical Grid Issues for further information. Soil Conditions Both the feasibility of building foundations of the turbines, and road construction to reach the site with heavy trucks must be taken into account with any wind turbine project. Pitfalls in Using Meteorology Data Meteorologists already collect wind data for weather forecasts and aviation, and that information is often used to assess the general wind conditions for wind energy in an area. Precision measurement of wind speeds, and thus wind energy is not nearly as important for weather forecasting as it is for wind energy planning, however. Wind speeds are heavily influenced by the surface roughness of the surrounding area, of nearby obstacles (such as trees, lighthouses or other buildings), and by the contours of the local terrain. Unless you make calculations which compensate for the local conditions under which the meteorology measurements were made, it is difficult to estimate wind conditions at a nearby site. In most cases using meteorology data directly will underestimate the true wind energy potential in an area. We'll return to how the professionals do their wind speed calculations on the following pages. |

|  | Offshore Wind Conditions |
| --- | --- |
| 500 kW offshore wind turbine at Tunø Knob, Denmark. Photograph © 1996 Vestas Wind Systems A/S | Wind Conditions at Sea The surfaces of seas and lakes are obviously very smooth, thus the roughness of a seascape is very low (at constant wind speeds). With increasing wind speeds some of the energy in the wind is used to build waves, i.e. the roughness increases. Once the waves have been built up, the roughness decreases again. We thus have a surface with varying roughness, (just as you have it in areas covered with more or less snow). Generally speaking, however, the roughness of the water surface is very low, and obstacles to the wind are few. When doing wind calculations we have to account for islands, lighthouses etc. just like you would account for upwind obstacles or changes in roughness on land. Low Wind Shear Means Lower Hub Height With low roughness, wind shear at sea is very low, i.e. the wind speed does not change very much with changes in the hub height of wind turbines. It may therefore be most economic to use fairly low towers of perhaps 0.75 times the rotor diameter for wind turbines located at sea, depending upon local conditions. (Typically towers on land sites are about the size of the rotor diameter, or taller). Low Turbulence Intensity = Longer Lifetime for Turbines The wind at sea is generally less turbulent than on land. Wind turbines located at sea may therefore be expected to have a longer lifetime than land based turbines. The low turbulence at sea is primarily due to the fact that temperature variations between different altitudes in the atmosphere above the sea are smaller than above land. Sunlight will penetrate several metres below the sea surface, whereas on land the radiation from the sun only heats the uppermost layer of the soil, which thus becomes much warmer. Consequently the temperature difference between the surface and the air will be smaller above sea than above land. This is the reason for lower turbulence. Wind Shade Conditions at Sea The conventional WAsP model used for onshore wind modelling is in the process of being modified for offshore wind conditions, according to its developer, Risø National Laboratory. The different production results obtained from the experience of the first major offshore wind park at Vindeby, Denmark , and the subsequently built wind park at Tunø Knob, Denmark , has led to new investigations with anemometer masts being placed offshore in a number of locations in Danish waters since 1996. The preliminary results indicate that wind shade effects from land may be more important, even at distances up to 20 kilometres, than was previously thought. On the other hand, it appear that the offshore wind resource may be some 5 to 10 per cent higher than was previously estimated. |

| Wind Map of Western Europe |  |  |
| --- | --- | --- |
| Wind Resources at 50 (45) m Above Ground Level. Colour; Sheltered terrain; Open plain; At a sea coast; Open sea; Hills and ridges. |
|  |  | How to Read the Wind Map of Western Europe This wind map of Western Europe was originally published as part of the European Wind Atlas. The details on how to interpret the colours are given in the legend above. Please note that the data for Norway, Sweden and Finland are from a later study, and are calculated for 45 m height above ground level, and assume an open plain. The purple zones are the areas with the strongest winds while the blue zones have the weakest winds. The dividing lines between the different zones are not as sharp as they appear on the map. In reality, the areas tend to blend smoothly into one another. You should note, however, that the colours on the map assume that the globe is round without obstacles to the wind, speed up effects , or varying roughness of the terrain. You may therefore easily find good, windy sites for wind turbines on hills and ridges in, say the yellow or green areas of the map, while you have little wind in sheltered terrain in the purple areas. The Power of the Wind In case you cannot explain why the calculated mean power of the wind in the table is approximately twice the power of the wind at the given mean wind speed, you should read the four to six pages starting with the Weibull Distribution. Reality is More Complicated Actual local differences in the terrain will mean that the picture will be much more complicated, if we take a closer look. As an example, we will now take a closeup view of Denmark on the next page. |

| Describing Wind Variations: Weibull Distribution The General Pattern of Wind Speed Variations It is very important for the wind industry to be able to describe the variation of wind speeds. Turbine designers need the information to optimise the design of their turbines, so as to minimise generating costs. Turbine investors need the information to estimate their income from electricity generation. If you measure wind speeds throughout a year, you will notice that in most areas strong gale force winds are rare, while moderate and fresh winds are quite common. The wind variation for a typical site is usually described using the so-called Weibull distribution, as shown in the image. This particular site has a mean wind speed of 7 metres per second, and the shape of the curve is determined by a so called shape parameter of 2. Statistical Description of Wind Speeds People who are familiar with statistics will realise that the graph shows a probability density distribution. The area under the curve is always exactly 1, since the probability that the wind will be blowing at some wind speed including zero must be 100 per cent. Half of the blue area is to the left of the vertical black line at 6.6 metres per second. The 6.6 m/s is called the median of the distribution. This means that half the time it will be blowing less than 6.6 metres per second, the other half it will be blowing faster than 6.6 metres per second. You may wonder then, why we say that the mean wind speed is 7 metres per second. The mean wind speed is actually the average of the wind speed observations we will get at this site. As you can see, the distribution of wind speeds is skewed, i.e. it is not symmetrical. Sometimes you will have very high wind speeds, but they are very rare. Wind speeds of 5.5 metres per second, on the other hand, are the most common ones. 5.5 metres is called the modal value of the distribution. If we multiply each tiny wind speed interval by the probability of getting that particular wind speed, and add it all up, we get the mean wind speed. The statistical distribution of wind speeds varies from place to place around the globe, depending upon local climate conditions, the landscape, and its surface. The Weibull distribution may thus vary, both in its shape, and in its mean value. If the shape parameter is exactly 2, as in the graph on this page, the distribution is known as a Rayleigh distribution. Wind turbine manufacturers often give standard performance figures for their machines using the Rayleigh distribution. |  |  |
| --- | --- | --- |
|  |  | Balancing the Weibull Distribution Another way of finding the mean wind speed is to balance the pile of blue bricks to the right, which shows exactly the same as the graph above. Each brick represents the probability that the wind will be blowing at that speed during 1 per cent of the time during the year. 1 m/s wind speeds are in the pile to the far left, 17 m/s is to the far right. The point at which the whole pile will balance exactly will be at the 7th pile, i.e. the mean wind speed is 7 m/s. Try This! If you have a Netscape 3, 4 or Internet Explorer 4 browser, the next page will let you experiment with different values for the Weibull parameters to get a grasp of what the wind speed probability distribution looks like. |

Weibull Distribution Plotter Programme

Top of Form

Choose between entering 
mean wind speed (2.0-12.0 m/s) or 
scale parameter A in the first box 
, then enter shape k = (1.0-3.0). 
Click to draw.

Bottom of Form

This page will give you an idea of the way different Weibull distributions look. The mean wind speed or the scale parameter, A, is used to indicate how windy the site is, on average. The shape parameter, k, tells how peaked the distibution is, i.e. if the wind speeds always tend to be very close to a certain value, the distibution will have a high k value, and be very peaked.

Start by clicking Weibull to see the result of our example on the previous page. Then try changing one parameter at a time, and watch what happens.

To print the results of the plotter programme you should make a screen dump.

The Average Bottle Fallacy

What is the average energy content of the wind at your wind turbine site?

Most people who are new to wind energy think they could easily live without the Weibull distribution. After all, if we know the average wind speed, we also know the average power of the wind, don't we? So, can't we just use the power (or energy) at the mean wind speed to figure out how much power (or energy) will hit the wind turbine?

In other words, couldn't we just say, that with an average wind speed of 7 m/s we get an average power input of 210 Watts per square metre of rotor area? (You may find that figure in the table on the power of the wind in the Reference Manual ).

The answer is no! We would underestimate wind resources by almost 100 per cent. If we did that, we would be victims of what we could call the Average Bottle Fallacy: Look at the smallest and largest bottle in the picture. Both have exactly the same shape. One is 0.24 m tall, the other is 0.76 m tall. How tall is the average bottle?

If you answer 0.5 m tall, you are a victim of the Average Bottle Fallacy. Bottles are interesting because of their volume, of course. But the volume varies with the cube (the third power) of their size. So, even though the largest bottle is only 3.17 times larger than the small bottle, its volume is actually 3.17 3 =32 times larger than the small bottle.

The average volume is therefore 16.5 times that of the small bottle. This means that a bottle with an average volume would have to be 2.55 times the height of the small bottle, i.e. 0.61 m tall. (Since 2.55 3 = 16.5).

The point we are trying to make, is that you cannot simply take an average of wind speeds, and then use the average wind speed for your power calculations. You have to weigh each wind speed probability with the corresponding amount of power. On the next two pages we shall calculate the energy in the wind. First we use the bottle example to grasp the idea, then we use simple math.

Mean (Average) Power of the Wind

Balancing the Power Distribution

The reason why we care about wind speeds is their energy content, just like with the bottles on the previous page: We cared about their content in terms of volume. Now, the volume of a bottle varies with the cube of the size, just like wind power varies with the cube of the wind speed. Let us take the Weibull distribution of wind speeds, and for each speed we place a bottle on a shelf each time we have a 1 per cent probability of getting that wind speed. The size of each bottle corresponds to the wind speed, so the weight of each bottle corresponds to the amount of energy in the wind.

To the right, at 17 m/s we have some really heavy bottles, which weigh almost 5000 times as much as the bottles at 1 m/s. (At 1 m/s the wind has a power of 0.61 W/m 2 . At 17 m/s its power is 3009 W/m 2 ).

Finding the wind speed at which we get the mean of the power distribution is equivalent to balancing the bookshelves. (Remember how we did the balancing act on the Weibull distribution page ?). In this case, as you can see, although high winds are rare, they weigh in with a lot of energy.

So, in this case with an average wind speed of 7 m/s, the power weighted average of wind speeds is 8.7 m/s. At that wind speed the power of the wind is 402 W/m 2 , which is almost twice as much as we figured out in our naive calculation on the top of the previous page.

Betz' Law

The Ideal Braking of the Wind

The more kinetic energy a wind turbine pulls out of the wind, the more the wind will be slowed down as it leaves the left side of the turbine in the picture. (If you wonder about the stream tube in the picture, you have not read the page on how the wind turbine deflects the wind ).

If we tried to extract all the energy from the wind, the air would move away with the speed zero, i.e. the air could not leave the turbine. In that case we would not extract any energy at all, since all of the air would obviously also be prevented from entering the rotor of the turbine.

In the other extreme case, the wind could pass though our tube above without being hindered at all. In this case we would likewise not have extracted any energy from the wind.

We can therefore assume that there must be some way of braking the wind which is in between these two extremes, and is more efficient in converting the energy in the wind to useful mechanical energy. It turns out that there is a surprisingly simple answer to this: An ideal wind turbine would slow down the wind by 2/3 of its original speed. To understand why, we have to use the fundamental physical law for the aerodynamics of wind turbines:

Betz' Law

Betz' law says that you can only convert less than 16/27 (or 59%) of the kinetic energy in the wind to mechanical energy using a wind turbine.

Betz' law was first formulated by the German Physicist Albert Betz in 1919. His book "Wind-Energie" published in 1926 gives a good account of the knowledge of wind energy and wind turbines at that moment.

It is quite surprising that one can make such a sweeping, general statement which applies to any wind turbine with a disc-like rotor.

To prove the theorem requires a bit of math and physics, but don't be put off by that, as Betz himself writes in his book. Betz' own proof of the theorem is included in the Reference Manual on this web site.

| Power Density Function Power of the Wind From the page on the energy in the wind , we know that the energy potential per second (the power ) varies in proportion to the cube (the third power) of the wind speed, and in proportion to the density of the air. (Its weight per unit of volume). We may now combine everything we have learned so far: If we multiply the power of each wind speed with the probability of each wind speed from the Weibull graph, we have calculated the distribution of wind energy at different wind speeds = the power density.Notice, that the previous Weibull curve changes shape, because the high wind speeds have most of the power of the wind. From Power Density to Power Output This graph was drawn using the wind turbine power calculator on this web site. The area under the grey curve (all the way to the axis at the bottom) gives us the amount of wind power per square metre wind flow we may expect at this particular site. In this case we have a mean wind speed of 7 m/s and a Weibull k=2, so we get 402 W/m 2 . You should note that this is almost twice as much power as the wind has when it is blowing constantly at the average wind speed. The graph consists of a number of narrow vertical columns, one for each 0.1 m/s wind speed interval. The height of each column is the power (number of watts per square metre), which that particular wind speed contributes to the total amount of power available per square metre. The area under the blue curve tells us how much of the wind power we can theoretically convert to mechanical power. (According to Betz' law , this is 16/27 of the total power in the wind). The total area under the red curve tells us how much electrical power a certain wind turbine will produce at this site. We will learn how to figure that out in a moment when we get to the page on power curves. The Important Messages in the Graph The most important thing to notice is that the bulk of wind energy will be found at wind speeds above the mean (average) wind speed at the site. This is not as surprising as it sounds, because we know that high wind speeds have much higher energy content than low wind speeds. |  |  |
| --- | --- | --- |
|  |  | The Cut In Wind Speed Usually, wind turbines are designed to start running at wind speeds somewhere around 3 to 5 metres per second. This is called the cut in wind speed. The blue area to the left shows the small amount of power we lose due to the fact the turbine only cuts in after, say 5 m/s. The Cut Out Wind Speed The wind turbine will be programmed to stop at high wind speeds above, say 25 metres per second, in order to avoid damaging the turbine or its surroundings. The stop wind speed is called the cut out wind speed. The tiny blue area to the right represents that loss of power. |

The Power Curve of a Wind Turbine

The power curve of a wind turbine is a graph that indicates how large the electrical power output will be for the turbine at different wind speeds.

The graph shows a power curve for a typical Danish 600 kW wind turbine.

Power curves are found by field measurements, where an anemometer is placed on a mast reasonably close to the wind turbine (not on the turbine itself or too close to it, since the turbine rotor may create turbulence, and make wind speed measurement unreliable).

If the wind speed is not fluctuating too rapidly, then one may use the wind speed measurements from the anemometer and read the electrical power output from the wind turbine and plot the two values together in a graph like the one to the left.

Uncertainty in Measurement of Power Curves

In reality, one will see a swarm of points spread around the blue line, and not the neat curve in the graph.

The reason is that in practice the wind speed always fluctuates, and one cannot measure exactly the column of wind that passes through the rotor of the turbine.

(It is not a workable solution just to place an anemometer in front of the turbine, since the turbine will also cast a "wind shadow" and brake the wind in front of itself).

In practice, therefore, one has to take an average of the different measurements for each wind speed, and plot the graph through these averages.

Furthermore, it is difficult to make exact measurements of the wind speed itself. If one has a 3 per cent error in wind speed measurement, then the energy in the wind may be 9 per cent higher or lower (remember that the energy content varies with the third power of the wind speed).

Consequently, there may be errors up to plus or minus 10 per cent even in certified power curves.

Verifying Power Curves

Power curves are based on measurements in areas with low turbulence intensity, and with the wind coming directly towards the front of the turbine. Local turbulence and complex terrain (e.g. turbines placed on a rugged slope) may mean that wind gusts hit the rotor from varying directions. It may therefore be difficult to reproduce the power curve exactly in any given location.

Pitfalls in Using Power Curves

A power curve does not tell you how much power a wind turbine will produce at a certain average wind speed. You would not even be close, if you used that method!

Remember, that the energy content of the wind varies very strongly with the wind speed, as we saw in the section on the energy in the wind. So, it matters a lot how that average came about, i.e. if winds vary a lot, or if the wind blows at a relatively constant speed.

Also, you may remember from the example in the section on the power density function , that most of the wind energy is available at wind speeds which are twice the most common wind speed at the site.

Finally, we need to account for the fact that the turbine may not be running at standard air pressure and temperature, and consequently make corrections for changes in the density of air.

The Power Coefficient

The power coefficient tells you how efficiently a turbine converts the energy in the wind to electricity.

Very simply, we just divide the electrical power output by the wind energy input to measure how technically efficient a wind turbine is. In other words, we take the power curve , and divide it by the area of the rotor to get the power output per square metre of rotor area. For each wind speed, we then divide the result by the amount of power in the wind per square metre.

The graph shows a power coefficient curve for a typical Danish wind turbine. Although the average efficiency for these turbines is somewhat above 20 per cent, the efficiency varies very much with the wind speed. (If there are small kinks in the curve, they are usually due to measurement errors).

As you can see, the mechanical efficiency of the turbine is largest (in this case 44 per cent) at a wind speed around some 9 m/s. This is a deliberate choice by the engineers who designed the turbine. At low wind speeds efficiency is not so important, because there is not much energy to harvest. At high wind speeds the turbine must waste any excess energy above what the generator was designed for. Efficiency therefore matters most in the region of wind speeds where most of the energy is to be found.

Higher Technical Efficiency is not Necessarily the Way Forward

It is not an aim in itself to have a high technical efficiency of a wind turbine. What matters, really, is the cost of pulling kilowatt hours out of the winds during the next 20 years. Since the fuel is free, there is no need to save it. The optimal turbine is therefore not necessarily the turbine with the highest energy output per year.

On the other hand, each square metre of rotor area costs money, so it is of course necessary to harvest whatever energy you can - as long as you can keep costs per kilowatt hour down. We return to that subject later on the page about optimising wind turbines.

Guide to the Wind Turbine Power Calculator If you have room on your screen, you may open another browser window with the calculator , in order to look at it while you look at this guide. If you do not want to read all of these instructions, please read the advice at the bottom of the page in any case. Using the Power Curve and the Weibull distribution to Estimate Power and Energy Output In order to use the power curve properly, you have to combine your knowledge of the Weibull distribution with the power curve. This is what we will be doing using the power density calculator on the next page: For each tiny 0.1 metre interval of wind speeds we multiply the probability of that wind speed interval (from the Weibull curve) with the value from the power curve of the wind turbine. We then take the sum of all these multiplications to get the mean (or average) power output. If we multiply the power by 365.25 by 24 (the number of hours in a year) we get the total energy output for an average year. Site Data Use the pop up menu to fill out European wind distribution data automatically. The data calculated for roughness classes 0, 1, 2, and 3 was taken from the European wind atlas. If you use roughness class 1.5, we interpolate to find the data. If you have data for other parts of the world you would like to have included, please e-mail us. Air Density Data As we learned on a previous page, the energy in the wind varies in proportion to the density of air. Try changing the air temperature from, say 40 degrees Celsius, to -20 degrees Celsius. There are almost 25 per cent more air molecules in a cubic metre of the cold air than in a cubic metre of the warm air, so watch what happens to the energy output... If you wish to change the altitude above sea level, then start setting the temperature at sea level first. The programme will then automatically compute the likely temperature and pressure at the altitude you set. You may set the air density directly, if you know what you are doing. The programme then computes a likely set of data for the other variables. (You may also change the air pressure, but you'd better leave it alone. Your air pressure obviously has to fit to the local altitude and temperature). Wind Distribution Data The Weibull shape parameter is generally around 2 in Northern Europe, but situations vary, so you may really need a wind atlas to set this more accurately. You can either enter the mean wind speed, or the Weibull scale parameter (the programme then automatically computes) the other. The measurement height for your wind speed is very important, because wind speeds increase with heights above ground level, cf. the page on wind shear. Meteorology observations are generally made at 10 m height, but anemometer studies are often made at hub height of the wind turbine (in our example 50 metres). The average roughness of the surrounding terrain is important to determine the wind speed at turbine hub height, if it differs from the height at which wind speed measurements were made. You may either set the roughness length or the roughness class, depending on the local landscape type. (See the Reference Manual for guidelines on roughness classes).

|  | Wind Turbine Data This section of the calculator lets you specify the rated power of the main generator, the rotor diameter, the cut in wind speed , and the cut out wind speed , and the hub height of your machine. At the bottom of the page you may then specify the power curve of your machine. It is much easier, however, to use the first pop up menu which allows you to set all turbine specifications using a built-in table of data for typical Danish wind turbines. We have already put data for a typical 600 kW machine in the form for you, but you may experiment by looking at other machines. The second pop up menu will allow you to choose from the available hub heights for the machine you have chosen. You may also enter a hub height of your own, if you wish. Try experimenting a bit with different hub heights, and see how energy output varies. The effect is particularly noticeable if the machine is located in terrain with a high roughness class. (You can modify the roughness class in the wind distribution data to see for yourself). If you modify the standard machine specifications, the text on the first pop up menu changes to User example, to show that you are not dealing with a standard machine. It is safe to play with all of the variables, but it does not make much sense to change the generator size or rotor diameter for a standard machine, unless you also change the power curve. We only use the rotor diameter to show the power input, and to compute the efficiency of the machine (in terms of the power coefficient ). We only use the rated power of the generator to compute the capacity factor. Wind Turbine Power Curve For practical reasons (keeping your input data and your results in view at the same time) we have placed the listing of the turbine power curve at the bottom of the page. You can use this area to specify a turbine which is not listed in the built-in table. The only requirement is that wind speeds be ordered sequentially in ascending (increasing) order. The programme approximates the power curve with a straight line between each two successive points which have non zero values for the power output. Note: The programme only uses wind speeds up to 40 m/s in its calculations of the wind climate, so do not bother about fantasy machines that work beyond 30 m/s. Control Buttons Calculate recalculates the results on the form. You may also click anywhere else or use the tab key after you have entered data to activate the calculator. Note that if you change the power curve, the machine will not recalculate your data until you click calculate, or change other data. Reset Data sets the data back to the user example you first encountered on your screen. Power Density plots the power density graph for this site and machine in a separate window. Power Curve plots the power curve for the machine you have selected in a separate window. Power Coefficient plots the power coefficient , i.e. the efficiency of the machine at different wind speeds. Site Power Input Results Power input per square metre rotor area shows the amount of energy in the wind which theoretically would flow through the circle containing the rotor area, if the rotor were not present. (In reality, part of the airflow will be diverted outside the rotor area due to the high pressure area in front of the rotor). Maximum power input at x m/s shows at what wind speed we achieve the highest contribution to total power output. The figure is usually much higher than average wind speed, cf. the page on the power density function. Mean hub height wind speed shows how the programme recalculates your wind data to the proper hub height. If you have specified a hub height which is different from the height at which wind measurements were taken, the programme automatically recalculates all wind speeds in the Weibull distribution in accordance with the roughness class (or roughness length) you have specified. Turbine Power Output Results Power output per square metre of rotor area tells us how much of the power input per square metre the machine will convert to electricity. Generally, you will find that it is cost effective to build the machine to use about 30 per cent of the power available. (Please note, that the figure for site power input includes the power for wind speeds outside the cut in/cut out wind speed range, so you cannot divide by that figure to obtain the average power coefficient). Energy output per square metre rotor area per year, is simply the mean power output per square metre rotor area multiplied by the number of hours in a year. Energy output in kWh per year, tells us how much electrical energy the wind turbine will produce in an average year. That is probably the figure the owner cares more about than the rest. When the owner considers that figure, however, he will also have to take the price of the machine, its reliability, and the cost of operation and maintenance. We return to those subjects in the section on the economics of wind energy. The annual energy output calculated here may be slightly different from the real figures from the manufacturer. This is particularly the case if you vary the density of air. In that case the manufacturer will calculate different power curves for each density of air. The reason is, that with a pitch controlled turbine the pitching mechanism will automatically change the pitch angle of the blade with the change of air density, while for a stall controlled turbine, the manufacturer will set the angle of the blade slightly differently depending on the local average air density. This programme may be up to 3.6% below the correct figure from the manufacturer for low air densities, and up to 1.6% above the manufacturers' figures for high air densities. Capacity factor tells us how much the turbine uses the rated capacity of its (main) generator. You may read more on the page on annual energy output from a wind turbine. Note 1: Make sure that you use the same hub height, if you wish to compare how two machines with the same rotor diameter perform. Note 2: If you wish to compare machines with different rotor diameters you should look at the energy output per square metre of rotor area instead (you should still use the same hub height). Note 3: Low wind machines (large rotor diameter relative to generator size) will generally perform badly at high wind sites and vice versa. Most low wind machines are not designed for use in high wind areas with strong gusts. |
| --- | --- |

| Top of Form Wind Turbine Power Calculator |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Do not operate the form until this page and its programme have loaded completely. CALCULATOR |  |  |  |  |  |  |  |  |  |
| Site Data |  |  |  |  |  |  |  |  |  |
| Air Density Data °C temp atm altitude (=kPa pressure)kg/m3 density |  |  |  |  |  |  |  |  |  |
| Wind Distribution Data for Site Weibull shape parameter m/s mean =Weibull scale parameter m height, Roughness length m = class |  |  |  |  |  |  |  |  |  |
| Wind Turbine Data kW m/s cut in wind speed,m/s cut out wind speed m rotor diameter,m hub height |  |  |  |  |  |  |  |  |  |
| Site Power Input Results Power input* W/m2 rotor area Max. power input at* m/s Mean hub ht wind speed* m/s |  |  |  | Turbine Power output Results Power output* W/m2 rotor area Energy output* kWh/m2/year Energy output* kWh/year Capacity factor* per cent |  |  |  |  |  |
| Wind Turbine Power Curve |  |  |  |  |  |  |  |  |  |
| m/s... | ...kW |  | m/s... | ...kW |  | m/s... | ...kW Bottom of Form |  |  |

Annual Energy Output from a Wind Turbine

We are now ready to calculate the relationship between average wind speeds and annual energy output from a wind turbine. To draw the graph to the right, we have used the power calculator on the previous page, and the power curve from the default example 600 kW wind turbine. We have used a standard atmosphere with an air density of 1.225 kg/m 3 .

For each of the Weibull parameters 1.5, 2.0, and 2.5 we have calculated the annual energy output for different average wind speeds at turbine hub height.

As you can see, output may vary up to 50 per cent depending on the shape parameter at a low average wind speed of 4.5 m/s, while it may vary some 30 per cent at a very high average wind speed of 10 m/s at hub height.

Output varies almost with the cube of the wind speed

Now, let us look at the red curve with k=2, which is the curve normally shown by manufacturers:

With an average wind speed of 4.5 m/s at hub height the machine will generate about 0.5 GWh per year, i.e. 500,000 kWh per year. With an average wind speed of 9 metres per second it will generate 2.4 GWh/year = 2,400,000 kWh per year. Thus, doubling the average wind speed has increased energy output 4.8 times.

If we had compared 5 and 10 metres per second instead, we would have obtained almost exactly 4 times as much energy output.

The reason why we do not obtain exactly the same results in the two cases, is that the efficiency of the wind turbine varies with the wind speeds, as described by the power curve. Note, that the uncertainty that applies to the power curve also applies to the result above.

You may refine your calculations by accounting for the fact that e.g. in temperate climates the wind tends to be stronger in winter than in summer, and stronger during the daytime than at night.

The Capacity Factor

Another way of stating the annual energy output from a wind turbine is to look at the capacity factor for the turbine in its particular location. By capacity factor we mean its actual annual energy output divided by the theoretical maximum output, if the machine were running at its rated (maximum) power during all of the 8766 hours of the year.

Example: If a 600 kW turbine produces 1.5 million kWh in a year, its capacity factor is = 1500000 : ( 365.25 * 24 * 600 ) = 1500000 : 5259600 = 0.285 = 28.5 per cent.

Capacity factors may theoretically vary from 0 to 100 per cent, but in practice they will usually range from 20 to 70 per cent, and mostly be around 25-30 per cent.

The Capacity Factor Paradox

Although one would generally prefer to have a large capacity factor, it may not always be an economic advantage. This is often confusing to people used to conventional or nuclear technology.

In a very windy location, for instance, it may be an advantage to use a larger generator with the same rotor diameter (or a smaller rotor diameter for a given generator size). This would tend to lower the capacity factor (using less of the capacity of a relatively larger generator), but it may mean a substantially larger annual production, as you can verify using the Power calculator on this web site.

Whether it is worthwhile to go for a lower capacity factor with a relatively larger generator, depends both on wind conditions, and on the price of the different turbine models of course.

Wind Turbine Components

Click on the parts of the open wind turbine to learn about the nacelle, rotor blades, hub, low speed shaft, gearbox, high speed shaft with its mechanical brake, electrical generator, yaw mechanism, electronic controller, hydraulics system, cooling unit, tower, anemometer and wind vane.

Aerodynamics of Wind Turbines: Lift

The rotor consisting of the rotor blades and the hub are placed upwind of the tower and the nacelle on most modern wind turbines. This is primarily done because the air current behind the tower is very irregular (turbulent).

What makes the rotor turn?

The answer seems obvious - the wind.

But actually, it is a bit more complicated than just the air molecules hitting the front of the rotor blades. Modern wind turbines borrow technologies known from aeroplanes and helicopters, plus a few advanced tricks of their own, because wind turbines actually work in a very different environment with changing wind speeds and changing wind directions.

Lift

Have a look at the animation of the cut-off profile (cross section) of the wing of an aircraft. The reason why an aeroplane can fly is that the air sliding along the upper surface of the wing will move faster than on the lower surface.

This means that the pressure will be lowest on the upper surface. This creates the lift, i.e. the force pulling upwards that enables the plane to fly.

The lift is perpendicular to the direction of the wind. The lift phenomenon has been well known for centuries to people who do roofing work: They know from experience that roof material on the lee side of the roof (the side not facing the wind) is torn off quickly, if the roofing material is not properly attached to its substructure.

Aerodynamics of Wind Turbines: Stall

Stall

Now, what happens if an aircraft tilts backward in an attempt to climb higher into the sky quickly? The lift of the wing will indeed increase, as the wing is tilted backwards, but in the picture you can see that all of a sudden the air flow on the upper surface stops sticking to the surface of the wing. Instead the air whirls around in an irregular vortex (a condition which is also known as turbulence ). All of a sudden the lift from the low pressure on the upper surface of the wing disappears. This phenomenon is known as stall.

An aircraft wing will stall, if the shape of the wing tapers off too quickly as the air moves along its general direction of motion. (The wing itself, of course, does not change its shape, but the angle of the the wing in relation to the general direction of the airflow (also known as the angle of attack) has been increased in our picture above). Notice that the turbulence is created on the back side of the wing in relation to the air current.

Stall can be provoked if the surface of the aircraft wing - or the wind turbine rotor blade - is not completely even and smooth. A dent in the wing or rotor blade, or a piece of self-adhesive tape can be enough to start the turbulence on the backside, even if the angle of attack is fairly small. Aircraft designers obviously try to avoid stall at all costs, since an aeroplane without the lift from its wings will fall like a rock.

On the page on power control we shall return to the subject of how wind turbine engineers deliberately make use of the stall phenomenon when designing rotor blades.

Aerodynamics of Wind Turbines

Adding Wind Speeds and Directions (Wind Velocities)

The wind which hits the rotor blades of a wind turbine will not come from the direction in which the wind is blowing in the landscape, i.e. from the front of the turbine. This is because the rotor blades themselves are moving.

To understand this, consider the picture of a bicycle which is equipped with a blue banner (or a wind vane) to indicate the direction of the wind: If we have completely calm weather, and the bicycles moves forwards, with, say, 7 metres per second (14 knots), the bicycle will be moving through the air at 7 metres per second. On the bicycle we can measure a wind speed of 7 metres per second relative to the bicycle. The banner will point straight backwards, because the wind will come directly from the front of the bicycle.

Now, let us look at the bicycle again directly from above, and let us assume that the bicycle moves forward at a constant speed of, once again, 7 metres per second. If the wind is blowing directly from the right, also at 7 metres per second, the banner will clearly be blown partly to the left, at a 45 degree angle relative to the bicycle. With a bit less wind, e.g. 5 metres per second, the banner will be blown less to the left, and the angle will be some 35 degrees. As you can see from the picture, the direction of the wind, the resulting wind as measured from the bicycle, will change whenever the speed of the wind changes.

What about the wind speed measured from the bicycle?

The wind is, so to speak, blowing at a rate of 7 metres per second from the front and 5 to 7 metres per second from the right. If you know a bit of geometry or trigonometry you can work out that the wind speed measured on the bicycle will be between 8.6 and 9.9 metres per second.

Enough about changing wind directions, now what about the wind turbine rotor?

| Rotor Aerodynamics To study how the wind moves relative to the rotor blades of a wind turbine, we have fixed red ribbons to the tip of the rotor blades of our model wind turbine, and yellow ribbons some 1/4 out the length of the blade from the hub. |  |
| --- | --- |
|  | We then let the ribbons float freely in the air (in the cartoon we abstract from the air currents created by the blades themselves, and the centrifugal force). The two images on this page give you one view from the side of the turbine, and another view from the front of the turbine. Since most wind turbines have constant rotational speed, the speed with which the tip of the rotor blade moves through the air (the tip speed) is typically some 64 m/s, while at the centre of the hub it is zero. 1/4 out the length of the blade, the speed will then be some 16 m/s. The yellow ribbons close to the hub of the rotor will be blown more towards the back of the turbine than the red ribbons at the tips of the blades. This is obviously because at the tip of the blades the speed is some 8 times higher than the speed of the wind hitting the front of the turbine. Why are Rotor Blades Twisted? Rotor blades for large wind turbines are always twisted. Seen from the rotor blade, the wind will be coming from a much steeper angle (more from the general wind direction in the landscape), as you move towards the root of the blade, and the centre of the rotor. As you learned on the page on stall , a rotor blade will stop giving lift, if the blade is hit at an angle of attack which is too steep. Therefore, the rotor blade has to be twisted, so as to acheive an optimal angle of attack throughout the length of the blade. However, in the case of stall controlled wind turbines in particular, it is important that the blade is built so that it will stall gradually from the blade root and outwards at high wind speeds. |

| Rotor Blades Changing the Wind Speed Changes Wind Direction Relative to the Rotor Blade In this next picture we have taken one rotor blade from the previous page off its hub, and we look from the hub towards the tip, at the back side (the lee side) of the rotor blade. The wind in the landscape blows between, say 8 m/s and 16 m/s (from the bottom of the picture), while the tip of the blade rotates towards the left side of the picture. In the picture you can see how the angle of attack of the wind changes much more dramatically at the root of the blade (yellow line) than at the tip of the blade (red line), as the wind changes. If the wind becomes powerful enough to make the blade stall , it will start stalling at the root of the blade. Lift Direction Now, let us cut the rotor blade at the point with the yellow line. In the next picture the grey arrow shows the direction of the lift at this point. The lift is perpendicular to the direction of the wind. As you can see, the lift pulls the blade partly in the direction we want, i.e. to the left. It also bends the rotor blade somewhat, however. |  |
| --- | --- |
|  | Rotor Blade Profiles (Cross Sections) As you can see, wind turbine rotor blades look a lot like the wings of an aircraft. In fact, rotor blade designers often use classical aircraft wing profiles as cross sections in the outermost part of the blade. The thick profiles in the innermost part of the blade, however, are usually designed specifically for wind turbines. Choosing profiles for rotor blades involves a number of compromises including reliable lift and stall characteristics, and the profile's ability to perform well even if there is some dirt on the surface (which may be a problem in areas where there is little rain). Rotor Blade Materials Most modern rotor blades on large wind turbines are made of glass fibre reinforced plastics, (GRP), i.e. glass fibre reinforced polyester or epoxy. Using carbon fibre or aramid (Kevlar) as reinforcing material is another possibility, but usually such blades are uneconomic for large turbines. Wood, wood-epoxy, or wood-fibre-epoxy composites have not penetrated the market for rotor blades, although there is still development going on in this area. Steel and aluminium alloys have problems of weight and metal fatigue respectively. They are currently only used for very small wind turbines. |

Power Control of Wind Turbines

Wind turbines are designed to produce electrical energy as cheaply as possible. Wind turbines are therefore generally designed so that they yield maximum output at wind speeds around 15 metres per second. (30 knots or 33 mph). Its does not pay to design turbines that maximise their output at stronger winds, because such strong winds are rare.

In case of stronger winds it is necessary to waste part of the excess energy of the wind in order to avoid damaging the wind turbine. All wind turbines are therefore designed with some sort of power control. There are two different ways of doing this safely on modern wind turbines.

Pitch Controlled Wind Turbines

On a pitch controlled wind turbine the turbine's electronic controller checks the power output of the turbine several times per second. When the power output becomes too high, it sends an order to the blade pitch mechanism which immediately pitches (turns) the rotor blades slightly out of the wind. Conversely, the blades are turned back into the wind whenever the wind drops again.

The rotor blades thus have to be able to turn around their longitudinal axis (to pitch) as shown in the picture.

Note, that the picture is exaggerated:

During normal operation the blades will pitch a fraction of a degree at a time - and the rotor will be turning at the same time.

Designing a pitch controlled wind turbine requires some clever engineering to make sure that the rotor blades pitch exactly the amount required. On a pitch controlled wind turbine, the computer will generally pitch the blades a few degrees every time the wind changes in order to keep the rotor blades at the optimum angle in order to maximise output for all wind speeds.

The pitch mechanism is usually operated using hydraulics.

Stall Controlled Wind Turbines

(Passive) stall controlled wind turbines have the rotor blades bolted onto the hub at a fixed angle.

The geometry of the rotor blade profile, however has been aerodynamically designed to ensure that the moment the wind speed becomes too high, it creates turbulence on the side of the rotor blade which is not facing the wind as shown in the picture on the previous page. This stall prevents the lifting force of the rotor blade from acting on the rotor.

If you have read the section on aerodynamics and aerodynamics and stall , you will realise that as the actual wind speed in the area increases, the angle of attack of the rotor blade will increase, until at some point it starts to stall.

If you look closely at a rotor blade for a stall controlled wind turbine you will notice that the blade is twisted slightly as you move along its longitudinal axis. This is partly done in order to ensure that the rotor blade stalls gradually rather than abruptly when the wind speed reaches its critical value. (Other reasons for twisting the blade are mentioned in the previous section on aerodynamics).

The basic advantage of stall control is that one avoids moving parts in the rotor itself, and a complex control system. On the other hand, stall control represents a very complex aerodynamic design problem, and related design challenges in the structural dynamics of the whole wind turbine, e.g. to avoid stall-induced vibrations. Around two thirds of the wind turbines currently being installed in the world are stall controlled machines.

Active Stall Controlled Wind Turbines

An increasing number of larger wind turbines (1 MW and up) are being developed with an active stall power control mechanism.

Technically the active stall machines resemble pitch controlled machines, since they have pitchable blades. In order to get a reasonably large torque (turning force) at low wind speeds, the machines will usually be programmed to pitch their blades much like a pitch controlled machine at low wind speeds. (Often they use only a few fixed steps depending upon the wind speed).

When the machine reaches its rated power , however, you will notice an important difference from the pitch controlled machines: If the generator is about to be overloaded, the machine will pitch its blades in the opposite direction from what a pitch controlled machine does. In other words, it will increase the angle of attack of the rotor blades in order to make the blades go into a deeper stall, thus wasting the excess energy in the wind.

One of the advantages of active stall is that one can control the power output more accurately than with passive stall, so as to avoid overshooting the rated power of the machine at the beginning of a gust of wind. Another advantage is that the machine can be run almost exactly at rated power at all high wind speeds. A normal passive stall controlled wind turbine will usually have a drop in the electrical power output for higher wind speeds, as the rotor blades go into deeper stall.

The pitch mechanism is usually operated using hydraulics or electric stepper motors.

As with pitch control it is largely an economic question whether it is worthwhile to pay for the added complexity of the machine, when the blade pitch mechanism is added.

Other Power Control Methods

Some older wind turbines use ailerons (flaps) to control the power of the rotor, just like aircraft use flaps to alter the geometry of the wings to provide extra lift at takeoff.

Another theoretical possibility is to yaw the rotor partly out of the wind to decrease power. This technique of yaw control is in practice used only for tiny wind turbines (1 kW or less), as it subjects the rotor to cyclically varying stress which may ultimately damage the entire structure.

| The Wind Turbine Yaw Mechanism The wind turbine yaw mechanism is used to turn the wind turbine rotor against the wind. Yaw Error The wind turbine is said to have a yaw error, if the rotor is not perpendicular to the wind. A yaw error implies that a lower share of the energy in the wind will be running through the rotor area. (The share will drop to the cosine of the yaw error, for those of you who know math). If this were the only thing that happened, then yaw control would be an excellent way of controlling the power input to the wind turbine rotor. That part of the rotor which is closest to the source direction of the wind, however, will be subject to a larger force (bending torque) than the rest of the rotor. On the one hand, this means that the rotor will have a tendency to yaw against the wind automatically, regardless of whether we are dealing with an upwind or a downwind turbine. On the other hand, it means that the blades will be bending back and forth in a flapwise direction for each turn of the rotor. Wind turbines which are running with a yaw error are therefore subject to larger fatigue loads than wind turbines which are yawed in a perpendicular direction against the wind. Yaw Mechanism |  |
| --- | --- |
| Photograph Soren Krohn © 1998 DWIA | Almost all horizontal axis wind turbines use forced yawing, i.e. they use a mechanism which uses electric motors and gearboxes to keep the turbine yawed against the wind. The image shows the yaw mechanism of a typical 750 kW machine seen from below, looking into the nacelle. We can see the yaw bearing around the outer edge, and the wheels from the yaw motors and the yaw brakes inside. Almost all manufacturers of upwind machines prefer to brake the yaw mechanism whenever it is unused. The yaw mechanism is activated by the electronic controller which several times per second checks the position of the wind vane on the turbine, whenever the turbine is running. Cable Twist Counter Cables carry the current from the wind turbine generator down through the tower. The cables, however, will become more and more twisted if the turbine by accident keeps yawing in the same direction for a long time. The wind turbine is therefore equipped with a cable twist counter which tells the controller that it is time to untwist the cables. Occasionally you may therefore see a wind turbine which looks like it has gone berserk, yawing continuously in one direction for five revolutions. Like other safety equipment in the turbine there is redundancy in the system. In this case the turbine is also equipped with a pull switch which is activated if the cables become too twisted. |

| Wind Turbine Towers |  |
| --- | --- |
| Wind turbine towers, Navarra, Spain Photograph Soren Krohn © 1999 DWIA | The tower of the wind turbine carries the nacelle and the rotor. Towers for large wind turbines may be either tubular steel towers, lattice towers, or concrete towers. Guyed tubular towers are only used for small wind turbines (battery chargers etc.) |
|  | Tubular Steel Towers Most large wind turbines are delivered with tubular steel towers, which are manufactured in sections of 20-30 metres with flanges at either end, and bolted together on the site. The towers are conical (i.e. with their diameter increasing towards the base) in order to increase their strength and to save materials at the same time. Photograph © NEG-Micon A/S 1998 |
|  | Lattice Towers Lattice towers are manufactured using welded steel profiles. The basic advantage of lattice towers is cost, since a lattice tower requires only half as much material as a freely standing tubular tower with a similar stiffness. The basic disadvantage of lattice towers is their visual appearance , (although that issue is clearly debatable). Be that as it may, for aesthetic reasons lattice towers have almost disappeared from use for large, modern wind turbines. Photograph © Nordex A/S 1998 |
|  | Guyed Pole Towers Many small wind turbines are built with narrow pole towers supported by guy wires. The advantage is weight savings, and thus cost. The disadvantages are difficult access around the towers which make them less suitable in farm areas. Finally, this type of tower is more prone to vandalism, thus compromising overall safety. Photograph Soren Krohn © 1999 DWIA |
|  | Hybrid Tower Solutions Some towers are made in different combinations of the techniques mentioned above. One example is the three-legged Bonus 95 kW tower which you see in the photograph, which may be said to be a hybrid between a lattice tower and a guyed tower. Photograph © Bonus Energy A/S 1998 |
|  | Cost Considerations The price of a tower for a wind turbine is generally around 20 per cent of the total price of the turbine. For a tower around 50 metres' height, the additional cost of another 10 metres of tower is about 15,000 USD. It is therefore quite important for the final cost of energy to build towers as optimally as possible. Lattice towers are the cheapest to manufacture, since they typically require about half the amount of steel used for a tubular steel tower. Aerodynamic Considerations Generally, it is an advantage to have a tall tower in areas with high terrain roughness, since the wind speeds increases farther away from the ground, as we learned on the page about wind shear. Lattice towers and guyed pole towers have the advantage of giving less wind shade than a massive tower. Structural Dynamic Considerations The rotor blades on turbines with relatively short towers will be subject to very different wind speeds (and thus different bending) when a rotor blade is in its top and in its bottom position, which will increase the fatigue loads on the turbine. Choosing Between Low and Tall Towers Obviously, you get more energy from a larger wind turbine than a small one, but if you take a look at the three wind turbines below, which are 225 kW, 600 kW, and 1,500 kW respectively, and with rotor diameters of 27, 43, and 60 metres, you will notice that the tower heights are different as well. |
|  | Clearly, we cannot sensibly fit a 60 metre rotor to a tower of less than 30 metres. But if we consider the cost of a large rotor and a large generator and gearbox, it would surely be a waste to put it on a small tower, because we get much higher wind speeds and thus more energy with a tall tower. (See the section on wind resources ). Each metre of tower height costs money, of course, so the optimum height of the tower is a function of - tower costs per metre (10 metre extra tower will presently cost you about 15,000 USD) - how much the wind locally varies with the height above ground level, i.e. the average local terrain roughness (large roughness makes it more useful with a taller tower), - the price the turbine owner gets for an additional kilowatt hour of electricity. Manufacturers often deliver machines where the tower height is equal to the rotor diameter. aesthetically, many people find that turbines are more pleasant to look at, if the tower height is roughly equal to the rotor diameter. Occupational Safety Considerations The choice of tower type has consequences for occupational safety: This is discussed in detail on the page on Wind Turbines and Occupational Safety. |

| Size of Wind Turbines |  |
| --- | --- |
| Service crew working on a 32 m rotor blade on a 1.5 MW wind turbine Photograph Christian Kjaer © 2000 DWIA | Power Output Increases with the Swept Rotor Area When a farmer tells you how much land he is farming, he will usually state an area in terms of hectares or acres. With a wind turbine it is much the same story, though doing wind farming we farm a vertical area instead of a horizontal one. The area of the disc covered by the rotor, (and wind speeds, of course), determines how much energy we can harvest in a year. The picture gives you an idea of the normal rotor sizes of wind turbines: A typical turbine with a 600 kW electrical generator will typically have a rotor diameter of some 44 metres (144 ft.). If you double the rotor diameter, you get an area which is four times larger (two squared). This means that you also get four times as much power output from the rotor. Rotor diameters may vary somewhat from the figures given above, because many manufacturers optimise their machines to local wind conditions: A larger generator, of course, requires more power (i.e. strong winds) to turn at all. So if you install a wind turbine in a low wind area you will actually maximise annual output by using a fairly smallgenerator for a given rotor size (or a larger rotor size for a given generator) For a 600 kW machine rotor diameters may vary from 39 to 48 m (128 to 157 ft.) The reason why you may get more output from a relatively smaller generator in a low wind area is that the turbine will be running more hours during the year. Reasons for Choosing Large Turbines - There are economies of scale in wind turbines, i.e. larger machines are usually able to deliver electricity at a lower cost than smaller machines. The reason is that the cost of foundations, road building, electrical grid connection, plus a number of components in the turbine (the electronic control system etc.), are somewhat independent of the size of the machine. - Larger machines are particularly well suited for offshore wind power. The cost of foundations does not rise in proportion to the size of the machine, and maintenance costs are largely independent of the size of the machine. - In areas where it is difficult to find sites for more than a single turbine, a large turbine with a tall tower uses the existing wind resource more efficiently. You may take a look at some megawatt-sized wind turbines in the picture gallery. Reasons for Choosing Smaller Turbines - The local electrical grid may be too weak to handle the electricity output from a large machine. This may be the case in remote parts of the electrical grid with low population density and little electricity consumption in the area. - There is less fluctuation in the electricity output from a wind park consisting of a number of smaller machines, since wind fluctuations occur randomly, and therefore tend to cancel out. Again, smaller machines may be an advantage in a weak electrical grid. - The cost of using large cranes, and building a road strong enough to carry the turbine components may make smaller machines more economic in some areas. - Several smaller machines spread the risk in case of temporary machine failure, e.g. due to lightning strikes. - aesthetical landscape considerations may sometimes dictate the use of smaller machines. Large machines, however, will usually have a much lower rotational speed, which means that one large machine really does not attract as much attention as many small, fast moving rotors. (See the section on wind turbines in the landscape ). |

| Wind Turbine Safety |  |
| --- | --- |
| Photograph Soren Krohn © 1998 DWIA | The components of a wind turbine are designed to last 20 years. This means that they will have to endure more than 120,000 operating hours, often under stormy weather conditions. If you compare with an ordinary automobile engine, it usually only operates only some 5,000 hours during its lifetime. Large wind turbines are equipped with a number of safety devices to ensure safe operation during their lifetime. Sensors One of the classical, and most simple safety devices in a wind turbine is the vibration sensor in the image above, which was first installed in the Gedser wind turbine. It simply consists of a ball resting on a ring. The ball is connected to a switch through a chain. If the turbine starts shaking, the ball will fall off the ring and switch the turbine off. There are many other sensors in the nacelle, e.g. electronic thermometers which check the oil temperature in the gearbox and the temperature of the generator. Rotor Blades Safety regulations for wind turbines vary between countries. Denmark is the only country in which the law requires that all new rotor blades are tested both statically, i.e. applying weights to bend the blade, and dynamically, i.e. testing the blade's ability to withstand fatigue from repeated bending more than five million times. You may read more about this on the page on Testing Wind Turbine Rotor Blades. Overspeed Protection It is essential that wind turbines stop automatically in case of malfunction of a critical component. E.g. if the generator overheats or is disconnected from the electrical grid it will stop braking the rotation of the rotor, and the rotor will start accelerating rapidly within a matter of seconds. In such a case it is essential to have an overspeed protection system. Danish wind turbines are requited by law to have two independent fail safe brake mechanisms to stop the turbine. |
|  | Aerodynamic Braking System: Tip Brakes The primary braking system for most modern wind turbines is the aerodynamic braking system, which essentially consists in turning the rotor blades about 90 degrees along their longitudinal axis (in the case of a pitch controlled turbine or an active stall controlled turbine ), or in turning the rotor blade tips 90 degrees (in the case of a stall controlled turbine ). These systems are usually spring operated, in order to work even in case of electrical power failure, and they are automatically activated if the hydraulic system in the turbine loses pressure. The hydraulic system in the turbine is used turn the blades or blade tips back in place once the dangerous situation is over. Experience has proved that aerodynamic braking systems are extremely safe. They will stop the turbine in a matter of a couple of rotations, at the most. In addition, they offer a very gentle way of braking the turbine without any major stress, tear and wear on the tower and the machinery. The normal way of stopping a modern turbine (for any reason) is therefore to use the aerodynamic braking system. Mechanical Braking System The mechanical brake is used as a backup system for the aerodynamic braking system, and as a parking brake, once the turbine is stopped in the case of a stall controlled turbine. Pitch controlled turbines rarely need to activate the mechanical brake (except for maintenance work), as the rotor cannot move very much once the rotor blades are pitched 90 degrees. |

| Wind Turbine Occupational Safety Towers Large, modern wind turbines normally use conical tubular steel towers. The primary advantage of this tower over a lattice tower is that it makes it safer and far more comfortable for service personnel to access the wind turbine for repair and maintenance. The disadvantage is cost. Occupational Safety |  |
| --- | --- |
|  | The primary danger in working with wind turbines is the height above ground during installation work and when doing maintenance work. New Danish wind turbines are required to have fall protection devices, i.e. the person climbing the turbine has to wear a parachutist-like set of straps. The straps are connected with a steel wire to an anchoring system that follows the person while climbing or descending the turbine. The wire system has to include a shock absorber, so that persons are reasonably safe in case of a fall. |
| Photograph Soren Krohn © 1999 DWIA | A Danish tradition (which has later been taken up by other manufacturers), is to place the access ladders at a certain distance from the wall. This enables service personnel to climb the tower while being able to rest the shoulders against the inside wall of the tower. In this image you see the editor of our Spanish web site verifying that this is actually a very practical solution. Protection from the machinery, fire protection and electrical insulation protection is governed by a number of national and international standards. During servicing it is essential that the machinery can be stopped completely. In addition to a mechanical brake, the rotor can be locked in place with a pin, to prevent any movement of the mechanical parts whatsoever. |

| Wind Turbine Generators |  |
| --- | --- |
| You can see the internal cooling fan moving inside this generator. It is mounted at the end of the rotor, which is hidden inside the shining magnetic steel cylinder, called the stator. The radiator-like surface cools the generator. It is hard to see the details on a real life generator like the one to the right. Therefore, we'll take it apart and make some simplified models on the next pages. | The wind turbine generator converts mechanical energy to electrical energy. Wind turbine generators are a bit unusual, compared to other generating units you ordinarily find attached to the electrical grid. One reason is that the generator has to work with a power source (the wind turbine rotor) which supplies very fluctuating mechanical power (torque). These pages assumes that you are familiar with the basics of electricity, electromagnetism, and in particular alternating current. If any of the expressions volt (V), phase, three phase, frequency, or Hertz (Hz) sound strange to you, you should take a look at the Reference Manual on Electricity , and read about alternating current , three phase alternating current , electromagnetism , and induction , before you proceed with the following pages. Generating Voltage (tension) On large wind turbines (above 100-150 kW) the voltage (tension) generated by the turbine is usually 690 V three-phase alternating current (AC). The current is subsequently sent through a transformer next to the wind turbine (or inside the tower) to raise the voltage to somewhere between 10,000 and 30,000 volts, depending on the standard in the local electrical grid. Large manufacturers will supply both 50 Hz wind turbine models (for the electrical grids in most of the world) and 60 Hz models (for the electrical grid in America). Cooling System Generators need cooling while they work. On most turbines this is accomplished by encapsulating the generator in a duct, using a large fan for air cooling, but a few manufacturers use water cooled generators. Water cooled generators may be built more compactly, which also gives some electrical efficiency advantages, but they require a radiator in the nacelle to get rid of the heat from the liquid cooling system. Starting and Stopping the Generator If you connected (or disconnected) a large wind turbine generator to the grid by flicking an ordinary switch, you would be quite likely to damage both the generator, the gearbox and the current in the grid in the neighbourhood. You will learn how turbine designers deal with this challenge in the page on Power Quality Issues , later. Design Choices in Generators and Grid Connection Wind turbines may be designed with either synchronous or asynchronous generators, and with various forms of direct or indirect grid connection of the generator. Direct grid connection mean that the generator is connected directly to the (usually 3-phase) alternating current grid. Indirect grid connection means that the current from the turbine passes through a series of electric devices which adjust the current to match that of the grid. With an asynchronous generator this occurs automatically. |

Synchronous Generators

3-Phase Generator (or Motor) Principles

All 3-phase generators (or motors) use a rotating magnetic field.

In the picture to the left we have installed three electromagnets around a circle. Each of the three magnets is connected to its own phase in the three phase electrical grid.

As you can see, each of the three electromagnets alternate between producing a South pole and a North pole towards the centre. The letters are shown in black when the magnetism is strong, and in light grey when the magnetism is weak. The fluctuation in magnetism corresponds exactly to the fluctuation in voltage of each phase. When one phase is at its peak, the other two have the current running in the opposite direction, at half the voltage. Since the timing of current in the three magnets is one third of a cycle apart, the magnetic field will make one complete revolution per cycle.

Synchronous Motor Operation

The compass needle (with the North pole painted red) will follow the magnetic field exactly, and make one revolution per cycle. With a 50 Hz grid, the needle will make 50 revolutions per second, i.e. 50 times 60 = 3000 rpm (revolutions per minute).

In the picture above, we have in fact managed to build what is called a 2-pole permanent magnet synchronous motor. The reason why it is called a synchronous motor, is that the magnet in the centre will rotate at a constant speed which is synchronous with (running exactly like the cycle in) the rotation of the magnetic field.

The reason why it is called a 2-pole motor is that it has one North and one South pole. It may look like three poles to you, but in fact the compass needle feels the pull from the sum of the magnetic fields around its own magnetic field. So, if the magnet at the top is a strong South pole, the two magnets at the bottom will add up to a strong North pole.

The reason why it is called a permanent magnet motor is that the compass needle in the centre is a permanent magnet, not an electromagnet. (You could make a real motor by replacing the compass needle by a powerful permanent magnet, or an electromagnet which maintains its magnetism through a coil (wound around an iron core) which is fed with direct current).

The setup with the three electromagnets is called the stator in the motor, because this part of the motor remains static (in the same place). The compass needle in the centre is called the rotor, obviously because it rotates.

Synchronous Generator Operation

If you start forcing the magnet around (instead of letting the current from the grid move it), you will discover that it works like a generator, sending alternating current back into the grid. (You should have a more powerful magnet to produce much electricity). The more force (torque) you apply, the more electricity you generate, but the generator will still run at the same speed dictated by the frequency of the electrical grid.

You may disconnect the generator completely from the grid, and start your own private 3-phase electricity grid, hooking your lamps up to the three coils around the electromagnets. (Remember the principle of magnetic / electrical induction from the reference manual section of this web site). If you disconnect the generator from the main grid, however, you will have to crank it at a constant rotational speed in order to produce alternating current with a constant frequency. Consequently, with this type of generator you will normally want to use an indirect grid connection of the generator.

In practice, permanent magnet synchronous generators are not used very much. There are several reasons for this. One reason is that permanent magnets tend to become demagnetised by working in the powerful magnetic fields inside a generator. Another reason is that powerful magnets (made of rare earth metals, e.g. Neodynium) are quite expensive, even if prices have dropped lately.

Wind Turbines With Synchronous Generators

Wind turbines which use synchronous generators normally use electromagnets in the rotor which are fed by direct current from the electrical grid. Since the grid supplies alternating current, they first have to convert alternating current to direct current before sending it into the coil windings around the electromagnets in the rotor.

The rotor electromagnets are connected to the current by using brushes and slip rings on the axle (shaft) of the generator.

| Changing Generator Rotational Speed A Four Pole Generator The speed of a generator (or motor) which is directly connected to a three-phase grid is constant, and dictated by the frequency of the grid, as we learned on the previous page. If you double the number of magnets in the stator , however, you can ensure that the magnetic field rotates at half the speed. In the picture to the left, you see how the magnetic field now moves clockwise for half a revolution before it reaches the same magnetic pole as before. We have simply connected the six magnets to the three phases in a clockwise order. This generator (or motor) has four poles at all times, two South and two North. Since a four pole generator will only take half a revolution per cycle, it will obviously make 25 revolutions per second on a 50 Hz grid, or 1500 revolutions per minute (rpm). When we double the number of poles in the stator of a synchronous generator we will have to double the number of magnets in the rotor , as you see on the picture. Otherwise the poles will not match. (We could use to two bent "horseshoe" magnets in this case). Other Numbers of Poles Obviously, we could repeat what we just did, and introduce another pair of poles, by adding 3 more electromagnets to the stator. With 9 magnets we get a 6 pole machine, which will run at 1000 rpm on a 50 Hz grid. The general result is the following: |  |
| --- | --- |
|  | Synchronous Generator Speeds (rpm) Pole number; 50 Hz; 60 Hz. 2; 3000; 3600. 4; 1500; 1800. 6; 1000; 1200. 8; 750; 900. 10; 600; 720. 12; 500; 600. The term "synchronous generator speed" thus refers to the speed of the generator when it is running synchronously with the grid frequency. It applies to all sorts of generators, however: In the case of asynchronous (induction) generators it is equivalent to the idle speed of the generator. High or Low Speed Generators? Most wind turbines use generators with four or six poles. The reasons for using these relatively high-speed generators are savings on size and cost. The maximum force (torque) a generator can handle depends on the rotor volume. For a given power output you then have the choice between a slow-moving, large (expensive) generator, or a high-speed (cheaper) smaller generator. |

| Asynchronous (Induction) Generators |  |  |
| --- | --- | --- |
| The picture to the right illustrates the basic principles in the asynchronous generator, much as we saw it presented on the previous pages. In reality, only the rotor part looks different, as you will see on the this page. |  | Note: Before reading this page, you should have completed the previous three pages on Wind Turbine Generators. Most wind turbines in the world use a so-called three phase asynchronous (cage wound) generator, also called an induction generator to generate alternating current. This type of generator is not widely used outside the wind turbine industry, and in small hydropower units, but the world has a lot of experience in dealing with it anyway: The curious thing about this type of generator is that it was really originally designed as an electric motor. In fact, one third of the world's electricity consumption is used for running induction motors driving machinery in factories, pumps, fans, compressors, elevators, and other applications where you need to convert electrical energy to mechanical energy. One reason for choosing this type of generator is that it is very reliable, and tends to be comparatively inexpensive. The generator also has some mechanical properties which are useful for wind turbines. (Generator slip , and a certain overload capability). |
|  |  | The Cage Rotor |
| The key component of the asynchronous generator is the cage rotor. (It used to be called a squirrel cage rotor but after it became politically incorrect to exercise your domestic rodents in a treadmill, we only have this less captivating name). |  | It is the rotor that makes the asynchronous generator different from the synchronous generator. The rotor consists of a number of copper or aluminium bars which are connected electrically by aluminium end rings. In the picture at the top of the page you see how the rotor is provided with an "iron" core, using a stack of thin insulated steel laminations, with holes punched for the conducting aluminium bars. The rotor is placed in the middle of the stator, which in this case, once again, is a 4-pole stator which is directly connected to the three phases of the electrical grid. Motor Operation When the current is connected, the machine will start turning like a motor at a speed which is just slightly below the synchronous speed of the rotating magnetic field from the stator. Now, what is happening? If we look at the rotor bars from above (in the picture to the right) we have a magnetic field which moves relative to the rotor. This induces a very strong current in the rotor bars which offer very little resistance to the current, since they are short circuited by the end rings. The rotor then develops its own magnetic poles, which in turn become dragged along by the electromagnetic force from the rotating magnetic field in the stator. Generator Operation Now, what happens if we manually crank this rotor around at exactly the synchronous speed of the generator, e.g. 1500 rpm (revolutions per minute), as we saw for the 4-pole synchronous generator on the previous page? The answer is: Nothing. Since the magnetic field rotates at exactly the same speed as the rotor, we see no induction phenomena in the rotor, and it will not interact with the stator. But what if we increase speed above 1500 rpm? In that case the rotor moves faster than the rotating magnetic field from the stator, which means that once again the stator induces a strong current in the rotor. The harder you crank the rotor, the more power will be transferred as an electromagnetic force to the stator, and in turn converted to electricity which is fed into the electrical grid. Generator Slip The speed of the asynchronous generator will vary with the turning force (moment, or torque) applied to it. In practice, the difference between the rotational speed at peak power and at idle is very small, about 1 per cent. This difference in per cent of the synchronous speed , is called the generator's slip. Thus a 4-pole generator will run idle at 1500 rpm if it is attached to a grid with a 50 Hz current. If the generator is producing at its maximum power, it will be running at 1515 rpm. It is a very useful mechanical property that the generator will increase or decrease its speed slightly if the torque varies. This means that there will be less tear and wear on the gearbox. (Lower peak torque). This is one of the most important reasons for using an asynchronous generator rather than a synchronous generator on a wind turbine which is directly connected to the electrical grid. Automatic Pole Adjustment of the Rotor Did you notice that we did not specify the number of poles in the stator when we described the rotor? The clever thing about the cage rotor is that it adapts itself to the number of poles in the stator automatically. The same rotor can therefore be used with a wide variety of pole numbers. Grid Connection Required On the page about the permanent magnet synchronous generator we showed that it could run as a generator without connection to the public grid. An asynchronous generator is different, because it requires the stator to be magnetised from the grid before it works. You can run an asynchronous generator in a stand alone system, however, if it is provided with capacitors which supply the necessary magnetisation current. It also requires that there be some remanence in the rotor iron, i.e. some leftover magnetism when you start the turbine. Otherwise you will need a battery and power electronics, or a small diesel generator to start the system). |

Changing the Number of Generator Poles

You may be thinking that a stator with twice as many magnets would be twice as expensive, but that is not really the case. Generators (and motors) are usually made with a very large number of stator magnets anyway, as you see in the picture. (We have not yet added the stator coil windings on the iron).

The reason for this stator arrangement is that we wish to minimise the air gap between the rotor and the stator. At the the same time we need to provide cooling of the magnets. The stator iron in reality consists of a large number of thin (0.5 mm) insulated steel sheets which are stacked to form the stator iron. This layering is done to prevent current eddies in the stator iron from decreasing the efficiency of the generator.

The problem of providing more generator poles on an asynchronous cage wound generator then really boils down to connecting the neighbouring magnets differently: Either we take a bunch of magnets at a time, connecting them to the same phase as we move around the stator, or else we change to the next phase every time we get to the next magnet.

Two Speed, Pole Changing Generators

Some manufacturers fit their turbines with two generators, a small one for periods of low winds, and a large one for periods of high winds.

A more common design on newer machines is pole changing generators, i.e. generators which (depending on how their stator magnets are connected) may run with a different number of poles, and thus a different rotational speed.

Some electrical generators are custom built as two-in-one, i.e. they are able to run as e.g. either 400 kW or 2000 kW generators, and at two different speeds. This design has become ever more widespread throughout the industry.

Whether it is worthwhile to use a double generator or a higher number of poles for low winds depends on the local wind speed distribution , and the extra cost of the pole changing generator compared to the price the turbine owner gets for the electricity. (You should keep in mind that the energy content of low winds is very small).

A good reason for having a dual generator system, however, is that you may run your turbine at a lower rotational speed at low wind speeds. This is both more efficient (aerodynamically), and it means less noise from the rotor blades (which is usually only a problem at low wind speeds).

Incidentally, you may have a few pole changing motors in your house without even knowing it: Washing machines which can also spin dry clothes usually have pole changing motors which are able to run at low speed for washing and at high speed for spinning. Similarly, exhaust fans in your kitchen may be built for two or three different speeds. (In the latter case with a variable speed fan, you can use what you have learned about the energy in the wind : If you want to move twice as much air out of your house per minute using the same fan, it will cost you eight times as much electricity).

Variable Slip Generators for Wind Turbines

Manufacturers of electric motors have for many years been faced with the problem that their motors can only run at certain almost fixed speeds determined by the number of poles in the motor.

As we learned on the previous page, the motor (or generator) slip in an asynchronous (induction) machine is usually very small for reasons of efficiency, so the rotational speed will vary with around 1 per cent between idle and full load.

The slip, however is a function of the (DC) resistance (measured in ohms) in the rotor windings of the generator. The higher resistance, the higher the slip. so one way of varying the slip is to vary the resistance in the rotor. In this way one may increase generator slip to e.g. 10 per cent.

On motors, this is usually done by having a wound rotor, i.e. a rotor with copper wire windings which are connected in a star , and connected with external variable resistors, plus an electronic control system to operate the resistors. The connection has usually been done with brushes and slip rings, which is a clear drawback over the elegantly simple technical design of an cage wound rotor machine. It also introduces parts which wear down in the generator, and thus the generator requires extra maintenance.

Opti Slip®

An interesting variation of the variable slip induction generator avoids the problem of introducing slip rings, brushes, external resistors, and maintenance altogether.

By mounting the external resistors on the rotor itself, and mounting the electronic control system on the rotor as well, you still have the problem of how to communicate the amount of slip you need to the rotor. This communication can be done very elegantly, however, using optical fibre communications, and sending the signal across to the rotor electronics each time it passes a stationary optical fibre.

Running a Pitch Controlled Turbine at Variable Speed

As mentioned on the next page, there are a number of advantages of being able to run a wind turbine at variable speed.

One good reason for wanting to be able to run a turbine partially at variable speed is the fact that pitch control (controlling the torque in order not to overload the gearbox and generator by pitching the wind turbine blades) is a mechanical process. This means that the reaction time for the pitch mechanism becomes a critical factor in turbine design.

If you have a variable slip generator, however, you may start increasing its slip once you are close to the rated power of the turbine. The control strategy applied in a widely used Danish turbine design is to run the generator at half of its maximum slip when the turbine is operating near the rated power. When a wind gust occurs, the control mechanism signals to increase generator slip to allow the rotor to run a bit faster while the pitch mechanism begins to cope with the situation by pitching the blades more out of the wind. Once the pitch mechanism has done its work, the slip is decreased again. In case the wind suddenly drops, the process is applied in reverse.

Although these concepts may sound simple, it is quite a technical challenge to ensure that the two power control mechanisms co-operate efficiently.

Improving Power Quality

You may protest that running a generator at high slip releases more heat from the generator, which runs less efficiently. That is not a problem in itself, however, since the only alternative is to waste the excess wind energy by pitching the rotor blades out of the wind.

One of the real benefits of using the control strategy mentioned here is that you get a better power quality, since the fluctuations in power output are "eaten up" or "topped up" by varying the generator slip and storing or releasing part of the energy as rotational energy in the wind turbine rotor.

| Indirect Grid Connection of Wind Turbines |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Generating Alternating Current (AC) at Variable Frequency Most wind turbines run at almost constant speed with direct grid connection. With indirect grid connection, however, the wind turbine generator runs in its own, separate mini AC-grid, as illustrated in the graphic. This grid is controlled electronically (using an inverter), so that the frequency of the alternating current in the stator of the generator may be varied. In this way it is possible to run the turbine at variable rotational speed. Thus the turbine will generate alternating current at exactly the variable frequency applied to the stator. The generator may be either a synchronous generator or an asynchronous generator , and the turbine may have a gearbox , as in the image above, or run without a gearbox if the generator has many poles, as explained on the next page. Conversion to Direct Current (DC) AC current with a variable frequency cannot be handled by the public electrical grid. We therefore start by rectifying it, i.e. we convert it into direct current, DC. The conversion from variable frequency AC to DC can be done using thyristors or large power transistors. Conversion to Fixed Frequency AC We then convert the (fluctuating) direct current to an alternating current (using an inverter) with exactly the same frequency as the public electrical grid. This conversion to AC in the inverter can also be done using either thyristors or transistors. Thyristors or power transistors are large semiconductor switches that operate without mechanical parts. The kind of alternating current one gets out of an inverter looks quite ugly at first sight - nothing like the smooth sinusoidal curve we learned about when studying alternating current. Instead, we get a series of sudden jumps in the voltage and current, as you saw in the animation above. Filtering the AC The rectangular shaped waves can be smoothed out, however, using appropriate inductances and capacitors, in a so-called AC filter mechanism. The somewhat jagged appearance of the voltage does not disappear completely, however, as explained below. Advantages of Indirect Grid Connection: Variable Speed The advantage of indirect grid connection is that it is possible to run the wind turbine at variable speed. The primary advantage is that gusts of wind can be allowed to make the rotor turn faster, thus storing part of the excess energy as rotational energy until the gust is over. Obviously, this requires an intelligent control strategy, since we have to be able to differentiate between gusts and higher wind speed in general. Thus it is possible to reduce the peak torque (reducing wear on the gearbox and generator), and we may also reduce the fatigue loads on the tower and rotor blades. The secondary advantage is that with power electronics one may control reactive power (i.e. the phase shifting of current relative to voltage in the AC grid), so as to improve the power quality in the electrical grid. This may be useful, particularly if a turbine is running on a weak electrical grid. Theoretically, variable speed may also give a slight advantage in terms of annual production, since it is possible to run the machine at an optimal rotational speed, depending on the wind speed. From an economic point of view that advantage is so small, however, that it is hardly worth mentioning. Disadvantages of Indirect Grid Connection The basic disadvantage of indirect grid connection is cost. As we just learned, the turbine will need a rectifier and two inverters, one to control the stator current, and another to generate the output current. Presently, it seems that the cost of power electronics exceeds the gains to be made in building lighter turbines, but that may change as the cost of power electronics decreases. Looking at operating statistics from wind turbines using power electronics (published by the the German ISET Institute), it also seems that availability rates for these machines tend to be somewhat lower than conventional machines, due to failures in the power electronics. Other disadvantages are the energy lost in the AC-DC-AC conversion process, and the fact that power electronics may introduce harmonic distortion of the alternating current in the electrical grid, thus reducing power quality. The problem of harmonic distortion arises because the filtering process mentioned above is not perfect, and it may leave some "overtones" (multiples of the grid frequency) in the output current. |  |  |  |  |

Gearboxes for Wind Turbines

Why Use a Gearbox?

The power from the rotation of the wind turbine rotor is transferred to the generator through the power train, i.e. through the main shaft, the gearbox and the high speed shaft, as we saw on the page with the Components of a Wind Turbine.

But why use a gearbox? Couldn't we just drive the generator directly with the power from the main shaft?

If we used an ordinary generator, directly connected to a 50 Hz AC ( alternating current ) three phase grid with two, four, or six poles, we would have to have an extremely high speed turbine with between 1000 and 3000 revolutions per minute (rpm), as we can see in the page on Changing Generator Rotational Speed. With a 43 metre rotor diameter that would imply a tip speed of the rotor of far more than twice the speed of sound, so we might as well forget it.

Another possibility is to build a slow-moving AC generator with many poles. But if you wanted to connect the generator directly to the grid, you would end up with a 200 pole generator (i.e. 300 magnets) to arrive at a reasonable rotational speed of 30 rpm.

Another problem is, that the mass of the rotor of the generator has to be roughly proportional to the amount of torque (moment, or turning force) it has to handle. So a directly driven generator will be very heavy (and expensive) in any case.

Less Torque, More Speed

The practical solution, which is used in the opposite direction in lots of industrial machinery, and in connection with car engines is to use a gearbox. With a gearbox you convert between slowly rotating, high torque power which you get from the wind turbine rotor - and high speed, low torque power, which you use for the generator.

The gearbox in a wind turbine does not "change gears". It normally has a single gear ratio between the rotation of the rotor and the generator. For a 600 or 750 kW machine, the gear ratio is typically approximately 1 to 50.

The picture below shows a 1.5 MW gearbox for a wind turbine. This particular gearbox is somewhat unusual, since it has flanges for two generators on the high speed side (to the right). The orange gadgets just below the generator attachments to the right are the hydraulically operated emergency disc brakes. In the background you see the lower part of a nacelle for a 1.5 MW turbine.

The Electronic Wind Turbine Controller

The wind turbine controller consists of a number of computers which continuously monitor the condition of the wind turbine and collect statistics on its operation. As the name implies, the controller also controls a large number of switches, hydraulic pumps, valves, and motors within the wind turbine.

As wind turbine sizes increase to megawatt machines, it becomes even more important that they have a high availability rate, i.e. that they function reliably all the time.

Communicating with the Outside World

The controller communicates with the owner or operator of the wind turbine via a communications link, e.g. sending alarms or requests for service over the telephone or a radio link. It is also possible to call the wind turbine to collect statistics, and check its present status. In wind parks one of the turbines will usually be equipped with a PC from which it is possible to control and collect data from the rest of the wind turbines in the park. This PC can be called over a telephone line or a radio link.

Internal Communications

There is usually a controller both at the bottom of the tower and in the nacelle. On recent wind turbine models, the communication between the controllers is usually done using fibre optics. The image to the right shows a fibre optics communications unit. On some recent models, there is a third controller placed in the hub of the rotor. That unit usually communicates with the nacelle unit using serial communications through a cable connected with slip rings and brushes on the main shaft.

Fail Safe Mechanisms and Redundancy

Computers and sensors are usually duplicated (redundant) in all safety or operation sensitive areas of newer, large machines. The controller continuously compares the readings from measurements throughout the wind turbine to ensure that both the sensors and the computers themselves are OK. The picture at the top of the page shows the controller of a megawatt machine, and has two central computers. (We removed the cover on one of the two computers to show the electronics).

What is Monitored?

It is possible to monitor or set somewhere between 100 and 500 parameter values in a modern wind turbine. The controller may e.g. check the rotational speed of the rotor, the generator, its voltage and current. In addition, lightning strikes and their charge may be registered. Furthermore measurements may be made of of outside air temperature, temperature in the electronic cabinets, oil temperature in the gearbox, the temperature of the generator windings, the temperature in the gearbox bearings, hydraulic pressure, the pitch angle of each rotor blade (for pitch controlled or active stall controlled machines), the yaw angle (by counting the number of teeth on yaw wheel), the number of power cable twists, wind direction, wind speed from the anemometer, the size and frequency of vibrations in the nacelle and the rotor blades, the thickness of the brake linings, whether the tower door is open or closed (alarm system).

Control Strategies

Many of the business secrets of the wind turbine manufacturers are to be found in the way the controller interacts with the wind turbine components. Improved control strategies are responsible for an important part of the increase in wind turbine productivity in recent years.

An interesting strategy pursued by some manufacturers is to adapt the operational strategy to the local wind climate. In this way it may e.g. be possible to minimise uneconomic tear and wear on the machine during (rare) periods of rough weather.

| Controlling Power Quality from Wind Turbines Most people think of the controller as the unit which runs the wind turbine, e.g. yaws it against the wind, checks that the safety systems are OK, and starts the turbine. The controller does indeed do all these things, but it also looks after the power quality of the current generated by the wind turbine. Grid Connection and Power Quality In the section about power quality you will learn how electricity companies require that wind turbines connect "softly" to the grid, and how they have certain requirements that the alternating current and voltage move in step with one another. The image to the right shows the high voltage section of a controller for a megawatt machine. This part of the controller operates e.g. the thyristors which ensure soft coupling to the electrical grid. |  |  |
| --- | --- | --- |
|  |  | Reactive Power Control Voltage and current are typically measured 128 times per alternating current cycle, (i.e. 50 x 128 times per second or 60 x 128 times per second, depending on the electrical grid frequency). On this basis, a so called DSP processor calculates the stability of the grid frequency and the active and reactive power of the turbine. (The reactive power component is basically a question of whether the voltage and the current are in phase or not). In order to ensure the proper power quality, the controller may switch on or switch off a large number of electrical capacitors which adjust the reactive power, (i.e. the phase angle between the voltage and the current). As you can see in the image to the left, the switchable capacitor bank is quite a large control unit in itself in a megawatt sized machine. |
|  |  | Electromagnetic Compatibility (EMC) There are very powerful electromagnetic fields around power cables and generators in a wind turbine. This means that the electronics in the controller system has to be insensitive to electromagnetic fields. Conversely, the electronics should not emit electromagnetic radiation which can inhibit the functioning of other electronic equipment. The image to the left shows a radiation free room with metal walls in the laboratory of one of the largest wind turbine controller manufacturers. The equipment in the room is used to measure electromagnetic emissions from the components of the controllers. |

| Wind Turbine Design: Basic Load Considerations |  |
| --- | --- |
| Water pumping windmill, South Australia, Photograph Soren Krohn © 1997 DWIA | Whether you are building wind turbines or helicopters, you have to take the strength, the dynamic behaviour, and the fatigue properties of your materials and the entire assembly into consideration. |
|  | Extreme Loads (Forces) |
| Comodoro Rivadavia, Argentina (NEG Micon 750 kW turbines) Photograph Soren Krohn © 1998 DWIA | Wind turbines are built to catch the wind's kinetic (motion) energy. You may therefore wonder why modern wind turbines are not built with a lot of rotor blades, like the old "American" windmills you have seen in the Western movies. Turbines with many blades or very wide blades, i.e. turbines with a very solid rotor, however, will be subject to very large forces, when the wind blows at a hurricane speed. (Remember, that the energy content of the wind varies with the third power (the cube) of the wind speed). Wind turbine manufacturers have to certify that their turbines are built, so that they can withstand extreme winds which occur, say, during 10 minutes once every 50 years. To limit the influence of the extreme winds turbine manufacturers therefore generally prefer to build turbines with a few, long, narrow blades. In order to make up for the narrowness of the blades facing the wind, turbine manufacturers prefer to let the turbines rotate relatively quickly. Fatigue Loads (Forces) Wind turbines are subject to fluctuating winds , and hence fluctuating forces. This is particularly the case if they are located in a very turbulent wind climate. Components which are subject to repeated bending, such as rotor blades, may eventually develop cracks which ultimately may make the component break. A historical example is the huge German Growian machine (100 m rotor diameter) which had to be taken out of service after less than three weeks of operation. Metal fatigue is a well known problem in many industries. Metal is therefore generally not favoured as a material for rotor blades. When designing a wind turbine it is extremely important to calculate in advance how the different components will vibrate, both individually, and jointly. It is also important to calculate the forces involved in each bending or stretching of a component. This is the subject of structural dynamics, where physicists have developed mathematical computer models that analyse the behaviour of an entire wind turbine. These models are used by wind turbine manufacturers to design their machines safely. Structural Dynamics: An Example *) A 50 metre tall wind turbine tower will have a tendency to swing back and forth, say, every three seconds. The frequency with which the tower oscillates back and forth is also known as the eigenfrequency of the tower. The eigenfrequency depends on both the height of the tower, the thickness of its walls, the type of steel, and the weight of the nacelle and rotor. Now, each time a rotor blade passes the wind shade of the tower, the rotor will push slightly less against the tower. If the rotor turns with a rotational speed such that a rotor blade passes the tower each time the tower is in one of its extreme positions, then the rotor blade may either dampen or amplify (reinforce) the oscillations of the tower. The rotor blades themselves are also flexible, and may have a tendency to vibrate, say, once per second. As you can see, it is very important to know the eigenfreqencies of each component in order to design a safe turbine that does not oscillate out of control. *) A very dramatic example of structural dynamic forces at work under influence of the wind (undampened torsion oscillations) is the famous crash of the Tacoma Bridge close to Seattle in the United States. You may find a short movie clip (700 K) on the disaster on the Internet. |

| Wind Turbines: Horizontal or Vertical Axis Machines? Horizontal Axis Wind Turbines Most of the technology described on these pages is related to horizontal axis wind turbines (HAWTs, as some people like to call them). The reason is simple: All grid-connected commercial wind turbines today are built with a propeller-type rotor on a horizontal axis (i.e. a horizontal main shaft). The purpose of the rotor, of course, is to convert the linear motion of the wind into rotational energy that can be used to drive a generator. The same basic principle is used in a modern water turbine, where the flow of water is parallel to the rotational axis of the turbine blades. Vertical Axis Wind Turbines |  |
| --- | --- |
| Eole C, a 4200 kW Vertical axis Darrieus wind turbine with 100 m rotor diameter at Cap Chat, Québec, Canada. The machine (which is the world's largest wind turbine) is no longer operational. Photograph Soren Krohn © 1997 DWIA | As you will probably recall, classical water wheels let the water arrive at a right angle (perpendicular) to the rotational axis (shaft) of the water wheel. Vertical axis wind turbines (VAWTs as some people call them) are a bit like water wheels in that sense. (Some vertical axis turbine types could actually work with a horizontal axis as well, but they would hardly be able to beat the efficiency of a propeller-type turbine). The only vertical axis turbine which has ever been manufactured commercially at any volume is the Darrieus machine, named after the French engineer Georges Darrieus who patented the design in 1931. (It was manufactured by the U.S. company FloWind which went bankrupt in 1997). The Darrieus machine is characterised by its C-shaped rotor blades which make it look a bit like an eggbeater. It is normally built with two or three blades. The basic theoretical advantages of a vertical axis machine are: 1) You may place the generator, gearbox etc. on the ground, and you may not need a tower for the machine. 2) You do not need a yaw mechanism to turn the rotor against the wind. The basic disadvantages are: 1) Wind speeds are very low close to ground level, so although you may save a tower, your wind speeds will be very low on the lower part of your rotor. 2) The overall efficiency of the vertical axis machines is not impressive. 3) The machine is not self-starting (e.g. a Darrieus machine will need a "push" before it starts. This is only a minor inconvenience for a grid connected turbine, however, since you may use the generator as a motor drawing current from the grid to to start the machine). 4) The machine may need guy wires to hold it up, but guy wires are impractical in heavily farmed areas. 5) Replacing the main bearing for the rotor necessitates removing the rotor on both a horizontal and a vertical axis machine. In the case of the latter, it means tearing the whole machine down. (That is why EOLE 4 in the picture is standing idle). |

| Wind Turbines: Upwind or Downwind Machines? Upwind Machines Upwind machines have the rotor facing the wind. The basic advantage of upwind designs is that one avoids the wind shade behind the tower. By far the vast majority of wind turbines have this design. On the other hand, there is also some wind shade in front of the tower, i.e. the wind starts bending away from the tower before it reaches the tower itself, even if the tower is round and smooth. Therefore, each time the rotor passes the tower, the power from the wind turbine drops slightly. The basic drawback of upwind designs is that the rotor needs to be made rather inflexible, and placed at some distance from the tower (as some manufacturers have found out to their cost). In addition an upwind machine needs a yaw mechanism to keep the rotor facing the wind. Downwind Machines |  |
| --- | --- |
| Small downwind turbine (22 kW). You may notice that the rotor is "coning" away from the tower. Photograph Soren Krohn © 1998 DWIA | Downwind machines have the rotor placed on the lee side of the tower. They have the theoretical advantage that they may be built without a yaw mechanism, if the rotor and nacelle have a suitable design that makes the nacelle follow the wind passively. For large wind turbines this is a somewhat doubtful advantage, however, since you do need cables to lead the current away from the generator. How do you untwist the cables, when the machine has been yawing passively in the same direction for a long period of time, if you do not have a yaw mechanism? (Slip rings or mechanical collectors are not a very good idea if you are working with 1000 ampere currents). A more important advantage is that the rotor may be made more flexible. This is an advantage both in regard to weight, and the structural dynamics of the machine, i.e. the blades will bend at high wind speeds, thus taking part of the load off the tower. The basic advantage of the downwind machine is thus, that it may be built somewhat lighter than an upwind machine. The basic drawback is the fluctuation in the wind power due to the rotor passing through the wind shade of the tower. This may give more fatigue loads on the turbine than with an upwind design. |

| Wind Turbines: How many blades? Why Not an Even Number of Blades? Modern wind turbine engineers avoid building large machines with an even number of rotor blades. The most important reason is the stability of the turbine. A rotor with an odd number of rotor blades (and at least three blades) can be considered to be similar to a disc when calculating the dynamic properties of the machine. A rotor with an even number of blades will give stability problems for a machine with a stiff structure. The reason is that at the very moment when the uppermost blade bends backwards, because it gets the maximum power from the wind, the lowermost blade passes into the wind shade in front of the tower. The Danish Three-Bladed Concept Most modern wind turbines are three-bladed designs with the rotor position maintained upwind (on the windy side of the tower) using electrical motors in their yaw mechanism. This design is usually called the classical Danish concept , and tends to be a standard against which other concepts are evaluated. The vast majority of the turbines sold in world markets have this design. The basic design was first introduced with the renowned Gedser wind turbine. Another characteristic is the use of an asynchronous generator. You may read more about the Danish concept in the articles section of this web site. |  |
| --- | --- |
|  | Two-Bladed (Teetering) Concept Two-bladed wind turbine designs have the advantage of saving the cost of one rotor blade and its weight, of course. However, they tend to have difficulty in penetrating the market, partly because they require higher rotational speed to yield the same energy output. This is a disadvantage both in regard to noise and visual intrusion. Lately, several traditional manufacturers of two-bladed machines have switched to three-bladed designs. Two- and one-bladed machines require a more complex design with a hinged (teetering hub) rotor as shown in the picture, i.e. the rotor has to be able to tilt in order to avoid too heavy shocks to the turbine when a rotor blades passes the tower. The rotor is therefore fitted onto a shaft which is perpendicular to the main shaft, and which rotates along with the main shaft. This arrangement may require additional shock absorbers to prevent the rotor blade from hitting the tower. |
|  | One-Bladed Concept Yes, one-bladed wind turbines do exist, and indeed, they save the cost of another rotor blade! If anything can be built, engineers will do it. One-bladed wind turbines are not very widespread commercially, however, because the same problems that are mentioned under the two-bladed design apply to an even larger extent to one-bladed machines. In addition to higher rotational speed, and the noise and visual intrusion problems, they require a counterweight to be placed on the other side of the hub from the rotor blade in order to balance the rotor. This obviously negates the savings on weight compared to a two-bladed design. |

| Optimising Wind Turbines Optimisation and Economics |  |
| --- | --- |
| Victoria in Southern Australia would never have been populated in the late 19th century, were it not for the water pumping windmills - and these windmills are really optimised for their purpose. Photograph Soren Krohn © 1998 DWIA | The water pumping windmills to the left look very different from modern, large wind turbines. But they are quite sensibly designed for the purpose they serve: The very solid rotor with many blades means that they will be running even at very low wind speeds, and thus pumping a fair amount of water all year round. Clearly, they will be very inefficient at high wind speeds, and they will have to shut themselves down, and yaw out of the wind in order to avoid damage to the turbine, due to the very solid rotor. But that does not really matter: We do not want them to empty the wells and flood the water tank during a gale. The ideal wind turbine design is not dictated by technology alone, but by a combination of technology and economics: Wind turbine manufacturers wish to optimise their machines, so that they deliver electricity at the lowest possible cost per kilowatt hour (kWh) of energy. But manufacturers are not very concerned about how efficiently they use the wind resource: The fuel is free, after all. It is not necessarily a good idea to maximise annual energy production, if that means that one has to build a very expensive wind turbine. In the next sections we shall look at some of the choices manufacturers have to make. Relative Generator and Rotor Size A small generator, (i.e. a generator with low rated power output in kW) requires less force to turn than a large one. If you fit a large wind turbine rotor with a small generator it will be producing electricity during many hours of the year, but it will capture only a small part of the energy content of the wind at high wind speeds. A large generator, on the other hand, will be very efficient at high wind speeds, but unable to turn at low wind speeds. Clearly, manufacturers will look at the distribution of wind speeds and the energy content of the wind at different wind speeds to determine the ideal combination of the size of the rotor and the size of the generator at different wind turbine sites. Fitting a wind turbine with two (or more) generators can sometimes be an advantage, but whether it really pays to do it depends on the electricity price. Tower Heights In the section on wind shear , you have learned that taller towers generally increase a wind turbine's energy production. Once again, whether a taller tower is worth the extra cost depends both on the roughness class, and the cost of electricity. |

| Testing Wind Turbine Rotor Blades Fatigue Testing of Rotor Blades |  |
| --- | --- |
| Video Soren Krohn © 1999 DWIA Click on image to restart video | The video to the left (122 K) shows how a 32 m rotor blade is fatigue tested by being bent cyclically in a flapwise direction for 5 million full cycles. A full flapwise test thus takes about three months. If you look closely to the left you can see another (shorter) rotor blade being bent cyclically in an edgewise (chordwise) direction. In both cases the blades are bent using a cycle close to the natural frequency of the blade. The natural frequency is the frequency with which the blade will oscillate back and forth, if you push it once in a certain direction and let go. The natural frequencies are different in the flapwise and edgewise direction: The blade tends to be much stiffer in the edgewise direction, thus it has a higher natural frequency for edgewise bending. Each blade is set in motion by an electric motor mounted on the blade which swings a weight up and down. The foundations which carry the blade socket have to be very solid: The foundation for the large blade socket consists of 2,000 tonnes of concrete. This video was shot at the rotor blade test facility of the Risoe National Laboratory Sparkær Test Centre in Jutland, Denmark. (Type approval requirements for rotor blades are very strict in Denmark, requiring physical testing of rotor blades for both fatigue properties (fatigue testing) and strength properties (static testing). Other countries usually have less stringent requirements for type approval of rotor blades). Rotor Blade Materials Rotor blades are usually made using a matrix of fibre glass mats which are impregnated with a material such as polyester (GRP = Glass fibre reinforced polyester). The polyester is hardened after it has impregnated the fibre glass. Epoxy may be used instead of polyester. Likewise the basic matrix may be made wholly or partially from carbon fibre, which is a lighter, but costlier material with high strength. Wood-epoxy laminates are also being used for large rotor blades. The Purpose of Testing Rotor Blades The purpose of rotor blade testing is to verify that laminations in the blade are, safe, i.e. that the layers of the rotor blade do not separate (delamination). Also, the test verifies that the fibres do not break under repeated stress. Measuring Strains |
| Photograph Soren Krohn © 1999 DWIA | Strain gauges, (i.e. flat electrical resistors which are glued on to the surface of the rotor blades being tested), are used to measure very accurately the bending and stretching of the rotor blades. Monitoring Fatigue Testing |
| Photograph Soren Krohn © 1999 DWIA | The measurement results from the strain gauges are continuously monitored on computers. Nonlinear variations in the pattern of bending may reveal a damage in the rotor blade structure. |
|  | Infrared Inspection (Thermography) Infrared cameras are used to reveal local build-up of heat in the blade. This may either indicate an area with structural dampening, i.e. an area where the blade designer has deliberately laid out fibres which convert the bending energy into heat in order to stabilise the blade, or it may indicate an area of delamination or an area which is moving toward the breaking point for the fibres. Modal Forms of Rotor Blade Vibrations From the year 2000 blade testing (in Denmark) also includes a verification of the different modal forms of vibration of each blade. This is done using a special type of equipment which excites the blade vibrations at different frequencies and in different directions. Different modal forms of oscillation are also known when building musical instruments: A string on a violin may oscillate with is basic tone, i.e. the centre of the string moving up and down, but it will usually also oscillate with the first overtone or first harmonic, with two centres of oscillation located at a distance of 1/4 from each end of the string, moving at twice the frequency of the basic tone or natural frequency. The reason why manufacturers of wind turbines are interested in studying and verifying the various forms of vibration frequencies in rotor blades, is that they have to make sure that the turbine on which the blade is to be mounted does not have some of the same natural frequencies as the rotor blade. Otherwise, a resonance may occur in the whole structure of the turbine, leading to undampened vibrations which may eventually wreck the whole wind turbine. We will return to this issue on the page on structural dynamics in the design section later in this guided tour. Static Testing of Rotor Blades Rotor blades are also tested for strength (and thus their ability to withstand extreme loads) by being bent once with a very large force. This test is made after the blades has been subject to fatigue testing, in order to verify the strength for a blade which has been in operation for a substantial amount of time. |

| Manufacturing Wind Turbine Towers Rolling Conical Tower Sections |  |
| --- | --- |
| All photographs Soren Krohn © 1999 DWIA | Most modern wind turbine towers are conical tubular steel towers, as we learned on the page about wind turbine towers. This image from a tower manu- facturer's workshop shows how a steel plate is rolled into a conical subsection for a wind turbine tower. It is a bit tricky to achieve the conical shape, since the tension (pressure) of the steel rollers has to be different at the two sides in order to make the plate bend properly. Towers are assembled from these smaller, conical subsections which are cut and rolled into the right shape, and then welded together. Towers are usually manufactured in 20 to 30 m sections (65 to 100 ft.), the limiting factor being transportation on roads or rail. Typical modern tower weights are 40 metric tonnes for a 50 m (165 ft.) tower for a turbine with a 44 m rotor diameter (600 kW), and 80 metric tonnes for a 60 metre tower for a 72 m rotor diameter (2000 kW). |
|  | Designed by the Turbine Manufacturer Towers for wind turbines are generally designed by each turbine manufacturer, since the entire wind turbine has to be type approved as a unit. (The reasons are explained in the page about structural dynamics ). So even if some towers are manufactured by independent producers, they are always specific for each manufacturer. Independent tower manufacturers are often also manufacturers of oil tanks or pressure vessels, since the machinery and safety inspection procedures are very similar. Weight Matters Tower weights (per installed power in kW) have declined by about 50% during the past five years due to more advanced design methods. Still, towers are a fairly heavy part of the wind turbine, so transportation costs are important. For larger markets it generally does not pay to transport towers more than 1000 km (600 miles) by road. In case the distance is larger (and the project is a large one), towers are usually manufactured locally. |
|  | Banana Peel Shaped Plates In order to end up with a cone-shaped section, the plate used for rolling has to be curved along the longest edges, and the short edges are not parallel. Most tower manufacturers use programmable laser cutting tools in order to obtain the appropriate shape for the steel plate. |

| Welding Wind Turbine Towers Steel Sections are Powder Welded |  |
| --- | --- |
| All photographs Soren Krohn © 1999 DWIA | Each tower section is welded with a seam lengthwise, plus a circular welding seam to connect to the next section of the tower. This is done by placing the tower sections on a rolling bed which slowly rotates the tower, while an operator with a powder welding machine welds the sections from the outside... |
|  | ...and another operator welds a corresponding set of seams on the inside. |
|  | Checking Welding Seams for Safety Welding seams in towers are checked using ultrasonic or x-ray devices. Important seams are checked 100%, while other seams are checked on a sample basis. |

| Installing and Assembling Wind Turbine Towers Attaching Towers to their Foundations |  |
| --- | --- |
| All pictures Soren Krohn © 1999 DWIA | Towers are usually bolted onto the concrete foundations on which they are placed. There are other methods, however, as in this case where part of the bottom section of the tower is cast into the concrete foundation, and where the lowest section of the tower is subsequently welded together directly on the site. This method requires that the tower be fitted with special guides and clamps to hold the two tower sections in place while the welding is being done. It also requires a small mobile tower factory including a generator, welding gear, and x-ray inspection equipment for checking the welding seams. |
|  | Flanges Wind turbine tower sections are bolted together using hot rolled steel flanges, which are welded to the end of each tower section. The flanges are made from killed steel. The image shows a pair of flanges. |
|  | Bolt Assembly The next image shows how the tower sections are bolted together inside the tower. The quality of the flanges and the bolt tensions are important parameters for the safety of wind turbine towers. |

| Research and Development in Wind Energy For wind turbine manufacturers, the basic aim of research and development of wind turbines is to be able to manufacture ever more cost effective machines. Basic Aerodynamics Research |  |
| --- | --- |
| Photograph of computer simulation of airflows around a rotor blade © Risoe National Laboratory, Denmark | Wind turbines engineers use techniques such as stall , which aircraft designers try to avoid at all costs. Stall is a very complex phenomenon, because it involves airflows in three dimensions on wind turbine rotor blades. (e.g. the centrifugal force will induce an airflow which makes the air molecules move radially along the rotor blade from its root towards the tip of the blade). 3D computer simulations of airflows are rarely used in the aircraft industry, so wind turbine researchers have to develop new methods and computer simulation models to deal with these issues. Computational Fluid Dynamics, or CFD, is a group of methods that deal with simulating airflows around e.g. rotor blades for wind turbines. The picture shows a computer simulation of the airflows and pressure distributions around a wind turbine rotor blade moving towards the left. Aerodynamic Improvement Devices A number of technologies known from the aircraft industry are increasingly being applied to improve the performance of wind turbine rotors. One example is vortex generators, which are small fins, often only about 0.01 metre (0.4 inch) tall, which are fitted to the surface of aircraft wings. The fins are alternately slightly skewed a few degrees to the right and the left. The fins create a thin current of turbulent air on the surface of the wings. The spacing of the fins is very accurate to ensure that the turbulent layer automatically dissolves at the back edge of the wing. Curiously, this creation of minute turbulence prevents the aircraft wing from stalling at low wind speeds. Wind turbine blades are prone to stalling even at low wind speeds close to the root of the blade where the profiles are thick. Consequently, on some of the newest rotor blades you may find a stretch of one metre or so along the back side of the blade (near the root) equipped with a number of vortex generators. |

| Offshore Wind Power Research |  |
| --- | --- |
| Vindeby Offshore Wind Farm Photograph © 1992 Bonus Energy A/S | Megawatt sized wind turbines, cheaper foundations and new knowledge about offshore wind conditions is improving the economics of offshore wind power. While wind energy is already economic in good onshore locations, wind energy is about to cross another frontier: The economic frontier set by shorelines. Researchers and developers are about to challenge conventional wisdom on electricity generating technologies: Offshore wind energy is rapidly becoming competitive with other power generating technologies. The Danish Plan 21 According to The Danish Governments' Action Plan for Energy, Energy 21 (see the Links page), 4,000 MW of offshore wind power should be installed before year 2030. Onshore the target was 1,500 MW. All together Denmark would then be able to cover more than 50 per cent of the total electricity consumption by wind energy. In 2003, the installation in Denmark amounted to approx. 2,600 onshore and just under 400 MW offshore. A total of 5,500 MW of wind power in the Danish electricity system means that the wind turbines periodically will cover more than 100 per cent of Danish electricity demand. Therefore, the future Danish offshore power plants should be an integrated part of the Scandinavian electricity system, which is based on huge amounts on hydro power. With a total investment of some 48 billion DKK (= 7 billion USD) for the 4,000 MW offshore capacity the Danish action plan will be the world's largest investment in wind power ever. Offshore Timetable in Denmark I 1998, the Danish utility companies received an executive order from the Danish Government to build 750 MW offshore wind power, divided between five demonstration wind farms. The two first of the large demonstration farms were Horns Rev with 160 MW (2002) and Nysted with 158 MW (2003). In 2002, the executive order was altered to only these two wind farms. The three remaining wind farms will be put out to tender (expected at the end of 2003). A report drafted by the Danish power companies for the Minister of Environment and Energy identifies four main areas in Danish sea territory suitable for wind power with a potential of 8,000 MW. The philosophy behind the selected areas is simple: For environmental reasons the Committee has concentrated the capacity in few and remote areas with water depths between 5 and 11 metres. The areas have been selected to avoid national park areas, shipping routes, microwave links, military areas, etc. The distance from coastal areas varies from 7 to 40 km. This also minimises the visual impact onshore. The most recent research into foundations indicates that it may be economic to install offshore turbines even at 15 metres water depth. This mean that the offshore potential is some 16,000 MW in the selected areas in the Danish Waters. |

| Wind Turbine Offshore Foundations |  |
| --- | --- |
|  | The major challenge for offshore wind energy is cutting costs: Undersea cabling and foundations have until recently made offshore wind energy an expensive option. New studies of foundation technology, however, plus megawatt-sized wind turbines are now on the point of making offshore wind energy competitive with onshore sites, at least for shallow water depths up to 15 metres (50 ft.). Since offshore wind turbines generally yield 50 per cent higher output than turbines on nearby onshore sites (on flat land), offshore siting may be quite attractive, cf. the page on offshore wind conditions. |
|  | Steel is Cheaper Than Concrete Two Danish power company groups and three engineering firms made a pioneering study on the design and costing of offshore wind turbine foundations in 1996-1997. The report concluded that steel is far more competitive than concrete for larger offshore wind farms. It appears that all of the new technologies will be economic until at least 15 metres water depth, and possibly beyond such depths. In any case, the marginal cost of moving into deeper waters is far smaller than what was previously estimated. With these concepts foundation and grid connection costs for large 1.5 megawatt turbines are only 10 to 20 per cent higher than the corresponding costs for the 450-500 kW turbines used at Vindeby and Tunø Knob offshore wind parks in Denmark. 50 Year Design Lifetime Contrary to popular belief, corrosion is not a major concern with offshore steel structures. Experience from offshore oil rigs has shown that they can be adequately protected using cathodic (electrical) corrosion protection. Surface protection (paint) on offshore wind turbines will routinely be delivered with a higher protection class than for onshore turbines. Oil rig foundations are normally built to last 50 years. This is also the design lifetime for the steel foundations used in these studies. Reference Turbine The reference turbine for the study is a modern 1.5 MW three-bladed upwind turbine with a hub height of about 55 metres (180 ft.) and a rotor diameter of some 64 metres (210 ft.). The hub height of the reference turbine is low compared with the typical onshore turbine of that size. In Northern Germany the typical hub height of a 1.5 MW turbine varies from 60 to 80 m (200 to 260 ft.). Because of the very smooth surface (low roughness ) of water surfaces it is cost-efficient to use lower towers. You may verify these conclusions using the Wind Turbine Power Calculator which already has a built in example of a 1.5 MW offshore wind turbine. |

| Offshore Foundations: Traditional Concrete |  |
| --- | --- |
| Foundation being floated out to Tunoe Knob Photograph © 1996 by Flemming Hagensen | The first offshore pilot projects in Denmark (and the world) used concrete gravity caisson foundations. As the name indicates, the gravity foundation relies on gravity to keep the turbine in an upright position. |
|  | Vindeby and Tunoe Knob Offshore Wind Farms Vindeby Offshore Wind Farm and Tunoe Knob Wind Farm are examples of this traditional foundation technique. The caisson foundations were built in dry dock near the sites using reinforced concrete and were floated to their final destination before being filled with sand and gravel to achieve the necessary weight. The principle is thus much like that of traditional bridge building. The foundations used at these two sites are conical to act as breakers for pack ice. This is necessary because solid ice is regularly observed in the Baltic Sea and the Kattegat during cold winters. Disadvantage of Concrete Using traditional concrete foundation techniques the cost of the completed foundation is approximately proportional with the water depth squared - the quadratic rule. The water depths at Vindeby and Tunoe Knob vary from 2.5 m to 7.5 m. This implies that each concrete foundation has an average weight of some 1050 metric tonnes. According to the quadratic rule the concrete platforms tend to become prohibitively heavy and expensive to install at water depths above 10 metres. Therefore, alternative techniques had to be developed in order to break through the cost barrier, as we shall see on the next pages. |

| Offshore Foundations: Gravitation + Steel |  |
| --- | --- |
|  | Most of the existing offshore wind parks use gravitation foundations. A new technology offers a similar method to that of the concrete gravity caisson. Instead of reinforced concrete it uses a cylindrical steel tube placed on a flat steel box on the sea bed. Weight Considerations A steel gravity foundation is considerably lighter than concrete foundations. Although the finished foundation has to have a weight of around 1,000 tonnes, the steel structure will only weigh some 80 to 100 tonnes for water depths between 4 and 10 m. (Another 10 tonnes have to be added for structures in the Baltic Sea, which require pack ice protection). The relatively low weight allows barges to transport and install many foundations rapidly, using the same fairly lightweight crane used for the erection of the turbines. The gravity foundations are filled with olivine, a very dense mineral, which gives the foundations sufficient weight to withstand waves and ice pressure. Size Considerations The base of a foundation of this type will be 14 by 14 m (or a diameter of 15 m for a circular base) for water depths from 4 to 10 m. (Calculation based on a wind turbine with a rotor diameter of 65 m). Seabed Preparation The advantage of the steel caisson solution is that the foundation can be made onshore, and may be used on all types of seabed although seabed preparations are required. Silt has to be removed and a smooth horizontal bed of shingles has to be prepared by divers before the foundation can be placed on the site. Erosion Protection The seabed around the base of the foundation will normally have to be protected against erosion by placing boulders or rocks around the edges of the base. This is, of course, also the case for the concrete version of the gravitation foundation. This makes the foundation type relatively costlier in areas with significant erosion. Costs by Water Depth for Steel Gravitational Foundations The cost penalty of moving to larger water depths is minimal compared to traditional concrete foundations. The reason is, that the foundation base does not have to increase in size proportion to the water depth to lean against ice pressure or waves. The cost estimates for this type of foundation is for instance 2,343,000 DKK (= 335,000 USD) for a 1.5 MW machine placed at 8 m water depth in the Baltic Sea (1997 figures). The costs include installation. The graph shows how the cost varies with water depth. Interestingly, the dimensioning factor (which decides the required strength and weight of the foundation) is not the turbine itself but ice and wave pressure forces. |

| Offshore Foundations: Mono Pile |  |
| --- | --- |
|  | The mono pile foundation is a simple construction. The foundation consists of a steel pile with a diameter of between 3.5 and 4.5 metres. The pile is driven some 10 to 20 metres into the seabed depending on the type of underground. The mono pile foundation is effectively extending the turbine tower under water and into the seabed. An important advantage of this foundation is that no preparations of the seabed are necessary. On the other hand, it requires heavy duty piling equipment, and the foundation type is not suitable for locations with many large boulders in the seabed. If a large boulder is encountered during piling, it is possible to drill down to the boulder and blast it with explosives. Costs by Water Depth for Mono Pile Foundations The dimensioning factor of the foundation varies from the North Sea to the Baltic Sea. In the North Sea it is the wave size that determines the dimension of the mono pile. In the Baltic Sea the pack ice pressure decides the size of the foundation. This is the reason why the mono pile foundation cost increases more rapidly in the Baltic Sea than in the North Sea. The costs include installation (1997 prices). Erosion Considerations Erosion will normally not be a problem with this type of foundation. Swedish Offshore Project A 2.5 MW pilot project with five Danish wind turbines using the mono pile technology has been installed in the Baltic sea south of the Swedish island of Gotland. Using the mono pile foundation technique at Gotland involved drilling a hole of 8 to 10 metres depth for each of the turbines (Wind World 500 kW). Each steel pile is slotted into the the solid rock. When the foundations are in place the turbines can be bolted on top of the mono piles. The whole operation takes about 35 days under average Baltic weather conditions. |

| Offshore Foundations: Tripod |  |
| --- | --- |
| Image © 1997 Ramboll | The tripod foundation draws on the experiences with light weight and cost efficient three-legged steel jackets for marginal offshore fields in the oil industry. From a steel pile below the turbine tower emanates a steel frame which transfers the forces from the tower into three steel piles. The three piles are driven 10 to 20 metres into the seabed depending on soil conditions and ice loads. Advantages of the Tripod The advantage of the three-legged model is that it is suitable for larger water depths. At the same time only a minimum of preparations are required at the site before installation. Multi-pile technology The foundation is anchored into the seabed using a relatively small steel pile (0.9 m diameter) in each corner. Because of the piling requirement, the tripod foundation is not suited for locations with many large boulders. Erosion Considerations Erosion will normally not be a problem with this type of foundation. Suitable for Larger Water Depths This type of foundation is not suitable at water depths lower than 6-7 metres. The main reason for this is that service vessels at low water depths will face problems approaching the foundation due to the steel frame. Cost by Water Depth for Tripod Foundations As in previous page, the basic difference between costs in the North Sea and the Baltic Sea is that waves determine dimensioning in the North Sea, whereas ice is decisive in the Baltic Sea. The costs include in |

| Wind Turbines in the Electrical Grid: Wind Energy Variations The vast majority of the installed power of wind turbines in the world is grid connected, i.e. the turbines feed their electricity directly into the public electrical grid. Wind Energy Production During a Fine Summer Week |  |  |
| --- | --- | --- |
| Wind presently covers about 20 per cent of the electricity consumption in the Western part of Denmark.The ELSAM electricity supply area comprises the Western part of Denmark with the Jutland Peninsula and the neighbouring island of Fyn. The area has a population of 3 million. | The graph above shows a summer week of electricity output from the 650 MW (megawatts) of wind turbines installed in the Western part of Denmark. The blue curve at the top left shows the power output on 25 June 1997, while the orange curve shows the output the preceding day. Electrical power consumption was 2,700 MW at the time this curve was printed from the power company control centre. Wind was supplying 270 MW i.e. wind was supplying exactly 10 per cent of the electricity consumption of 3 million people at 13:45 hours when we visited the control centre. Wind Matches Daily Electricity Consumption Patterns At the bottom of the graph you can see the power output of the five preceding days. On average, the month of June has the lowest wind power output during the year in Denmark. Some days of fresh winds, however, began in the early morning hours of 24 June. The typical weather pattern is that winds are low at night, and higher during the day, as you can see from the five days of moderate winds. This means that wind electricity generally fits well into the electricity consumption pattern, i.e. wind electricity tends to be more valuable to the electrical grid systems than if it were being produced at a random level. |  |
| Seasonal Variation in Wind Energy |  |  |
|  |  | Wind Matches Seasonal Electricity Consumption Patterns In temperate zones summer winds are generally weak compared to winter winds. Electricity consumption is generally higher in winter than in summer in these regions. In the cooler areas of the globe, electrical heating is therefore ideal in combination with wind energy, because the cooling of houses varies with the wind speed much like the electricity production of wind turbines vary with wind speeds. In electricity systems that are not based on hydropower and wind there may be good reasons to avoid electrical heating, however: Conventional power plant wastes a lot of heat, and thus fuel (at least 60%), i.e. for every unit of useful heat consumed by a household, the power station will waste 1.5 units of heat (and fuel). Annual Variation in Wind Energy Just like harvest yields vary from year to year in agriculture, you will find that wind patters may vary from year to year. Typically, the variations are less than the changes in agricultural production. In the case of Denmark, you will see that output from wind turbines typically have a variation (a standard deviation) of some 9 to 10 per cent. You may see the monthly and yearly variations in Denmark during more than 20 years on the web site Vindstyrke. |

| Wind Turbines and Power Quality Issues The buyer of a wind turbine does not need to concern himself with local technical regulations for wind turbines and other equipment connected to the electrical grid. This responsibility is generally left to the turbine manufacturer and the local power company. For the people who are technically minded, we go into some of the electrotechnical issues involved in connecting a turbine to the grid on this page. Power Quality The term "power quality" refers to the voltage stability, frequency stability, and the absence of various forms of electrical noise (e.g. flicker or harmonic distortion) on the electrical grid. More broadly speaking, power companies (and their customers) prefer an alternating current with a nice sinusoidal shape, such as the one in the image above. (If you are not familiar with the basics of alternating current (AC) it may be useful to consult the Reference Manual about this subject before continuing). Starting (and Stopping) a Turbine Most electronic wind turbine controllers are programmed to let the turbine run idle without grid connection at low wind speeds. (If it were grid connected at low wind speeds, it would in fact run as a motor, as you can read about on the generator page ). Once the wind becomes powerful enough to turn the rotor and generator at their rated speed, it is important that the turbine generator becomes connected to the electrical grid at the right moment. Otherwise there will be only the mechanical resistance in the gearbox and generator to prevent the rotor from accelerating, and eventually overspeeding. (There are several safety devices, including fail-safe brakes, in case the correct start procedure fails, which you may have read in the section on Wind Turbine Safety ). Soft Starting with Thyristors If you switched a large wind turbine on to the grid with a normal switch, the neighbours would see a brownout (because of the current required to magnetise the generator) followed by a power peak due to the generator current surging into the grid. You may see the situation in the drawing in the accompanying browser window, where you see the flickering of the lamp when you operate the switch to start the wind turbine. The same effect can possibly be seen when you switch on your computer, and the transformer in its power supply all of a sudden becomes magnetised. Another unpleasant side effect of using a "hard" switch would be to put a lot of extra wear on the gearbox, since the cut-in of the generator would work as if you all of a sudden slammed on the mechanical brake of the turbine. |  |  |  |
| --- | --- | --- | --- |
| Large power thyristors in wind turbines get very hot when they are activated. They have to be equipped with aluminium heat sinks and fans as you see in the picture to the right. Photograph Soren Krohn © 1998 DWIA | To prevent this situation, modern wind turbines are soft starting, i.e. they connect and disconnect gradually to the grid using thyristors, a type of semiconductor continuous switches which may be controlled electronically. (You may in fact have a thyristor in your own home, if you own a modern light dimmer, where you can adjust the voltage on your lamps continuously). Thyristors waste about 1 to 2 per cent of the energy running through them. Modern wind turbines are therefore normally equipped with a so called bypass switch, i.e. a mechanical switch which is activated after the turbine has been soft started. In this way the amount of energy wasted will be minimised. Weak Grids, Grid Reinforcement If a turbine is connected to a weak electrical grid, (i.e. it is vary far away in a remote corner of the electrical grid with a low power-carrying ability), there may be some brownout / power surge problems of the sort mentioned above. In such cases it may be necessary to reinforce the grid, in order to carry the fluctuating current from the wind turbine. Your local power company has experience in dealing with these potential problems, because they are the exact mirror-image of connecting a large electricity user, (e.g. a factory with large electrical motors) to the grid. Flicker Flicker is an engineering expression for short lived voltage variations in the electrical grid which may cause light bulbs to flicker. This phenomenon may be relevant if a wind turbine is connected to a weak grid, since short-lived wind variations will cause variations in power output. There are various ways of dealing with this issue in the design of the turbine, mechanically, electrically, and using power electronics. Preventing "Islanding" Islanding is a situation which may occur if a section of the electrical grid becomes disconnected from the main electrical grid, e.g. because of accidental or intended tripping of a large circuit breaker in the grid (e.g. due to lightning strikes or short circuits in the grid). If wind turbines keep on running in the isolated part of the grid, then it is very likely that the two separate grids will not be in phase after a short while. Once the connection to the main grid is re-established it may cause huge current surges in the grid and the wind turbine generator. It would also cause a large release of energy in the mechanical drive train (i.e. the shafts, the gear box and the rotor of the wind turbine) much like "hard switching" the turbine generator onto the grid would do. The electronic controller of the wind turbine will therefore constantly have to monitor the voltage and frequency of the alternating current in the grid. In case the voltage or frequency of the local grid drift outside certain limits within a fraction of a second, the turbine will automatically disconnect from the grid, and stop itself immediately afterwards. (Normally by activating the aerodynamic brakes as explained in the section on wind turbine safety ). |  |  |
| Grid Connection of Offshore Wind Parks The Grid The picture to the right shows the Danish electrical transmission grid. Major power stations are shown in yellow. Total generating capacity was some 10,000 MW in 1998. Present and future offshore wind parks with a total of some 4,100 MW are shown in white and blue. The western and eastern part of the country are not directly connected, but are connected to the German and Swedish electrical transmission systems using AC (alternating currency transmission lines). The rest of the connections to Sweden, Norway, and Germany are DC (direct current) connections. Grid connection of offshore wind parks is not a major technical problem per se , in the sense that the technologies which are involved are well known. Optimising these technologies for remote offshore sites will be important, however, to ensure reasonable economics. The first commercial-sized offshore wind farms in Denmark - Horns Rev and Nysted - are located 10-20 km (6-12 miles) from shore, at water depths from 6 to 14 metres. The wind farms are 160 MW (Horns Rev) and 158 MW (Nysted) and they consist of respectively 2 MW and 2.2 MW wind turbines. Cabling Undersea cabling connecting offshore parks to the main electrical grid is a well known technology. Undersea cables will have to be buried in order to reduce the risk of damage due to fishing equipment, anchors, etc. If bottom conditions permit, it will be most economic to wash cables into the seabed (using high pressure water jets) rather than digging or ploughing cables into the bottom of the sea. Voltages Within the offshore wind farm of Horns Rev a 36 kV connection is used. In the middle of the wind farm, there is a platform with a 36-150 kV transformer station, as well as a number of service facilities, like a helicopter platform. A 150 kV connection is used to transport the electricity to the shore. Reactive Power, HVDC |  |  |  |
| Reactive Power is related to phase-shifting of alternating current, which makes it more difficult to transport usable energy through the electrical grid. See the Reference Manual on this web site for the technical details. |  | The undersea cables will have a high electrical capacitance, which may be useful to supply reactive power to the parks. It may be optimal to have some form of variable reactive power compensation built into the system, depending on the precise grid configuration. If the distance to the main grid is considerable, an interesting alternative could be to connect the parks to the mainland using high voltage direct current connections (HVDC). Remote Surveillance Remote surveillance of the parks is even more important offshore than on land. Radio links for this purpose have been in operation at the Tunoe Knob and Vindeby offshore wind farms for some years. At Horns Rev, the control centre is connected to each wind turbines via fibre-optics cables. The large wind turbines make it cost-effective to install e.g. extra sensors on each piece of equipment, (and continuously analyse its minute vibrations which tend to change pattern as the part is worn down). This technology is well known in the industry to ensure optimum maintenance of machinery. Preventive Maintenance Since weather conditions may prevent service personnel from approaching the wind turbines at times of bad weather, it is extremely important to ensure a high availability rate of offshore wind turbines. Preventive maintenance check programmes may need to be optimised for remote offshore locations. |  |

| Wind Turbines and the Environment: Landscape Hints About Landscape Architecture and Wind Turbines |  |
| --- | --- |
| Photograph Søren Krohn © 1999 DWIA | Wind turbines are always highly visible elements in the landscape. Otherwise they are not located properly from a meteorological point of view, cf. the page on wind turbine siting. The image to the left shows the wind farm at Kappel, Denmark. It is perhaps the most aesthetically pleasing layout of any wind farm known to this author. The shape of the dike along the coastline is repeated in the line of turbines. There is one disturbing element in the picture above: The single turbine next to the farmhouse, which interrupts the otherwise smooth pattern of turbines. (That turbine was there before the wind farm was built). |
|  | Simple Geometrical Patterns In flat areas it is often a good idea to place turbines in a simple geometrical pattern which is easily perceived by the viewer. Turbines placed equidistantly in a straight line work well, but the example in the picture above may be even more elegant, where landscape contours invite such a solution. There are limits to the usefulness of being dogmatic about using simple geometrical patterns, however: In hilly landscapes it is rarely feasible to use a simple pattern, and it usually works better to the the turbines follow the altitude contours of the landscape, or the fencing or other characteristic features of the landscape. Whenever turbines are placed in several rows, one will rarely be able to perceive the pattern when the park is viewed from normal eye level. Only when one is standing at the end of a row, does it really appear as an ordered layout. In the next panorama picture, you will probably only be able to discern three rows of turbines, while the rest appear to be scattered around the landscape. |
| Photograph © 1997 Suzanne Clemmesen | Light Grey Paint The picture above shows one of the larger groupings of Danish built wind turbines at Näsudden on the island of Gotland in Sweden. The grey paint on the turbines make them blend well into the landscape. Size of Wind Turbines Large wind turbines enable the same amount of energy to be produced with fewer wind turbines. There may be economic advantages to this, such as lower maintenance costs. From an aesthetic point of view, large wind turbines may be an advantage in the landscape, because they generally have lower rotational speed than smaller turbines. Large turbines therefore do not attract the eye the way fast-moving objects generally do. People's Perception of Wind Turbines in the Landscape To a large extent it is a matter of taste how people perceive that wind turbines fit into the landscape. Numerous studies in Denmark, the UK, Germany, and the Netherlands have revealed that people who live near wind turbines are generally more favourable towards them than city dwellers. You may find more details about these studies in the article Public Attitudes Toward Wind Power on this web site. A beautiful book of photographic examples of Wind Turbines in the Landscape may be purchased from Birk Nielsens Tegnestue, Aarhus, Denmark. The price is approximately 150 DKK, plus postage. |

| 3D Visualisation of Aviation Obstruction Markings on Wind Turbines |  |
| --- | --- |
| Virtual reality image by Uni-C. | Wind turbines have grown considerably in size over the past 20 years. In 1980 the average size wind turbine had a rotor diameter of 10.5 metres - today there are many wind turbines with rotor diameters of more than 80 metres, roughly 25% larger than the wingspan of a Boeing 747. Consequently, an increasing number of wind turbines exceed the 100 m (300 ft.) limit of height for when obstruction marking may be required by the aviation authorities. The first time in Denmark was in 1999, when six 2 MW turbines were erected at Hagesholm on Zealand. According to Danish aviation regulations on obstruction marking, constructions of less than 100 metres tall are not marked, whereas constructions taller than 150 metres are always marked. For constructions which are between 100 and 150 metres tall the Danish Civil Aviation Administration passes judgement on a case-by-case basis whether, and if so, how a construction must be marked. |
|  | How Should Wind Turbines be Marked? Danish authorities, wind turbine manufacturers and project developers began to seriously address the question of how to mark wind turbines with a total height of above 100 metres in 2000 during the planning phase for the two first large scale Danish offshore wind farms. The two 160 MW wind parks were erected in 2002 and 2003 respectively, at Horns Rev in the North Sea and off the Nysted coast of the island Lolland. It was clear from the outset that obstruction marking of wind turbines with a total height of above 100 metres is a balancing act between a number of different considerations. On the one hand, defence authorities require clear obstruction markings justified by the need for military training exercises as well as rescue operations. On the other hand, the envirnomental authorities want obstruction markings which are disturbance-free to nature and neighbours. The consulting engineering firms behind the Horns Rev and Nysted projects thus had a pressing need to get clarification on how the two first large offshore wind farms in the world were to be marked. The Danish wind turbine manufacturers wished to assist in resolving the question in particular in regard to making clear regulations and realistic demand specifications for light emission intensity and pulse frequency. At the same time, it was deemed important to find solutions friendly to the eye and the neighbouring environment. Accordingly, the Danish Wind Industry Association and the two consulting engineering firms in the autumn of 2000 initiated a 3D-visualisation project: Visualisation of Obstruction Lights on Wind Turbines. The project obtained 1 million DKK (135 ,000 EUR) in support from the Public Service Obligation fund controlled by Eltra, one of the Danish transmission system operators. 1) 1) The concept "Public Service Obligation" in this case covers the legal obligation for the transmission system operator to conduct or fund research which is of importance to society in the TSO's field of responsibility. Why 3D Visualisation? The immediate objective of the project was to develop a so-called wind turbine configurator, which may be used to visualise a predetermined wind farm with different types of obstruction lights of varying colour and intensity. The resulting configurations can then be visualised from different angles and distances as well as under different weather and light conditions. An advanced visualisation tool seemed to be the appropriate solution to get a realistic picture enabling evaluation of visibility of different markings as well as of their environmental impact. Firstly, it can be used to visualise new wind farms before they are erected. Secondly, it is much cheaper to visualise different solutions than to mount different models on existing wind farms and conduct military fly-by exercises. As such the visualization tool will be useful to balance the debate between different authorities and actors. Ultimately the tool will help find the most appropriate solution for obstruction marking of wind turbines. Nysted Case A Danish software company was given the task to develop the necessary 3D software in collaboration with the project participants. The final visualisation software comprises a number of modules: A wind turbine module, a light emission module, and a wind farm module. In addition, the software can handle a landscape database, e.g. the sea, a coastline or a location on land. The project used Nysted Offshore Wind Farm south of Lolland as a case for its model. The offshore farm will be connected to the grid in 2003. Nysted Offshore Wind Farm will consist of 72 wind turbines, each having a rated power of 2.2 MW. The hub height is 68.8 m (226 ft), the rotor diameter is 82.4 m (270 ft). Maximum tip height is therefore 110 m (361 ft). The offshore farm will be laid out as 8 rows of 9 turbines - the closest some 10 kilometres off shore. The 72 wind turbines have an production equivalent to the consumption of 110.000 Danish households. Video clips from the Project The most realistic visualisation viewing is achieved in 3D on a large screen in a virtual reality centre. This is particularly important with regard to reproduction of light, as the impression of light is subjective to the viewing situation. A lighted wind farm is much more visible to the eye if viewed at night from a dark beach than if viewed from a lighted city street. In a VR centre, adjustments can be made to compensate from these factors. The visualisation programme and the Nysted case has on several occasions been projected in a VR centre for an audience of authorities and other potential users. As such the programme has provided input to the ongoing debate. If you view the video clip from a computer screen instead, the necessary adjustments cannot be made. The simulation quality will depend greatly on the light intensity of the screen, light falling onto the screen, etc. Furthermore, the comparatively smaller size of the screen obviously also compromises the quality. The simulation is therefore far less realistic than on a computer screen than in a VR centre. The video clips (3 minutes, 3 MB in QuickTime) show the solutions which were finally adopted for the Horns Rev and Nysted-Rødsand projects, plus some solutions which were not accepted. Click the image below to see the movie. The movie will be available in English in July. The link below points to the Danish version. |

Sound from Wind Turbines

Noise is a Minor Problem Today

It is interesting to note that the sound emission levels for all new Danish turbine designs tend to cluster around the same values. This seems to indicate that the gains due to new designs of e.g. quieter rotor blade tips are spent in slightly increasing the tip speed (the wind speed measured at the tip of the rotor blade), and thus increasing the energy output from the machines.

In the guided tour section on Wind Turbine Design we have explained how turbines today are engineered to reduce sound emissions.

It thus appears that noise is not a major problem for the industry, given the distance to the closest neighbours (usually a minimum distance of about 7 rotor diameters or 300 m = 1000 ft. is observed).

The concepts of sound perception and measurement are not widely known in the public, but they are fairly easy to understand, once you get to grips with it. You can actually do the calculations yourself in a moment.

Planning Wind Turbine Installation in Regard to Sound

Fortunately, it is usually reasonably easy to predict the sound effect from wind turbines in advance. On one of the following pages you may even try for yourself, using the Sound Map Calculator , which was used to draw the picture.

Each square measures 43 by 43 metres, corresponding to one rotor diameter. The bright red areas are the areas with high sound intensity, above 55 dB(A). The dashed areas indicate areas with sound levels above 45 dB(A), which will normally not be used for housing etc. (We get to the explanation of the sound level and dB(A) in a moment).

As you can see, the zone affected by sound extends only a few rotor diameters' distance from the machine.

Background Noise: Masking Noise Drowns out Turbine Noise

No landscape is ever completely quiet. Birds and human activities emit sound, and at winds speeds around 4-7 m/s and up the noise from the wind in leaves, shrubs, trees, masts etc. will gradually mask (drown out) any potential sound from e.g. wind turbines.

This makes it extremely difficult to measure sound from wind turbines accurately. At wind speeds around 8 m/s and above, it generally becomes a quite abstruse issue to discuss sound emissions from modern wind turbines, since background noise will generally mask any turbine noise completely.

The Influence of the Surroundings on Sound Propagation

Sound reflection or absorption from terrain and building surfaces may make the sound picture different in different locations. Generally, very little sound is heard upwind of wind turbines. The wind rose is therefore important to chart the potential dispersion of sound in different directions.

Human Perception of Sound and Noise

Most people find it pleasant listen to the sound of waves at the seashore, and quite a few of us are annoyed with the noise from the neighbour's radio, even though the actual sound level may be far lower.

Apart from the question of your neighbour's taste in music, there is obviously a difference in terms of information content. Sea waves emit random "white" noise, while you neighbour's radio has some systematic content which your brain cannot avoid discerning and analysing. If you generally dislike your neighbour you will no doubt be even more annoyed with the noise. Sound experts for lack of a better definition define "noise" as "unwanted sound".

Since the distinction between noise and sound is a highly psychological phenomenon, it is not easy to make a simple and universally satisfactory modelling of sound phenomena. In fact, a recent study done by the Danish research institute DK Teknik seems to indicate that people's perception of noise from wind turbines is governed more by their attitude to the source of the noise, rather than the actual noise itself.

Measuring and Calculating Sound Levels

The dB(A) Scale

Public authorities around the world use the so-called dB(A), or decibel (A), scale to quantify sound measurement. To give you an idea of the scale, look at the table below.

| Sound Level | Threshold of Hearing | Whisper | Talking | City Traffic | Rock Concert | Jet Engine 10 m Away |
| --- | --- | --- | --- | --- | --- | --- |
| dB(A) | 0 | 30 | 60 | 90 | 120 | 150 |

The dB(A) scale measures the sound intensity over the whole range of different audible frequencies (different pitches), and then it uses a weighing scheme which accounts for the fact that the human ear has a different sensitivity to each different sound frequency. Generally, we hear better at medium (speech range) frequencies than at low or high frequencies. The dB(A) system says, that the sound pressure at the most audible frequencies are to be multiplied by high numbers while the less audible frequencies are multiplied by low numbers, and everything is then added up to get an index number.

(The (A) weighing scheme is used for weak sounds, such as wind turbines. There exist other weighing schemes for loud sounds called (B) and (C), although they are rarely used).

The dB-scale is a logarithmic, or relative scale. This means, that as you double the sound pressure (or the energy in the sound) the index increases by approximately 3. A sound level of 100 dB(A) thus contains twice the energy of a sound level of 97 dB(A). The reason for measuring sound this way is that our ears (and minds) perceive sound in terms of the logarithm of the sound pressure, rather than the sound pressure itself.

Most people will say, that if you increase the dB(A) by 10, you double the subjective loudness of the sound.

In case you are interested in the exact definitions, take a look at the Reference Manual on Acoustics of this web site.

Sound Propagation and Distance: Inverse Square Law

The energy in sound waves (and thus the sound intensity) will drop with the square of the distance to the sound source. In other words, if you move 200 m away from a wind turbine, the sound level will generally be one quarter of what it is 100 m away. A doubling of your distance will thus make the dB(A) level drop by 6.

At one rotor diameter distance (43 m) from the base of a wind turbine emitting 100 dB(A) you will generally have a sound level of 55-60 dB(A) corresponding to a (European) clothes dryer. 4 rotor diameters (170 m) away you will have 44 dB(A), corresponding to a quiet living room in a house. 6 rotor diameters (260 m) away you will have some 40 dB(A).

The precise relationship between sound level and distance from the sound source is given in a table on the Reference Manual on Acoustics of this web site.

In practice, sound absorption and reflection (from soft or hard surfaces) may play a role on a particular site, and may modify the results shown here.

Adding Sounds from Several Sources

If we have two wind turbines rather than one, located at the same distance from our ears, naturally the sound energy reaching us will double. As we have just learned, this means that two turbines will increase the sound level by 3 dB(A). Four turbines instead of one (at the same distance) will increase the sound level by 6 dB(A). You will actually need ten turbines placed at the same distance from you, in order to perceive that the subjective loudness has doubled (i.e. the dB level has increased by 10).

If you wish to learn the details about adding sounds together, take a look at the Reference Manual on Acoustics in this web site.

The Pure Tone Penalty

The fact that the human ear (and mind) discerns pure tones more easily than (random) white noise, means the authorities may wish to take that into account when doing sound estimates. They consequently often have rules which specify that you add a certain number to the dB(A) figure in case you have pure tones present in a sound.

Wind Turbine Noise Information in Practice

In accordance with international standards manufacturers generally specify a theoretical dB(A) level for sound emissions which assumes that all sound originates from a central point, although in practice, of course, it will originate from the whole surface of the machine and its rotor.

Sound pressure thus calculated is typically around 96-101 dB(A) for modern wind turbines. The figure itself is rather uninteresting, since there will not be a single point, where you can experience that sound level! Rather, it is useful for predicting the sound level at different distances from the wind turbine.

Pure tones have generally be eradicated completely for modern wind turbines, at least in the case of the modern turbines listed in the catalogue on the Wind Power Calculator page.

Legal Noise Limits

At distances above 300 m the maximum theoretical noise level from high quality wind turbines will generally be significantly below 45 dB(A) outdoors, corresponding to the legislation in Denmark. (For built-up areas with several houses, a noise limit of 40 dB(A) is the legal limit in Denmark).

Noise regulations vary from country to country. In practice the same machine designs can be used everywhere.

Current Practice: Calculations Rather than Measurement

Calculating potential sound emission from wind turbines is generally important in order to obtain planning permission (from the public authorities) for installing wind turbines in densely populated areas.

Generally speaking, it is far easier to calculate the potential sound emissions than to measure them in practice.

The reason why it is difficult to measure the sound is that the sound level has to be some 10 dB(A) above the background noise in order to measure it properly. The background noise from leaves, birds, and traffic will frequently be above 30 dB(A), however. In most places in the world public authorities therefore rely on calculations rather than measurements, when granting planning permission for wind turbines.

Energy Payback Period for Wind Turbines

Two to Three Months Required

Modern wind turbines rapidly recover all the energy spent in manufacturing, installing, maintaining, and finally scrapping them. Under normal wind conditions it takes between two and three months for a turbine to recover all of the energy involved.

This is one of the main results of a life cycle analysis of wind turbines done by the Danish Wind Industry Association.

The study includes the energy content in all components of a wind turbine, and it includes the global energy content in all links of the production chain.

You may download the 16 page report.

Input Output Analysis Method

To find the results, the study employs a so called input output model of the Danish economy published by the Danish Central Bureau of Statistics. The input output model divides the economy into 117 sub sectors, and accounts for the flows of 27 different energy goods (fuels etc.) between the 117 sectors.

The basic advantage of using this method instead of engineering calculations, is that we are able to account properly for the amount of energy used by producers of components and manufacturing equipment, buildings etc. in all links of the production chain. The result is a large 117 by 117 table of energy flows. (Doing a mathematical operation on the table called matrix inversion we obtain the amount of energy per dollar of output).

The Energy Balance for Offshore Wind Turbines

Offshore wind turbines may have a slightly more favourable energy balance than onshore turbines, depending on local wind conditions. In Denmark and the Netherlands, where wind turbines onshore are typically placed in flat terrain, offshore wind turbines will generally yield some 50 per cent more energy than a turbine placed on a nearby onshore site. The reason is the low roughness of the sea surface.

On the other hand, the construction and installation of foundations require 50 per cent more energy than onshore turbines.

It should be remembered, however, that offshore wind turbines have a longer expected lifetime than onshore turbines, in the region of 25 to 30 years. The reason is that the low turbulence at sea gives lower fatigue loads on the wind turbines.

Analysis of 1980 Vintage Turbines

1980 wind turbines do surprisingly well in the studies of the energy balance. The analysis shows that while small Danish 1980 turbines of 10-30 kW took almost a year to recover the energy spent in manufacturing, installing and decommissioning them, turbines of 55 kW took some 6 months to recover all of the energy.

Birds and Wind Turbines

Birds often collide with high voltage overhead lines, masts, poles, and windows of buildings. They are also killed by cars in the traffic.

Birds are seldom bothered by wind turbines, however. Radar studies from Tjaereborg in the western part of Denmark, where a 2 megawatt wind turbine with 60 metre rotor diameter is installed, show that birds - by day or night - tend to change their flight route some 100-200 metres before the turbine and pass above the turbine at a safe distance.

In Denmark there are several examples of birds (falcons) nesting in cages mounted on wind turbine towers.

The only known site with bird collision problems is located in the Altamont Pass in California. Even there, collisions are not common, but they are of extra concern because the species involved are protected by law.

A study from the Danish Ministry of the Environment says that power lines, including power lines leading to wind farms, are a much greater danger to birds than the wind turbines themselves.

Some birds get accustomed to wind turbines very quickly, others take a somewhat longer time. The possibilities of erecting wind farms next to bird sanctuaries therefore depend on the species in question. Migratory routes of birds will usually be taken into account when siting wind farms, although bird studies from Yukon, Canada, show that migratory birds do not collide with wind turbines (Canadian Wind Energy Association Conference, 1997).

| Birds and Offshore Wind Turbines |  |
| --- | --- |
| Ornithologists' (Bird watchers) tower erected next to the offshore wind farm at Tunø Knob, Denmark, for a three-year avian study which was completed in 1997. Photograph Soren Krohn © 1997 DWIA | Offshore wind turbines have no significant effect on water birds. That is the overall conclusion of a three year offshore bird life study made at the Danish offshore wind farm Tunø Knob. The offshore wind park has been placed in this particular area because of a very substantial population of eiders (Somateria mollissima) and a small population of scoters ( Melanitta nigra ). At Tunø Knob more than 90 per cent of the birds are eiders, and about 40 per cent of the North Atlantic population of eiders are wintering in the Danish part of the Kattegat Sea. The Studies were conducted by the National Environmental Research Institute at Kalø, Denmark. Eight Different Studies The very thorough study consists of both aerial surveys, bird counts from observation towers, and observations of the spatial distribution of birds at the offshore site as well as at a similar control site in the same region. In the three year period some eight experiments were carried out. The central experiment was a so called before-after-control-impact study. From a watch tower placed one kilometre from the turbines and from aeroplanes scientists mapped the population of eiders the winter before the erection of the turbines and the following two winters. Declining Population During the three year period the number of Eiders declined by 75 per cent and the number of scoters declined by more than 90 per cent. But more interestingly, the population of water birds fell in all of the shoal of the Tunø Knob and not just around the turbines. This indicated that other factors than the turbines had to be taken into account. At the same time the area was repeatedly surveyed by divers in order to determine variations in the amount of blue mussels (Mytilus edulis) which the birds prey on. Less Food The amount of blue mussels showed also great natural variation over the three years. Especially the population of smaller mussels which are the eiders' preferred prey fell significantly in the three year period. With these findings in mind the scientific group concluded that the changes in size and composition of the blue mussel population could explain the variation in the number of eiders before and after the construction of the wind farm. Safe Distance Controlled experiments stopping the wind turbines for a certain period has been performed. In another experiment decoys was used to attract the eiders, which are very social birds. The result of the experiment using groups of decoys at different distances from the wind farm showed that the eiders were reluctant to pass at distances of 100 m or closer to the turbines. The on/off experiment showed that there was no detectable effect of revolving rotors on the abundance of eiders in the area. In fact the eiders - like people - apparently prefer rotating turbines (but that result was clearly insignificant). The overall conclusion of the final two experiments were that on one hand the eiders do keep a safe distance to the turbines, but on the other hand they do not get scared away from their foraging areas by revolving rotors. Also, the eiders showed normal landing behaviour until 100 m from the turbines. Mussels Matter The prevalence of eiders in the different zones around the turbines could be fully accounted for by the relative abundance of food. The English edition of this study "Impact Assessment of an Off-shore Wind Park on Sea Ducks, NERI Technical Report No. 227 1998" is available from NERI, the National Environmental Research Institute, Denmark. |

Shadow Casting from Wind Turbines

Wind turbines, like other tall structures will cast a shadow on the neighbouring area when the sun is visible. If you live very close to the wind turbine, it may be annoying if the rotor blades chop the sunlight, causing a flickering (blinking) effect while the rotor is in motion.

A bit of careful planning, and the use of good software to plan your wind turbine site can help you resolve this problem, however. If you know where the potential flicker effect is of a certain size, you may be able to place the turbines to avoid any major inconvenience for the neighbours.

Few Rules

Shadow casting is generally not regulated explicitly by planning authorities. In Germany, however, there has been a court case in which the judge tolerated 30 hours of actual shadow flicker per year at a certain neighbour's property. In the 30 hours, it appears, one should only include flicker which occur during the hours where the property is actually used by people (who are awake).

Predicting Shadow Flicker

Fortunately, we are able to predict quite accurately the probability of when and for how long there may be a flicker effect. We may not know in advance whether there is wind, or what the wind direction is, but using astronomy and trigonometry we can compute either a likely, or a "worst case" scenario, i.e. a situation where there is always sunshine, when the wind is blowing all the time, and when the wind and the turbine rotor keep tracking the sun by yawing the turbine exactly as the sun moves.

Figuring out the exact shape, place, and time of the shadow from a wind turbine requires a lot of computation, but at least one professional wind software programme can do this very accurately, even in hilly terrain, and with house windows of any size, shape, location and inclination facing in any direction. (See the Links page for the address of wind software companies).

Do it Yourself

On one of the following pages we have included another shadow calculator, which will give you a possibility of computing a shadow map of your particular area in flat terrain. The calculator gives you a lot of options to produce realistic estimates of actual shadow casting. Fortunately, you will discover that shadow casting problems are generally restricted to a few areas close to the turbine.

Since the calculation of shadow casting requires lots of computer power, we have included a number of important general results on the following pages.

| Calculating Shadows from Wind Turbines Daily Shadow Variation - Worst Case This simulation of shadow casting shows how the rotor shadow moves (worst case) from sunrise to sunset on a particular day at a certain location on the globe. The image is seen directly from above, with the centre of the wind turbine tower placed at the tiny black dot in the centre. The shadow positions are shown for every half hour during the day. Shadows, of course, are long around sunrise and sunset, and short at noon. This particular set of images was made for 55° Northern latitude for 21 September, assuming a 43 m rotor diameter on a 50 m tower, using the shadow simulation programme on this web site. Doing a worst case simulation we assume that the rotor yaws so as to track the movement of the sun exactly. This is is equivalent to assuming that the rotor is a solid balloon (or a Darrieus turbine ). Annual and Daily Shadows - Worst Case |  |
| --- | --- |
| Map of maximum (worst case) shadows around a 600 kW wind turbine placed at 55 degrees Northern latitude. The turbine has a 43 m rotor diameter and a 50 m tower. The map is 1200 m wide (East - West) and 750 m in the North - South direction. The map was computed using the Wind Turbine Shadow Calculator on this web site. | This map shows how shadows are typically distributed around a wind turbine throughout a year, assuming a worst case direction of the rotor. You will notice a number of kidney-shaped or bell-shaped areas around the wind turbine in the centre of the map. Each of the grey areas represents a certain maximum number of minutes of shadow from the wind turbine rotor. Since this map was computed for 55 degrees latitude in the Northern hemisphere, there is no shadow South of the turbine. Timing Shadows You will notice from the white lines on the map, that we can easily predict the time of day when shadows may occur. The shadow will e.g. obviously be directly North of the turbine at solar noon, when the sun reaches its maximum height in the sky. (Solar noon varies a bit during the year relative to our clocks, but it is fairly close to 12 o' clock, local time). The shadow will be to the bottom left at 4 o'clock in the morning on a summer day, so shadows to the Southwest are a minor problem in the Northern hemisphere. (The shadows occur in summer only, and at 4 in the morning most neighbours will be asleep anyway). The commercial software we referred to earlier will tell you exactly the dates and times when shadows may occur. |

Refining Shadow Calculations for Wind Turbines

Random Rotor Direction (Random Azimuth)

It is very unlikely that the wind and thus the rotor will track the sun in practice. We may therefore get a more realistic result if we modify our calculations by assuming that the rotor can assume any position at any time. In the small picture to the far right you can see a situation where the rotor is directly facing the sun. The tiny white dot near the bottom right is the centre of the wind turbine tower.

Now, let us assume that we yaw the rotor out of its position by one degree, take a snapshot of the shadow image, then yaw it by another degree, take another snapshot etc., until we have done a full 360 degree turn. Then we overlay all our 360 snapshots, and what we end up with will look similar to the small image to the left: The centre will get the most of the shadow, but as we move towards the outer edge (where the vertical edges of the rotor disc cast their shadows) the overall shadow intensity will decrease.

Shadow casting is on average reduced to 63% of the worst case results, if you assume a random rotor direction. Ideally, we should have a wind rose , (preferably hourly for each day or month) to do an exact calculation.

Fixed Rotor Direction (Fixed Azimuth)

In practice the wind turbine rotor will follow the wind direction (if the wind speed is above the cut in speed ). This image shows the shape of an area (in red) which gives 10 hours or more of shadows per year at 55° Northern latitude with the rotor yaw ( azimuth ) fixed at an angle of -45 degrees (i.e. with the wind permanently coming from the Southwest or Northeast). As you can see, there will be almost no shadows at an angle of +45 degrees, i.e. in the direction parallel to the rotor plane.

Shadow casting is typically reduced to around 62% of the worst case results, if we assume a fixed rotor direction.

Actual Rotor Direction (Wind Rose)

Usually we will already have a wind rose with a frequency distribution of the wind in the different directions of the compass when we are planning a wind turbine site. Using that information, we may calculate a more exact shadow picture. In the case of our test example, Copenhagen, shadows are reduced to some 64 per cent of the comparable worst case value.

Turbine Operating Hours

The rotor will not be running all the time, so we may multiply the number of minutes of shadow flicker by a factor of typically 0.75, depending on the local wind climate, (and ideally using the correct factor for daytime during each month).

Actual Sunshine Hours

When studying shadows, we should only count the fraction of the time when the sun is actually shining brightly, ideally using the correct fraction for each hour of the day during the year. In 1853 the first reliable sunshine recording device was invented (and improved in 1879), which means that in many parts of the world the meteorological institutes have very accurate long term statistics on the number of hours of bright sunshine during the year.

The number of bright sunshine hours varies with the geographical location and the season (summer or winter). We have included data for three Danish sites (Christiansø, Copenhagen, and Viborg) where the number of sunshine hours vary from 44 to 40, and 36 per cent of the time.

Combining Turbine operating hours, Actual Rotor Direction, and Actual Sunshine Hours

If we use both turbine operating hours, the actual rotor direction, and the actual bright sunshine hours we get a result (in the case of Denmark) which is some 18 per cent of the worst case assumption, using 75% operating hours in both cases. (The percentages given above are the results of simulations for Copenhagen on a 720 by 720 metre square with a turbine in the centre with 43 m rotor diameter and 50 m hub height).

The two images below compare a worst case simulation (with 75% operating hours) with an actual simulation for Copenhagen (also 75% operating hours) using both sunshine and wind statistics. The red area is the zone with 30 hours of shadow or more per year. Each map represents 720 by 720 metres.

The important conclusion of this simulation is that actual sunshine hours play a very important role in diminishing the amount of shadows north of the turbine (in the Northern hemisphere). The reason why this is important is that there are very few hours of sunshine when the sun is low in the sky to the south during winter.

| Shadow Variations from Wind Turbines Monthly Shadow Variation This movie shows the areas affected by shadow casting from a wind turbine. The movie shows how the area varies month by month - in this case in relatively high latitudes (55°) in the Northern hemisphere. The darkest areas represent the areas with most shadows. In winter the sun stays in the Southern part of the sky, and the shadows are distributed in a V-shaped area to the North of the turbine. In summer the sun rises very early in the morning to the Northeast and sunset is in the Northwest. This means that the summer shadows will be distributed in an A-shaped area, with the turbine in the tip of the "A". In locations closer to the equator there will be far less shadow North and South of the turbine. |  |
| --- | --- |
|  | Shadow Geometry Varies by Latitude Each latitude on the globe has its own shadow signature in terms of the area affected by a certain period of shadows from an object (30 hours per year). Close to the equator the signature resembles a butterfly. Farther away from the equator it becomes more kidney-shaped, and close to the poles it almost becomes a circle. All of the graphs above were computed using the shadow calculator on this web site, and assume a "worst case" or a random rotor position. Shadow Size Grows with Rotor Diameter The size of the rotor shadow and the number of shadow minutes per year in the vicinity of the turbine varies in proportion to the rotor area, as shown in the three pictures above. The red areas indicate the annual shadow patterns with more than 30 hours of shadow (worst case) from wind turbine rotors of 43, 53, and 63 m mounted on 50 m towers and computed for 55° latitude. Hub Height of Minor Importance The hub height of a wind turbine is of minor importance for the shadow from the rotor. The same shadow will be spread over a larger area, so in the vicinity of the turbine, say, up to 1,000 m, the number of minutes per year with shadows will actually decrease. The four pictures show shadow casting during a year (worst case) from a wind turbine with a 43 m rotor diameter, placed with four different hub heights and computed for 55° latitude. The red areas represent areas with more than 30 hours of shadows. If you are farther away from a wind turbine rotor than about 500-1000 metres, the rotor of a wind turbine will not appear to be chopping the light, but the turbine will be regarded as an object with the sun behind it. Therefore, it is generally not necessary to consider shadow casting at such distances. |

Guide to the Wind Turbine Shadow Calculator

The calculator on the following page allows you to simulate shadows from a wind turbine on a plane, horizontal landscape any minute, hour, day, month, or year anywhere on the globe.

Warning: Huge Plots Will Take Their Time - and lots of RAM

If you wish to compute shadows for a whole year, it may take your computer from 20 minutes to a couple of hours or more, depending on the speed of your browser and your machine, and how fine a map resolution and time resolution you choose. A fine map resolution (down to 3 pixels square) or a large plot area increases processing time and the required amount of RAM on your computer significantly.

Colouring the Plot

The grey colours in your plot are selected automatically by the programme, so that the most shadow affected areas are shown in pure black, while the least affected areas are shown in white, regardless of whether you run the programme for 1 minute or a year. The unaffected areas remain green.

Screen Settings

If you have a screen with millions of colours, you will find that the grey shadows vary very smoothly across the screen. If you like to be able to see the different "bands" of shadow minute values, like we have done in our images on this web site, set your monitor to thousands of colours, or even 256 colours.

You Can Save Your Shadow Maps

If you have generated a shadow map which you want to look at later, or compare with another map, you may save the page (e.g. onto your desktop), just like any other web page in HTML format, if you use Internet Explorer 4. Just choose Save from your file menu, (and take care where you save it, and what you name it).

Read the Number of Minutes of Shadows in Each Cell

if you have an Internet Explorer 4 browser, and you leave this option on when you generate the map, you can do an exact readout (in the status line of your browser) of the number of minutes there may be shadows in each cell by moving the cursor around on the shadow map.

You May Recolour Your Result

The plot uses a number of standard colours which look logical on a colour screen. The colours, however, may not be optimal if you wish to print the result on a black and white printer. We have therefore included a facility which allows you to change the colour scheme without redoing the long calculations: You may use a particular colour in a "shadow zone" around the turbine. If you use a large high resolution map, it may take a few minutes for your programme to do the recolouring (IE 4 is slower than Netscape 4 for this).

Paint Your Shadow Zone

You may modify your plot to show you any zone with a certain minimum number of minutes of shadows in a certain colour. Be warned, however, that with a large high resolution map, it will take several minutes to complete that process.

Other Calculator Usage

Incidentally, this calculator is very practical for photographers who wish to know where the sun is before they go out to take a picture of their favourite motive in ideal lighting conditions. (We tested it when photographing wind turbines, of course). You may also use it if you wish to know how to place a terrace in your garden (regardless of whether you want shadow or sun).

Location

You may either specify your turbine location using the pop up menu which gives the longitude and latitude of a number of cities around the globe, or you may enter the longitude and latitude in degrees and minutes directly, together with your time zone.

Time Zone

The time zone is automatically included, if you use the pop up menu with city names. You may enter your time zone relative to GMT from the pop up menus, or you may enter the standard time zone meridian, i.e. the longitude relative to Greenwich which your local time system uses as a reference, which is generally a multiple of 15 degrees, corresponding to a one hour time difference. (India and a few other places have a time zone which is a multiple of 7.5 degrees, i.e. half an hour).

Time

You may enter date and time to see the sunrise and sunset times, plus the current direction of the light coming from the sun.

Wind Turbine

Enter the hub height and the rotor diameter. A typical hub height for a 600-750 kW wind turbine is 45 to 60 m, a typical rotor diameter is 43 to 48 m. (You may find typical hub heights and rotor diameters using theWind Turbine Power Calculator turbine pop up menu).

If you wish to study shadows in areas which are lower than the base of the wind turbine, you can cheat, and increase the hub height of the turbine. Conversely, you can lower the hub height, if you wish to study areas which are higher than the base of the turbine.If you enter, say 0.5 for the rotor diameter, you may use the programme to study the behaviour of a shadow from the top of a mast, or the corner of a building. (Or you can use it to build your own sundial).

Shadow Plot

You can specify the time range for which you like your shadow images computed. You can select a minute, an hour, a day, a month, or a year.

You may set the plot area to fit your screen size (and/or paper output). If you have enough RAM (and time) you may even specify a map larger than your screen. The default size prints well on A4 paper in landscape format.The resolution parameter determines the area covered by each 3-25 pixel square. We recommend that you let each square represent less than half the rotor diameter to get a decent plot. Or, even more cleverly, you may set it to match your map resolution, and print your output on an acetate (overhead) foil as an overlay to a map of a prospective wind turbine location. (One printed pixel is 1/72 of an inch (1 inch = 2.54 cm)).The step length in minutes determines how many rotor images the programme projects onto your ground surface. The default step length of 4 minutes corresponds to the sun

azimuth changing on average 1 degree between each simulation. You may save processing time if you choose a longer step length. For a 1 month or 1 year simulation results are generally not affected much by using 8 minute steps - and it is 8 times faster than 1 minute steps. If the shadow image is not smooth, (or if it is asymmetrical in the East-West direction even if you are not running with a fixed rotor direction or a wind rose), your step length may be too large. If you double the step length, the programme assumes that the rotor shadow stays in the same place for twice as long, i.e. for each rotor image projected onto the ground, it adds the step length to a shadow counter for that particular area.

You may choose rotor direction as random (default), which means the rotor may be facing in any direction (random azimuth), you may choose worst case, where the rotor always faces the sun.

You may choose a fixed rotor azimuth angle from -90 to 90 degrees. The angle is measured relative to South, and the solar angle is positive before noon, regardless of hemisphere. 0 means that the wind is coming form the South or North. Southeast/Northwest is 45 degrees in the Northern hemisphere, and -45 degrees in the Southern hemisphere. East/West is 90 or -90 degrees. To help you select the correct angle, you may use the pop up menu.

Finally, you may choose to enter a wind rose with a frequency distribution for your wind directions. Since a normal propeller type wind turbine is symmetrical about its rotor plane, you should add the percentages for North and South, and so forth in each of your directions. The programme accepts 8, 12 and 16 compass directions, which means that you specify 4, 6, or 8 percentages. The program checks that the sum is exactly 100, before it is willing to do the simulation. Please note that wind roses are specified with the North as 0 degrees, and that the degrees are given in a clockwise direction (retrograde direction).

You should specify the fraction of daytime hours the turbine will be running. 0.75 is a typical fraction. The basic result in terms of minutes of shadows is multiplied by this fraction.

You should specify the fraction of daytime hours with bright sunshine. The basic result in terms of minutes of shadows is multiplied by this fraction.

If you have accurate statistics on the number of bright sunshine hours per month, you may instead use that data in your calculations, by filling out the sunshine table at the bottom of the page. In that case the programme uses the table data for each month instead of the average. We have included sunshine data for 3 Danish locations (you select them from the pop up menu). If you have reliable monthly data available for your location, please e-mail us (giving the source) so that we may include it in the city pop up menu. Remember to check the box that says you want to use the table for your calculations. (A clever trick: If you wish to see the pattern of shadows during e.g. June, July, and August only, you may set the sunshine percentages for the three months only, and leave the rest of the months at zero, and then run a simulation for a year, using the sunshine table).

You may set a maximum distance from the wind turbine for the shadow plot, since it is usually not relevant to look at distances above 7 to 10 rotor diameters or 1,000 m at the most.

Finally, you may choose to have your output displayed with mouse-sensitive shadow readout (for I.E. 4 browsers), which means that you may read the number of minutes of shadow in each cell on the map in your browser's status line by placing the mouse cursor on a particular cell. Using this mechanism increases RAM demand.

Sunrise

In this programme the sunrise time is defined as the moment a straight line to the centre of the sun passes the horizon in the upwards direction on the date you have entered in your data. In your local newspaper, you may find that the sunrise is defined as being some minutes earlier, when the upper edge of the sun reaches the horizon. In addition, the refraction (bending of the light) in the atmosphere means that you can actually see the sun before it reaches the horizon. The sunrise is in local time, or daylight saving time, if the Daylight saving time box is checked.

Noon

The solar noon is when the sun reaches it highest point in the sky, i.e. the solar altitude is at its maximum. Noon is in local time, or daylight saving time, if the Daylight saving time box is checked.

Sunset

In this programme sunset time is defined as the moment a straight line to the centre of the sun passes the horizon in the downwards direction on the date you have entered in your data. In your local newspaper, you may find that the sunset is defined as being some minutes later, when the upper edge of the sun reaches the horizon. In addition, the refraction (bending of the light) in the atmosphere means that you can actually see the sun after it goes below the horizon. The sunset is in local time, or daylight saving time, if the Daylight saving time box is checked.

Declination

The declination is the angle between the earth's equatorial plane, and the earth-sun line.As the earth rotates, it spins around its axis which points to the North Star. This axis is inclined 23.45° relative to the plane in which it orbits the sun. The angle between the equatorial plane and the earth-sun line thus varies between +/-23.45° during the year, being approximately zero on the 21/3 and 23/9 (Equinox), and reaching its extreme values on 21/6 and 21/12 (Solstice). (Its precise value varies a bit from year to year since a year is 365.25 days long).

Sunrise/Sunset Duration

This is the number of minutes and seconds it takes for the solar disc to move the 0.531° between the bottom and the top of the sun at sunrise or sunset. At the equator the sunrise and sunset last little more than two minutes. As you move towards the polar regions, the duration increases significantly, particularly in winter, as you may verify by altering the latitude.

Solar Azimuth

The solar azimuth is the angle in the horizontal plane between the South and the sun at the moment in time you have entered in your data. The angle is positive before noon, negative after noon (regardless of hemisphere).

Solar Altitude

The solar altitude is the angle between the horizontal plane and the sun.

Direction form the Sun (Sun Vector)

If you are standing in the centre of the turbine with your back towards the South, and you move x units of length to the right (East), y ahead (North), and z up (or rather -z down), then a straight line from your new position to the centre of the turbine will be pointing directly to the sun. The values for x, y, and z are given in the three boxes.

What does a Wind Turbine Cost?

The Price Banana

The graph above gives an impression of the price range of modern, Danish grid connected wind turbines as of February 1998. As you can see prices vary for each generator size. The reasons are e.g. different tower heights, and different rotor diameters. One extra metre of tower will cost you roughly 1 500 USD. A special low wind machine with a relatively large rotor diameter will be more expensive than a high wind machine with a small rotor diameter.

Economies of Scale

As you move from a 150 kW machine to a 600 kW machine, prices will roughly triple, rather than quadruple. The reason is, that there are economies of scale up to a certain point, e.g. the amount of manpower involved in building a 150 kW machine is not very different from what is required to build a 600 kW machine. E.g. the safety features, and the amount of electronics required to run a small or a large machine is roughly the same. There may also be (some) economies of scale in operating wind parks rater than individual turbines, although such economies tend to be rather limited.

Price Competition and Product Range

Price competition is currently particularly tough, and the product range particularly large around 1000 kW. This is where you are likely to find a machine which is optimised for any particular wind climate.

Typical 1000 kW Machines on the Market Today

Even if prices are very similar in the range from 600 to 750 kW, you would not necessarily want to pick a machine with as large a generator as possible. A machine with a large 750 kW generator (and a relatively small rotor diameter) may generate less electricity than, say a 600 kW machine, if it is located in a low wind area. The working horse today is typically a 1000 kilowatt machine with a tower height of some 60 to 80 metres and a rotor diameter of around 54 metres.

1 000 Dollars per Kilowatt Average

The average price for large, modern wind farms is around 1 000 USD per kilowatt electrical power installed. (Note, that we are not talking about annual energy production, yet. We'll return to that in a couple of pages. Energy production is measured in kilowatt hours. If this sounds confusing, take a look at the Reference Manual of this web site).

For single turbines or small clusters of turbines the costs will usually be somewhat higher. On the next page we will discuss installation costs further.

| Installation Costs for Wind Turbines |  |
| --- | --- |
| Novar Wind Farm, Scotland, under construction in a moor, July 1997. Photograph by Steffen Damborg | Installation costs include foundations, normally made of reinforced concrete, road construction (necessary to move the turbine and the sections of the tower to the building site), a transformer (necessary to convert the low voltage (690 V) current from the turbine to 10-30 kV current for the local electrical grid, telephone connection for remote control and surveillance of the turbine, and cabling costs, i.e. the cable from the turbine to the local 10-30 kV power line. Installation Costs Vary Obviously, the costs of roads and foundations depend on soil conditions, i.e. how cheap and easy it is to build a road capable of carrying 30 tonne trucks. Another variable factor is the distance to the nearest ordinary road, the cost of getting a mobile crane to the site, and the distance to a power line capable of handling the maximum energy output from the turbine. A telephone connection and remote control is not a necessity, but is is often fairly cheap, and thus economic to include in a turbine installation. Transportation costs for the turbine may enter the calculation, if the site is very remote, though usually they will not exceed some 15 000 USD. Economies of Scale It is obviously cheaper to connect many turbines in the same location, rather than just one. On the other hand, there are limits to the amount of electrical energy the local electrical grid can handle (see the section on Wind Turbines in the Electrical Grid ). If the local grid is too weak to handle the output from the turbine, there may be need for grid reinforcement, i.e. extending the high voltage electrical grid. It varies from country to country who pays for grid reinforcement - the power company or the owner of the turbine. |

Operation and Maintenance Costs for Wind Turbines

Modern wind turbines are designed to work for some 120 000 hours of operation throughout their design lifetime of 20 years. That is far more than an automobile engine which will generally last for some 4 000 to 6 000 hours.

Operation and Maintenance Costs

Experience shows that maintenance cost are generally very low while the turbines are brand new, but they increase somewhat as the turbine ages.

Studies done on the 5000 Danish wind turbines installed in Denmark since 1975 show that newer generations of turbines have relatively lower repair and maintenance costs that the older generations. (The studies compare turbines which are the same age, but which belong to different generations).

Older Danish wind turbines (25-150 kW) have annual maintenance costs with an average of around 3 per cent of the original turbine investment. Newer turbines are on average substantially larger, which would tend to lower maintenance costs per kW installed power (you do not need to service a large, modern machine more often than a small one). For newer machines the estimates range around 1.5 to 2 per cent per year of the original turbine investment.

Most of maintenance cost is a fixed amount per year for the regular service of the turbines, but some people prefer to use a fixed amount per kWh of output in their calculations, usually around 0.01 USD/kWh. The reasoning behind this method is that tear and wear on the turbine generally increases with increasing production.

Economies of Scale

Other than the economies of scale which vary with the size of the turbine, mentioned above, there may be economies of scale in the operation of wind parks rather than individual turbines. These economies are related to the semi-annual maintenance visits, surveillance and administration, etc.

Turbine Reinvestment (Refurbishment, Major Overhauls)

Some wind turbine components are more subject to tear and wear than others. This is particularly true for rotor blades and gearboxes.

Wind turbine owners who see that their turbine is close the end of their technical design lifetime may find it advantageous to increase the lifetime of the turbine by doing a major overhaul of the turbine, e.g. by replacing the rotor blades.

The price of a new set of rotor blades, a gearbox, or a generator is usually in the order of magnitude of 15-20 per cent of the price of the turbine.

Project Lifetime, Design Lifetime

The components of Danish wind turbines are designed to last 20 years. It would, of course, be possible to design certain components to last much longer, but it would really be a waste, if other major components were to fail earlier.

The 20 year design lifetime is a useful economic compromise which is used to guide engineers who develop components for the turbines. Their calculations have to prove that their components have a very small probability of failure before 20 years have elapsed.

The actual lifetime of a wind turbine depends both on the quality of the turbine and the local climatic conditions, e.g. the amount of turbulence at the site, as explained in the page on turbine design and fatigue loads.

Offshore turbines may e.g. last longer, due to low turbulence at sea. This may in turn lower costs, as shown in the graph on the page on the Economics of Offshore Wind Turbines.

Income from Wind Turbines

Energy Output from a Wind Turbine

If you have read the page on Annual energy output from a wind turbine , this graph will already be familiar to you.

The graph shows how annual energy production in million kilowatt hours varies with the windiness of the site. With a mean wind speed of, say 6.75 metres per second at hub height you get about 1.5 million kilowatt hours of energy per year.

As you can see, annual energy output varies roughly with the cube of the wind speed at turbine hub height. Just how sensitive energy production is to wind speed varies with the probability distribution for the wind, as explained in the page on the Weibull distribution. In this graph we have three examples with different k-values (shape factors). We will be working with the red curve (k=2) in our example.

The Availability Factor

The figures for annual energy output assume that wind turbines are operational and ready to run all the time. In practice, however, wind turbines need servicing and inspection once every six months to ensure that they remain safe. In addition, component failures and accidents (such as lightning strikes) may disable wind turbines.

Very extensive statistics show that the best turbine manufacturers consistently achieve availability factors above 98 per cent, i.e. the machines are ready to run more than 98 per cent of the time. Total energy output is generally affected less than 2 per cent, since wind turbines are never serviced during high winds.

Such a high degree of reliability is remarkable, compared to other types of machinery, including other electricity generating technologies. The availability factor is therefore usually ignored when doing economic calculations, since other uncertainties (e.g. wind variability) are far larger.

Not all wind turbine manufacturers around the world have a good, long reliability record, however, so it is always a good idea to check the manufacturers' track record and servicing ability before you go out and buy a new wind turbine.

Wind Energy and Electrical Tariffs

This page is relevant for private investors in wind energy, but not for power companies, which of course know everything about their own tariff system.

Electrical Energy Tariffs

Electricity companies are generally more interested in buying electricity during the periods of peak load (maximum consumption) on the electrical grid, because this way they may save using the electricity from the less efficient generating units. According to a study on the social costs and benefits of wind energy by the Danish AKF institute (see the Links page), wind electricity may be some 30 to 40 per cent more valuable to the grid, than if it were produced completely randomly.

In some areas, power companies apply variable electricity tariffs depending on the time of day, when they buy electrical energy from private wind turbine owners.

Normally, wind turbine owners receive less than the normal consumer price of electricity, since that price usually includes payment for the power company's operation and maintenance of the electrical grid, plus its profits.

Environmental Credit

Many governments and power companies around the world wish to promote the use of renewable energy sources. Therefore, they offer a certain environmental premium to wind energy, e.g. in the form of refund of electricity taxes etc. on top of normal rates paid for electricity delivered to the grid.

Capacity Credit

To understand the concept of capacity credit, let us look at its opposite, power tariffs: Large electricity customers are usually charged both for the amount of energy (kWh) they use, and for the maximum amount of power (kW) they draw from the grid, i.e. customers who want to draw a lot of energy very quickly have to pay more. The reason they have to pay more is, that it obliges the power company to have a higher total generating capacity (more power plant) available.

Power companies have to consider adding generating capacity whenever they give new consumers access to the grid. But with a modest number of wind turbines in the grid, wind turbines are almost like "negative consumers", as explained in the section on Wind turbines in the electrical grid : They postpone the need to install other new generating capacity.

Many power companies therefore pay a certain amount per year to the wind turbine owner as a capacity credit. The exact level of the capacity credit varies. In some countries it is paid on the basis of a number of measurements of power output during the year. In other areas, some other formula is used. Finally, in a number of areas no capacity credit is given, as it is assumed to be part of the energy tariff. In any case, the capacity credit is usually a fairly modest amount per year.

Reactive Power Charges

Most wind turbines are equipped with so called asynchronous generators, also called induction generators, cf. the section on electrical parts of a wind turbine. These generators require current from the electrical grid to create a magnetic field inside the generator in order to work. As a result of this, the alternating current in the electrical grid near the turbine will be affected (phase-shifted). This may at certain times decrease (though in some cases increase) the efficiency of electricity transmission in the nearby grid, due to reactive power consumption.

In most places around the world, the power companies require that wind turbines be equipped with switchable electric capacitor banks which partly compensate for this phenomenon. (For technical reasons they do not want full compensation). If the turbine does not live up to the power company specifications, the owner may have to pay extra charges.

Normally, this is not a problem which concerns wind turbine owners, since the experienced manufacturers routinely will deliver according to local power company specifications.

Basic Economics of Investment

Social Return from Investment in Wind Energy

On the next two pages, we look at the economics of an investment in wind energy from the point of view of society as a whole, as economists typically do. If you do not like economics, or if you know everything about it in advance, skip this page.

We do not account for environmental benefits, we shall do that later. We do not look at financing or taxation. These items vary enormously from one country to the other, but they do not make any nation richer or poorer: They only serve to redistribute income.

What society gets in return for investment in wind energy is pollution-free electricity; let us find out how much that costs.

Private Investors' Guide

If you are a private investor in wind energy you can still use our calculations - pre tax, that is: Generally speaking, investments which have a high rate of return before tax will have an even higher rate of return after taxes.

This is a surprise to most people.

The reason is, however, that depreciation regulations for all sorts of business tend to be very favourable in most countries. With rapid tax depreciation you get a higher return on your investment, because you are allowed to deduct the loss of value of your asset faster than it actually loses it value. This is nothing special for wind turbines. It is true for all sorts of business investment.

Once again, do note, that our calculations in real terms omit financing and taxes. As a prudent investor, you would probably want to plan your cash flow to make sure you can pay your debts. This you obviously have to calculate in money terms, i.e. in nominal terms.

Working with Investments

With any investment, you pay something now to get something else later. We assume that a dollar in your pocket today is more valuable to you than a dollar tomorrow. The reason why we say that, is that you could invest that dollar somewhere or put it into a bank account and earn interest on it.

To tell the difference between today's and tomorrow's dollars, we therefore use the interest rate. If we do that, 1 dollar a year from now is worth 1/(1+r) to you today. r is the interest rate, for example 5 per cent per year.

Thus 1 dollar a year from now is worth 1/1.05 = 0.9523 dollars today. 1 dollar 2 years from now is worth 1/(1.05*1.05) = 0.9070 and so forth...

But what about inflation? To deal with that we shall simply only work with dollars which have the same purchasing power as a dollar does today. Economists call that working with real values, instead of nominal ones.

Work in Real Values, not Nominal Values

An investment in a wind turbine gives you a real return, i.e. electricity, and not just a financial (cash) return. This is important, because if you expect some general inflation of prices during the next 20 years, you may expect electricity prices to follow the same trend.

Likewise, we would expect operation and maintenance costs to follow roughly the same price trend as electricity. If we expect all prices to move in parallel (with the same growth rates) over the next 20 years, then we can do our calculations quite simply: We do not need to adjust or calculations for inflation, we simply do all of our calculations in the price level of our base year, i.e. the year of our investment.

In other words, when we work with real values, we work with money which represent a fixed amount of purchasing power.

Use the Real Rate of Interest, not the Nominal Rate

Since we are studying the real rate of return (profitability) of wind energy, we have to use the real rate of interest, i.e. the interest rate minus the expected rate of inflation. (If both rates are high, say, above 10 per cent, you cannot really subtract the percentages, you should divide like this (1+r)/(1+i) but let's not make this into a course in economics).

Typical real rates of interest for calculation purposes these days are in the vicinity of 5 per cent per annum or so. You may say that in countries like Western Europe you could even go down to 3 per cent. Some people have a very high demand for profitability, so they might wish to use a higher real rate of interest, say, 7 per cent. Using the bank rate of interest is nonsense, unless you then do nominal calculations, i.e. add price changes everywhere, including to the price of electricity.

Wind Energy Economics

There is no Such Thing as a Single Price for Wind Energy

As we learned from the page on energy output , annual electricity production will vary enormously depending on the amount of wind on your turbine site. Therefore, there is not a single price for wind energy, but a range of prices, depending on wind speeds.

The graph to the right shows how the cost of electricity produced by a typical Danish 600 kW wind turbine varies with annual production. (We used the example built into the Wind Energy Economics Calculator to find the points for the graph).

The relationship is really very simple: If you produce twice as much energy per year, you pay half the cost per kilowatt hour. (If you believe that maintenance costs increase with turbine use, the graph might not be exactly true, but close to true).

If we use the graph above, plus the example from the page on income from wind turbines we find the relationship between wind speeds and costs per kWh below.

Remember, that everything on this page is based on our examples, so you cannot use the graph to predict costs for any particular project.

As an example, if your real rate of interest is 6 per cent per annum, rather than 5, costs are approximately 7.5 per cent higher than shown in the graph. When you use the Wind Energy Economics Calculator in a moment, you can use your own data to compute the cost of electricity.

The example is for a 600 kW wind turbine with project lifetime of 20 years; investment = 585,000 USD including installation; operation & maintenance cost = 6750 USD/year; 5% p.a. real rate of interest; annual turbine energy output taken from power density calculator using a Rayleigh wind distribution (shape factor = 2).

You should note that wind speeds at 50 metre hub height will be some 28 to 35 per cent higher* than at 10 metre height, which is usually used for meteorological observations, cf. the wind speed calculator page. Look at the grey axis at the bottom of the graph to see how wind speeds at 10 metre height may translate into higher wind speeds. A wind speed of e.g. 6.25 m/s at 10 metre height in roughness class 1 will translate into 8 m/s at 50 metre hub height.

Pitfalls in Wind Energy Cost Analysis

Many studies of the cost of wind energy and other renewables are poor science, because of a lack of understanding of both the technology and the economics involved. Frequently people who understand the economics do not understand the technology and vice versa - and sometimes neither!

This page warns you against the most common pitfalls. Even trained economists have fallen into these pits, and misleading comparisons of costs of different energy technologies are unfortunately not uncommon.

What are the Costs of Wind Power Generation?

- economic depreciation of your investment

- interest on the capital invested

- operation and maintenance costs

If you think that the amount required to buy a wind turbine is a cost or an expenditure, you are wrong, and you do not understand basic bookkeeping or economics. In that case, stay away from cost analysis! Profit is not a cost either. If you think it is, keep away from the rest of this page, and take a course in economics first.

Depreciation

Economic depreciation is a bit trickier. You simply cannot calculate the economic depreciation of your investment unless you know the income from your investment. That comes as a surprise to many people, including some economists. But depreciation is simply defined as the decline in the capital value of your investment using the internal rate of return as the discounting factor. If you do not know the income from the investment, you do not know the rate of return, thus you cannot calculate economic depreciation.

The source of the misunderstanding is that people mix up tax depreciation or accounting depreciation with economic depreciation. But tax or accounting depreciation is simply a set of mechanical rules which you do not use when you want to find the true cost of energy per kWh.

Prices and Costs are Two Very Different Concepts

Many non-economists use the words cost and price as synonyms. They are not. The price of a product is determined by supply and demand for the product. Many people naively assume that the price of a product is somehow a result of adding a normal or reasonable profit to a cost. That is definitely not the case, unless you are running a Government controlled monopoly.

Prices of Wind Turbines Cannot be Calculated by Dividing Turnover by Volume

Some people take the turnover figures from manufacturers and divide them by sales (in MW) in order to obtain a price per MW installed. But the results are complete nonsense. Some of the reasons why this cannot be done are:

- Some of the manufacturers deliveries are complete turnkey projects including planning, turbine nacelles, rotor blades, towers, foundations, transformers, switchgear and other installation costs including road building and power lines. Other deliveries are nacelles only, or all variations in between. Manufacturers' sales figures also include service and sales of spare parts.

- Manufacturers' sales include licensing income, but the corresponding MW are not registered in company accounts.

- Sales may vary significantly between markets for e.g. high wind turbines and low wind turbines. The prices of different types of turbines are very different.

- The patterns of sales, types of turbines, and types of contracts vary significantly and unsystematically from year to year.

Prices should be obtained from price lists. It is useless to make some simple average of prices from such a list, however, since some turbine models are not sold whereas others are sold in huge volume. It does not make sense to take an average of prices for turbines of, say 1,000 kW, even if they have the same tower height. It makes much more sense to look at the price per square metre rotor area, as explained in a subsequent section.

Productivity and Costs Depend on the Price of Electricity and Not Vice Versa

If you look at annual production per square metre rotor area in Denmark, it tends to be significantly higher than in, say, Germany. That, strictly speaking, has nothing to do with different wind resources. It is caused by different prices for electricity. In Denmark it is simply not profitable to locate wind turbines in low wind areas, whereas it is profitable to use low wind areas in Germany due to high electricity prices.

Germany has a very high electricity price for renewables (electricity tariff per kWh of energy delivered to the grid). You will therefore find that in Germany it is profitable to equip wind turbines with very tall towers. The high electricity price also makes it profitable to locate wind turbines in low wind areas. In that case, the the most economic turbines will have larger rotor diameters relative to the generator size than in other areas of the world.

Wind turbines sold on the German market may therefore look more expensive than they do for other markets, if you look at the price per kW installed (rated) power. But that is really a very deceptive statistic, because what you really see, are machines which are optimised for the German low wind sites. The price per square metre rotor area located at a given hub height is what matters, not the price per kW installed power. This is explained in detail in one of the next section below.

Installation Cost Variation

You get a similarly deceptive picture when you look at installation costs. The curious thing is that you do not necessarily have a high cost of generating electricity because of high installation cost. Quite the contrary: You tend to incur high installation costs whenever you have a good wind resource (and thus cheap generating costs) in a remote area,.

In Wales installation costs tend to be very high - several hundred per cent higher than in Denmark - despite a very low electricity price. This is simply because there is a lot of wind if you place the wind turbines on top of the nicely rounded Welch hills (see the hill effect ). It is really profitable to build an expensive road through the moors, and build expensive foundations in order to use the high-wind areas. In other words: You can afford high installation costs, precisely when you have a good wind resource.

In some cases installation costs include costs for extension of the electrical grid and/or grid reinforcement. Since the costs of cabling can be quite significant, it matters a lot whether a wind farm is located next to an existing medium voltage power line (9-30 kV), or far from a power line.

Consequently it makes no sense to use average installation costs, if we are not talking about areas with roughly the same wind climate, the same electricity price per kWh delivered to the grid, and the same distance to the grid.

Wind Energy is a Resource Extracting Technology

Many people ask: "What is the average cost of wind energy?". That Question is just as meaningless as "What is the average cost of crude oil?"

In Kuwait the cost may be 1 USD per barrel, in the North Sea, it may be 15 USD per barrel. The reason why the costs are so different is that it is far more complex to extract the oil from the North Sea than in Kuwait. It does not make any sense to average out the cost of oil production in Kuwait and in the North Sea in order to find some average cost. The average will certainly not be a guide to the price of crude oil! Even if the market price of oil drops below 15 USD per barrel, it may still pay to produce oil from the North Sea, what matters in that case is not the average cost per barrel of oil but the marginal variable cost of extracting another barrel of oil.

Using Statistics from one Area is not a Reliable Guide to Costs in Another Area

The cost of wind energy in Germany is high, because prices for electricity are high. The cost of wind energy in the UK is low, because the price of electricity is low. And of course you get very few wind turbines installed if you have low prices for electricity, because high wind sites are scarce, and you may not be able to find sites which are profitable.

The price per kW Rated Power is a Very Poor Guide to Investment in Wind Power - the Price per Square Metre Rotor Area matters

Many researchers who are interested in the decline in costs of wind power wish to study the decline in the price of wind turbines. They therefore ask for an apparently simple statistic: The price of a wind turbine per kW installed power. That figure is usually difficult to get hold of, and a very poor guide to cost developments for several reasons.

It is very difficult to give a single figure for price per kW installed power, because the price of a turbine varies much more with its rotor diameter than with the rated power of its generator. The reason is that annual production depends much more on the rotor diameter than the generator size. Studies which compare the average price per kW installed power for different technologies are usually misleading, if they include wind power.

Systematic kW Nonsense - an Example

As an example of why it is misleading to use the price per kW rated power for a wind turbine, compare the annual energy production from two machines from the same manufacturer, both mounted on a 50 m tower. (The first one is a high wind machine, the second one a universal machine). You can use the Wind Turbine Power Calculator to verify the results:

- Vestas V39, a 600 kW turbine with a 39 m rotor diameter

- Vestas V47, a 660 kW turbine with a 47 m rotor diameter

The result is that annual energy production from the second machine is 45.2% higher than the first machine, despite the fact that the generator is only 10% larger. If you compare the two rotor areas, however, you may observe that the rotor area of the second machine is exactly 45.2% larger than the first machine.

So, if we assume that the price for the second machine is 33% higher than for the first machine you would get very different results, if you compare

- The price per kW rated power has increased 21%

- The price per sq m rotor area has decreased 8.4%

- The price per kWh energy has decreased 8.4%

New wind turbines are increasingly being built with pitch control rather than stall control. This means that the generator size can be varied more freely in relation to the rotor size. In general, there is a tendency to use larger rotor areas for a given generator size. That means that you will get a completely wrong (overestimated) price development when you compare the price per kW installed power for old turbines with new turbines. The relevant price measure is the price per square metre swept rotor area, not the price per kW installed (rated) power.

Mistakes with Capacity Factors

Analysts are frequently interested in the capacity factor for wind power. The capacity factor for a generating technology is equal to the annual energy production divided by the theoretical maximum energy production if the generator were running at its rated power all the year.

Depending on the wind statistics for a particular site, the ideal capacity factor for a wind turbine is somewhere around 25-30%, because that capacity factor minimises cost per kWh. It is definitely not desirable to increase the capacity factor for a wind turbine, as it would be for technologies where the fuel is not free! This apparent capacity factor paradox is explained more in detail on the page on Annual Energy Output from a Wind Turbine.

Capacity factors will be very different for different machines cf. the example above, but likewise the prices (or costs) of those machines will be very different. In the final analysis, what counts is the cost per kWh of energy produced, not the capacity factor.

Land Rents Depend on the Profitability of a Project and not Vice Versa

It is a very common mistake to treat compensation to land owners where the turbines are placed as a cost of wind energy. Actually, it is only a minor share of the compensation which is a cost, namely the loss of crop on the area that can no longer be farmed, plus a possible nuisance compensation in case the farmer has to make extra turns when plowing the fields underneath the wind turbines.

If the compensation exceed what you would normally pay to install a power line pylon, the excess is really an income transfer, which is quite a different matter to economists. It is not a cost to society as such, but it is a transfer of income (profits) from the wind turbine owner to the land owner. Such a profit transfer called a land rent by economists. A rent payment does not transfer real resources from one use to another.

Some people ask what the normal compensation for placing a wind turbine on agricultural land is. The answer is, that there is no "normal" compensation. The compensation depends on the quality of the site. If there is a lot of wind, and there is cheap grid access nearby, the land owner can bargain for a high compensation, because the turbine owner can afford it due to the profitability of the site. If there is little wind, and/or high installation costs, the compensation will just be the nuisance value of the turbine.

Guide to the Wind Energy Economics Calculator

This page is a guide to the Wind Energy Economics Calculator on the next page. It may may sense for you to look at the calculator first, and then click on the question marks to jump back here and get the full explanation of how it works.

Built-in Examples

To kick start you to get working right away we have included some data examples for wind turbines, which you may select from the pop up menu. The offshore example is taken from the Danish power companies' report on offshore wind turbines.

Project Lifetime

Danish wind turbines have a design lifetime of 20 years. With offshore wind conditions (low turbulence) it is likely that the the turbines will last longer, probably 25 to 30 years.

Since offshore foundations are designed to last 50 years, it may be interesting to calculate two generations of turbines on the same set of foundations, possibly with a repair overhaul after 25 years.

Read more on the page on Operation and Maintenance.

Wind Turbine Price

Prices may vary due to transportation costs, different tower heights, different rotor diameters etc. You can use the example prices, or you can type a price of your own directly in the box to the right.

Read more on the page What does a Wind Turbine Cost?

Installation Cost

Costs may vary with the location, particularly with costs for road construction and grid connection. 30% of turbine cost is a fair average for Denmark.

Read more on the page on Installation Costs.

Income from Electricity Sales

This optional data item is interesting for individuals who want to invest in a wind turbine. You may also include capacity credit, if any. Specify the number of kilowatt hours you found using the Power Density Calculator , and the tariff (payment) per kilowatt hour. You may also enter an amount directly in the box to the far right in the form.The data is not needed to compute the cost of electricity.

Read more on the page on Income from Wind Turbines , and the page on Electricity Tariffs.

Operation and Maintenance

You may include either a fixed amount per year (by typing directly into the box to the right) or a percentage of the cost of the turbine. Costs could include a service contract with the manufacturer. You may specify a fixed cost per kilowatt hour instead, if you wish.

Read more on the page on Operation and Maintenance.

Net Present Value

Here you specify the real rate of interest to tell the programme how to evaluate future income and expenditure.

The Net Present Value of your project is the value of all payments, discounted back to the beginning of the investment. If the figure is positive, your project has a real rate of return which is larger than your real rate of interest. If the value is negative, the project has a lower rate of return.

To compute the real rate of return, the programme takes the first payment listed at the bottom of the calculator (number 01) and divides it by (1+the real rate of interest). It then divides the next payment (number 02) by (1+the real rate of interest) to the second power, and so forth, and adds it all up together with the initial investment (number 00).

Real Rate of Return

The real rate of return tells you the real rate of interest which makes the net present value of your project exactly zero. In other words, the real rate of return tells you how much real interest you earn on your investment. (The programme does not use your real rate of interest for anything, it computes one for you).

Computing that rate is a bit tricky, since it requires that the programme makes a guess to find the answer that makes the net present value zero. If it guesses to high, the net present value becomes negative. If it guesses too low, it becomes positive. But the programme uses a very clever, blazingly fast technique called Newton-Rapson iteration which means that the guesses improve dramatically each time. After 5 guesses it has found your answer with 5 digit precision.

Electricity Cost per kWh

The cost is calculated by finding the sum of the total investment and the discounted value of operation and maintenance costs in all years. We then divide the result by the sum of the discounted value of all future electricity production, i.e. we divide each year's electricity production by (1+i) to the n th power, where n is the period number (01 to 50. If you have specified an income from electricity sales, that amount is not used, or more accurately, it is subtracted from all non-zero amounts specified in the list of payments in period 01 to 50.

Payments

The payments in these boxes are the results of your specifications above, and they are used to calculate the net present value and the real rate of return. The boxes are also used to calculate the cost of electricity after subtracting any income from electricity sales from all non zero boxes in period 01 to 50.

Economics of Offshore Wind Energy

New Danish Reports on Offshore Wind Energy

In 1997 the Danish electrical power companies and the Danish Energy agency finalised plans for large scale investment in offshore wind energy in Danish waters.

The plans imply that some 4 100 MW of wind power are to be installed offshore before the year 2030. Wind would by then cover some 50 per cent of Danish electricity consumption (out of a total of 31 TWh/year).

Improving Economics of Offshore Wind Energy

On the previous page, the calculator already includes an example showing the expected average cost of offshore wind energy in Denmark, using presently available technology.

The most important reason why offshore wind energy is becoming economic is that the cost of foundations has decreased dramatically. The estimated total investment required to install 1 MW of wind power offshore in Denmark is around 12 million DKK today, (equivalent to 4 million DEM, or 1.7 million USD). This includes grid connection etc.

Since there is substantially more wind at sea than on land, however, we arrive at an average cost of electricity of some 0.36 DKK/kWh = 0.05 USD/kWh = 0.09 DEM/kWh. (5% real discount rate, 20 year project lifetime, 0.08 DKK/kWh = 0.01 USD/kWh = 0.02 DEM in operation and maintenance cost).

Accounting for Longer Project Lifetime

It would appear, however that turbines at sea would have a longer technical lifetime, due to lower turbulence.

If we assume a project lifetime of, say, 25 years instead of 20, this makes costs 9 per cent lower, at some 0.325 DKK/kWh.

The cost sensitivity to project lifetime is plotted in the accompanying graph, which was made using the calculator on the previous page.

Danish power companies, however, seem to be optimising the projects with a view to a project lifetime of 50 years. This can be seen from the fact that they plan to require 50 year design lifetime for both foundations, towers, nacelle shells, and main shafts in the turbines.

If we assume that the turbines have a lifetime of 50 years, and add an overhaul (refurbishment) after 25 years, costing some 25 per cent of the initial investment (this figure is purely a numerical example), we get a cost of electricity of 0.283 DKK/kWh, which is similar to average onshore locations in Denmark.

Employment in the Wind Industry

30,000 Jobs Worldwide in 1995

The wind industry in 1995 employed some 30,000 people worldwide. This estimate is based on a study from the Danish Wind Industry Association, which was published in 1995.

The study accounts for both direct and indirect employment. By indirect employment we mean the people who are employed manufacturing components for wind turbines, and the people who are involved in installing wind turbines worldwide.

9,000 Jobs in Denmark

The Danish wind industry had some 8 500 people employed in 1995. It may be interesting to see how they are divided between different components:

| Component | Employment |
| --- | --- |
| Turbine assembly | 3 600 |
| Rotor blades | 2 000 |
| Electronic controllers | 700 |
| Brakes, hydraulics etc. | 200 |
| Towers | 1 500 |
| Installation of turbines | 300 |
| Other | 300 |
| Total | 8 300 |

In reality wind turbine production creates about 50 per cent more jobs, since Danish manufacturers import many components, e.g. gearboxes, generators, hubs etc. from abroad. In addition, jobs are created through the installation of wind turbines in other countries.

How was the Study done?

You may think we went out and asked the wind turbine manufacturers to get the figures. Well, we did, but only to check our calculations. The point is, that you are likely to end up with the wrong answer, if you rely on asking people about something as complex as job creation throughout the economy. You may see remarkably large errors in other estimates made as naive back-of-an-envelope calculations elsewhere on the Web. (Out of courtesy, we won't include the link, here).

Actually, we started very differently using a so called input-output model, which is what most economists in government service, or central statistical bureaus would do. In an input output model we actually follow the flow of deliveries from each sector of the economy to other sectors of the economy. In our case, we have a 117 by 177 table of deliveries between the sectors, and the statisticians have double checked that they sum up to the total production in the economy. With this table on may follow sub-sub-contracting all the way back in the economy in an infinite number of links. (Using a mathematical technique called matrix inversion):

How to get Hold of the Study

You may download the employment study (44K) in Adobe Acrobat pdf-format for printing in the original layout on your own printer. To download to a PC: Click the Download button using the right mouse button. To download to a Macintosh, hold the mouse button down, and select Save this Link as...

History of Wind Turbines

Please respect that we have exclusive copyright on all of this web site. You may quote us, giving proper attribution to the Danish Wind Industry Association web site www.windpower.org, but it is illegal to use any picture, plot, graphics or programming on any other web site or in any commercial or non commercial medium, printed, electronic or otherwise.

- A Wind Energy Pioneer: Charles F. Brush

- The Wind Energy Pioneer: Poul la Cour

- The Wind Energy Pioneers - 1940-1950

- The Wind Energy Pioneers - The Gedser Wind Turbine

- Wind Turbines From the 1980s

- The California Wind Rush

- Modern Wind Turbines

- Offshore Wind Turbines

- Megawatt-Sized Wind Turbines

- Multi-Megawatt Wind Turbines

| Modern Wind Turbines |  |  |  |
| --- | --- | --- | --- |
|  |  | Denmark has (in 2003) around 3,000 MW wind power, which is supplied by approximately 5,500 wind turbines. Individuals and cooperatives own around 80% of the capacity. Avedøre Holme, Denmark |  |
|  |  | The picture shows the Avedøre Wind Farm, just 5 kilometres from the city centre of Copenhagen, Denmark. The 12 Bonus 300 kW wind turbines, (and one 1,000 kW power company test wind turbine) are located next to a 250 MW coal-fired power plant. (Photograph Søren Krohn, © 1997 DWIA) (48K, JPEG) Large Onshore Wind Farms |  |
|  |  | Rejsby Hede wind farm consist of 40 turbines from Bonus Energy each of 600 kW. The turbines were erected in 1995 near Tønder in southern Jutland. Totalling 24 MW it was the largest wind farm in Denmark at the time. Today the largest onshore wind farm in Denmark is Syltholm on the southern island Lolland. The farm consist of 35 NEG Micon 750 kW turbines i.e. a total capacity of 26,25 MW. |  |
| Offshore Wind Turbines |  |  |  |
|  |  |  | Offshore wind energy is a promising application of wind power, particularly in countries with high population density, and difficulties in finding suitable sites on land. Construction costs are higher at sea, but energy production is also higher. The largest offshore wind farms in Denmark are Horns Rev by the west coast of Jutland and Nysted close to Lolland – 160 and 158 MW respectively. A tendering procedure for new offshore wind farms will be commenced in late 2003. The Danish energy plan, Energi21, from 1996 set up a target for 4,000 MW offshore wind power in 2030. These 4,000 MW are expected to produce 13.5 TWh per year equivalent to 40% of the Danish electricity consumption. You can read more about offshore wind turbines in Research in Offshore Nysted Offshore Wind Farm |
| Nysted offshore wind farm is located 10 km south of Lolland on Rødsand Bank |  |  | The most recent large offshore farm is Nysted Offshore Wind Farm at Rødsand built in 2003. The wind farm is located app. 10 km south of the town of Nysted on Lolland and consists of 8 rows with 9 turbines each. The total power of the 72 wind turbines each of 2.3 MW thus reaches 165,5 MW. The annual electricity production of the wind farm is 600GWh, enough to supply 145,000 (Danish) households. The wind turbine towers are about 70 m tall, and the rotor blades 41 m long. The picture is from the initial phase of the installation. Learn more at www.nystedwindfarm.com (Photograph Jakob Holst, © 2003 DWIA) Horns Rev – Denmark's Largest Wind Farm |
| Horns Rev is situated of the coast of, Blåvandhuk, the most western point of Denmark. |  |  | The largest wind farm in Denmark is the offshore wind farm of Horns Rev, which was completed in 2002. It is situated in the North Sea, 14-20 km off the coast of Jutland. With its 80 Vestas 2MW turbines, the wind farm has a total capacity of 160 MW. That makes it the largest offshore wind farm in the world today (2003). The farm supplies the equivalent of 150,000 (Danish) households. The larger production compared to Nysted is due to better wind conditions. Learn more at www.hornsrev.dk (Photograph Claus Bøjle Møller, © 2003 DWIA) Samsø |
| Samsø offshore wind farm is located 3,5 km south of the island Samsø. |  |  | The energy consumption of the small Danish island Samsø is more than 100% matched by local production of renewable energy. A major reason for this is remarkable fact is a locally owned offshore wind farm consisting of 10 Bonus 2,3 MW turbines. See online production data at www.samsohavvind.dk/windfarm (Photograph Søren Krohn, © 2003 DWIA) Middelgrunden |
| The urban offshore wind farm Middelgrunden is located close to the port of Copenhagen. |  |  | Middelgrunden is located 2 km off shore east of Copenhagen. It consists of 20 Bonus 2 MW wind turbines arranged to form an arch. With a total power of 40 MW the wind farm can generate 90 TWh a year. That is equivalent to the annual electricity consumption of 20,000 (Danish) households or three per cent of the total electricity consumption of Copenhagen. Learn more at www.middelgrunden.dk (Photograph Søren Krohn, © 2000 DWIA) Tunø Knob |
| The world's second offshore wind farm is located between the Jutland peninsula and the small island of Tunø in Denmark |  |  | The Tunø Knob offshore wind farm (36K, JPEG) in the Kattegat Sea off the Coast of Denmark was built in 1995 by the utility company Midtkraft. The picture shows the construction work with a floating crane. The Wind farm consists of 10 Vestas 500 kW pitch controlled wind turbines. The turbines were modified for the marine environment, each turbine being equipped with an electrical crane to be able to replace major parts such as generators without the need for a floating crane. In addition, the gearboxes were modified to allow a 10 per cent higher rotational speed than on the onshore version of the turbine. This will give an additional electricity production of some 5 per cent. This modification could be carried out because noise emissions are not a concern with a wind park located 3 kilometres offshore from the island of Tunø, and 6 kilometres off the coast of the mainland Jutland peninsula. The park has been performing extremely well, and production results have been substantially higher than expected, cf. the page on offshore wind conditions. (Photograph copyright Vestas Wind Systems A/S) Vindeby |
| The world's first offshore wind farm is located North of the island of Lolland in the Southern part of Denmark |  |  | The Vindeby wind farm (32K, JPEG) in the Baltic Sea off the coast of Denmark was built in 1991 by the utility company SEAS. The wind farm consists of 11 Bonus 450 kW stall controlled wind turbines, and is located between 1.5 and 3 kilometres North of the coast of the island of Lolland near the village of Vindeby. The turbines were modified to allow room for high voltage transformers inside the turbine towers, and entrance doors are located at a higher level than normally. These same modifications were carried over to the subsequent Tunø Knob project. Two anemometer masts were placed at the site to study wind conditions, and turbulence, in particular. A number of interesting results on offshore wind conditions have been obtained through these studies which were carried out by Risø National Laboratory. The park has been performing flawlessly. Electricity production is about 20 per cent higher than on comparable land sites, although production is somewhat diminished by the wind shade from the island of Lolland to the South. |
| Multi-Megawatt Wind Turbines |  |  |  |
|  | NEG Micon 2 MW The prototype of the NEG Micon 2 MW turbine (1024 x 768 pixels, 132K JPEG) was commissioned in August 1999. It has a 72 m (236 ft.) rotor diameter. In this case (Hagesholm, Denmark) it is mounted on a 68 m tower. In the background you see the foundations for two sister machines.The turbine is intended for offshore applications. From the outside it resembles the 1500 kW NEG Micon machine so much, that you'd have to see the turbine in its stopped state (with the blades pitched out of the wind) in order to notice the difference: The rotor blades are pitchable, since the machine has active stall power control , whereas its 1500 kW cousin has passive stall power control. (Aerial photograph Soren Krohn © 1999 DWIA) |  |  |
|  | Bonus 2 MW The prototype of the Bonus 2 MW turbine (88 K) was commissioned in the fall of 1998. It has a 72 m (236 ft.) rotor diameter. In this case (Wilhelmshaven, Germany) it is mounted on a 60 m tower. The turbine is intended for offshore applications, and has Combi Stall® power control (Bonus trademark for active stall power control ). The machine resembles the Bonus 1 MW and 1.3 MW machines considerably. (Aerial photograph Soren Krohn © 1999 DWIA) |  |  |
|  | Nordex 2,5 MW The prototype of the Nordex 2,5 MW turbine (132 K) was commissioned in the spring of 2000. The rotor diameter of the wind turbine is 80 m. The image shows the prototype at Grevenbroich, Germany, which has a 80 m tower. The turbine has pitch power control. (Photo |  |  |