The applicable primary standards are:

AS 4509 Stand-alone Power Systems
part 1 Safety requirements 
part 2 Design guidelines 
part 3 Installation and maintenance

AS 4086 Secondary batteries for SPS
part 2 Installation and maintenance

AS 5033 Installation of photovoltaic (PV) arrays

If you're a BCSE Accreditated Installer you MUST have and use a copy of these standards. They can be purchased from www.standards.com.au 

Other relevant standards include:

AS 3000 Electrical Wiring Rules

AS 1768 Lightning Protection

ASNZS 1170.2 Wind Loads

AS 4777 Grid Connections of Energy Systems via Inverters

Maintenance of Systems

It is the responsibility of all accredited installers to advise the owner of a system if does not comply with the current Standards. This could be as a result of weathering, damage or an upgrading of the Standards themselves.

It should be noted that with the advent of AS5033 there has been a strengthening of the Standards applicable to PV array installations. Similarly AS4777 for the Standards on Grid-connection of energy systems via inverters. AS4509 is also currently being revised so stay tuned for news on the changes to it. 

Manufacturing and Importing Standards

Since the advent of the new rebate schemes, the Clean Energy Council has received many enquiries from importers wishing to bring components and entire PV systems into the Country.

There are several requirements to be met:

1. The Australian Standard for Photovoltaic Installations (AS5033)

Currently this Standard states that PV modules shall comply with IEC61215 or IEC61646 and should be should be Class II. Class II is defined in clause 1.4.8. The normative reference to IEC 61829 is for information only. 

3. The Australian Standard for Grid-connected Inverters (AS4777)

Grid-connected inverters must comply with AS4777, AS3100 or IEC equivalent and have a current Certificate of Suitability from a state regulator.

4. The Trade Practices Act

We suggest you familiarise yourself with the content of the Trade Practices Act 1974 with regards to importing. Our interpretation includes the need for the importer to provide a warranty as though they were the manufacturer, ensure the goods are of marketable quality and that they are used for the purpose for which they were intended.

Trade Practices Act 

5. The Australian Greenhouse Office requirements for eligibility for rebates.

Each rebate program has its own web pages outlining its requirements in terms of Standards and compliance.

Renewable Remote Power Generation Program (RRPGP) 

Photovoltaic Power Generation Program .

6. BCSE Accreditation

Rebates will only be paid when the systems are designed and installed by BCSE Accredited personnel. The installation reports must be physically signed off by the installer at the installation site after the system has been commissioned.

Grid Connect Inverters - Testing Standards
Standards Australia has released three standards which are pertinent to grid connected inverter systems. These are:

-AS 4777.1 - 2002 Grid connection of energy systems via inverters Part 1: Installation requirements.
-AS 4777.2 - 2002 Grid connection of energy systems via inverters Part 2: Inverter requirements.
-AS 4777.3 - 2002 Grid connection of energy systems via inverters Part 1: Grid protection requirements.

Inverters must be tested against AS 4777.2 - 2002 (or equivalent) by an appropriate testing laboratory.

Testing Laboratories
The list of inverters available below, also includes a list of approved testing laboratories. RESLab is able to test to AS4777.

Inverters
The Clean Energy Council does not approve inverters - it is the electricity utility which is responsible for approving inverters used in its jurisdiction. Approved inverters should have a Certificate of Suitability number and a date of testing. The testing date should be less than 5 years old.

The Clean Energy Council does not accept any liability to any person for the information or advice (or the use of such information or advice). The information provided here is done so on the basis that all persons using the information undertake responsibility for assessing the relevance and accuracy of its content. Please note carefully that Certificates of Suitability may no longer be current. Please don't hesitate to contact the CEC for more information on 03 9929 4115.

The current list of Grid Connect Inverters and approved testing laboratories has been now been updated. Companies wishing to be included should contact Mike at the CEC with accurate details of their approved products. - updated April 28th, 2008

click to download - Click to download file. There are two pages of inverters listed. You may need to zoom in on this pdf file in order to read it effectively.