# Energy efficiency

- From Wikipedia, the free encyclopedia

Jump to: navigation, search

This article is about energy efficiency as a ratio. For the term as used in energy economics, see Efficient energy use.

See also: Energy conversion efficiency

In physics and engineering, including mechanical and electrical engineering, energy efficiency is a dimensionless number, with a value between 0 and 1 or, when multiplied by 100, is given as a percentage. The energy efficiency of a process, denoted by eta, is defined as

where output is the amount of mechanical work (in watts) or energy released by the process (in joules), and input is the quantity of work or energy used as input to run the process.

Due to the principle of conservation of energy, energy efficiency within a closed system can never exceed 100%.

## Energy efficiency and global warming

Main article: Global warming

Making homes, vehicles, and businesses more energy efficient is seen as a largely untapped solution to addressing global warming, energy security, and fossil fuel depletion. Many of these ideas have been discussed for years, since the 1973 oil crisis brought energy issues to the forefront. In the late 1970s, physicist Amory Lovins popularized the notion of a "soft path" on energy, with a strong focus on energy efficiency. Among other things, Lovins popularized the notion of negawatts -- the idea of meeting energy needs by increasing efficiency instead of increasing energy production.

Energy efficiency has proved to be a cost-effective strategy for building economies without necessarily growing energy consumption, as environmental business strategist Joel Makower has noted. For example, the state of California began implementing energy-efficiency measures in the mid-1970s, including building code and appliance standards with strict efficiency requirements. During the following years, California's energy consumption has remained approximately flat on a per capita basis while national U.S. consumption doubled. As part of its strategy, California implemented a three-step plan for new energy resources that puts energy efficiency first, renewable electricity supplies second, and new fossil-fired power plants last.

Still, efficiency often has taken a secondary position to new power generation as a solution to global warming in creating national energy policy. Some companies also have been reluctant to engage in efficiency measures, despite the often favorable returns on investments that can result. Lovins' Rocky Mountain Institute points out that in industrial settings, "there are abundant opportunities to save 70% to 90% of the energy and cost for lighting, fan, and pump systems; 50% for electric motors; and 60% in areas such as heating, cooling, office equipment, and appliances." In general, up to 75% of the electricity used in the U.S. today could be saved with efficiency measures that cost less than the electricity itself.

Other studies have emphasized this. A report published in 2006 by the McKinsey Global Institute, asserted that "there are sufficient economically viable opportunities for energy-productivity improvements that could keep global energy-demand growth at less than 1 percent per annum" -- less than half of the 2.2 percent average growth anticipated through 2020 in a business-as-usual scenario. Energy productivity -- which measures the output and quality of goods and services per unit of energy input -- can come from either reducing the amount of energy required to produce something, or from increasing the quantity or quality of goods and services from the same amount of energy.

The Vienna Climate Change Talks 2007 Report, under the auspices of the United Nations Framework Convention on Climate Change (UNFCCC), clearly shows "that energy efficiency can achieve real emission reductions at low cost" [1]

## [edit] Energy efficiency and peak oil

Peak oil depletion scenarios graph which depicts cumulative published depletion studies by ASPO and other depletion analysts.

Main article: Peak oil

In addition to concerns over global warming, a potentially more immediate problem drew increasing attention during the mid-2000s: the possibly imminent peaking of world production of petroleum, natural gas, and (eventually) coal. Because petroleum is critical to vital sectors of industrial economies (especially transportation, industrial agriculture, and petrochemicals), and replacing petroleum in these applications takes time, an unexpected decline in the world production of petroleum could result in drastic increases in petroleum prices, leading to economic hardship which may range from moderate to catastrophic, depending on how rapidly oil production declines after the peak. The Hirsch report recommended several mitigation strategies, which will necessarily involve reducing the demand for petroleum through increasing the efficiency of use, as well as expanding the supply of alternatives.

## [edit] Energy efficiency in power supplies

Main article: Computer power supply

Computer power supplies are generally about 70–75% efficient;[2] to produce 75 W of DC output they require 100 W of AC input and dissipate the remaining 25 W in heat. Higher-efficiency power supplies can be over 80% efficient; higher energy efficiency uses less power directly, and requires less power to cool as well.

# Overview of Regulatory Requirements - Labelling and MEPS

It is currently mandatory for all of the following electrical products offered for sale in Australia to carry an approved energy label:

- refrigerators and freezers

- clothes washers

- clothes dryers

- dishwashers

- air conditioners (single phase mandatory, three phase voluntary)

The following products are also regulated on the basis of Minimum Energy Performance Standards (MEPS) - this means that they have regulated minimum energy efficiency levels:

- refrigerators and freezers (from 1 October 1999, revision 1 January 2005)

- mains pressure electric storage water heaters (from 1 October 1999)

- small mains pressure electric storage water heaters (<80L) and low pressure and heat exchanger types (from 1 October 2005) (Refer to   AGO letter regarding levels and test methods)

- three phase electric motors (0.73kW to <185kW) (from 1 October 2001, revision April 2006)

- single phase air conditioners (from 1 October 2004, revision 1 April 2006 and 2007 and 2008)

- three phase air conditioners up to 65kW cooling capacity (from 1 October 2001, revision 1 October 2007)

- ballasts for linear fluorescent lamps (from 1 March 2003). Note that in addition to MEPS, ballasts also have to be marked with an energy efficiency index (EEI).

- linear fluorescent lamps - from 550mm to 1500mm inclusive with a nominal lamp power >16W (from 1 October 2004)

- distribution transformers - 11kV and 22kV with a rating from 10kA to 2.5MVA (from 1 October 2004)

- commercial refrigeration (self contained and remote systems) (from 1 October 2004)

Details of the relevant standards are available on this web site.

The following products are proposed for regulation in the future, subject to normal regulatory processes and approval by the Ministerial Council on Energy.

- external power supplies (from 1 December 2008), New Zealand (April 2009);

- Set top boxes (from 1 December 2008), New Zealand (April 2009);

- Televisions;

- Standby;

- Boiling and chilled water dispensers;

- Vending machines;

- Commercial icemakers;

- A range of lamp types;

Find out how to make an application for an energy label and/or MEPS.

Regulations specify the general requirements for the energy labelling of appliances, including offences and penalties if a party does not comply with the requirements. Technical requirements for energy labelling are set out in "Part 2" of the relevant appliance standard, which is referenced in state regulations. State based legislation is necessary because the Australian constitution gives Australian States clear responsibility for resource management issues, including energy. More information on regulations and relevant contacts is available on this web site. (Note that regulatory standards are generally Part number 2 of the relevant standard, but this does vary by product).

A number of other products are being considered for energy labelling and MEPS programs (or will have existing levels revised).

Details are available on this web site as details become available - see what's new.

This web site does not cover gas appliances or safety but there are links to for further information.

# Requirements for Refrigerators and Freezers - Energy Labelling and MEPS

Refrigerators and freezers (and their combinations) are regulated for energy labelling in Australia. They are also regulated for MEPS (minimum efficiency standards). The requirements for energy labelling and MEPS are set out in AS/NZS 4474.2.

Product Definition: Refrigerators, freezers and refrigerator-freezers which intended for household or similar use and which:

- operate using the vapour compression cycle; and

- use mains electricity (230/240 Volts at 50 Hz) as the primary power source.

Test Standards: Note that all energy labelling standards for refrigerators and freezers are published jointly by Standards Australia and Standards New Zealand.

AS/NZS4474: Performance of household electrical appliances- Refrigerating appliances Part 1: Energy Consumption and Performance

AS/NZS4474: Performance of household electrical appliances- Refrigerating appliances Part 2: Energy labelling and minimum energy performance standard requirements

Part 1 of the standard defines the test procedures for the determination of energy consumption and performance of refrigerators, freezers and refrigerator-freezers in Australia.

Part 2 of the standard sets out the requirements for energy labelling of refrigerators, freezers and refrigerator-freezers in Australia. An approved Energy Label for refrigerators must be displayed on all products which are offered for sale in Australia.

These standards can be purchased from Standards Australia.

Find out how to make an application for energy labelling and MEPS.

## Overview of the Test Procedure for Refrigerators

Products are classified into one of nine Groups which are defined in the standard. A number of performance requirements must be met by refrigerators, freezers and refrigerator-freezers before being tested for energy consumption. These include:

- Volume - manufacturer declared values must be within defined tolerances of the measured compartment volumes.

- Pull down test - the unit is left off in an ambient temperature of 43°C with the doors open, the doors are then closed and the unit is switched on. The unit must reach certain internal temperatures within each compartment (as specified for its Group) after a period of 6 hours (including any compressor trips). This test is originally based on the US AHAM HRF-1 pull down test.

- Temperature Operation Test - the unit must be able to maintain acceptable internal temperatures in each compartment (as specified for its Group) under external ambient temperatures of 10°C, 32°C and 43°C. This test is identical to the ISO Temperature Operation Test.

Energy consumption is measured at specified internal compartment target temperatures (as specified for its Group) while operating at an ambient temperature of 32°C. During the energy consumption test, the freezer compartment does not contain test packages and any automatic defrost mechanism is allowed to operate. Energy consumption is measured over a whole number of defrost cycles and there are separate procedures for adaptive defrost systems (where time between defrosts exceeds 24 hours). There are no door openings in the test procedure. All tests are undertaken with a power supply at 240 Volts and 50 Hz.

Amendment 1 of AS/NZS 4474.1 (released 17 December 2001) introduces a number of changes to the test procedure as follows:

- Products that comply with the forthcoming MEPS 2005 requirements may be tested at 230V.

- Adaptive defrost definition has been revised to include products that have a defrost cycle of >24 hours for the energy consumption test and >36 hours under the least onerous setting at an ambient of 32°C. Adaptive defrost models are given an additional 5% allowance in the 2005 MEPS level.

- Any sequence of events that occurs before every activation of the defrost heater is considered to be the start of the defrost cycle.

- Group 5 has been broken into Group 5B (bottom freezer) and 5T (usually top freezer).

The Preface of AS/NZS 4474.2-2001 describes the recent changes in the refrigerator MEPS levels in Australia.

Note that energy consumption values measured to the Australian Standard will be different to values measured under other test procedures. The Foreword of AS/NZS 4474.1 outlines the major differences between the Australian Standard and US Department of Energy (AHAM) and International Standards Organisation test procedures for refrigerators.

# Requirements for Clothes Washers - Energy Labelling

Clothes washers are regulated for energy labelling in Australia.

Product Definition: Clothes washers which are intended for household or similar use.

Test Standards: Note that all energy labelling standards for clothes washers are published jointly by Standards Australia and Standards New Zealand.

AS/NZS2040: Performance of household electrical appliances- Clothes washing machines Part 1: Energy Consumption and Performance

AS/NZS2040: Performance of household electrical appliances- Clothes washing machines Part 2: Energy labelling requirements

Part 1 of the standard defines the test procedures for the determination of energy consumption and performance of clothes washers in Australia.

Part 2 of the standard sets out the requirements for energy labelling of clothes washers in Australia. An approved Energy Label for clothes washers must be displayed on all products which are offered for sale in Australia.

These standards can be purchased from Standards Australia.

Find out how to make an application for an energy label.

Details of the new mandatory rinse performance requirements from 2006 are now available.

## Overview of the Test Procedure for Clothes Washers

Products are classified into either drum type (generally front loading) or non drum type (all other types eg top loaders with impellers or agitators, twin tub machines). A number of performance requirements must be met by clothes washers during a test for energy consumption. These include:

- Wash performance - soil removal from soiled swatches (swatch type AS9), which are attached to a clothes load of rated capacity, must exceed 80% (there are also limits on the variability of the wash).

- Spin performance - the water extraction index (defined as ratio of the remaining water in the load after the final spin to the bone dry mass) must not exceed 1.1.

- Rinse performance - a maximum concentration of 2.25 mg PBIS per kg of load. This a new mandatory requirement from July 2006 - more details are on the rinse performance page.

- Severity of washing - the severity of washing index must not exceed 0.3 after a single run.

- Water consumption - shall not exceed 110% of the value stated by the manufacturer.

- Water pressure - machine shall be capable of operating at the maximum and minimum water pressure stated by the manufacturer.

Energy consumption is measured on the program recommended for a normally soiled cotton load at the rated capacity. The minimum wash temperature for energy labelling tests is 35°C.

Note that energy consumption measured to the Australian Standard will be different to values measured under other test procedures, as the bulk of the energy consumption for a clothes washer test is used to heat the water and energy consumption is therefore closely correlated with wash temperature. The Australian Standard allows external hot water connections - energy input in these cases is calculated as the heat energy of the hot water used. Cold water energy corrections are undertaken for those operations where either external hot water is drawn into the machine or where water is heated internally. The base temperature for the calculation of water energy is 20°C under AS/NZS 2040 (note that IEC60456 uses 15°C as the base temperature). Hot water is supplied at 60°C. Dynamic water pressure is 320 kPa. All tests are undertaken with a power supply at 240 Volts and 50 Hz.

The Preface of AS/NZS 2040 of Part 1 describes the recent changes in the clothes washer test procedure in Australia. It also describes the relationship between the Australian Standard and the International Electrotechnical Commission (IEC) standard for clothes washers.

The Preface of AS/NZS 2040 of Part 2 describes the current requirements for energy labelling for MEPS in Australia and New Zealand.

# Requirements for Clothes Dryers - Energy Labelling

Clothes dryers are regulated for energy labelling in Australia.

Product Definition: Electric rotary clothes dryers which are intended for household or similar use.

Test Standards: Note that all energy labelling standards for clothes dryers are published jointly by Standards Australia and Standards New Zealand.

AS/NZS2442: Performance of household electrical appliances- Rotary clothes dryers Part 1: Energy Consumption and Performance

AS/NZS2442: Performance of household electrical appliances- Rotary clothes dryers Part 2: Energy labelling requirements

Part 1 of the standard defines the test procedures for the determination of energy consumption and performance of clothes dryers in Australia (see summary below).

Part 2 of the standard sets out the requirements for energy labelling of clothes dryers in Australia. An approved Energy Label for clothes dryers must be displayed on all products which are offered for sale in Australia.

These standards can be purchased from Standards Australia.

Find out how to make an application for an energy label.

## Overview of the Test Procedure for Clothes Dryers

The standard is applicable to electric rotary clothes dryers which are intended for household or similar use. It does not apply to static dryers or gas powered dryers. A number of performance requirements must be met by clothes dryers during a test for energy consumption. These include:

- Drying clothes in a single operation - A timer dryer shall be capable of reaching the required final moisture content (6% of bone dry mass) for a load of rated capacity in a single timer setting. An autosensing dryer shall have at least one program which is capable of drying to below the required final moisture content (6% of bone dry mass) for a load of rated capacity in one program setting.

- Maximum fabric temperature - when tested to the standard, the drum or fabric temperature shall not exceed 130°C under the most severe operating condition.

- Energy efficiency - the dryer tested energy consumption (kilowatt hours per kg of moisture removed) shall not exceed 1.36.

Energy consumption values determined under AS/NZS 2442.1 will not be equivalent to energy determined under other test procedures due differences in initial moisture content, which is currently set at 90% of bone dry mass (approximately 84% of normalised mass for cotton under the test procedure IEC61121). Note that some of the current energy labelling registrations in Australia have been determined with an initial moisture content of 100%. Energy consumption is measured on the hottest program recommended for a cotton load of rated capacity. The test ambient air temperature is now 20°C ± 2°C and the humidity is 60% ± 5%. All tests are undertaken with a power supply at 240 Volts and 50 Hz.

The Preface of AS/NZS 2442 describes the recent changes in the clothes dryer test procedure in Australia. The Foreword of AS/NZS 2442 describes the relationship between the Australian, ANSI and the International Electrotechnical Commission (IEC) standards for clothes dryers.

# Requirements for Dishwashers - Energy Labelling

Dishwashers are regulated for energy labelling in Australia.

Product Definition: Electric dishwashers which are intended for household or similar use.

Test Standards: Note that all energy labelling standards for dishwashers are published jointly by Standards Australia and Standards New Zealand.

AS/NZS2007: Performance of household electrical appliances- Dishwashers Part 1: Energy Consumption and Performance

AS/NZS2007: Performance of household electrical appliances- Dishwashers Part 2: Energy labelling requirements Part 1 of the standard defines the test procedures for the determination of energy consumption and performance of dishwashers in Australia.

Part 2 of the standard sets out the requirements for energy labelling of dishwashers in Australia. An approved Energy Label for dishwashers must be displayed on all products which are offered for sale in Australia.

These standards can be purchased from Standards Australia.

Find out how to make an application for an energy label.

## Overview of the Test Procedure for Dishwashers

The standard is applicable to electric dishwashers which are intended for household or similar use. A number of performance requirements must be met by dishwashers during a test for energy consumption. These include:

- Washing index - the washing index of the test machine must exceed the specified value measured on the reference machine which is tested in parallel. The reference machine is a dishwasher which specially constructed and calibrated for this purpose (Miele G590).

- Drying index - the drying index of the test machine must exceed 50% (this is conducted as a separate test)

- Rated capacity - all specified load items shall be supported.

- Water consumption - shall not exceed 110% of the value stated by the manufacturer.

- Water pressure - machine shall be capable of operating at the maximum and minimum water pressure stated by the manufacturer.

Energy consumption is determined on the program recommended by the manufacturer for energy labelling that is capable of meeting the performance requirements specified above. From April 2004, all dishwashers are to be re-labelled using the "normal" program when tested to the 2003 edition of AS/NZS 2007 Part 1. The ambient air temperature is 20°C ± 2°C and the humidity is 60% ± 5%. All tests are undertaken with a power supply at 240 Volts and 50 Hz.

The Preface of AS/NZS 2007.2 describes the current requirements for energy labelling for dishwashers in Australia and New Zealand.

The Preface and Foreword of AS/NZS 2007.1 describes the recent changes in the dishwasher test procedure in Australia. It also describes the relationship between the Australian Standard and the International Electrotechnical Commission (IEC) standard for dishwashers (IEC60436-2003). It should be noted that the IEC has undertaken a major revision of the dishwasher standard over the past few years and many of the recent changes have been included in AS/NZS 2007.1-2005.

# Requirements for Airconditioners - Program Overview and Test Procedures

Single phase non-ducted airconditioners for household use are regulated for energy labelling and MEPS in Australia. All three phase and single phase ducted airconditioners up to 65kW cooling capacity are regulated for MEPS. Manufacturers can choose to label three phase and ducted airconditioners, but this is not mandatory. More stringent MEPS requirements for three phase airconditioners apply from October 2007 while more stringent MEPS requirements for single phase units apply from April 2006 or October 2007 (depending on the product type). MEPS details are shown on the airconditioner MEPS page and are set out in AS/NZS 3823.2-2005.

Product Definition for Energy Labelling: Single phase non-ducted airconditioners of the vapour compression type intended for household or similar use. It covers units with a single or multiple refrigeration systems such as window wall units (unitary) and single split systems. It includes units with a single outdoor unit and multiple indoor units with a single indoor control and also covers water sourced heat pumps.

Product Definition for MEPS: Single and three phase non-ducted or ducted room airconditioners of the vapour compression type of up to 65kW cooling capacity (commercial or residential). It covers units with a single or multiple refrigeration systems with a single indoor control such as single packaged units (unitary), packaged ducted units (unitary), double and triple split systems and single split systems. It also covers water sourced heat pumps.

Exemptions: The following airconditioner types are exempted from energy labelling and MEPS:

- Close controlled airconditioners (for computer rooms), pending the development of MEPS levels for these products. (refer to AS/NZS3823.2-2005 for a precise definition of these products)

- Multi-split systems (i.e., those having more than one indoor unit with more than one independent indoor controls) pending the development of a suitable test method.

- Evaporative coolers or any other cooling systems which are not of the vapour compression type.

- Ground water or ground loop sourced heat pumps within the Scope of AS/NZS 3823.1.3 (ISO 13256-1). Note that water loop heat pumps are not exempt and these are covered by AS/NZS 3823.1.3.

- Unbalanced airconditioners and spot coolers (these include many portable types).

- Commercial chillers which are used in commercial buildings to provide chilled-water for space cooling equipment.

- Models that have been granted exemption by the relevant Australian/New Zealand regulatory authority due to their specific design for applications such as process heating or cooling or for heating or cooling of spaces to conditions that are not intended for human comfort.

Test Standards: Note that all energy labelling and MEPS standards for airconditioners are published jointly by Standards Australia and Standards New Zealand.

AS/NZS3823: Performance of household electrical appliances - room airconditioners Part 1.1: Non ducted airconditioners and heat pumps - Testing and rating for performance

AS/NZS3823: Performance of electrical appliances - airconditioners and heat pumps Part 1.2: Test Methods - Ducted airconditioners and air-to-air heat pumps - Testing and rating for performance

AS/NZS3823: Performance of electrical appliances - airconditioners and heat pumps Part 1.3: Water-source heat pumps - Testing and rating for performance: Water-to-air and brine-to-air heat pumps

AS/NZS3823: Performance of electrical appliances - airconditioners and heat pumps Part 2: Energy labelling and minimum energy performance standard (MEPS) requirements

AS/NZS3823: Performance of electrical appliances - airconditioners and heat pumps Part 3: Calculation of performance for minimum energy performance standard (MEPS) requirements

Note that Part 1.1 and 1.2 have recently been amended to include standby measurement requirements. Part 1.3 covers any water source product.

Part 1.1, Part 1.2 and Part 1.3 of the standard defines the test procedures for the determination of energy consumption and capacity of airconditioners in Australia, depending on the product type.

Part 2 of the standard sets out the requirements for energy labelling and MEPS for airconditioners in Australia. An approved Energy Label for single phase non ducted airconditioners must be displayed on all products which are intended for household use and which are offered for sale in Australia. Note that all products registered to AS/NZS 3823.2-2005 must carry the revised label shown.

All airconditioners up to 65kW cooling capacity have to be registered for MEPS except for those that are exempt (see above). Part 3 sets out an alternative simulation method which can be used in lieu of physical tests to demonstrate compliance with MEPS requirements (this is not permitted where products are registered for energy labelling). The suppliers of single phase ducted and three phase models may elect to energy label their products, but all the normal rules for energy label then apply.

These standards can be purchased from Standards Australia.

Find out how to make an application for energy labelling and/or MEPS.

The  MEPS 2004 Airconditioner Registration Fact Sheet  provides more information on making an application. (factsheet-ac.pdf 31kB)

## Overview of the Test Procedures for Room Airconditioners

The standard is applicable to non-ducted and ducted room airconditioners of the vapour compression type with a single or multiple refrigeration circuits. The cooling and heating capacity and energy is determined under AS/NZS 3823.1.1 or AS/NZS 3823.1.2. These standards have been cloned from ISO5151 and ISO13253 respectively and are technically equivalent. A number of minor amendments have been included (mostly regarding tolerances and minor editorial changes), but many of these changes are scheduled to appear in the forthcoming revised edition of the ISO standards.

AS/NZS 3823.1.3 now covers all products that use a water source for heating and/or cooling. It is based on ISO13256-1.

Part 1.1 and Part 1.2 have been amended to include standby power measurements. Part 1.3 also includes standby power measurements.

For applications which cover both energy labelling and MEPS:

- one unit has to be tested (physical test) and results supplied with the labelling application

- the model must be capable of meeting the maximum cooling test requirement and a test report to demonstrate this shall be supplied.

- all tests must be carried out in a calorimeter (enthalpy method is not acceptable)

- cooling capacity and energy consumption is determined to condition T1

- heating capacity and energy consumption is determined to condition H1 (ISO condition "high")

- for each unit tested, the measured capacity shall be > 0.95 of the rated value

- for each unit tested, the measured energy consumption shall be < 1.05 of the rated value

For applications which are for MEPS only:

- one unit has to be tested (or simulated) and results supplied with the MEPS application

- a physical test in a calorimeter or the enthalpy method is acceptable

- a simulation to AS/NZS 3823.3 is an acceptable alternative to a physical test where this standard covers the product type and configuration

- cooling capacity and energy consumption is determined to condition T1

- heating capacity and energy consumption is determined to condition H1 (ISO condition "high")

- for each unit tested, the measured capacity shall be > 0.95 of the rated value

- for each unit tested, the measured energy consumption shall be < 1.05 of the rated value

The Preface of AS/NZS 3823 of Part 1.1 describes the relationship between the Australian Standard and the International Standards Organisation standards for airconditioners. It should be noted that ISO has undertaken a revision of both the ducted (ISO5151) and non ducted airconditioner standard (ISO13253) over the past few years and many of the changes proposed by International Standards Organisation TC86 SC6 working group 1 have been included in AS/NZS 3823.1.1-1998 and AS/NZS 3823.1.2-2001.

The Preface of AS/NZS 3823 of Part 2 describes the current requirements for energy labelling for MEPS in Australia and New Zealand.

# Minimum Energy Performance Standards (MEPS) Regulations in Australia - Overview

MEPS programs are made mandatory in Australia by state government legislation and regulations which give force to the relevant Australian Standards. Regulations specify the general requirements for MEPS for appliances, including offences and penalties if a party does not comply with the requirements. Technical requirements for MEPS are set out in the relevant appliance standard, which is referenced in state regulations. State based legislation is necessary because the Australian constitution gives Australian States clear responsibility for resource management issues, including energy.

It is mandatory for the following products manufactured in or imported into Australia to meet the MEPS levels specified in the relevant Australian Standards:

- refrigerators and freezers (from 1 October 1999, revision 1 January 2005)

- mains pressure electric storage water heaters (from 1 October 1999)

- small mains pressure electric storage water heaters (<80L) and low pressure and heat exchanger types (from 1 October 2005) (Refer to   AGO letter regarding levels and test methods)

- three phase electric motors (0.73kW to <185kW) (from 1 October 2001, revision April 2006)

- single phase air conditioners (from 1 October 2004, revision 1 April 2006 and 2007 and 2008)

- three phase air conditioners up to 65kW cooling capacity (from 1 October 2001, revision 1 October 2007)

- ballasts for linear fluorescent lamps (from 1 March 2003). Note that in addition to MEPS, ballasts also have to be marked with an energy efficiency index (EEI).

- linear fluorescent lamps - from 550mm to 1500mm inclusive with a nominal lamp power >16W (from 1 October 2004)

- distribution transformers - 11kV and 22kV with a rating from 10kA to 2.5MVA (from 1 October 2004)

- commercial refrigeration (self contained and remote systems) (from 1 October 2004)

# Requirements for Refrigerators and Freezers - Minimum Energy Performance Standards (MEPS)

From 1 October 1999, refrigerators, freezers and refrigerator-freezers manufactured in or imported into Australia must comply with Minimum Energy Performance Requirements which are set out in AS/NZS 4474 Part 2. More stringent requirements, which are broadly harmonised with US 2001 MEPS levels, will be introduced on 1 January 2005. The levels for both 1999 and 2005 are defined in AS/NZS 4474.2-2001. The main factors for both the 1999 and 2005 MEPS levels are shown in the tables below.

## MEPS Formula

For a refrigerating appliance model, the energy consumption (determined in accordance with AS/NZS 4474 Part 1) shall not exceed the MEPS cut off level, which is determined as follows:

MEPS cut-off level = [Kf + (Kv x Vadj tot)] x Ka + Ad tot + Awi (kWh/y) . . . 3(1)

where:

kWh = Kilowatt hours

Kf = Fixed allowance factor for its appliance group. (Refer to Table below) (kWh/y)

Kv = Variable allowance factor. (Refer to Table below) (kWh/y/L)

Ka = Adaptive defrost adjustment factor, which = 1.0 for MEPS 1999 and for non-adaptive defrost models in MEPS 2005, = 1.05 for adaptive defrost models for MEPS 2005.

Vadj tot = Total adjusted volume. (gross litres)

Ad tot = An allowance which is made where the external doors provided on an appliance differ from the regular arrangement for that appliance group - see the standard for details. The door allowance can be positive or negative.

Awi = An allowance of 120 kWh/year which applies where an appliance has a "through-the-door ice dispenser" i.e. it has an automatic ice-maker coupled with a device for delivery on demand of ice externally through a door. This allowance also applies if the through the door dispenser also dispenses chilled water.

## Refrigerator Groups

The following provides an approximate description of refrigerator and freezer Groups - see Table 1.1 in AS/NZS 4474.1 for the precise definition:

- Group 1: Refrigerator without a low temperature compartment, automatic defrost.

- Group 2: Refrigerator with or without an icemaking compartment, manual defrost

- Group 3: Refrigerator with a short or long term frozen food compartment, manual defrost

- Group 4: Refrigerator-freezer, fresh food compartment is automatic defrost, freezer manual defrost ("partial automatic defrost")

- Group 5B: Refrigerator-freezer, both compartments automatic defrost (frost free), bottom mounted freezer

- Group 5T: Refrigerator-freezer, both compartments automatic defrost (frost free), not side by side configuration or bottom mounted freezer (ie top mounted freezer)

- Group 5S: Refrigerator-freezer, both compartments automatic defrost (frost free), side by side configuration

- Group 6U: Separate vertical freezer, manual defrost

- Group 6C: Separate chest freezer, all defrost types

- Group 7: Separate vertical freezer, automatic defrost (frost free)

Compartment in this context refers to compartment (with separate external door) or an internal sub-compartment.

## MEPS Factors

Fixed and variable allowance factors are determined from the Group classification of the appliance in the Tables below.

### MEPS CUT-OFF LEVEL FACTORS - October 1999

| Refrigerator Group | Fixed allowance factor (Kf) kWh/y | Variable allowance factor (Kv) kWh/y/L |
| --- | --- | --- |
| 1 | 368 | 0.892 |
| 2 | 300 | 0.728 |
| 3 | 330 | 0.800 |
| 4 | 424 | 1.020 |
| 5T & 5B | 424 | 1.256 |
| 5S | 465 | 1.378 |
| 6C | 248 | 0.670 |
| 6U | 439 | 0.641 |
| 7 | 439 | 1.020 |

New MEPS levels for 2005 have been negotiated with industry and these have been publicly released in AS/NZS 4474.2-2001 and are set out in the table below. These levels are broadly harmonised with US 2001 MEPS levels. A copy of the standard can be obtained from SAI Global under licence from Standards Australia.

### MEPS CUT-OFF LEVEL FACTORS - January 2005

| Refrigerator Group | Fixed allowance factor (Kf) kWh/y | Variable allowance factor (Kv) kWh/y/L |
| --- | --- | --- |
| 1 | 278 | 0.335 |
| 2 | 289 | 0.290 |
| 3 | 283 | 0.344 |
| 4 | 277 | 0.330 |
| 5T | 311 | 0.357 |
| 5B | 411 | 0.357 |
| 5S | 569 | 0.169 |
| 6C | 190 | 0.483 |
| 6U | 281 | 0.298 |
| 7 | 356 | 0.478 |

# MEPS Requirements for Electric Storage Water Heaters

Since 1 October 1999, unvented (displacement) electric storage water heaters manufactured in or imported into Australia must comply with Minimum Energy Performance Requirements which are set out in AS/NZS 4692.2 (originally also set out AS 1056.1 - Amendment 3).

Since 1 October 2005 the following changes and additions to water heater MEPS were made:

- MEPS levels for small main pressure water heaters (63 litres or less) were increased from the 1999 levels - the new levels are set out in the table below.

- MEPS for vented water heaters (25 to 630 litres hot water delivery inclusive) were introduced for the first time - see the equation below.

- MEPS for heat exchange water heaters with a heat storage volume from 45 to 710 litres inclusive were introduced for the first time - see the equation below.

These levels are set out in the notice of new MEPS levels for miscellaneous and mains pressure electric water heaters:   Letter from the NAEEEC Chair and are fully documented in AS/NZS4692. Note that there are a number of requirements for new models registered to revised MEPS levels that came into force on 1 October 2005 - these are fully detailed in AS/NZS 4692.2. AS/NZS 4692.2 also sets out MEPS levels for New Zealand which are different from those in Australia (these are also set out in NZS 4602 and NZS 4606.1).

The following products are exempt from MEPS:

- Water heaters that use electric resistive heating to provide less than 50% of the energy supplied in a typical year (eg heat pump and solar water heaters) when simulated to AS 4234 under Climate Zone 3 with an energy delivery of 22.5 MJ/day for an electric boosting element and energisation profile specified by the manufacturer.

- Water heaters that are not intended for electric power grid connection (eg for use in vehicles and marine applications).

## Mains Pressure Electric Water Heaters

The 1999 and 2005 MEPS levels for mains pressure (unvented displacement) electric water heaters when tested to AS1056.1 are set out in the table below. AS/NZS 4692.2 set outs MEPS levels when tested to AS/NZS 4692.1.

#### Minimum Energy Performance Standards:
Maximum Heat Loss for Electric Storage Water Heaters
(unvented tanks without an attached feed tank)

| Rated Hot Water Delivery (litres) | Maximum Allowable Standing Heat Loss (kilowatt hours/day) October 1999 | Maximum Allowable Standing Heat Loss (kilowatt hours/day) October 2005 |
| --- | --- | --- |
| <25 | N/A | 0.98 |
| 25 | 1.4 | 0.98 |
| 31.5 | 1.5 | 1.05 |
| 40 | 1.6 | 1.12 |
| 50 | 1.7 | 1.19 |
| 63 | 1.9 | 1.33 |
| 80 | 1.47 | 1.47 |
| 100 | 1.61 | 1.61 |
| 125 | 1.75 | 1.75 |
| 160 | 1.96 | 1.96 |
| 200 | 2.17 | 2.17 |
| 250 | 2.38 | 2.38 |
| 315 | 2.66 | 2.66 |
| 400 | 2.87 | 2.87 |
| 500 | 3.15 | 3.15 |
| 630 | 3.43 | 3.43 |

Only the pre-defined hot water delivery values shown above are permitted for mains pressure models. These values may be increased by 0.2 kWh/day for each additional heating element (where there are more than 1) and 0.2 kWh/day for each temperature or temperature/pressure release valve mounted on a hot water fitting. The rated hot water delivery is defined in AS1056.1 (this is less than the hot water storage volume). It also defines the test procedures for the determination of standing heat loss.

## Vented Electric Water Heaters

MEPS levels for vented electric water heaters from 1 October 2005 are set out in the following equation when tested to AS1056.1: (AS/NZS 4692.2 set outs MEPS levels when tested to AS/NZS 4692.1)

MEPS (maximum daily heat loss) = 0.255 x V0.4032

Where:
Where V is the hot water delivery of the vented electric water heater.

The values in the above Equation apply to water heaters with a single heating unit and may be increased by 0.2 kWh/24 h for each additional heating unit and by 0.3 kWh/24 h for an attached feed tank. Vented water heaters with an attached feed tank which have a hot water delivery of less than 125 litres are exempted from MEPS.

## Heat Exchange Water Heaters

MEPS levels for heat exchange electric water heaters from 1 October 2005 are set out in the following equation when tested to AS1361: (AS/NZS 4692.2 set outs MEPS levels when tested to AS/NZS 4692.1)

MEPS (maximum daily heat loss) = 0.4057 x V0.3322

Where:
Where V is the heat storage volume of the heat exchange electric water heater.

The values in the above Equation apply to water heaters with a single heating unit and may be increased by 0.2 kWh/24 h for each additional heating unit, by 0.3 kWh/24 h for an attached feed tank, by 0.2 kWh/24 h for an expansion relief valve and by 0.1 kWh/24 h for each other water fitting (excluding a drain fitting).

# MEPS Requirements for Three Phase Electric Motors from 0.73kW to <185kW

Since 1 October 2001, three phase electric motors from 0.73kW to <185kW manufactured in or imported into Australia must comply with Minimum Energy Performance (MEPS) requirements which are set out in AS/NZS 1359.5-2000. MEPS does not apply to submersible motors, integral motor-gear systems, variable or multi-speed motors or those rated only for short duty cycles (IEC 60034-2 duty rating S2). Rewound motors are not required to comply with MEPS.

The Minimum Energy Performance Standards requirements are set out as minimum efficiency levels. These are set out in the tables below for Test Method B (AS 1359.102.1) and Test Method A (AS/NZS 1359.102.3) (see details on motor test methods).

From 1 April 2006, MEPS levels for three phase electric motors were revised to become more stringent. The "High Efficiency" level from 2001 became the MEPS level on 1 April 2006. From 1 April 2005 a revised "High Efficiency" level came into force. These new MEPS and high efficiency requirements are set out in detail in AS/NZS 1359.5-2004 which was published in September 2004. This standard supercedes AS/NZS 1359.5-2000.

MEPS levels and "High Efficiency" levels are set out in the tables as follows:

- Table A1: MEPS 2001, Test Method A

- Table A2: MEPS 2006, Test Method A and "High Efficiency" 2001, Test Method A

- Table A3: "High Efficiency" 2005, Test Method A

- Table B1: MEPS 2001, Test Method B

- Table B2: MEPS 2006, Test Method B and "High Efficiency" 2001, Test Method B

- Table B3: "High Efficiency" 2005, Test Method B

A copy of the standard can be purchased from SAI Global under licence from Standards Australia.

## Table A1: Efficiency Levels for Three Phase Electric Motors - Test Method A

| Rated output kW | Minimum efficiency % |  |  |  |
| --- | --- | --- | --- | --- |
|  | 2 pole | 4 pole | 6 pole | 8 pole |
| 0.73 | 72.3 | 72.7 | 70.7 | 66.7 |
| 0.75 | 72.3 | 72.7 | 70.7 | 66.7 |
| 1.1 | 74.6 | 74.6 | 73.6 | 69.9 |
| 1.5 | 76.9 | 76.9 | 75.7 | 73.0 |
| 2.2 | 79.5 | 79.5 | 78.1 | 76.1 |
| 3 | 81.2 | 81.2 | 79.9 | 78.2 |
| 4 | 82.8 | 82.8 | 81.6 | 80.1 |
| 5.5 | 84.4 | 84.4 | 83.3 | 82.0 |
| 7.5 | 85.8 | 85.8 | 84.7 | 83.7 |
| 11 | 87.2 | 87.2 | 86.4 | 85.6 |
| 15 | 88.3 | 88.3 | 87.7 | 87.1 |
| 18.5 | 89.0 | 89.0 | 88.6 | 88.0 |
| 22 | 89.5 | 89.5 | 89.1 | 88.7 |
| 30 | 90.5 | 90.5 | 90.2 | 89.9 |
| 37 | 91.1 | 91.1 | 90.8 | 90.6 |
| 45 | 91.7 | 91.7 | 91.5 | 91.2 |
| 55 | 92.2 | 92.2 | 92.0 | 91.8 |
| 75 | 92.9 | 92.9 | 92.8 | 92.7 |
| 90 | 93.4 | 93.2 | 93.2 | 93.0 |
| 110 | 93.8 | 93.8 | 93.7 | 93.5 |
| 132 | 94.2 | 94.1 | 94.1 | 93.8 |
| 150 | 94.5 | 94.5 | 94.4 | 94.1 |
| <185 | 94.5 | 94.5 | 94.4 | 94.1 |
| NOTES: 1. For intermediate vales of rated output, the efficiency shall be determined by linear interpolation. 2. Tolerances specified in Table 1.1 of AS/NZS 1359.5 are applicable to the above values only in the case of a verification test. |  |  |  |  |

## Table A2: Efficiency Levels for Three Phase Electric Motors - Test Method A

| Rated output kW | Minimum efficiency % |  |  |  |
| --- | --- | --- | --- | --- |
|  | 2 pole | 4 pole | 6 pole | 8 pole |
| 0.73 | 78.8 | 80.5 | 76.0 | 71.8 |
| 0.75 | 78.8 | 80.5 | 76.0 | 71.8 |
| 1.1 | 80.6 | 82.2 | 78.3 | 74.7 |
| 1.5 | 82.6 | 83.5 | 79.9 | 76.8 |
| 2.2 | 84.0 | 84.9 | 81.9 | 79.4 |
| 3 | 85.3 | 86.0 | 83.5 | 81.3 |
| 4 | 86.3 | 87.0 | 84.7 | 82.8 |
| 5.5 | 87.2 | 87.9 | 86.1 | 84.5 |
| 7.5 | 88.3 | 88.9 | 87.3 | 86.0 |
| 11 | 89.5 | 89.9 | 88.7 | 87.7 |
| 15 | 90.3 | 90.8 | 89.6 | 88.9 |
| 18.5 | 90.8 | 91.2 | 90.3 | 89.7 |
| 22 | 91.2 | 91.6 | 90.8 | 90.2 |
| 30 | 92.0 | 92.3 | 91.6 | 91.2 |
| 37 | 92.5 | 92.8 | 92.2 | 91.8 |
| 45 | 92.9 | 93.1 | 92.7 | 92.4 |
| 55 | 93.2 | 93.5 | 93.1 | 92.9 |
| 75 | 93.9 | 94.0 | 93.7 | 93.7 |
| 90 | 94.2 | 94.4 | 94.2 | 94.1 |
| 110 | 94.5 | 94.7 | 94.5 | 94.5 |
| 132 | 94.8 | 94.9 | 94.8 | 94.8 |
| 150 | 95.0 | 95.2 | 95.1 | 95.2 |
| <185 | 95.0 | 95.2 | 95.1 | 95.2 |
| NOTES: 1. For intermediate vales of rated output, the efficiency shall be determined by linear interpolation. 2. Tolerances specified in Table 1.1 of AS/NZS 1359.5 are applicable to the above values only in the case of a verification test. |  |  |  |  |

## Table A3: Efficiency Levels for Three Phase Electric Motors - Test Method A

| Rated output kW | Minimum efficiency % |  |  |  |
| --- | --- | --- | --- | --- |
|  | 2 pole | 4 pole | 6 pole | 8 pole |
| 0.73 | 81.4 | 82.9 | 78.8 | 75.0 |
| 0.75 | 81.4 | 82.9 | 78.8 | 75.0 |
| 1.1 | 83.0 | 84.5 | 80.9 | 77.6 |
| 1.5 | 84.8 | 85.6 | 82.4 | 79.6 |
| 2.2 | 86.2 | 86.9 | 84.2 | 81.9 |
| 3 | 87.2 | 87.8 | 85.6 | 83.6 |
| 4 | 88.1 | 88.7 | 86.7 | 85.0 |
| 5.5 | 88.9 | 89.5 | 87.9 | 86.5 |
| 7.5 | 89.9 | 90.4 | 89.0 | 87.8 |
| 11 | 90.9 | 91.3 | 90.2 | 89.3 |
| 15 | 91.6 | 92.1 | 91.0 | 90.4 |
| 18.5 | 92.1 | 92.4 | 91.6 | 91.1 |
| 22 | 92.4 | 92.8 | 92.1 | 91.5 |
| 30 | 93.1 | 93.4 | 92.8 | 92.4 |
| 37 | 93.6 | 93.8 | 93.3 | 92.9 |
| 45 | 93.9 | 94.1 | 93.7 | 93.5 |
| 55 | 94.2 | 94.4 | 94.1 | 93.9 |
| 75 | 94.8 | 94.9 | 94.6 | 94.6 |
| 90 | 95.0 | 95.2 | 95.0 | 94.9 |
| 110 | 95.3 | 95.5 | 95.3 | 95.3 |
| 132 | 95.5 | 95.6 | 95.5 | 95.5 |
| 150 | 95.7 | 95.9 | 95.8 | 95.9 |
| <185 | 95.7 | 95.9 | 95.8 | 95.9 |
| NOTES: 1. For intermediate vales of rated output, the efficiency shall be determined by linear interpolation. 2. Tolerances specified in Table 1.1 of AS/NZS 1359.5 are applicable to the above values only in the case of a verification test. |  |  |  |  |

## Table B1: Efficiency Levels for Three Phase Electric Motors - Test Method B

| Rated output kW | Minimum efficiency % |  |  |  |
| --- | --- | --- | --- | --- |
|  | 2 pole | 4 pole | 6 pole | 8 pole |
| 0.73 | 74.0 | 74.4 | 72.4 | 68.4 |
| 0.75 | 74.0 | 74.4 | 72.4 | 68.4 |
| 1.1 | 76.2 | 76.2 | 75.2 | 71.5 |
| 1.5 | 78.5 | 78.5 | 77.3 | 74.6 |
| 2.2 | 81.0 | 81.0 | 79.6 | 77.6 |
| 3 | 82.6 | 82.6 | 81.4 | 79.7 |
| 4 | 84.2 | 84.2 | 83.0 | 81.5 |
| 5.5 | 85.7 | 85.7 | 84.6 | 83.3 |
| 7.5 | 87.0 | 87.0 | 86.0 | 85.0 |
| 11 | 88.4 | 88.4 | 87.6 | 86.8 |
| 15 | 89.4 | 89.4 | 88.8 | 88.2 |
| 18.5 | 90.0 | 90.0 | 89.6 | 89.0 |
| 22 | 90.5 | 90.5 | 90.1 | 89.7 |
| 30 | 91.4 | 91.4 | 91.1 | 90.8 |
| 37 | 92.0 | 92.0 | 91.7 | 91.5 |
| 45 | 92.5 | 92.5 | 92.3 | 92.0 |
| 55 | 93.0 | 93.0 | 92.8 | 92.6 |
| 75 | 93.6 | 93.6 | 93.5 | 93.4 |
| 90 | 94.1 | 93.9 | 93.9 | 93.7 |
| 110 | 94.4 | 94.4 | 94.3 | 94.1 |
| 132 | 94.8 | 94.7 | 94.7 | 94.4 |
| 150 | 95.0 | 95.0 | 94.9 | 94.7 |
| <185 | 95.0 | 95.0 | 94.9 | 94.7 |
| NOTES: 1. For intermediate vales of rated output, the efficiency shall be determined by linear interpolation. 2. Tolerances specified in Table 1.1 of AS/NZS 1359.5 are applicable to the above values only in the case of a verification test. |  |  |  |  |

## Table B2: Efficiency Levels for Three Phase Electric Motors - Test Method B

| Rated output kW | Minimum efficiency % |  |  |  |
| --- | --- | --- | --- | --- |
|  | 2 pole | 4 pole | 6 pole | 8 pole |
| 0.73 | 80.5 | 82.2 | 77.7 | 73.5 |
| 0.75 | 80.5 | 82.2 | 77.7 | 73.5 |
| 1.1 | 82.2 | 83.8 | 79.9 | 76.3 |
| 1.5 | 84.1 | 85.0 | 81.5 | 78.4 |
| 2.2 | 85.6 | 86.4 | 83.4 | 80.9 |
| 3 | 86.7 | 87.4 | 84.9 | 82.7 |
| 4 | 87.6 | 88.3 | 86.1 | 84.2 |
| 5.5 | 88.5 | 89.2 | 87.4 | 85.8 |
| 7.5 | 89.5 | 90.1 | 88.5 | 87.2 |
| 11 | 90.6 | 91.0 | 89.8 | 88.8 |
| 15 | 91.3 | 91.8 | 90.7 | 90.0 |
| 18.5 | 91.8 | 92.2 | 91.3 | 90.7 |
| 22 | 92.2 | 92.6 | 91.8 | 91.2 |
| 30 | 92.9 | 93.2 | 92.5 | 92.1 |
| 37 | 93.3 | 93.6 | 93.0 | 92.7 |
| 45 | 93.7 | 93.9 | 93.5 | 93.2 |
| 55 | 94.0 | 94.2 | 93.9 | 93.7 |
| 75 | 94.6 | 94.7 | 94.4 | 94.4 |
| 90 | 94.8 | 95.0 | 94.8 | 94.7 |
| 110 | 95.1 | 95.3 | 95.1 | 95.1 |
| 132 | 95.4 | 95.5 | 95.4 | 95.4 |
| 150 | 95.5 | 95.7 | 95.6 | 95.7 |
| <185 | 95.5 | 95.7 | 95.6 | 95.7 |
| NOTES: 1. For intermediate vales of rated output, the efficiency shall be determined by linear interpolation. 2. Tolerances specified in Table 1.1 of AS/NZS 1359.5 are applicable to the above values only in the case of a verification test. |  |  |  |  |

## Table B3: Efficiency Levels for Three Phase Electric Motors - Test Method B

| Rated output kW | Minimum efficiency % |  |  |  |
| --- | --- | --- | --- | --- |
|  | 2 pole | 4 pole | 6 pole | 8 pole |
| 0.73 | 82.9 | 84.5 | 80.4 | 76.5 |
| 0.75 | 82.9 | 84.5 | 80.4 | 76.5 |
| 1.1 | 84.5 | 85.9 | 82.4 | 79.1 |
| 1.5 | 86.2 | 87.0 | 83.8 | 81.0 |
| 2.2 | 87.5 | 88.2 | 85.5 | 83.3 |
| 3 | 88.5 | 89.1 | 86.9 | 84.9 |
| 4 | 89.3 | 89.9 | 87.9 | 86.2 |
| 5.5 | 90.1 | 90.7 | 89.1 | 87.7 |
| 7.5 | 90.9 | 91.5 | 90.1 | 88.9 |
| 11 | 91.9 | 92.2 | 91.2 | 90.3 |
| 15 | 92.5 | 92.9 | 92.0 | 91.4 |
| 18.5 | 92.9 | 93.3 | 92.5 | 92.0 |
| 22 | 93.3 | 93.6 | 92.9 | 92.4 |
| 30 | 93.9 | 94.2 | 93.6 | 93.2 |
| 37 | 94.2 | 94.5 | 94.0 | 93.7 |
| 45 | 94.6 | 94.8 | 94.4 | 94.2 |
| 55 | 94.9 | 95.0 | 94.8 | 94.6 |
| 75 | 95.4 | 95.5 | 95.2 | 95.2 |
| 90 | 95.5 | 95.7 | 95.5 | 95.5 |
| 110 | 95.8 | 96.0 | 95.8 | 95.8 |
| 132 | 96.1 | 96.1 | 96.1 | 96.1 |
| 150 | 96.1 | 96.3 | 96.2 | 96.3 |
| <185 | 96.1 | 96.3 | 96.2 | 96.3 |
| NOTES: 1. For intermediate vales of rated output, the efficiency shall be determined by linear interpolation. 2. Tolerances specified in Table 1.1 of AS/NZS 1359.5 are applicable to the above values only in the case of a verification test. |  |  |  |  |

# MEPS Requirements for Single Phase and Three Phase Air Conditioners to 65kW Cooling Capacity

Since 1 October 2001, three phase air conditioners with a cooling capacity of up to 65kW manufactured in or imported into Australia must comply with Minimum Energy Performance (MEPS) requirements. Current and future MEPS requirements are set out in AS/NZS 3823.2-2005. MEPS covers three phase non-ducted or ducted room air conditioners of the vapour compression type of up to 65kW cooling (commercial or residential) within the scope of AS/NZS 3823.1.1 or AS/NZS 3823.1.2 or AS/NZS 3823.1.3, as applicable.

Since 1 October 2004, all single phase air conditioners manufactured in or imported into Australia must comply with Minimum Energy Performance (MEPS) requirements. Current and future MEPS requirements are set out in AS/NZS 3823.2-2005. MEPS covers single phase non-ducted or ducted room air conditioners of the vapour compression type (commercial or residential) within the scope of AS/NZS 3823.1.1 or AS/NZS 3823.1.2 or AS/NZS 3823.1.3, as applicable.

AS/NZS 3823.1.1 and AS/NZS 3823.1.2 and AS/NZS 3823.1.3 cover equipment utilizing one or more refrigeration systems with one outdoor unit and one or more indoor units controlled by a single thermostat/controller. They cover equipment utilizing single, multiple and variable capacity components. Examples of equipment types covered include those with a single indoor control such as single packaged units, packaged ducted units, double and triple split systems and single split systems and water sourced heat pumps.

MEPS requirements for single phase and three phase air conditioners have been revised and are documented in the revised edition of AS/NZS 3823.2 which was published on 30 December 2005. The changes include accelerated MEPS levels for many non-ducted single phase models effective from April 2006. Other single phase and all three phase types have revised MEPS levels effective from October 2007. The details are set out in the latest Regulatory Impact Statement. The details included below are from the 2005 edition of the standard.

### Special E3 Notices Regarding this Product:

- Stakeholder Letter -  Notice of increased levels Minimum Energy Performance Standards for Air Conditioners - 1 October 2007

- Stakeholder Letter -  Postponement of the Proposed 2008 MEPS Levels for Air Conditioners - May 2007

- The   letter dated 19 January 2006 confirms that MCE have now agreed to the proposal for MEPS to commence on 1 April 2006.

- Notice of early adoption of increased MEPS for single phase airconditioners from 1 April 2006.   Letter from the NAEEEC Chair outlines the proposal under consideration by Energy Ministers for the revised MEPS date for non-ducted single phase air conditioners less than 7.5kW for household use.

Products manufactured or imported must comply with the relevant requirements and MEPS levels at the time of manufacture or importation. Once registered to these requirements, products manufactured or imported prior to any new MEPS date can continue to be sold after new MEPS levels come into force. Details related to stock that has been imported into Australia or manufactured in Australia before 1st April 2006 are set out in the Administrative Guidelines (refer to MEPS transition arrangements, clause 3.2.2).

Test procedures for single and three phase units are the same: see requirements for air conditioners.

The following air conditioner types are exempted from energy labelling and MEPS:

- Close controlled airconditioners (for computer rooms), pending the development of MEPS levels for these products. (refer to AS/NZS3823.2-2005 for a precise definition of these products)

- Multi-split systems (i.e., those having more than one indoor unit with more than one independent indoor controls) pending the development of a suitable test method.

- Evaporative coolers or any other cooling systems which are not of the vapour compression type.

- Ground water or ground loop sourced heat pumps within the Scope of ISO 13256-1. Note that water loop heat pumps are not exempt and these are covered by AS/NZS 3823.1.3.

- Unbalanced airconditioners and spot coolers (these include many portable types).

- Commercial chillers which are used in commercial buildings to provide chilled-water for space cooling equipment.

- Models that have been granted exemption by the relevant Australian/New Zealand regulatory authority due to their specific design for applications such as process heating or cooling or for heating or cooling of spaces to conditions that are not intended for human comfort.

The Minimum Energy Performance Standards requirements are set out as minimum EER levels. These are set out in the table below.

### MEPS: MINIMUM EER FOR AIR COOLED CONDENSER AIRCONDITIONERS (from AS/NZS 3823.2-2005)

| Configuration | Cooling only or reverse cycle | Phase | Rated cooling capacity (kW) | Min EER 1 Oct 2001 | Min EER 1 Oct 2004 | Min EER 1 Apr 2006 | Min EER 1 Oct 2007 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Non ducted unitary (window/wall> | C/O | 1 | < 7.5 | n/a | 2.45 | 2.75b | 2.75 |
|  | C/O | 1 | 7.5 to <10 | n/a | 2.45 | 2.45 | 2.75 |
|  | C/O | 1 | ≥ 10.0a | n/a | 2.45 | 2.45 | 2.75 |
|  | R/C | 1 | < 7.5 | n/a | 2.30 | 2.75b | 2.75 |
|  | R/C | 1 | 7.5 to < 10 | n/a | 2.30 | 2.30 | 2.75 |
|  | R/C | 1 | ≥ 10.0a | n/a | 2.30 | 2.30 | 2.75 |
|  | Both | 3 | < 10 | 2.25 | 2.25 | 2.25 | 2.75 |
|  | Both | 3 | 10 to 12.5 | 2.30 | 2.30 | 2.30 | 2.75 |
|  | Both | 3 | 12.6 to 15.5 | 2.35 | 2.35 | 2.35 | 2.75 |
|  | Both | 3 | 15.6 to 18 | 2.40 | 2.40 | 2.40 | 2.75 |
|  | Both | 3 | 18.1 to 18.9c | 2.45 | 2.45 | 2.45 | 2.75 |
| Non ducted split | C/O | 1 | < 4 | n/a | 2.45 | 3.05b | 3.05 |
|  | C/O | 1 | 4 to < 7.5 | n/a | 2.45 | 2.75b | 2.75 |
|  | C/O | 1 | 7.5 to < 10 | n/a | 2.45 | 2.45 | 2.75 |
|  | C/O | 1 | ≥ 10.0a | n/a | 2.45 | 2.45 | 2.75 |
|  | R/C | 1 | < 4 | n/a | 2.30 | 3.05b | 3.05 |
|  | R/C | 1 | 4 to < 7.5 | n/a | 2.30 | 2.75b | 2.75 |
|  | R/C | 1 | 7.5 to < 10 | n/a | 2.30 | 2.30 | 2.75 |
|  | R/C | 1 | ≥ 10.0a | n/a | 2.30 | 2.30 | 2.75 |
|  | Both | 3 | < 4 | 2.25 | 2.25 | 2.25 | 3.05 |
|  | Both | 3 | 4 to < 7.5 | 2.25 | 2.25 | 2.25 | 2.75 |
|  | Both | 3 | 7.5 to < 10 | 2.25 | 2.25 | 2.25 | 2.75 |
|  | Both | 3 | 10 - 12.5 | 2.30 | 2.30 | 2.30 | 2.75 |
|  | Both | 3 | 12.6 - 15.5 | 2.35 | 2.35 | 2.35 | 2.75 |
|  | Both | 3 | 15.6 - 18 | 2.40 | 2.40 | 2.40 | 2.75 |
|  | Both | 3 | 18.1 - 18.9 | 2.45 | 2.45 | 2.45 | 2.75 |
|  | Both | 3 | 19 - 25 | 2.45 | 2.45 | 2.45 | 3.05 |
|  | Both | 3 | 25.1 - 30 | 2.50 | 2.50 | 2.50 | 3.05 |
|  | Both | 3 | 30.1 - 37.5 | 2.55 | 2.55 | 2.55 | 3.05 |
|  | Both | 3 | 37.6 - 39 | 2.60 | 2.60 | 2.60 | 3.05 |
|  | Both | 3 | 39.1 - 45.5 | 2.60 | 2.60 | 2.60 | 2.75 |
|  | Both | 3 | 45.6 - 65 | 2.65 | 2.65 | 2.65 | 2.75 |
| Ducted (split & unitary) | C/O | 1 | 0 to < 10 | n/a | 2.45 | 2.45 | 2.50 |
|  | C/O | 1 | ≥ 10.0a | n/a | 2.45 | 2.45 | 2.50 |
|  | R/C | 1 | 0 to < 10 | n/a | 2.30 | 2.30 | 2.50 |
|  | R/C | 1 | ≥ 10.0a | n/a | 2.30 | 2.30 | 2.50 |
|  | Both | 3 | 0 to < 10 | 2.25 | 2.25 | 2.25 | 2.50 |
|  | Both | 3 | 10 - 12.5 | 2.30 | 2.30 | 2.30 | 2.75 |
|  | Both | 3 | 12.6 - 15.5 | 2.35 | 2.35 | 2.35 | 2.75 |
|  | Both | 3 | 15.6 - 18 | 2.40 | 2.40 | 2.40 | 2.75 |
|  | Both | 3 | 18.1 - 18.9 | 2.45 | 2.45 | 2.45 | 2.75 |
|  | Both | 3 | 19 - 25 | 2.45 | 2.45 | 2.45 | 3.05 |
|  | Both | 3 | 25.1 - 30 | 2.50 | 2.50 | 2.50 | 3.05 |
|  | Both | 3 | 30.1 - 37.5 | 2.55 | 2.55 | 2.55 | 3.05 |
|  | Both | 3 | 37.6 - 39 | 2.60 | 2.60 | 2.60 | 3.05 |
|  | Both | 3 | 39.1 - 45.5 | 2.60 | 2.60 | 2.60 | 2.75 |
|  | Both | 3 | 45.6 - 65 | 2.65 | 2.65 | 2.65 | 2.75 |
| NOTES: a Amendment 2 b Increased MEPS levels for 1 April 2006 apply to any product marketed for household use. Products which are purely commercial are subjected to the 2006 MEPS levels indicated apply from 1 October 2007. MEPS for 2007 apply to all product types irrespective of use. c For three phase non-ducted unitary products > 18.9kW, MEPS are the same as for non-ducted-split systems. 1. AS/NZS 3823.2 defines terms and applicable dates used in this table. 2. Not applicable is indicated by n/a - no coverage of this product type from the date specified. 3. Amendment 3 (to be published in late 2007) deletes the published 2008 MEPS levels (not shown above). These MEPS levels (with any adjustments) are expected to be included in a revision of AS/NZS3823.2 in 2008 with a delay of 12 to 18 months - see stakeholder letter dated May 2007 above. |  |  |  |  |  |  |  |

### MEPS: MINIMUM EER FOR WATER COOLED CONDENSER AIRCONDITIONERS (from AS/NZS 3823.2-2005)

| Supply (No. of phases) | Cooling capacity/Type | Oct. 2001 (see Note 2) | Oct. 2004 | Oct. 2007 |
| --- | --- | --- | --- | --- |
| 1 | All capacities-Cooling only (ducted and non-ducted) | N/A | 2.45 | 3.50 |
|  | All capacities-Reverse cycle (ducted and non-ducted) | N/A | 2.30 | 3.50 |
| 3 | < 10 kW | 2.25 | 2.25 | 3.50 |
|  | 10-12.5 kW | 2.30 | 2.30 | 3.50 |
|  | 12.6-15.5 kW | 2.35 | 2.35 | 3.50 |
|  | 15.6-18.0 kW | 2.40 | 2.40 | 3.50 |
|  | 18.1-18.9 kW | 2.45 | 2.45 | 3.50 |
|  | 19.0-25.0 kW | 2.45 | 2.45 | 3.50 |
|  | 25.1-30.0 kW | 2.50 | 2.50 | 3.50 |
|  | 30.1-37.5 kW | 2.55 | 2.55 | 3.50 |
|  | 37.6-39.0 kW | 2.60 | 2.60 | 3.50 |
|  | 39.0-45.5 kW | 2.60 | 2.60 | 3.20 |
|  | 45.6-65.0 kW | 2.65 | 2.65 | 3.20 |
| NOTES: 1. This table includes requirements for airconditioners with water cooled condensers and water sourced heat pumps. 2. In New Zealand, MEPS became mandatory on 1 July 2002. |  |  |  |  |

### FUTURE MEPS LEVELS BEYOND 2007 (CLASS A EFFICIENCY)
MINIMUM EER COOLING THREE PHASE AIRCONDITIONERS FROM 1 OCTOBER 2007 (from AS/NZS 3823.2-2005)

| Rated cooling capacity (kW) | Minimum EER cooling (W/W) |
| --- | --- |
| kW ≤ 10.0 | Under consideration |
| 10.0 < kW ≤ 19.0 | 3.35 |
| 19.0 < kW ≤ 39.0 | 3.65 |
| kW > 39.0 | 3.35 |

The rated capacity is defined under condition T1 in AS/NZS 3823.1.1 or AS/NZS3823.1.2 or AS/NZS3823.1.3, as applicable.

For MEPS purposes:

- at least one unit has to be tested (or simulated) and results supplied with the application

- physical tests in a calorimeter or using the enthalpy method are acceptable

- simulation to AS/NZS 3823.3 is an acceptable alternative to physical tests

- cooling capacity and energy consumption is determined to condition T1

- heating capacity and energy consumption is determined to condition H1 (ISO condition "high")

- for each unit tested, the measured capacity shall be > 0.95 of the rated value

- for each unit tested, the measured energy consumption shall be < 1.05 of the rated value

Note that where a product is registered for energy labelling as well as MEPS, only physical tests are permitted (ie simulation is not allowed). For products registered for energy labelling, a test report must also be submitted to show that the model is capable of meeting the maximum cooling test requirement.

Note that heating only units (with no cooling function) are not required to be registered for MEPS.

A copy of the standard can be purchased from SAI Global under licence from Standards Australia.

Find out how to make an application for energy labelling and/or MEPS.

You can also download the  MEPS 2004 Air Conditioner Registration Fact Sheet  for more information on making an application. (factsheet-ac.pdf 31kB)

# MEPS Requirements for Single Phase and Three Phase Air Conditioners to 65kW Cooling Capacity

Since 1 October 2001, three phase air conditioners with a cooling capacity of up to 65kW manufactured in or imported into Australia must comply with Minimum Energy Performance (MEPS) requirements. Current and future MEPS requirements are set out in AS/NZS 3823.2-2005. MEPS covers three phase non-ducted or ducted room air conditioners of the vapour compression type of up to 65kW cooling (commercial or residential) within the scope of AS/NZS 3823.1.1 or AS/NZS 3823.1.2 or AS/NZS 3823.1.3, as applicable.

Since 1 October 2004, all single phase air conditioners manufactured in or imported into Australia must comply with Minimum Energy Performance (MEPS) requirements. Current and future MEPS requirements are set out in AS/NZS 3823.2-2005. MEPS covers single phase non-ducted or ducted room air conditioners of the vapour compression type (commercial or residential) within the scope of AS/NZS 3823.1.1 or AS/NZS 3823.1.2 or AS/NZS 3823.1.3, as applicable.

AS/NZS 3823.1.1 and AS/NZS 3823.1.2 and AS/NZS 3823.1.3 cover equipment utilizing one or more refrigeration systems with one outdoor unit and one or more indoor units controlled by a single thermostat/controller. They cover equipment utilizing single, multiple and variable capacity components. Examples of equipment types covered include those with a single indoor control such as single packaged units, packaged ducted units, double and triple split systems and single split systems and water sourced heat pumps.

MEPS requirements for single phase and three phase air conditioners have been revised and are documented in the revised edition of AS/NZS 3823.2 which was published on 30 December 2005. The changes include accelerated MEPS levels for many non-ducted single phase models effective from April 2006. Other single phase and all three phase types have revised MEPS levels effective from October 2007. The details are set out in the latest Regulatory Impact Statement. The details included below are from the 2005 edition of the standard.

### Special E3 Notices Regarding this Product:

- Stakeholder Letter -  Notice of increased levels Minimum Energy Performance Standards for Air Conditioners - 1 October 2007

- Stakeholder Letter -  Postponement of the Proposed 2008 MEPS Levels for Air Conditioners - May 2007

- The   letter dated 19 January 2006 confirms that MCE have now agreed to the proposal for MEPS to commence on 1 April 2006.

- Notice of early adoption of increased MEPS for single phase airconditioners from 1 April 2006.   Letter from the NAEEEC Chair outlines the proposal under consideration by Energy Ministers for the revised MEPS date for non-ducted single phase air conditioners less than 7.5kW for household use.

Products manufactured or imported must comply with the relevant requirements and MEPS levels at the time of manufacture or importation. Once registered to these requirements, products manufactured or imported prior to any new MEPS date can continue to be sold after new MEPS levels come into force. Details related to stock that has been imported into Australia or manufactured in Australia before 1st April 2006 are set out in the Administrative Guidelines (refer to MEPS transition arrangements, clause 3.2.2).

Test procedures for single and three phase units are the same: see requirements for air conditioners.

The following air conditioner types are exempted from energy labelling and MEPS:

- Close controlled airconditioners (for computer rooms), pending the development of MEPS levels for these products. (refer to AS/NZS3823.2-2005 for a precise definition of these products)

- Multi-split systems (i.e., those having more than one indoor unit with more than one independent indoor controls) pending the development of a suitable test method.

- Evaporative coolers or any other cooling systems which are not of the vapour compression type.

- Ground water or ground loop sourced heat pumps within the Scope of ISO 13256-1. Note that water loop heat pumps are not exempt and these are covered by AS/NZS 3823.1.3.

- Unbalanced airconditioners and spot coolers (these include many portable types).

- Commercial chillers which are used in commercial buildings to provide chilled-water for space cooling equipment.

- Models that have been granted exemption by the relevant Australian/New Zealand regulatory authority due to their specific design for applications such as process heating or cooling or for heating or cooling of spaces to conditions that are not intended for human comfort.

The Minimum Energy Performance Standards requirements are set out as minimum EER levels. These are set out in the table below.

### MEPS: MINIMUM EER FOR AIR COOLED CONDENSER AIRCONDITIONERS (from AS/NZS 3823.2-2005)

| Configuration | Cooling only or reverse cycle | Phase | Rated cooling capacity (kW) | Min EER 1 Oct 2001 | Min EER 1 Oct 2004 | Min EER 1 Apr 2006 | Min EER 1 Oct 2007 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Non ducted unitary (window/wall> | C/O | 1 | < 7.5 | n/a | 2.45 | 2.75b | 2.75 |
|  | C/O | 1 | 7.5 to <10 | n/a | 2.45 | 2.45 | 2.75 |
|  | C/O | 1 | ≥ 10.0a | n/a | 2.45 | 2.45 | 2.75 |
|  | R/C | 1 | < 7.5 | n/a | 2.30 | 2.75b | 2.75 |
|  | R/C | 1 | 7.5 to < 10 | n/a | 2.30 | 2.30 | 2.75 |
|  | R/C | 1 | ≥ 10.0a | n/a | 2.30 | 2.30 | 2.75 |
|  | Both | 3 | < 10 | 2.25 | 2.25 | 2.25 | 2.75 |
|  | Both | 3 | 10 to 12.5 | 2.30 | 2.30 | 2.30 | 2.75 |
|  | Both | 3 | 12.6 to 15.5 | 2.35 | 2.35 | 2.35 | 2.75 |
|  | Both | 3 | 15.6 to 18 | 2.40 | 2.40 | 2.40 | 2.75 |
|  | Both | 3 | 18.1 to 18.9c | 2.45 | 2.45 | 2.45 | 2.75 |
| Non ducted split | C/O | 1 | < 4 | n/a | 2.45 | 3.05b | 3.05 |
|  | C/O | 1 | 4 to < 7.5 | n/a | 2.45 | 2.75b | 2.75 |
|  | C/O | 1 | 7.5 to < 10 | n/a | 2.45 | 2.45 | 2.75 |
|  | C/O | 1 | ≥ 10.0a | n/a | 2.45 | 2.45 | 2.75 |
|  | R/C | 1 | < 4 | n/a | 2.30 | 3.05b | 3.05 |
|  | R/C | 1 | 4 to < 7.5 | n/a | 2.30 | 2.75b | 2.75 |
|  | R/C | 1 | 7.5 to < 10 | n/a | 2.30 | 2.30 | 2.75 |
|  | R/C | 1 | ≥ 10.0a | n/a | 2.30 | 2.30 | 2.75 |
|  | Both | 3 | < 4 | 2.25 | 2.25 | 2.25 | 3.05 |
|  | Both | 3 | 4 to < 7.5 | 2.25 | 2.25 | 2.25 | 2.75 |
|  | Both | 3 | 7.5 to < 10 | 2.25 | 2.25 | 2.25 | 2.75 |
|  | Both | 3 | 10 - 12.5 | 2.30 | 2.30 | 2.30 | 2.75 |
|  | Both | 3 | 12.6 - 15.5 | 2.35 | 2.35 | 2.35 | 2.75 |
|  | Both | 3 | 15.6 - 18 | 2.40 | 2.40 | 2.40 | 2.75 |
|  | Both | 3 | 18.1 - 18.9 | 2.45 | 2.45 | 2.45 | 2.75 |
|  | Both | 3 | 19 - 25 | 2.45 | 2.45 | 2.45 | 3.05 |
|  | Both | 3 | 25.1 - 30 | 2.50 | 2.50 | 2.50 | 3.05 |
|  | Both | 3 | 30.1 - 37.5 | 2.55 | 2.55 | 2.55 | 3.05 |
|  | Both | 3 | 37.6 - 39 | 2.60 | 2.60 | 2.60 | 3.05 |
|  | Both | 3 | 39.1 - 45.5 | 2.60 | 2.60 | 2.60 | 2.75 |
|  | Both | 3 | 45.6 - 65 | 2.65 | 2.65 | 2.65 | 2.75 |
| Ducted (split & unitary) | C/O | 1 | 0 to < 10 | n/a | 2.45 | 2.45 | 2.50 |
|  | C/O | 1 | ≥ 10.0a | n/a | 2.45 | 2.45 | 2.50 |
|  | R/C | 1 | 0 to < 10 | n/a | 2.30 | 2.30 | 2.50 |
|  | R/C | 1 | ≥ 10.0a | n/a | 2.30 | 2.30 | 2.50 |
|  | Both | 3 | 0 to < 10 | 2.25 | 2.25 | 2.25 | 2.50 |
|  | Both | 3 | 10 - 12.5 | 2.30 | 2.30 | 2.30 | 2.75 |
|  | Both | 3 | 12.6 - 15.5 | 2.35 | 2.35 | 2.35 | 2.75 |
|  | Both | 3 | 15.6 - 18 | 2.40 | 2.40 | 2.40 | 2.75 |
|  | Both | 3 | 18.1 - 18.9 | 2.45 | 2.45 | 2.45 | 2.75 |
|  | Both | 3 | 19 - 25 | 2.45 | 2.45 | 2.45 | 3.05 |
|  | Both | 3 | 25.1 - 30 | 2.50 | 2.50 | 2.50 | 3.05 |
|  | Both | 3 | 30.1 - 37.5 | 2.55 | 2.55 | 2.55 | 3.05 |
|  | Both | 3 | 37.6 - 39 | 2.60 | 2.60 | 2.60 | 3.05 |
|  | Both | 3 | 39.1 - 45.5 | 2.60 | 2.60 | 2.60 | 2.75 |
|  | Both | 3 | 45.6 - 65 | 2.65 | 2.65 | 2.65 | 2.75 |
| NOTES: a Amendment 2 b Increased MEPS levels for 1 April 2006 apply to any product marketed for household use. Products which are purely commercial are subjected to the 2006 MEPS levels indicated apply from 1 October 2007. MEPS for 2007 apply to all product types irrespective of use. c For three phase non-ducted unitary products > 18.9kW, MEPS are the same as for non-ducted-split systems. 1. AS/NZS 3823.2 defines terms and applicable dates used in this table. 2. Not applicable is indicated by n/a - no coverage of this product type from the date specified. 3. Amendment 3 (to be published in late 2007) deletes the published 2008 MEPS levels (not shown above). These MEPS levels (with any adjustments) are expected to be included in a revision of AS/NZS3823.2 in 2008 with a delay of 12 to 18 months - see stakeholder letter dated May 2007 above. |  |  |  |  |  |  |  |

### MEPS: MINIMUM EER FOR WATER COOLED CONDENSER AIRCONDITIONERS (from AS/NZS 3823.2-2005)

| Supply (No. of phases) | Cooling capacity/Type | Oct. 2001 (see Note 2) | Oct. 2004 | Oct. 2007 |
| --- | --- | --- | --- | --- |
| 1 | All capacities-Cooling only (ducted and non-ducted) | N/A | 2.45 | 3.50 |
|  | All capacities-Reverse cycle (ducted and non-ducted) | N/A | 2.30 | 3.50 |
| 3 | < 10 kW | 2.25 | 2.25 | 3.50 |
|  | 10-12.5 kW | 2.30 | 2.30 | 3.50 |
|  | 12.6-15.5 kW | 2.35 | 2.35 | 3.50 |
|  | 15.6-18.0 kW | 2.40 | 2.40 | 3.50 |
|  | 18.1-18.9 kW | 2.45 | 2.45 | 3.50 |
|  | 19.0-25.0 kW | 2.45 | 2.45 | 3.50 |
|  | 25.1-30.0 kW | 2.50 | 2.50 | 3.50 |
|  | 30.1-37.5 kW | 2.55 | 2.55 | 3.50 |
|  | 37.6-39.0 kW | 2.60 | 2.60 | 3.50 |
|  | 39.0-45.5 kW | 2.60 | 2.60 | 3.20 |
|  | 45.6-65.0 kW | 2.65 | 2.65 | 3.20 |
| NOTES: 1. This table includes requirements for airconditioners with water cooled condensers and water sourced heat pumps. 2. In New Zealand, MEPS became mandatory on 1 July 2002. |  |  |  |  |

### FUTURE MEPS LEVELS BEYOND 2007 (CLASS A EFFICIENCY)
MINIMUM EER COOLING THREE PHASE AIRCONDITIONERS FROM 1 OCTOBER 2007 (from AS/NZS 3823.2-2005)

| Rated cooling capacity (kW) | Minimum EER cooling (W/W) |
| --- | --- |
| kW ≤ 10.0 | Under consideration |
| 10.0 < kW ≤ 19.0 | 3.35 |
| 19.0 < kW ≤ 39.0 | 3.65 |
| kW > 39.0 | 3.35 |

The rated capacity is defined under condition T1 in AS/NZS 3823.1.1 or AS/NZS3823.1.2 or AS/NZS3823.1.3, as applicable.

For MEPS purposes:

- at least one unit has to be tested (or simulated) and results supplied with the application

- physical tests in a calorimeter or using the enthalpy method are acceptable

- simulation to AS/NZS 3823.3 is an acceptable alternative to physical tests

- cooling capacity and energy consumption is determined to condition T1

- heating capacity and energy consumption is determined to condition H1 (ISO condition "high")

- for each unit tested, the measured capacity shall be > 0.95 of the rated value

- for each unit tested, the measured energy consumption shall be < 1.05 of the rated value

Note that where a product is registered for energy labelling as well as MEPS, only physical tests are permitted (ie simulation is not allowed). For products registered for energy labelling, a test report must also be submitted to show that the model is capable of meeting the maximum cooling test requirement.

Note that heating only units (with no cooling function) are not required to be registered for MEPS.

A copy of the standard can be purchased from SAI Global under licence from Standards Australia.

Find out how to make an application for energy labelling and/or MEPS.

You can also download the  MEPS 2004 Air Conditioner Registration Fact Sheet  for more information on making an application. (factsheet-ac.pdf 31kB)

# MEPS Requirements for Ballasts for Linear Fluorescent Lamps

From 1 March 2003, ballasts for linear fluoescent lamps manufactured in or imported into Australia must comply with Minimum Energy Performance (MEPS) requirements which are set out in AS/NZS 4783.2-2002.

MEPS DOES apply to the following types of ballasts:

- ferromagnetic and electronic ballasts used with linear fluorescent lamps from 15W to 70W;

- rated for 50 Hz and 230/240/250V supply (or a range that includes these);

- ballsts supplied as separate components or as part of a luminaire.

Ballasts within the scope of MEPS must also be marked with their energy efficiency by way of their Energy Efficiency Index (EEI), the details of which are also specified in AS/NZS 4783.2-2002. AS/NZS 4783.2-2002 also requires that ballasts within the scope of MEPS be designed to comply with the relevant performance requirements of IEC60921 for ferromagnetic ballasts and IEC60929 for electronic ballasts. These standards are also published by Standards Australia as AS/NZS60921 and AS/NZS60929.

MEPS DOES NOT apply to the following types of ballasts:

- primarily for use on DC supplies or batteries;

- primarily for the production of light (radiation) outside the visible spectrum;

- to exit signs within the scope of AS/NZS 2293;

- to hazardous area lighting equipment within the scope of AS/NZS 2380, AS/NZS 60079 and AS/NZS 61241.

The Minimum Energy Performance Standards (MEPS) requirements are set out as maximum allowable total circuit power when tested in accordance with AS/NZS 4783.1. These are set out in the tables below when tested to AS/NZS4783.1 (see details on ballast test methods):

Maximum allowable total circuit power is shown in Tables 1-3 of AS/NZS 4783.2 below. A copy of the standard can be purchased from SAI Global under licence from Standards Australia.

### TABLE 1
BALLASTS FOR FLUORESCENT LAMPS - EEI CLASSIFICATION FOR RATED VOLTAGE >250 V

| Lamp type and arrangement | Nominal lamp power* Watts | ILCOS code | Maximum corrected total input power, Watts |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Energy Efficiency Index (EEI) classification |  |  |  |  |  |  |
|  |  |  | A1# | A2 | A3 | B1 | B2 | C | D |
| Linear | 15 | FD-15-E-G13-26/450 | <18.0 | <16.0 | <18.0 | <21.0 | <24.0 | <25.0 | >25.0 |
|  | 18 | FD-18-E-G13-26/600 | <21.0 | <19.0 | <21.0 | <24.0 | <27.0 | <28.0 | >28.0 |
|  | 30 | FD-30-E-G13-26/895 | <33.0 | <31.0 | <33.0 | <36.0 | <39.0 | <40.0 | >40.0 |
|  | 36 | FD-36-E-G13-26/1200 | <38.0 | <36.0 | <38.0 | <41.0 | <44.0 | <45.0 | >45.0 |
|  | 38 | FD-38-E-G13-26/1047 | <40.0 | <38.0 | <40.0 | <43.0 | <46.0 | <47.0 | >47.0 |
|  | 58 | FD-58-E-G13-26/1500 | <59.0 | <55.0 | <59.0 | <64.0 | <68.0 | <70.0 | >70.0 |
|  | 70 | FD-70-E-G13-26/1800 | <72.0 | <68.0 | <72.0 | <77.0 | <81.0 | <83.0 | >83.0 |
| NOTES: 1. Refer to AS/NZS 61231, International lamp coding system (ILCOS) 2. Applies only to mains frequency ferromagnetic ballasts with two-wire connection and with an external starter. |  |  |  |  |  |  |  |  |  |

* Nominal values shown may have different rating values. Refer to the relevant lamp data sheet.
# Refer Clause 5.3.2

### TABLE 2
BALLASTS FOR FLUORESCENT LAMPS - EEI CLASSIFICATION FOR RATED VOLTAGE >240 V AND < 250V

| Lamp type and arrangement | Nominal lamp power* Watts | ILCOS code | Maximum corrected total input power, Watts |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Energy Efficiency Index (EEI) classification |  |  |  |  |  |  |
|  |  |  | A1# | A2 | A3 | B1 | B2 | C | D |
| Linear | 15 | FD-15-E-G13-26/450 | <18.0 | <16.0 | <18.0 | <21.0 | <23.5 | <25.0 | >25.0 |
|  | 18 | FD-18-E-G13-26/600 | <21.0 | <19.0 | <21.0 | <24.0 | <26.5 | <28.0 | >28.0 |
|  | 30 | FD-30-E-G13-26/895 | <33.0 | <31.0 | <33.0 | <36.0 | <38.5 | <40.0 | >40.0 |
|  | 36 | FD-36-E-G13-26/1200 | <38.0 | <36.0 | <38.0 | <41.0 | <43.5 | <45.0 | >45.0 |
|  | 38 | FD-38-E-G13-26/1047 | <40.0 | <38.0 | <40.0 | <43.0 | <45.5 | <47.0 | >47.0 |
|  | 58 | FD-58-E-G13-26/1500 | <59.0 | <55.0 | <59.0 | <64.0 | <67.5 | <70.0 | >70.0 |
|  | 70 | FD-70-E-G13-26/1800 | <72.0 | <68.0 | <72.0 | <77.0 | <80.5 | <83.0 | >83.0 |
| NOTES: 1. Refer to AS/NZS 61231, International lamp coding system (ILCOS) 2. Applies only to mains frequency ferromagnetic ballasts with two-wire connection and with an external starter. |  |  |  |  |  |  |  |  |  |

* Nominal values shown may have different rating values. Refer to the relevant lamp data sheet.
# Refer Clause 5.3.2

### TABLE 3
BALLASTS FOR FLUORESCENT LAMPS - EEI CLASSIFICATION

| Lamp type and arrangement | Nominal lamp power* Watts | ILCOS code | Maximum corrected total input power, Watts |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Energy Efficiency Index (EEI) classification |  |  |  |  |  |  |
|  |  |  | A1# | A2 | A3 | B1 | B2 | C | D |
| Linear | 15 | FD-15-E-G13-26/450 | <18.0 | <16.0 | <18.0 | <21.0 | <23.5 | <25.0 | >25.0 |
|  | 18 | FD-18-E-G13-26/600 | <21.0 | <19.0 | <21.0 | <24.0 | <26.0 | <28.0 | >28.0 |
|  | 30 | FD-30-E-G13-26/895 | <33.0 | <31.0 | <33.0 | <36.0 | <38.0 | <40.0 | >40.0 |
|  | 36 | FD-36-E-G13-26/1200 | <38.0 | <36.0 | <38.0 | <41.0 | <43.0 | <45.0 | >45.0 |
|  | 38 | FD-38-E-G13-26/1047 | <40.0 | <38.0 | <40.0 | <43.0 | <45.0 | <47.0 | >47.0 |
|  | 58 | FD-58-E-G13-26/1500 | <59.0 | <55.0 | <59.0 | <64.0 | <67.0 | <70.0 | >70.0 |
|  | 70 | FD-70-E-G13-26/1800 | <72.0 | <68.0 | <72.0 | <77.0 | <80.0 | <83.0 | >83.0 |
| Compact 2 tube | 18 | FSD-18-E-2G11 | <21.0 | <19.0 | <21.0 | <24.0 | <26.0 | <28.0 | >28.0 |
|  | 24 | FSD-24-E-2G11 | <27.0 | <25.0 | <27.0 | <30.0 | <32.0 | <34.0 | >34.0 |
|  | 36 | FSD-36-E-2G11 | <38.0 | <36.0 | <38.0 | <41.0 | <43.0 | <45.0 | >45.0 |
|  | 40 | FSDH-40-L/P-2G11 | <46.0 | <44.0 | <46.0 | - | - | - | - |
|  | 55 | FSDH-55-L/P-2G11 | <63.0 | <59.0 | <63.0 | - | - | - | - |
| Compact 4 tube flat | 18 | FSS-18-E-2G10 | <21.0 | <19.0 | <21.0 | <24.0 | <26.0 | <28.0 | >28.0 |
|  | 24 | FSS-24-E-2G10 | <27.0 | <25.0 | <27.0 | <30.0 | <32.0 | <34.0 | >34.0 |
|  | 36 | FSS-36-E-2G10 | <38.0 | <36.0 | <38.0 | <41.0 | <43.0 | <45.0 | >45.0 |
| Compact 4 tube (not flat) | 10 | FSQ-10-E-G24q = 1 FSQ-10-I-G24d = 1 | <13.0 | <11.0 | <13.0 | <14.0 | <16.0 | <18.0 | >18.0 |
|  | 13 | FSQ-13-E-G24q = 1 FSQ-13-I-G24d = 1 | <16.0 | <14.0 | <16.0 | <17.0 | <19.0 | <21.0 | >21.0 |
|  | 18 | FSQ-18-E-G24q = 2 FSQ-18-I-G24d = 2 | <21.0 | <19.0 | <21.0 | <24.0 | <26.0 | <28.0 | >28.0 |
|  | 26 | FSQ-26-E-G24q = 3 FSQ-26-I-G24d = 3 | <29.0 | <27.0 | <29.0 | <32.0 | <34.0 | <36.0 | >36.0 |
| Compact 6 tube | 18 | FSM-18-I-GX24d = 2 FSM-18-E-G24q = 2 | <21.0 | <19.0 | <21.0 | <24.0 | <26.0 | <28.0 | >28.0 |
|  | 26 | FSM-26-I-GX24d = 3 FSM-13-E-G24q = 1 | <29.0 | <27.0 | <29.0 | <32.0 | <34.0 | <36.0 | >36.0 |
|  | 32 | FSSMH-32-L/P-GX24q=4 | <39.0 | <36.0 | <39.0 | - | - | - | - |
|  | 42 | FSMH-42-L/P-GX24q = 4 | <49.0 | <46.0 | <49.0 | - | - | - | - |
| Compact 2D (double D) | 10 | FSS-10-E-GR10q FSS-10-L/P/H-GR10q | <13.0 | <11.0 | <13.0 | <14.0 | <16.0 | <18.0 | >18.0 |
|  | 26 | FSS-16-I-GR8 FSS-16-E-GR10q FSS-16-L/P/H-GR10q | <19.0 | <17.0 | <19.0 | <21.0 | <23.0 | <25.0 | >25.0 |
|  | 21 | FSS-21-E-GR10q FSS-21-L/P/H-GR10q | <24.0 | <22.0 | <24.0 | <27.0 | <29.0 | <31.0 | >31.0 |
|  | 28 | FSS-28-I-GR8 FSS-28-E-GR10q FSS-28-L/P/H-GR10q | <31.0 | <29.0 | <31.0 | <34.0 | <36.0 | <38.0 | >38.0 |
|  | 38 | FSS-38-E-GR10q FSS-38-L/P/H-GR10q | <40.0 | <38.0 | <40.0 | <43.0 | <45.0 | <47.0 | >47.0 |
|  | 55 | FSS-55-E-GRY10q = 3 FSS-55-L/P/H-GRY10q = 3 | <63.0 | <59.0 | <63.0 | - | - | - | - |
| NOTE: Refer to AS/NZS 61231, International lamp coding system (ILCOS) |  |  |  |  |  |  |  |  |  |

* Nominal values shown may have different rating values. Refer to the relevant lamp data sheet.
# Refer Clause 5.3.2

# MEPS Requirements for Fluorescent Lamps

From 1 October 2004, linear fluorescent lamps manufactured in or imported into Australia must comply with Minimum Energy Performance (MEPS) requirements which are set out in AS/NZS 4782.2-2004. The scope of linear fluorescent lamps MEPS covers FD and FDH lamps ranging from 550mm to 1500mm in length (inclusive) and having a nominal lamp power of 16 Watts or more. The intention of MEPS is to improve end-use energy efficiency by eliminating lower efficiency fluorescent lamps from the market and to encourage the sale and purchase of higher efficiency fluorescent lamps.

The Minimum Energy Performance Standards (MEPS) for linear fluorescent lamps in AS/NZS 4782.2-2004 are set out as minimum luminous efficacy in lumens per Watt for various lamp sizes. There are also requirements for minimum Colour Rendering Index. The test methods for measurement luminous efficacy are set out in AS/NZS 4782.1.

These standards can be purchased from http://www.standards.com.au

Find out how to make an application for MEPS.

You can also download the  MEPS 2004 Fluorescent Lamps Registration Fact Sheet  for more information on making an application. (factsheet-fluoros.pdf 30kB)

Exclusions: MEPS in AS/NZS 4782.2-2004 does not apply to lamps that are clearly not intended for general illumination, specifically -

- lamps with a dominant colour or with an output that is predominantly outside the visible spectrum;

- lamps for colour matching and that have a colour rendering index greater than 90 and a colour appearance approximating to a point on the black body locus;

- lamps that are specifically for use in an industrial or agricultural process;

- lamps for medical applications; or

- lamps that have been given written exemption by the relevant regulatory authority on the grounds that they are for a specific purpose other than general illumination and are clearly distinguishable from lamps for general illumination.

AS/NZS 4782.2-2004 does not specify electrical safety requirements.

## MEPS Levels

When measured in accordance with AS/NZS 4782.1 the initial efficacy (at 100 hours) and the maintained efficacy (at 5000 hours) shall exceed the values specified in the table below. Lamps shall also have a Colour Rendering Index which exceeds the value in the table below.

### MEPS: MINIMUM LAMP EFFICACY AND COLOUR RENDERING REQUIREMENTS

| Lamp nominal length L (mm) mandatory | 550 < L < 700 | 700 < L < 1150 | 1150 < L < 1350 | 1350 < L < 1500 |
| --- | --- | --- | --- | --- |
| Lamp typical power (watts) (informative) | 16 - 24 | 17 - 40 | 28 - 50 | 35 - 80 |
| Initial efficacy Maintained efficacy | F 100 > 66.0 and FM > 57.5 | F 100 > 74.0 and FM > 61.0 | F 100 > 80.0 and FM > 70.0 | F 100 > 85.0 and FM > 70.0 |
| Minimum CRI | 69 | 69 | 69 | 69 |

# MEPS Requirements for Distribution Transformers

From 1 October 2004, distribution transformers manufactured in or imported into Australia must comply with Minimum Energy Performance (MEPS) requirements which are set out in AS 2374.1.2-2003. The scope of transformer MEPS covers oil-immersed and dry-type distribution transformers with power ratings from 10 kVA to 2500 kVA intended to be used on 11 kV and 22 kV networks. The intention of MEPS is to increase energy efficiency by eliminating low efficiency transformers from the market and to encourage the use of high efficiency transformers.

The standard also defines minimum efficiency levels for "High Power Efficiency Transformers". Only products which meet the specified efficiency levels can apply this term to promotional or advertising materials.

Transformers within the scope of MEPS are required to have on their rating plate a statement that indicates compliance with AS 2374.1.2.

The Minimum Energy Performance Standards (MEPS) for distribution transformers are set out as power efficiency levels at 50% of rated load in AS 2374.1.2 when tested in accordance with AS 2374.1 or AS 2735, as applicable.

A copy of these standards can be purchased from SAI Global under licence from Standards Australia.

MEPS does not apply to the following types of transformers:

- transformers other than those on 11 or 22 kv networks;

- instrument transformers;

- auto transformers;

- traction transformers mounted on rolling stock;

- starting transformers;

- testing transformers;

- welding transformers;

- three phase transformers with three or more windings per phase;

- arc-furnace transformers;

- earthing transformers;

- rectifier or converter transformers;

- uninterruptible power supply (ups) transformers;

- transformers with an impedance less than 3% or more than 8%;

- voltage regulating transformers;

- transformers designed for frequencies other than 50 hertz;

- gas-filled dry-type transformers; or

- flameproof transformers.

## MEPS Levels

MEPS levels, set out as minimum power efficiency levels at 50% of rated load for various transformer types, are set out below. Reference should be made to AS 2374.1.2-2003 for detailed conditions and test methods.

### MINIMUM POWER EFFICIENCY LEVELS FOR OIL-IMMERSED TRANSFORMERS

| Type | kVA | Power efficiency @ 50% load |
| --- | --- | --- |
| Single phase(and SWER) | 10 | 98.30 |
|  | 16 | 98.52 |
|  | 25 | 98.70 |
|  | 50 | 98.90 |
| Three phase | 25 | 98.28 |
|  | 63 | 98.62 |
|  | 100 | 98.76 |
|  | 200 | 98.94 |
|  | 315 | 99.04 |
|  | 500 | 99.13 |
|  | 750 | 99.21 |
|  | 1000 | 99.27 |
|  | 1500 | 99.35 |
|  | 2000 | 99.39 |
|  | 2500 | 99.40 |

NOTE: For intermediate power ratings the power efficiency level shall be calculated by linear interpolation.

### MINIMUM POWER EFFICIENCY LEVELS FOR DRY-TYPE TRANSFORMERS

| Type | kVA | Power efficiency @ 50% load |  |
| --- | --- | --- | --- |
|  |  | Um=12kV | Um=24kV |
| Single phase(and SWER) | 10 | 97.29 | 97.01 |
|  | 16 | 97.60 | 97.27 |
|  | 25 | 97.89 | 97.53 |
|  | 50 | 98.31 | 97.91 |
| Three phase | 25 | 97.17 | 97.17 |
|  | 63 | 97.78 | 97.78 |
|  | 100 | 98.07 | 98.07 |
|  | 200 | 98.46 | 98.42 |
|  | 315 | 98.67 | 98.59 |
|  | 500 | 98.84 | 98.74 |
|  | 750 | 98.96 | 98.85 |
|  | 1000 | 99.03 | 98.92 |
|  | 1500 | 99.12 | 99.01 |
|  | 2000 | 99.16 | 99.06 |
|  | 2500 | 99.19 | 99.09 |

Note: For intermediate power ratings the power efficiency level shall be calculated by linear interpolation.

## High Power Efficiency Levels

Minimum efficiency levels for "High Power Efficiency Transformers", set out as minimum power efficiency levels at 50% of rated load for various transformer types, are set out below. Reference should be made to AS 2374.1.2-2003 for detailed conditions and test methods.

### HIGH POWER EFFICIENCY LEVELS FOR OIL-IMMERSED TRANSFORMERS

| Type | kVA | Power efficiency @ 50% load |
| --- | --- | --- |
| Single phase(and SWER) | 10 | 98.42 |
|  | 16 | 98.64 |
|  | 25 | 98.80 |
|  | 50 | 99.00 |
| Three phase | 25 | 98.50 |
|  | 63 | 98.82 |
|  | 100 | 99.00 |
|  | 200 | 99.11 |
|  | 315 | 99.19 |
|  | 500 | 99.26 |
|  | 750 | 99.32 |
|  | 1000 | 99.37 |
|  | 1500 | 99.44 |
|  | 2000 | 99.49 |
|  | 2500 | 99.50 |

Note: For intermediate power ratings the power efficiency level shall be calculated by linear interpolation.

### HIGH POWER EFFICIENCY LEVELS FOR DRY-TYPE TRANSFORMERS

| Type | kVA | Power efficiency @ 50% load |  |
| --- | --- | --- | --- |
|  |  | Um=12kV | Um=24kV |
| Single phase(and SWER) | 10 | 97.53 | 97.32 |
|  | 16 | 97.83 | 97.55 |
|  | 25 | 98.11 | 97.78 |
|  | 50 | 98.50 | 98.10 |
| Three phase | 25 | 97.42 | 97.42 |
|  | 63 | 98.01 | 98.01 |
|  | 100 | 98.28 | 98.28 |
|  | 200 | 98.64 | 98.60 |
|  | 315 | 98.82 | 98.74 |
|  | 500 | 98.97 | 98.87 |
|  | 750 | 99.08 | 98.98 |
|  | 1000 | 99.14 | 98.04 |
|  | 1500 | 99.21 | 99.12 |
|  | 2000 | 99.24 | 99.17 |
|  | 2500 | 99.27 | 99.20 |

Note: For intermediate power ratings the power efficiency level shall be calculated by linear interpolation.

Find out how to make an application for MEPS.

You can also download the  MEPS 2004 Distribution Transformers Registration Fact Sheet  for more information on making an application. (factsheet-trans.pdf 29kB)

This page last modified 09 May 2007

# MEPS Requirements for Commercial Refrigeration

From 1 October 2004, refrigerated display cabinets manufactured in or imported into Australia must comply with Minimum Energy Performance (MEPS) requirements which are set out in AS 1731.14-2003. The scope of commercial refrigeration MEPS includes both remote and self-contained refrigerated display cabinets primarily used in commercial applications for the storage of frozen and unfrozen food.

The standard also defines minimum efficiency levels for "High Efficiency" refrigerated display cabinets. Only products which meet the specified efficiency levels can apply this term to promotional or advertising materials.

The Minimum Energy Performance Standards (MEPS) for commercial refrigeration are set out in AS 1731.14-2003 as total energy consumption per total display area (TEC/TDA) in kWh/day/square metre for various unit types. The test procedures for commercial refrigeration are the specified parts AS 1731.

A copy of these standards can be purchased from SAI Global under licence from Standards Australia.

AS1731 or MEPS does not apply to refrigerated vending machines, cabinets intended for use in catering and similar non-retail applications. However, MEPS for vending machines is under consideration.

## MEPS Levels

When measured in accordance with AS 1731.9 and AS 1731.12 the energy consumption of a remote or self contained refrigerated cabinet shall not exceed that specified below (as applicable). For the purpose of testing compliance, tests shall be conducted under climate Class 3 conditions, with lighting and anti-sweat heaters running for the duration of test period, unless controlled by a time-clock, smart sensor or similar automatic device. Where night-covers are supplied as a permanent fixture of the cabinet, the test shall be conducted as described in AS 1731.9, Section 4. Reference should be made to the relevant parts of AS 1731 for detailed conditions and test methods.

### MEPS: MAXIMUM ENERGY CONSUMPTION - REMOTE CABINETS

| Type | Maximum energy consumption TEC/TDA (kWh/day/m2) |
| --- | --- |
| RS 1 - Unlit shelves | 12.55 |
| RS 1 - Lit shelves | 17.76 |
| RS 2 - Unlit shelves | 12.73 |
| RS 2 - Lit shelves | 16.98 |
| RS 3 - Unlit shelves | 14.84 |
| RS 3 - Lit shelves | 18.39 |
| RS 4 - Solid door | no value |
| RS 4 - Glass door | 9.73 |
| RS 5 - Solid door | no value |
| RS 5 - Glass door | no value |
| RS 6 - Gravity coil | 14.21 |
| RS 6 - Fan coil | 14.16 |
| RS 7 - Gravity coil | no value |
| RS 7 - Fan coil | 14.79 |
| RS 8 - Gravity coil | 12.25 |
| RS 8 - Fan coil | 13.19 |
| RS 9 - Gravity coil | no value |
| RS 9 - Fan coil | 12.09 |
| RS 10 - High | no value |
| RS 10 - Medium | no value |
| RS 10 - Low | 18.67 |
| RS 11 | 38.13 |
| RS 12 | 66.33 |
| RS 13 - Solid sided | 19.48 |
| RS 13 - Glass sided | 19.58 |
| RS 14 - Solid sided | 15.49 |
| RS 14 - Glass sided | 37.08 |
| RS 15 - Solid door | no value |
| RS 15 - Glass door | 37.08 |
| RS 16 - Solid door | no value |
| RS 16 - Glass door | 40.56 |
| RS 17 - Solid door | no value |
| RS 17 - Glass door | no value |
| RS 18 | 48.58 |
| RS 19 | 36.15 |
| RS 20 | no value |

### MEPS: MAXIMUM ENERGY CONSUMPTION - SELF-CONTAINED CABINETS

| Maximum energy consumption TEC/TDA (kWh/day/m2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type |  |  |  |  |  |
|  | M-package temperature classes (See AS1731.6 Clause 5) |  | Type | M-package temperature classes (See AS1731.6 Clause 5) |  |
|  | M1 | M2 |  | L1 | L2 |
| HC1 | 11.50 | 11.50 | HF1 | no value | no value |
| HC2 | no value | no value | HF2 | no value | no value |
| HC3 | no value | no value | HF3 | no value | no value |
| HC4 | 15.50 | 15.50 | HF4 | 26.50 | 26.50 |
| HC5 | no value | no value | HF5 | no value | no value |
| HC6 | no value | no value | HF6 | 8.00 | 8.00 |
| VC1 | 37.50 | 28.00 | VF1 | no value | no value |
| VC2 | 27.00 | 25.50 | VF2 | no value | no value |
| VC3 | no value | no value | VF3 | no value | no value |
| VC4 (a) Solid door (b) Glass door | 17.00 17.00 | 17.50 17.50 | VF4 (a) Solid door (b) Glass door | 44.00 44.00 | 39.00 39.00 |
| YC1 | no value | no value | YF1 | no value | no value |
| YC2 | no value | no value | YF2 | no value | no value |
| YC3 | no value | no value | YF3 | no value | no value |
| YC4 | no value | no value | YF4 | no value | no value |

## High Efficiency Levels

A remote or self-contained refrigerated display cabinet may be designated 'high efficiency only if its energy consumption, measured in accordance with AS 1731.9 and AS 1731.12 does not exceed that value specified below. For the purpose of testing compliance, tests shall be conducted under climate Class 3 conditions, with lighting and anti-sweat heaters running for the duration of test period, unless controlled by a time-clock, smart sensor or similar automatic device. Where night-covers are supplied as a permanent fixture of the cabinet, the test shall be conducted as described in AS 1731.9, Section 4. Reference should be made to the relevant parts of AS 1731 for detailed conditions and test methods.

### MAXIMUM ENERGY CONSUMPTION FOR 'HIGH EFFICIENCY' REMOTE DISPLAY CABINETS

| Type | Maximum energy consumption TEC/TDA (kWh/day/m2) |
| --- | --- |
| RS 1 - Unlit shelves | 8.37 |
| RS 1 - Lit shelves | 10.66 |
| RS 2 - Unlit shelves | 8.49 |
| RS 2 - Lit shelves | 11.32 |
| RS 3 - Unlit shelves | 10.32 |
| RS 3 - Lit shelves | 12.26 |
| RS 4 - Solid door | no value |
| RS 4 - Glass door | 6.77 |
| RS 5 - Solid door | no value |
| RS 5 - Glass door | no value |
| RS 6 - Gravity coil | 9.88 |
| RS 6 - Fan coil | 9.85 |
| RS 7 - Gravity coil | no value |
| RS 7 - Fan coil | 9.86 |
| RS 8 - Gravity coil | 8.52 |
| RS 8 - Fan coil | 9.17 |
| RS 9 - Gravity coil | no value |
| RS 9 - Fan coil | 8.06 |
| RS 10 - High | no value |
| RS 10 - Medium | no value |
| RS 10 - Low | 12.99 |
| RS 11 | 26.52 |
| RS 12 | 46.14 |
| RS 13 - Solid sided | 12.99 |
| RS 13 - Glass sided | 13.62 |
| RS 14 - Solid sided | 11.45 |
| RS 14 - Glass sided | 12.86 |
| RS 15 - Solid door | no value |
| RS 15 - Glass door | 27.41 |
| RS 16 - Solid door | no value |
| RS 16 - Glass door | 29.98 |
| RS 17 - Solid door | no value |
| RS 17 - Glass door | no value |
| RS 18 | 39.75 |
| RS 19 | 29.57 |
| RS 20 | no value |

### MAXIMUM ENERGY CONSUMPTION FOR 'HIGH EFFICIENCY' SELF-CONTAINED DISPLAY CABINETS

| Maximum energy consumption TEC/TDA (kWh/day/m2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type |  |  |  |  |  |
|  | M-package temperature classes |  | Type | M-package temperature classes |  |
|  | M1 | M2 |  | L1 | L2 |
| HC1 | 8.50 | 8.50 | HF1 | no value | no value |
| HC2 | no value | no value | HF2 | no value | no value |
| HC3 | no value | no value | HF3 | no value | no value |
| HC4 | 11.40 | 11.40 | HF4 | 19.50 | 19.50 |
| HC5 | no value | no value | HF5 | no value | no value |
| HC6 | no value | no value | HF6 | 5.90 | 5.90 |
| VC1 | 27.60 | 20.60 | VF1 | no value | no value |
| VC2 | 19.90 | 18.80 | VF2 | no value | no value |
| VC3 | no value | no value | VF3 | no value | no value |
| VC4 (a) Solid Door (b) Glass Door | 7.30 10.70 | 7.30 10.70 | VF4 (a) Solid door (b) Glass Door | 32.40 32.40 | 28.70 28.70 |
| YC1 | no value | no value | YF1 | no value | no value |
| YC2 | no value | no value | YF2 | no value | no value |
| YC3 | no value | no value | YF3 | no value | no value |

You can also download the  MEPS 2004 Commercial Refrigeration Registration Fact Sheet  for more information on making an application. (factsheet-commrefrig.pdf 26kB)

# Overview of Regulatory Requirements - Labelling and MEPS

It is currently mandatory for all of the following electrical products offered for sale in Australia to carry an approved energy label:

- refrigerators and freezers

- clothes washers

- clothes dryers

- dishwashers

- air conditioners (single phase mandatory, three phase voluntary)

The following products are also regulated on the basis of Minimum Energy Performance Standards (MEPS) - this means that they have regulated minimum energy efficiency levels:

- refrigerators and freezers (from 1 October 1999, revision 1 January 2005)

- mains pressure electric storage water heaters (from 1 October 1999)

- small mains pressure electric storage water heaters (<80L) and low pressure and heat exchanger types (from 1 October 2005) (Refer to   AGO letter regarding levels and test methods)

- three phase electric motors (0.73kW to <185kW) (from 1 October 2001, revision April 2006)

- single phase air conditioners (from 1 October 2004, revision 1 April 2006 and 2007 and 2008)

- three phase air conditioners up to 65kW cooling capacity (from 1 October 2001, revision 1 October 2007)

- ballasts for linear fluorescent lamps (from 1 March 2003). Note that in addition to MEPS, ballasts also have to be marked with an energy efficiency index (EEI).

- linear fluorescent lamps - from 550mm to 1500mm inclusive with a nominal lamp power >16W (from 1 October 2004)

- distribution transformers - 11kV and 22kV with a rating from 10kA to 2.5MVA (from 1 October 2004)

- commercial refrigeration (self contained and remote systems) (from 1 October 2004)

Details of the relevant standards are available on this web site.

The following products are proposed for regulation in the future, subject to normal regulatory processes and approval by the Ministerial Council on Energy.

- external power supplies (from 1 December 2008), New Zealand (April 2009);

- Set top boxes (from 1 December 2008), New Zealand (April 2009);

- Televisions;

- Standby;

- Boiling and chilled water dispensers;

- Vending machines;

- Commercial icemakers;

- A range of lamp types;

Find out how to make an application for an energy label and/or MEPS.

Regulations specify the general requirements for the energy labelling of appliances, including offences and penalties if a party does not comply with the requirements. Technical requirements for energy labelling are set out in "Part 2" of the relevant appliance standard, which is referenced in state regulations. State based legislation is necessary because the Australian constitution gives Australian States clear responsibility for resource management issues, including energy. More information on regulations and relevant contacts is available on this web site. (Note that regulatory standards are generally Part number 2 of the relevant standard, but this does vary by product).

A number of other products are being considered for energy labelling and MEPS programs (or will have existing levels revised).

Details are available on this web site as details become available - see what's new.

This web site does not cover gas appliances or safety but there are links to for further information.

# Regulatory requirements for energy labelling and MEPS

The   MEPS and Energy Labeling Process document sets out the process for the development of MEPS levels in Australia and New Zealand.

## State contacts

State regulations for energy labelling and Minimum Energy Performance Standards (MEPS) typically include a range of provisions and requirements which are summarised below. It should be noted that the exact requirements and penalties for any offences should be checked with the relevant state authority: information provided here is only a guide. Currently, energy labelling applications for an approved energy label are accepted by New South Wales, South Australia, Victoria and Queensland. A registration for energy labelling in any of these states are accepted as valid in all Australian states and territories.

All energy labels must be approved by a regulatory authority: All products within the scope of energy labelling and/or MEPS must be registered. Applications for energy label registration and MEPS compliance are to include:

- test reports or data to the relevant standard (the number of units to be tested varies - see particular requirements by product);

- demonstration that the relevant performance requirements have been met by the model in addition to the measurement of energy consumption;

- a sample label (where applicable);

- the product meets the MEPS requirements (where applicable);

- the prescribed fee.

Cancellation of the label or MEPS approval: the regulatory authority can cancel the registration where there are reasonable grounds for doing so.

Offences: The following offences are typically contained in state regulations.

- person must not offer to supply (eg sell) a "specified" article which is not registered for energy labelling or MEPS (as applicable), or where the registration has been cancelled;

- energy label must not be obscured;

- other information must not be shown near the label that conflicts with data on the energy label;

- making a false or misleading declaration;

- registration holders must notify the regulatory authority of any change of contact details;

- registration holders must supply a sample for testing on the request of the regulatory authority;

- registration holders may be liable for costs of check testing by the regulatory authority if tests show that the model does not comply.

## State Regulatory Contacts

State contacts for energy labelling and Minimum Energy Performance Standards applications are:

NSW
Energy Labelling/MEPS Section
NSW Department of Water and Energy
Level 17, 227 Elizabeth Street
Sydney NSW 2000
(GPO Box 3889, Sydney NSW 2001)
Tel 02 8281 7706
Fax 02 8281 7750
Email: energylabelling@dwe.nsw.gov.au
Website: www.dwe.nsw.gov.au

Queensland
Electrical Safety Office
Department of Industrial Relations
LMB 2234
BRISBANE QLD 4001
Tel +61 7 3237 0280
Fax +61 7 3406 3808
Email: equipmentsafety@dir.qld.gov.au
Website: www.eso.qld.gov.au

South Australia
Energy Labelling Section
Office of the Technical Regulator
Level 8, 11 Waymouth St
Adelaide SA 5000
Postal address: PO Box 1 Walkerville SA 5081 
Tel 08 8226 5530
Fax 08 8226 5529
Email: robert.anderson@saugov.sa.gov.au
Website: www.technicalregulator.sa.gov.au

Victoria
Energy Labelling Section
Energy Safe Victoria
PO Box 262
Collins Street West VIC 8007
Tel 03 9203 9700
Fax 03 9686 2197
Email: energyefficiency@esv.vic.gov.au
Website: www.esv.vic.gov.au

New Zealand
Manager Products - Product Regulation
Energy Efficiency and Conservation Authority
PO Box 388
WELLINGTON NEW ZEALAND
Tel +64 4 470 2238
Fax +64 4 499 5330
Email: terry.collins@eeca.govt.nz
Website: www.eeca.govt.nz

If you are calling from outside of Australia, delete the initial 0 and replace with +61

## Energy Labelling and MEPS Legislation and Regulations

New South Wales
Energy and Utilities Administration Regulation 2006 made under the Electricity (Consumer Safety) Act 2004 . See Australasian Legal Information Institute (UTS) for an online version of the regulations.

Queensland
Electricity Regulation 1994 made under the Electricity Act 1994. See Chapter 6 of the regulations (Energy Efficiency and Performance of Electrical Equipment). An electronic copy can be obtained from the Queensland Office of the Parliamentary Counsel (look under Legislation starting with letter "E").

South Australia
Energy labelling regulations in South Australia are called Electrical Products Regulations 2001 under the Electrical Products Act 2000. See South Australian Attorney-General's Department for more details (the Act and Regulations can be viewed online under the alphabetical listings).

Victoria
Electricity Safety (Equipment Efficiency) Regulations 1999, regulation No.48/1999 under the Electricity Safety Act 1998. The ESV web site has links to all current safety and energy efficiency Acts and regulations.

This page last modified 13 May 2008

# Legal obligations for retailers

State regulations for energy labelling and MEPS primarily control the supply of appliances - the supply of appliances mainly occurs through retails outlets. So as a retailer it is in your interests to understand your obligations under the law.

The regulations set out the legal requirements in detail, but the requirements are quite straight forward. In summary, the main requirements include:

- Retailers are obliged to ensure that each product which is "prescribed" under energy efficiency laws and is offered for sale has an approved energy label affixed or it has been approved for MEPS (as applicable);

- The label must not be obscured when the product is on display;

- Conflicting information must not be shown with or near the label.

More details on typical regulatory requirements are available on this site, although the full requirements for each state should be determined from the relevant state or territory regulations

# Electrical Safety Requirements in Australia

A number of products, including most household appliances, are "prescribed" electrical articles in Australia and therefore must be approved to the relevant electrical safety approval and test specification before they can be sold in Australia. All Australian state and territory governments accept applications for electrical safety approvals. Further information can be obtained from state electrical safety regulators or through the Electrical Regulatory Authorities Council (ERAC).

# Requirements for Gas Space Heaters, Gas Water Heaters and Gas Cookers

Energy labels can be found on gas space heaters (AS4553 and AS4556) and gas water heaters (both storage and instantaneous) (AS4552). Gas energy labels are similar in format to those found on electrical appliances, except they are blue in colour and with annual energy shown in MJ. The gas labelling program is currently an industry voluntary scheme that was once managed by the Australian Gas Association (AGA). A review of the scheme is under way by the Gas Industry and Governments.

Note that the AGA is no longer responsible for publishing standards. This organisation now provides accredited product certfication services with respect to the gas, electrical and plumbing sectors within Australia. More information about product certification for gas products is available at:

- http://www.aga.asn.au (Australian Gas Association)

- http://www.saiglobal.com (SAI Global)

Energy tests, label requirements and relevant performance requirements are specified in following standards:

- AS 4551 (AG 101) Domestic Gas Cooking Appliances

- AS 4552 (AG 102) Gas Water Heaters

- AS 4553 (AG 103) Gas Space Heating Appliances

- AS 4554 (AG 104) Gas Laundry Dryers

- AS 4555 (AG 105) Domestic Gas Refrigerators

- AS 4556 (AG 106) Indirect gas-fired ducted air heater

- AS 4557 (AG 107) Domestic Outdoor Gas Barbecues

- AS 4558 (AG 108) Decorative gas log fires and other fuel effect appliances

These standards can be purchased from SAI Global webshop.

A trial web site listing gas water heaters available in Australia is also available.

## Latest News

For those stakeholders interested in accessing background information about the review of AS 4552 (Gas Water Heaters) through Standards Australia's committee process, recent presentations and links to key documents are available at  www.energyrating.gov.au/2007-regplans-wh.html

## Strategy and Work Program for Gas Appliances

The main objective of the Switch on Gas ten year strategic plan is to implement a nationally consistent regulation scheme for energy efficiency of gas appliances and equipment. This strategy is an important part of the package of measures being implemented by the MCE under the National Framework for Energy Efficiency (NFEE).

By progressively increasing the energy efficiency of gas appliances and equipment beyond business as usual levels, Switch on Gas responds to the evolving needs of the community by providing world class energy efficient gas products. Switch on Gas will also make a contribution to national efforts to reduce greenhouse gas emissions. Natural gas currently supplies about 30% of total household energy in Australia. Within 20 years it is projected that Switch on Gas has the potential to reduce the expenditure on natural gas of Australian consumers by up to $115 million per annum and consumption by over 5% below business as usual.

The Australian and New Zealand Equipment Energy Efficiency Program released its inaugural work plan for Switch on Gas, in April 2005 which outlined products targeted for potential regulation in both countries for the three year period, 2005/06 to 2007/08. An updated version of this work plan was released for comment in October 2006 (2006/12) and is a consequence of:

- The administrative and regulatory basis of the gas appliance efficiency programme;

- New information about gas appliance technology and testing;

- Gas consumer response to energy labels.

The final version of this revised document will be published by July 2007. This document is the first in a series of rolling plans.

## Changes to the Gas Labelling and MEPS Scheme for Appliances

In 2002, the Sustainable Energy Authority Victoria (SEAV) (now Sustainability Victoria (SV)), the Victorian Office of Gas Safety (OGS) and the Australian Greenhouse Office (AGO) commenced working together to review the gas appliance labelling and MEPS scheme, and to explore options for enhancing its effectiveness at driving energy efficiency improvements. The first step in this process was to commission a consultant study on the gas appliance scheme. The consultant's report was prepared by Mark Ellis and Associates (and others) and is available from the electronic library (Report 2002/17). This version of the report includes comments and a response on the report from the Australian Gas Association (AGA) and the Gas Appliance Manufacturers' Association of Australia (GAMAA).

In November 2002, a joint government-industry working group comprising SEAV, OGS, AGO, AGA and the GAMAA agreed to work cooperatively to enhance the effectiveness of the gas appliance efficiency scheme. A discussion paper developed on behalf of the joint working group sets out a range of options for the future direction of the gas appliance efficiency scheme. The aim of the discussion paper "Driving energy efficiency improvements to domestic gas appliances"(July 2003) is to stimulate debate on the future directions of the gas appliance efficiency program, and to canvas views from a broad spectrum of industry and other stakeholders.

In 2005 and 2006 a round robin of 10 gas water heaters was conducted in 3 NATA accredited test labs (report 2006/11) (available in the electronic library).

In 2006 and 2007 work has been undertaken on the development of a revised energy test method for gas water heaters which is expected to be released for public comment in late 2007 of early 2008.

An example of the gas appliance energy label

# Overview of Australian Standards used for Energy Efficiency

Australian Standards for products that carry an energy label or which are regulated for MEPS now generally have two parts:

- "Part 1" covers the test procedure and ambient conditions such as the test method, performance measures and test materials;

- "Part 2" contains the detailed technical requirements for energy labelling and Minimum Energy Performance Standards (MEPS) (where applicable).

For some products, the test procedure and the regulatory standards may not be part numbers 1 and 2 as these part numbers may have been allocated for other purposes. References on this web site to "Part 1" and "Part 2" refer to the relevant test procedure and the regulatory standard respectively. Note that most energy efficiency standards are now jointly published by Standards Australia and Standards New Zealand.

An energy labelling and/or Minimum Energy Performance Standards application form which has to be submitted to state regulators can be found in the Part 2 standard. Such applications are normally required to have a test report to the relevant Part 1 standard for three sample units. These tests reports have to be prepared by an acceptable laboratory. However, applications for energy labelling or MEPS are now done electronically.

Find out how to make an application for an energy label or MEPS.

Australian Standards provide a "one-stop shop" for industry and regulators, addressing not only testing and performance requirements, but also energy labelling and minimum energy performance requirements. The following standards are currently used for energy efficiency regulation in Australia:

AS/NZS 4474: refrigerator and freezers - more details
AS/NZS 2040: clothes washers - more details
AS/NZS 2442: clothes dryers - more details
AS/NZS 2007: dishwashers - more details
AS/NZS 3823: room air conditioners (single phase) - more details
AS/NZS 3823: packaged air conditioners (three phase) - more details
AS 1056: electric water heaters - more details (note that NZ Standards have separate numbers)
AS/NZS 1359: three phase electric motors - more details (note that some of these standards are AS only)
AS/NZS 4783: fluorescent lamp ballasts - more details
AS/NZS 4782: fluorescent lamps - more details
AS 2374 & AS 2735: distribution transformers - more details
AS 1731: commercial refrigeration - more details

Standards Australia have also published the test method for the measurement of standby power AS/NZS 62301-2005 and this is now available for purchase (based on the IEC standard of the same number). This standard will be referenced by some of the above test methods where standby requirements are specified. See the standby page for more details.

"Part 2" of the standard includes data on how to calculate star ratings and the comparative energy consumption (the energy number that appears on the label) for each model, details on the number of units to be tested, minimum performance requirements, application forms, check testing procedures, the design and shape of the energy label and how the label is to be affixed to the appliance (where applicable). Part 2 also contains any requirements for Minimum Energy Performance Standards for the particular appliance or equipment type (where applicable).

## MEPS Requirements

Minimum Energy Performance Standards requirements for mains pressure electric storage water heaters are still found in AS1056.1 - this standard is currently under review and is being re-structured into the same Part 1/Part 2 format that has been used for other appliances.

Mandatory Minimum Energy Performance Standards (MEPS) levels for refrigerators, freezers and mains pressure electric storage water heaters first came into force on 1 October 1999. Mandatory Minimum Energy Performance Standards levels for three phase air conditioners (up to 65kW) and three phase electric motors (0.73kW to <185kW) came into force on 1 October 2001. Mandatory Minimum Energy Performance Standards levels for linear fluorescent lamp ballasts came into force on 1 March 2003.

MEPS levels have been introduced for the following products:

- commercial refrigeration - 1 October 2004 (more details)

- linear fluorescent lamps - 1 October 2004 (more details)

- distribution transformers - 1 October 2004 (more details)

- air conditioners: single phase - 1 October 2004 and also revisions to levels for 2006, 2007 and 2008 (more details)

- air conditioners: three phase - 1 October 2007 (more details)

- refrigerators and freezers - 1 January 2005 (more details)

- small mains pressure water heaters (less than 80L) and vented and heat exchange electric water heaters - 1 October 2005 (more details)

- three phase electric motors - 1 April 2006 (more details) )

The following products are proposed for regulation in the future, subject to normal regulatory processes and approval by the Ministerial Council on Energy.

- external power supplies (from 1 December 2008), New Zealand (April 2009);

- Set top boxes (from 1 December 2008), New Zealand (April 2009);

- Televisions;

- Standby;

- Boiling and chilled water dispensers;

- Vending machines;

- Commercial icemakers;

- A range of lamp types;

A number of other products are being considered for energy labelling and MEPS programs (or will have existing levels revised). Details are available on this web site - see what's new and products being considered for regulation.

# The Energy Label

### HIGH QUALITY ARTWORK - Energy Label Images available for downloading.

The Energy Rating label enables consumers to compare the energy efficiency of domestic appliances on a fair and equitable basis. It also provides incentive for manufacturers to improve the energy performance of appliances.

The Energy Rating Label was first introduced in 1986 in NSW and Victoria. It is now mandatory in all states and territories for refrigerators, freezer, clothes washers, clothes dryers, dishwashers and air-conditioners (single phase only) to carry the label when they are offered for sale. Three phase airconditioners may carry an energy label if the suppliers chooses to apply for one.

The Energy Rating Label has two main features:

- The star rating gives a quick comparative assessment of the model's energy efficiency

- The comparative energy consumption (usually kilowatt hours/year) provides an estimate of the annual energy consumption of the appliance based on the tested energy consumption and information about the typical use of the appliance in the home. Airconditioners show the power consumption of the appliance (kW or kWh/hour).

The Star Rating of an appliance is determined from the energy consumption and size of the product. These values are measured under Australian Standards which define test procedures for measuring energy consumption and minimum energy performance criteria. Appliances must meet these criteria before they can be granted an Energy Rating Label.

## The Energy Rating Label

Following several years of negotiation between government and industry the familiar red and yellow Energy Rating Label was revised in 2000. Energy efficiency is now measured against a tougher standard. This change encourages improved technology and more efficient products, which will save consumers money and help reduce harmful greenhouse gas emissions. The old energy label was in use in Australia from 1986 until 2000.

The label is again being reviewed during 2003 to coordinate and plan future changes across all product groups. The elements under consideration are the green band at the base of the label (should this be retained?) and making the appliance capacity more prominent on the energy label. The first product to incorporate this latter change is airconditioners in October 2003. You can view a copy of the new airconditioner label on this web site.

Is energy efficiency important to consumers?
Programs for other appliances

Background to the energy labelling program - who runs the program?
History of the energy labelling program
Energy labelling - facts and fiction
How are star ratings calculated?
Tips for purchasing energy efficient appliances
Energy costs - the second price tag
Tips on how to save energy when using an appliance
Why is energy efficiency important?

How are appliances tested for energy consumption?
Refrigerators and freezers
Clothes washers
Clothes dryers
Dishwashers
Airconditioners

What do the labels look like for each of the labelled products?
Refrigerators and freezers
Airconditioners
Clothes Washers
Clothes Dryers
Dishwashers
Gas

# How are refrigerator star ratings calculated?

## Comparative energy consumption CEC

The energy consumption of a refrigerator is measured under conditions specified in an Australian Standard. Over a year, this value is known as the comparative energy consumption or CEC.

## Concept of adjusted volume

The measure of energy service for a refrigerator is the volume that has to be cooled. However, freezers operate at a much colder temperature than fresh food compartments, so the volume of these compartments is "adjusted" to take account of the extra energy required to achieve this task. The energy service provide by a refrigerator or freezer is called the total adjusted volume.

## Performance

To be eligible for an energy label a refrigerator has to meet a temperature operation test and a pull down test as defined in the Australian Standard. Refrigerators also have to meet the Minimum Energy Performance Standards (MEPS) requirements for minimum energy efficiency.

## Base Energy Consumption BEC and Star Rating

The Base Energy Consumption defines the "1 star" line for particular products. An additional star is awarded when the CEC of the model is reduced by a defined percentage from the BEC. Energy reductions per star vary from 14% to 23% per star, depending on the refrigerator group. For example, if the energy reduction per star was 20% (as is the case for Groups 2, 3 6U and 7), then a CEC that was 0.8 of the BEC or less would achieve 2 stars. Similar, a CEC of 0.64 (0.8 x 0.8) of the BEC or less would achieve 3 stars and so on.

For refrigerators and freezers, there are a total of 9 groups which have been defined under the standard. However, to determine star ratings, a number of these groups are compared on the same basis, especially where they are performing a similar task, so there are only 5 BEC rating scales which cover all 9 refrigerator and freezer groups.

Refrigerators and freezers are a special category of product under the energy labelling program as they are also subjected to Minimum Energy Performance Standards or MEPS. The effect of the MEPS requirements for refrigerators and freezers was considered when the BEC lines were developed - in general terms the MEPS lines are approximately equal to the BEC (1 star) for most groups.

## What about adaptive defrost?

Adaptive defrost is one of a number of new smart technologies for refrigerators where the appliance adjusts the frequency of automatic defrosts to match frost load on the evaporator. Adaptive defrost systems are recognised by the Australian Standard for refrigerators and freezers but at this stage a realistic test for these controls is not included in the test method. The standard assumes that at least one defrost per day will occur during normal operation, although some smart systems may have a longer time between defrosts under normal conditions of use. Adaptive defrost technologies will play a more prominent role in the next round of refrigerator and freezer MEPS scheduled for introduction in late 2004.

The detailed star rating equations are contained in the document "Equations for Appliance Star Ratings".

There is an overview of how star ratings are calculated for other products on this site.

# How are clothes washer star ratings calculated?

## Comparative energy consumption CEC

The energy consumption of a clothes washer is measured under conditions specified in an Australian Standard. Over a year, it is assumed that the clothes washer is used 7 times per week at rated capacity on a warm wash (warm CEC in red). A value for a cold wash energy of 7 times per week is also shown on the label (cold CEC in blue). The clothes washer is labelled on the "normal" or "regular" program (program specified for a normally soiled cotton load). The energy consumption of a clothes washer includes both electrical energy for motors and pumps and the energy embodied in any imported hot water or electrical energy used to heat the water internally. The majority of energy for a clothes washer is to heat water on a warm wash.

## Capacity

The measure of energy service for a clothes washer is rated load capacity. This is the value declared by the manufacturer and defines the test load used in the Australian Standard.

## Performance

To be eligible for an energy label, a clothes washer must be able to meet a minimum level of wash performance, a minimum level of spinning performance and a must not exceed the "wear and tear" limits that are defined in the Australian Standard.

## Base Energy Consumption (BEC) and Star Rating

The Base Energy Consumption defines the "1 star" line for particular products. An additional star is awarded when the CEC of the model is reduced by a defined percentage from the BEC. The energy reduction per star is 27% for clothes washers. For example, a model that had a CEC that was 0.73 of the BEC or less would achieve 2 stars. Similar, a CEC of 0.533 (0.73 x 0.73) of the BEC or less would achieve 3 stars and so on.

For clothes washers, front and top loading models are rated on the same basis. The warm wash energy consumption and a component of residual moisture (spin performance) is used to define the star rating in comparison with the BEC. Therefore a model that has a good spin performance may get a marginally higher star rating than a model of the same capacity and CEC with a poor spin performance.

The detailed star rating equations are contained in the document "Equations for Appliance Star Ratings".

There is an overview of how star ratings are calculated for other products on this site.

# Top Energy Saver Award Winner (TESAW) - award winning labelled appliances

- TESAW criteria for 2006 award year

- Migration to Energy Star

- What is the TESAW?

- How do I find an Award winning appliance?

- What do TESAW labels look like?

- How does TESAW work?

- Why do we need a separate label for TESAW?

- If I am a manufacturer, how do I apply for an Award?

- How do I renew my existing TESAW Award?

- Label Printing Specifications

Welcome to the TESAW home page. This page tells you how to find an award winning appliance, the award criteria by product type, the label (what to look for) and details for manufacturers on how to apply for an award.

## TESAW Criteria for 2006

To be eligible for a TESAW award, products must meet minimum energy efficiency criteria. Manufacturers also have to agree to a set of award conditions. Awards are year specific - the minimum energy efficiency criteria and the conditions will be updated each year. The 2006 award criteria have just been released.

The criteria for the 2006 Award Year have been released and apply to products registered or which have valid registrations during the 2006 award year.

New criteria for 2006 are now available for new registrations. Those products that have already accepted a 2005 award and which also meet the 2006 criteria can be upgraded for the 2006 award listing on confirmation that the product is still available on the market through the on-line registration system. Products with a 2005 award will remain on display until 31 March 2006.

The TESAW criteria for 2006 are unchanged relative to the 2005 levels for all products except for air conditioners, where more stringent MEPS levels are being introduced in 2006 for some product categories. Only the cooling criteria for non-ducted split system air conditioners have been increased to 5 stars 2006.

## Migration to Energy Star

In 2005 detailed discussions and negotiations were held with the US Environmental Protection Agency and the US Department of Energy. These resulted in an in-principle agreement that Australia and New Zealand could set local Energy Star criteria for products that were sold in the Australasian market (such as white goods where the USA had their own domestic criteria), subject to detailed review by EPA and DOE on a product by product basis. On this basis, EEEC decided to move towards the use of the Energy Star label as the primary endorsement label for appliances and equipment in Australia and to discontinue TESAW as an endorsement label. A complete overview of the regulatory and endorsement framework in Australia and New Zealand is in preparation to facilitate further discussions.

To allow a smooth transition from TESAW to Energy Star, EEEC has decided to continue the TESAW scheme for each product until suitable Energy Star criteria are finalised.

The discussion paper on star rating algorithms for refrigerators and freezers has draft Energy Star criteria for these products which are to be introduced in 2007.

## What is TESAW?

TESAW is an award system that governments have created to recognise the most efficient star rated products on the market. It applies to both electric and gas products that carry a star rating energy label. It is an award system that helps consumers quickly identify the most efficient products on the market. The award is updated every year and the scheme replaces the previous Galaxy Award system.

## How do I find an Award winning appliance?

Appliances with an Award are listed in the normal appliance listings on this website. If the product is an award winner then this will be shown in the first column. You can even sort this column so that all the award winners are at the top by clicking the arrows at the top of the column.

You can also view a list of all TESAW Award Winners for all products.

- I want to search for: Single phase airconditioners

- I want to search for: Clothes dryers

- I want to search for: Clothes washers

- I want to search for: Dishwashers

- I want to search for: Refrigerators and Freezers

- I want to search for: Electric Storage Water Heaters

## What do TESAW labels look like?

There are separate TESAW labels for electric and gas appliances. For electric appliances, manufacturers have the option of including the TESAW award year in the green band at the bottom of the energy label.

### Electric Product Label

Electrical products that are eligible for TESAW Award may carry one of two labels.

# Standby Power

PROCEEDINGS AND PRESENTATIONS International Conference on STANDBY POWER India Habitat Centre, New Delhi India 2nd - 3rd April, 2008. See also Standby Power Use and the IEA "1-Watt Plan" for more information.

Standby store data, which has been collected from retail stores over the period from 2001 to date is now available for download - this covers over 7000 products. An international project to add to this data set is now under way. See the main project page to download data and to find out more the Basket of Products project to track global standby power trends.

Many modern appliances use power all day and night, even when not in use. This 'Standby Power' may be substantial and required to maintain a convenient 'ready' state for instant, on demand use. However, in some cases, standby power serves no useful function or operates at excessive levels for the background task being performed. Standby becomes necessary to power certain core functions or to sense communication for those products that are waiting to provide full services. This power is consumed not while the appliance is being fully utilised but while it awaits instruction; while it is "standing by". Some appliances also perform a continuous function that requires small amounts of continuous power.

Australian governments have been developing policy on standby for some time. This web site provides some background to the development of this policy.

The Ministerial Council on Energy (MCE) has resolved that Australia will expand its commitment to reducing excessive standby by formulating coordinated product-specific plans to address excessive standby over ten years, 2002 - 2012, within the umbrella of the IEA "One Watt" initiative. Within this timeframe, specific product types may be identified as "at risk" of using excessive standby and will therefore be targeted for specific action. Each product will then be dealt with in potentially a two-stage action plan designed to reduce standby to levels acceptable for that product as quickly as economically viable. The first stage is development of product profiles for each major product group. Details of the strategy are outlined in the Ministerial Council on Energy strategy on standby titled "Money isn't all you're saving".

ENERGY STAR is an international standard for energy-efficient electronic equipment. It was created by the US Environmental Protection Agency (EPA) in 1992 and has now been adopted by several countries around the world, including Australia. In Australia, Commonwealth, State and Territory Government agencies are cooperating through the National ENERGY STAR Program to encourage the use of energy-efficient equipment at home and in business. ENERGY STAR is a key part of the standby strategy.

E3 Committee has the following links on standby:

- Background on standby power.

- Description of standby Product Profiles.

In September 2003 Standards Australia published an interim test method for the measurement of standby power AS/NZS 62301-2003 (int.) and this is now available for purchase. This standard will be referenced by test methods or regulatory standards where standby requirements are specified. This is based on the IEC CDV of the standard of the same number.

The following reports and documents can be downloaded from the E3 Committee electronic library under Standby:

- Standby store data, which has been collected from retail stores over the period from 2001 to date is now available for download - this covers to 6500 products.

- The standby Product Profiles that have been released for discussion to date.

- Appliance Standby Power Consumption Store Survey 2003. Report 2003/04.

- ECEEE paper prepared by Holt & Harrington - Australia's contribution on Standby Power.

- IEC Standby Test Method - committee draft 59/325/CD - March 2003.

- A Study of Office Equipment Operational Energy Use Issues. Report 2003/07.

- A Study of Home Entertainment Equipment Operational Energy Use Issues. Report 2003/03.

- MCE Standby Strategy, November 2002 - "Money isn't all you're saving". Report 2002/12.

- Standby Power Consumption - a discussion paper. NAEEEC long-term strategy to achieve Australia's One-Watt Goal 2002 to 2012, July 2002. Report 2002/09.

- Quantification of residential standby power consumption in Australia - April 2001.

- Green Office Guide - A guide to help you buy and use environmentally friendly office equipment. A joint initiative of Commonwealth, State, and Territory government agencies, 2001.

- The Leaking Electricity Initiative: an International Action to Reduce Standby Power Waste of Electrical Equipment, 1999.

# Standby Power Background

Australia is not alone in recognising the serious problem posed by excessive standby to national greenhouse gas reduction and energy efficiency goals. The International Energy Agency has facilitated member nations to coordinate a common approach to this problem. It advocates that members develop long-term plans to reduce standby over time - a recommendation that Australian jurisdictions have embraced.

In August 2000, all Australian jurisdictions agreed to:

"...pursue efficiencies in standby power consumption of energy-consuming products, through support for the International Energy Agency's One -Watt program, and endorse its incorporation into the...program of work."

Australia was the first nation to publicly state that it would pursue the 'one-watt' target under the banner of the IEA standby power initiative. This aspirational target sought to raise awareness about excessive standby amongst not only suppliers but also product purchasers. It was meant to demonstrate to suppliers both in Australia and internationally that excessive standby should be redressed through better design practice and other strategies. It was meant to demonstrate to consumers that Australian governments were acting to reduce excessive standby on their behalf.

Since Australia's announcement, other IEA member countries such as Japan and the USA have taken similar stands. For example, US President Bush released an Executive Order in July 2001 in which he stated:

"Each [federal] agency...shall purchase products that use no more than one watt in their standby power consuming mode."

Australia has also supported the development of the draft International Electrotechnical Commission standard defining the technical method for the measurement of standby (to be published in 2004 as IEC62301). It will adopt this standard when the international community has completed its development. It is hoped that this standard will define the agreed method of measurement for standby for all products.

With the international effort gaining momentum, the Equipment Energy Efficiency Committee (E3 Committee), on behalf of the Ministerial Council on Energy, sought ideas and a response to a discussion paper consultation process in 2002. In a process throughout 2002, government agencies consulted with stakeholders about ideas to reduce standby. The standby strategy is a result of that dialogue with stakeholders.

With most consumer products traded internationally, any Australian action can only be effective if in step with international endeavours. That said, Australia has been a "standard bearer" in this field and will continue to adopt actions to reduce standby to reasonable levels. It will continue to help other nations to identify and redress excessive standby power.

# Standby Power - Product Profiles

Standby "product profiles" are being developed for each product type or category. The profile will provide an overview of the product in terms of its standby characteristic (including a review of measurements that show the distribution of standby power), the purpose and functionality of their standby function, market status, ownership levels and trends in sales and product types. Product profiles will provide the initial basis for making an assessment of the need for further action in terms of reducing standby and will provide the basis for initial discussions between government and industry regarding concrete measures to reduce standby. Standby profiles essentially initiate the development of plans for each of the key products identified. Standby profiles are released on this web site as they become available.

The 2002 standby discussion paper proposed a list of potential product types for consideration as candidates for specific product profiles. This list was the subject of extensive comments by many stakeholders, as was the need to adopt a flexible approach to the plan for each product, especially in the timing of when to move from stage one (voluntary measures) to stage two (mandatory measures).

The revised product list in the MCE standby strategy continues to be prioritised into two classifications - 'immediate' for action in the first year of this strategy and 'subsequent' for action within the first three years of the overall strategy. The following criteria were used to determine the initial classification priority:

- high standby power consumption (particularly when this appears unnecessary to maintain functionality);

- high (or potentially high and/or growing) ownership or penetration levels of that product type;

- wide ranges of standby levels found within differing examples of that product type, demonstrating that reducing standby is technically possible.

## Targeted Products

The following table is the schedule proposed for the release of draft Standby Profiles for those products that have been identified as being potentially problematic.

| Product Group A - Prioritised for "Immediate" Action | Number of Products | Time Frame for Progressive Release of Individual Product Profiles within each Group |
| --- | --- | --- |
| Information Technology Group A1 | 6 | From early 2003 |
| Entertainment Group A2 | 7 | From early 2003 |
| Major Appliances Group A3 | 4 | From mid 2003 |
| Small Appliances Group A4 | 3 | From early 2003 |

1: Includes personal computers, PC monitors, photocopiers, printers, scanners and multi-function devices.
2: Includes analogue TVs, digital TVs, VCRs, DVDs, digital TV set top boxes and converters, pay TV set top boxes, integrated and portable stereos.
3: Includes clothes washers, clothes dryers, dishwashers and airconditioners.
4: Includes external power supplies, smoke detectors and microwave ovens.

| Product Group B - Prioritised for "Subsequent" Action | Number of Products | Time Frame for Progressive Release of Individual Product Profiles within each Group |  |
| --- | --- | --- | --- |
| Information Technology Group B5 | 4 | From mid 2004 |  |
| Entertainment Group B6 | Multiple | From mid 2004 |  |
| Major Appliances Group B7 | Multiple | From early 2005 |  |
| Small Appliances Group B8 | Multiple | From early 2005 |  |
| # Background information on standby data |  |  |  |

The Equipment Energy Efficiency (E3) Committee is responsible for the development and ongoing operation of the Household Appliance Standby Power Data web page under the Asia Pacific Partnership (APP) Standby Power Project, which is part of the Building and Appliances Task Force (BATF).

## Basket of Products Project

An international project to collect standby data to track global standby power trends has commenced. The aim of this project is collect data for a representative "basket of products" so that standby power levels can be measured and tracked for a range of countries around the world. A "basket" can be measured by any interested parties to compare trends in standby power within that country and across countries. Many of these products will be global commodities with small differences in standby power levels (some may be due to voltage and frequency effects). However, some products may be subject to regional differences as a result of local standby programs or other influences.

By pooling collected data, it will be possible to undertaken comparisons by country and region. Eventually, the data collected will also provide trends in standby power by product type over time so that the rate of improvement or deterioration can be quantified within and between markets as well as by product type and even brand.

More detail on the project is set out in the project discussion paper  Draft Basket of Products Approach to tracking global standby power trends, including the products to cover and the general approach.

If you wish to participate in this project, please review the  participants guide which sets out the steps you have to take, the support you will receive and the expectations with respect to sharing of data. If you wish to proceed, please contact us to express your interest and obtain the support materials. It will be important to keep participants informed of updates to the survey instrument to enable consistent data gathering, so a register of participants will be essential.

## Pooled Standby Data

This web site contains historical data collected primarily through a series of store surveys commissioned by E3 over recent years. Eventually a range of international data collected under the "basket of products" project will also be posted in this database. The surveys have been conducted where household appliances are physically measured to determine their power consumption while in various low power modes such as "standby" &/or "off". These surveys involve plugging each target appliance into a portable power meter and measuring the applicable low power modes and recording the data. Other details, including brand, model and other characteristics of interest are recorded as well. Appliances in several stores have been surveyed for the past number of years, enabling a comprehensive data set to be compiled.

### Access to the Data

Access to the data is provided under a range of conditions. Acceptance of these conditions is a prerequisite for access to the data.

To gain access to the standby data, you must agree to provide your details to be a registered user. Registration is free and your user details will remain confidential. Once you apply, an email will be sent to the email address you supply confirming that your application for a user name and password has been submitted. Confirmation that the user name is active will be forwarded by email, usually within 3 working days.

I want to apply for a user name and password

I have a user name and password - LOGIN

### Details of the Data Set

The current data set includes a total of 10 separate surveys conducted over the period 2001 to mid 2006. Data for 5,239 appliances is contained in the database. The data available for download was last updated in October 2006.

Manufacturers, governments or other organisations are invited to provide additional data for inclusion in the database. Please contact us to arrange of data to be included. Telephone +61 3 5623 2711 or contact the email address standbydata at energyrating.com.au.

A description of the modes and the modes collected for each product type are set out in the background.

The data set identifies the data source for each record, which is either store survey, or manufacturer supplied or other source.

Copyright © Commonwealth of Australia This work is copyright. Permission is given for fair dealing with this material as permitted under copyright legislation, including for the purposes of private study and research. Apart from those uses, no part may be reproduced without prior written permission from the Commonwealth. Requests and inquiries concerning reproduction rights should be directed to the Commonwealth as per the Contact Details below.

Contact Details:

The Manager, Communications, 
Equipment Energy Efficiency Team, 
Australian Greenhouse Office, 
Dept of the Environment and Heritage, 
GPO Box 787, 
Canberra City ACT 2601.

# Program Evaluation in Australia

Governments have been tracking trends in the efficiency of appliances on the market since 1993 in order to help in the evaluation of the impact of the energy labelling program. Each year a report is prepared detailing these trends. The latest report titled "Greening Whitegoods 2005" which includes data up to 2005 (NAEEEC report 2006/06) can be downloaded from the electronic library.

There are a number of other reports and papers that are available from the electronic library that may be of interest in terms of program evaluation:

- When You Keep Measuring It, You Know Even More About It! NAEEEP: Projected Impacts 2005-2020. This E3 Report (2005/05) evaluates energy and greenhouse savings from E3 energy programs.

- Energy Rating Labelling Program Audit 2004/2005 - results of surveillance of retail stores for MEPS and energy labelling compliance, Reports 2005/07 and 2005/08) (white goods and air conditioners).

- Matching World's Best Regulated Efficiency Standards: Australia's success in adopting new refrigerator MEPS. (Conference Paper by Harrington and Holt) presented to ACEEE 2002 which shows trends in energy consumption of refrigerators and freezers in Australia;

- Draft Framework for Evaluation of the Australian Energy Efficiency Standards and Labelling Program (NAEEEC Report Report 2002/20);

## Tracking appliance efficiency trends in Australia

### Key Findings from the Greening Whitegoods Report 1993 to 2005

For all appliances covered by the energy labelling program in Australia, the sales weighted energy consumption is decreasing. The key results by appliance type are summarised below. No data on trends in the air conditioner market is available in the report.

### Refrigerators

Market Trends: Total sales increased at 2.6% per annum over the 13 year period. Side by side (Group 5S) refrigerators sales doubled from 2003 to 2005 (to 12% share) although 2 door frost free refrigerator/freezers (top and bottom freezers) - Group 5T and 5B - still dominate the market with 63% of sales. Single door refrigerators with a short-term freezer compartment (Group 3) and two-door cyclic defrost refrigerator/freezers (Group 4) have virtually disappeared from the market. Average fresh food and freezer volumes now appear to be stable after freezer volumes increased during the 1990's. Prices (within each Group) have generally decreased slightly in real terms over the analysis period.

Energy: Energy consumption decreased at an average of 3.9% per annum from 1993 to 2005. Energy efficiency (taking account of changes in volume) increased at 4.6% per annum over the period. The average star rating under the old rating system climbed from 3.58 in 1993 to 4.39 in 2005. Under the new star rating system this increased from 1.76 in 1993 to 3.78 in 2005. The relative fall in energy from 2003 to 2005 was 23% for refrigerators, which is likely to be in response to new MEPS levels introduced on 1 January 2005. In 2003, 88% of refrigerators sold did not pass 2005 MEPS levels while in 2005 only 6% sold failed to meet 2005 MEPS.

### Freezers

Market Trends: Total sales grew at an average of 4.8% per annum, although the majority of this increase occurred in years 2004 and 2005 (sales before this date were fairly static). The average volume of freezers is fairly static overall. The sales of frost free vertical freezers (Group 7) is continuing to slowly increase while manual defrost vertical freezers (Group 6U) increased very sharply in 2004 and 2005 - this trend is not understood and is being investigated more closely. Chest freezer (Group 6C) sales constitute nearly 50% of the market and have been fairly steady. Prices (within each Group) decreased or were stable in real terms over the analysis period.

Energy: Energy consumption decreased at an average of 4.0% per annum from 1993 to 2005, with the most significant decline occurring in 2005, linked to the introduction of the more stringent freezer MEPS in January 2005. Energy efficiency (taking account of changes in volume) increased at 3.3% per annum over the period. The average star rating under the old rating system climbed from 4.24 in 1993 to 5.22 in 2005. Under the new star rating system this increased from 1.48 in 1993 to 3.4 in 2005.

### Clothes Washers

Market Trends: Total sales increased at 2.2% per annum over the analysis period. Front loading machines are dramatically increasing their market share and constituted 30.9% of all machines sold in Australia in 2005. The market share of front loaders in Western Australia and South Australia are the highest at 39% and 37% respectively. Average capacity is increasing steadily for both front and top loading machines and is now 6.7kg and 6.4kg respectively for these types. Water consumption decreased by 2.8% per annum over the period. Prices have decreased in real terms for top and front loaders over the analysis period. Twin tubs have increased in price in recent years but the average capacity for this type is now much larger and sales share remains at about 2%, so this only has a minor overall impact.

Energy: Energy consumption showed a slight decrease until 1998, but there was a significant increase in energy in 1999 and 2000. This increase was attributed to top loading machines only. Since 2000, energy consumption has been declining although a slight increase was noted in 2005. Most of the decrease is a result of increased market share of front loaders. The average star rating under the old rating system climbed from 3.39 in 1993 to 4.28 in 2005. Under the new star rating system this increased from 1.28 in 1993 to 2.4 in 2005, noting however that both old and new star ratings decreased in the years 1999 and 2000.

### Clothes Dryers

Market Trends: Total sales increased at an average of about 4% per annum from 1993 to 2005, but there has only been a small increase in sales since 1995. The market share of auto-sensing dryers has increased significantly from 10% in 1993 to 44% in 2005. Average load capacity has been static since 1993. Prices for timer dryers were generally steady in real terms over the analysis period while auto-sensing dryers have decreased in real terms over the analysis period.

Energy: Energy consumption decreased at an average of 0.7% per annum from 1993 to 2005. All dryers appeared to decline slightly in efficiency in 2000 as a result of the new test method (lower initial moisture content). The average star rating under the old rating system increased only slightly from 2.40 in 1993 to 2.49 in 2005. Under the new star rating system this increased from 1.52 in 1993 to 1.59 in 2005. These increases are primarily due to an increase in market share of auto-sensing dryers.

### Dishwashers

Market Trends: Total sales are continuing to increase with a growth of 5.4% per annum from 1993 to 2005. Average capacity (place settings) has been stable since 1996 although there has been a slight decline since 2000. Water consumption decreased by over 4.0% per annum over the period. Prices were steady in real terms to 2003 but fell slightly through to 2005.

Energy: Energy consumption decreased at an average of 3.6% per annum from 1993 to 2005. The average star rating under the old rating system climbed from 4.07 in 1993 to 5.4 in 2005. Under the new star rating system this increased from 1.88 in 1993 to 2.8 in 2005.

# Equipment Energy Efficiency Program (E3)

Australian Governments have decided that the energy efficiency of appliances and equipment must improve at rates well beyond what the market has delivered in the past.

Energy consumed by appliances and equipment is a major source of greenhouse gas emissions responsible for more than a quarter of net greenhouse gas emissions in Australia. Improved energy efficiency of appliances and equipment is a key objective for Australian Governments.

The Equipment Energy Efficiency Program (E3) is a collection of coordinated programs that deliver economic and environmental benefits to the community. The E3 program, which is co-funded by the Australian Government, state and territory governments and New Zealand Government, focuses on initiatives that require a nationally consistent framework to improve energy efficiency and reduce greenhouse emissions from household appliances and equipment, and commercial and industrial equipment.

E3 covers the technical, legal, and administrative aspects of national appliance and equipment energy efficiency initiatives, in particular mandatory minimum energy performance standards and energy efficiency labelling. Products are considered for inclusion within the program on the basis that the community will benefit from their regulation. The individual product energy efficiency target is either the equivalent of world-best regulatory target or a more stringent level developed specifically for Australia. This market intervention has proved to be an extremely cost-effective mechanism for reducing energy demand and greenhouse gases produced by consumer appliances, commercial and industrial equipment.

A key feature of the E3 program is the use of strategies that clearly articulate government policies and provide a map for reducing energy in all sectors for example:

- The E3 Work Plan and Policies which is updated every three years and the latest is from 2005/6 to 2007/8 (NAEEEC Report 2006-03).

Each year E3 has an annual report titled Achievements which summarises progress in the previous calendar year. The latest edition is for year 2005.

# Equipment Energy Efficiency (E3) Committee

The E3 Committee, consisting of officials from the Commonwealth, State and Territory government agencies and representatives from New Zealand, is responsible for managing the Australian end-use energy efficiency program. The Committee reports to other government structures and is ultimately directed by the Ministerial Council on Energy (the Energy Ministers from all jurisdictions).

The E3 Committee Charter provides the terms of reference for the committee. E3 Committee member organisations are listed below:

- The Australian Greenhouse Office which provides support for the E3 Committee's activities. An AGO Officer is the chair of the E3 Committee.

- NSW Ministry of Energy and Utilities

- NSW Sustainable Energy Development Authority

- Energy Safe Victoria

- Sustainable Energy Authority of Victoria

- Queensland Electrical Safety Office

- Queensland Department of Energy

- Western Australian Sustainable Energy Development Office

- South Australian Office of Energy Policy

- New Zealand Energy Efficiency and Conservation Authority

- Tasmanian Office of Energy

- ACT Department of Treasury

- Northern Territory Department of Business, Industry and Resource Development

## E3 Committee Charter

The charter of the E3 Committee encompasses the following functions:

- to provide assistance to all States and Territories, as required, in the development and regulatory implementation of technical, legal, and administrative aspects of equipment energy efficiency initiatives;

- to coordinate the national development and implementation of energy efficiency programs of a non-regulatory nature and enhance existing regulator programs. These may include voluntary labelling initiatives, market transformation projects, and similar voluntary actions;

- to coordinate national marketing and communication projects to support new, and enhance existing, energy efficiency programs;

- to review existing appliance energy consumption and improve standards and test procedures;

- to monitor program performance and achievements;

- to provide a forum to exchange information on enforcement/compliance issues and community information and marketing initiatives;

- to administer an effective, coordinated testing regime of the energy efficiency claims of suppliers;

- to coordinate broad consultative processes with industry and other interested parties in the development and implementation of energy labelling and associated programs.

This charter recognises the maturity of the program and the need for a "holistic" approach to government policies for greenhouse gas abatement in the appliance and equipment field. The focus of the program continues to be the delivery of nationally consistent regulation. The implementation of most voluntary programs remains an individual jurisdictional responsibility although voluntary programs that assist the regulatory program to maximise benefits are being added to E3 Committee work plans.