- 7762AA TRANSMISSION LINE DESIGN  PROJECT

- This project will have to be completed in Week 9. But the components of the project can be done concurrently with class lessons. You also need to think about how to do the following tasks whenever you complete the lectures in the respective week shown in brackets beside the following tasks. You also need to refer the respective notes in CD as indicated in underlined sentences.

- You need to use ruler to do scaled drawing for line route.

(1) Draw Line route to supply the proposed factory location from 3 Phase 11 KV line. Put on map (Week 5)

(2) Draw side view of the route that you select for power line route. Show ground levels.  (Week 4)

(3) Plan profile drawing for the power line  2.5 State pole and line installation techniques in CD refer profile drawing Fig 11.8. (Day 4)

(4) Determine the appropriate size of pole. Mention appropriate length of cross arm, height, diameter

(Refer my e-mail attachment- Page 44 –(I will send you) & Topic 2.2 Outline relevant factors related to installation, maintenance, cross arms, stays, pole types and choice of conductors sizes for commonly used configuration in CD. You will see clearance in Table 1-Open supply line. You need to convert foot to metre.1 metre= 3.3 Feet  (Week 2, 3)

(5) Select appropriate insulator and cross arm(Disc insulator EA2/D  EA3/D etc) (Week 2)

(6) Select the size of conductor based on 2MVA  33KV 3 Phase.Refer AS 3008

(Refer my e-mail attachment: AS3008 Pages) OR 2.3 Determine mechanical limitations and physical dimensions of lines in CD

(7)Calculate sag. (Use formula in Review Question (1) Question 10/11) (Week 2)

(8)Draw cross arm, pole, insulator. Refer 2.1. Identify relevant components use in over head line

design in CD. Week (3)

(9) Attach vibration damper, guy wire , line guard as necessary   Review Question (1) Number 14.

(10) Show footing Review Question (1) Number 15. (Week 2)

(11)Install cross arm to pole, insulator  Refer 2.5 State pole and line installation techniques (Part

2,3,4) in CD.

(12)Prepare list of materials. Cable, insulator, etc that you use. (Week 9)

(13) Sketch line diagram----- Containing-Route and pole diagram with dimension  (Week 9)

Take the power from 33KV MAIN located at the Shopping Centre

The above diagram is Incoming HV supply system for a Cement Factory. It contains Star –Delta Transformer, Interchange switches for switching 2 supply, sketch of protective relay system with trip circuit and outline for measuring instruments.

(1) Develop Detail installation diagram that contain appropriate protection instruments and measurement instruments

(2) Recommend appropriate specifications for protection and measuring instruments for power supply requiring 500Kw for the factory.