# Chapter 4 Practical Application

# Introduction

To illustrate the features of Chapter 4 I am using an application, which requires the detection of excessive heat in circuits due to high resistive joints or faulty elements. The project I am using to illustrate this function of detection is thermography or thermal infrared imaging.

To briefly explain thermography refer to the paper Understanding Thermography

by Roger Christiansz.

## Thermal Camera		Courtesy of FLIR

## Scope

It is widely known that as temperature in a conductor rises, so does its resistance. Conversely, as resistance increases (in most conductors) temperature rises. The majority of thermal electrical problems involve improper torque specifications or improper installation at the junction points. A loosely torqued connector effectively reduces the surface area in which current can flow and consequently results in an increase in the contact resistance. Oxidation build-up at the connection point can also cause a rise in resistance. The origin of most conductor, insulation and component problems can be traced to a poor connection.

This idea of linking the terms and parameters of a resistive network incorporating heat applications to chapters could also be used for the following:

- The selection of an indoor condensation heating element, courtesy of NHP (refer to Chapter 5);

- The selection of convection wall heaters, courtesy of Gerard industries (refer to Chapter 2);

- The selection of a motor condensation heating element, courtesy of Rockwell Automation and Reliance Electric (refer to Chapter 5);

- The selection of thermal protection, courtesy of Rockwell Automation and Reliance Electric (refer to Chapter 2);

- The selection of a Neutral Earth Resistor, courtesy of Fortress Systems Pty Ltd (refer to Chapter 2);

- The basic units of electricity (refer to Chapter 1 and Appendix 5);

- The fundamentals of electricity (refer to Chapter 2);

- Types of electric circuits (refer to Chapter 4);

- The use of test meters (refer to Chapter 7);

- Used as a simulated practical exercise in a classroom.

## Description and extent of work

Utilising a comprehensive predictive maintenance program, in which maintenance procedures are utilised to describe the process, industries have the ability to foresee failure of critical equipment components under stress modes that might otherwise be missed. Regular infrared inspections of their equipment will minimise the failures.

Typical electrical problems encountered include the following:

- Poor contacts (refer to Chapter 17);

- Loose, corroded, improper connections (refer to Chapter 3);

- Unbalanced loads (refer to Chapter 9);

- Overloading (refer to Chapter 10);

- Hysteresis (refer to Chapter 5);

- Eddy currents (refer to Chapters 8, 12 and 14);

- Induced voltages on non-energized equipment (refer to Chapter 5);

- High resistance joints (refer to Chapter 2);

- Inoperative capacitors (refer to Chapter 6);

- Defective fuses (refer to Chapters 2 and 13);

- Voltage tracking on insulators (refer to Chapter 7);

- Poor breaker connections (refer to Chapter 2);

- Abnormal transformer operating temperatures (refer to Chapter 14);

- Overheated bearings (refer to Chapter 12);

- Shorted or open motor windings (refer to Chapter 13);

- Ballast overheating (refer to Chapter 5);

- Dead cells in batteries (refer to Chapter 3);

- Circuit board testing (refer to Chapter 7).

Typical mechanical problems encountered include the following:

- Misalignment in rotating equipment;

- Conveyors;

- Pulley and drive belt misalignment;

- High bearing temperatures;

- High oil temperatures;

- Defective seals.

## Report

A comprehensive report was produced outlining the following:

  - Equipment identification (refer to Chapter 12 on scheduling);

  - Location (refer to Chapter 3 on floor plans and site plans);

  - Images produced;

  - Radiometric analysis (refer to Chapter 2 and the glossary of terms for thermography);

  - Thermal profile (refer to Chapter 1 for graphical representations and Chapter 2 for heat transfer);

  - Measurement parameters (refer to Chapters 8 and 9 on single phase and three-phase terms and theory);

  - Problem, Findings, Recommendations and Priorities.

## Regulations

Test in accordance with:

- AS/NZS 3000:2000; AS/NZS 3017:2001; AS/NZS 3008.1.1:1998; AS/NZS 3018:1997; AS/NZS 3190:1994; AS/NZS 3760:2001;

- Relevant Acts and Regulations for occupational health and safety, workplace safety, electricity etc. in each state or territory;

- Safety Tag and Lockout Procedures; Testing procedures for customer installations; Installation procedures for safety switches RCDs.

## General

- Consider the following when working with resistive networks:

  - Radiation, Convection and Conduction (refer to Chapters 1 and 2);

  - Electromagnetic spectrum;

  - Efficiency (refer to Chapter 1).

Electromagnetic spectrum

References:

Jenneson, Jim, 2003, Electrical Principles for the Electrical Trades 5th edn, Sydney: McGraw-Hill Australia, Chapters 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 12, 13, 14 and 17.

Internet references: